| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0155 |
| --- | --- |

| Title: | 3D-CE6.h related: Direct simplified depth coding in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Jian-Liang Lin2, Yi-Wen Chen2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { kai.zhang, jicheng.an, jl.lin, yiwen.chen, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a direct Simplified Depth Coding (dSDC) method in 3D-HEVC. In the current HTM, a two-step approach is applied to obtain the prediction values in the SDC prediction stage. After generating all the prediction samples in the coded block by a normal intra prediction mode, the average value of these samples in a segment is calculated as the prediction value. To simplify the SDC prediction approach, dSDC abandons the two-step method. Instead, the prediction values are calculated from the neighboring reconstructed samples directly. In this way, the high computational overhead of generating prediction samples and calculating the average value over these samples can be removed completely and the high bit-width of the averaging process is avoided. Experimental results show that dSDC can achieve 0.2% and 0.1% BD-rate gain on the synthesized view in the common test condition and all-intra test condition respectively.

# Introduction

Simplified depth coding (SDC) and a depth lookup table (DLT) are adopted into HTM [1] [2]. For each depth coding unit (CU), if SDC is selected, one of three different prediction modes, DC, Planar and DMM-1 can be selected. After the prediction, the residual is not coded using transform. As the SDC prediction stage always results in one or two depth segments per coded block, a single residual DC depth value is coded for each of these segments. Moreover, the DLT is used to map coded depth values in SDC to valid depth values of the original depth map.

A two-step approach is applied to obtain the prediction values in the SDC prediction stage. First, the normal intra-prediction procedure using neighboring reconstructed samples is invoked to get all the prediction samples in the coded block. DC, Planar and DMM-1 are three possible prediction modes in this step. Second, the average value of the prediction samples in a segment is calculated as the prediction value for this segment. For DC and Planar modes, there is only one segment in the coded block. For DMM-1 mode, there are two segments in the coded block, as the defined by DMM-1 mode. As a simplification, a sub-sampling method, which only uses one from each four prediction samples to get the average value, is adopted [3]. This method can reduce the addition operations in the averaging process significantly. Fig.1 demonstrates two examples for the two-step SDC prediction approach, where there are one and two segments respectively.

Fig.1 The two-step SDC prediction approach in HTM6. P, P0 and P1 represent the prediction values.

# Proposed methods

To simplify the SDC prediction approach further, a direct SDC (dSDC) method is proposed. In dSDC, the two-step method is abandoned. Instead, the prediction values are calculated from the neighboring reconstructed samples directly as depicted in Fig. 2. Apparently, prediction samples in the coded block are only intermediate variables to derive the one or two final prediction values in the current SDC. Since prediction samples in the coded block are derived from neighboring reconstructed samples, it is more reasonable and efficient to skip the normal intra-prediction procedure and derive the prediction values in SDC from the neighboring reconstructed samples directly. Before the derivation, neighboring reconstructed samples are initialized as before the normal intra-prediction procedure. The derivation methods for each mode are as follows.

Fig.2 Examples of dSDC. P, P0 and P1 represent the prediction values.

- For mode DMM-1.

P0 and P1 are derived following the same method in DMM-1. Without filling each prediction samples and calculating the average values, P0 and P1 are treated as the final prediction values directly.

- For mode DC.

P=(A+B+1)>>1,	If A and B are both available.

- For mode Planar.

P= (X+Y+1) >> 1.

A, B, X and Y are neighboring reconstructed samples in four different positions as depicted in Fig. 3.

Fig.3 Positions of neighboring reconstructed samples.

dSDC simplifies the derivation of prediction values in SDC significantly. First, the high computational overhead of generating prediction samples and calculating the average value over these samples can be removed completely by dSDC. Second, generating a dSDC prediction value is even much simpler than generating a normal prediction sample in mode DC and mode Planar. Third, the high bit-width of the averaging process is avoided in dSDC. In SDC, as many as 64x64/4 prediction samples may be summed together, thus 18 bits are required by the accumulator for 8-bit samples. In dSDC, however, the bit-width does not exceed that in normal intra-prediction.

# Experimental results

The proposed methods are integrated on HTM-6.0 [4]. Table 1 and Table 2 show the results of the dSDC under common test condition [5] and all-intra test condition [6] respectively. It can be seen that the proposed method can achieve 0.2% and 0.1% BD-rate gain on the synthesized view in the common test condition and all-intra test condition respectively.

Table 1. Performance of dSDC (CTC)

Table 2. Performance of dSDC (All-intra)

# Conclusion

In this proposal, a direct Simplified Depth Coding (dSDC) method is proposed. By deriving prediction values in SDC from neighboring reconstructed samples directly, the proposed method can simplify the prediction process of SDC significantly. In addition, 0.2% and 0.1% BD-rate gain on the synthesized view in the common test condition and all-intra test condition are achieved respectively.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1005, January, 2013.

- F. Jager, “3D-CE6.h: Simplified depth coding with an optional depth lookup table,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-B0036, October, 2012.

- X. Zheng, Y. Lin, J. Zheng, “CE6.H related: Reference samples sub-sampling for SDC and DMM,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0154, January, 2013.

- HTM-6.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.0/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1100, January, 2013.

- P. Merkle, “Description of Core Experiment 6 (CE6) on Depth Intra Coding,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1106, January, 2013.