| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0156 |
| --- | --- |

| Title: | 3D-CE1.h related: A high level syntax suggestion for stereo compatibility in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a high level syntax suggestion for stereo compatibility in 3D-HEVC. In HTM-6.0, depth dependent coding (DDC) methods such as BVSP and DoNBDV are adopted to improve the overall coding efficiency. However, the dependency between texture and depth pictures breaks the stereo compatibility in 3D-HEVC. With the proposed syntax, DDC tools can be switched on or off. An encoder can decide depending on the application scenario.

# Introduction

Backward view synthesis prediction (BVSP) and depth-oriented neighboring block disparity vector (DoNBDV) have been adopted into HTM [1][2][3]. BVSP uses the neighbouring blocks to derive a depth block to perform the backward warping operation. A new merging candidate indicating the BVSP mode is added in the merging candidate list. DoNBDV can enhance the accuracy of the NBDV by utilizing the coded depth map. By referring to the NBDV and the coded depth information, the inter-view information can be predicted more accurately with DoNBDV.

BVSP and DoNBDV both utilize the coded depth picture from the base view when a texture picture of a dependent view is encoded or decoded. These depth dependent coding (DDC) methods can improve the overall coding efficiency significantly compared with the depth independent coding (DIC) scheme.

Although DDC can outperform DIC in coding efficiency, the dependency between texture and depth pictures breaks the stereo compatibility in 3D-HEVC. In previous HTMs without DDC tools, texture pictures of dependent views can be encoded and decoded without depth pictures, which means stereo compatibility is supported. In HTM-6.0, however, texture pictures of dependent views cannot be encoded or decoded without depth pictures of the base view. Even if depth pictures are available, DDC might not be inefficient in the stereo application scenario. Fig. 1 shows an example, where only V0 and V1 are desirable. In a DIC scheme such as HTM-5.0, only bitstreams of V0 and V1 need to be sent. In a DDC scheme such as HTM-6.0, however, bitstream of D0 and bitstreams of V0 and V1 need to be sent. Although D0 can contribute to the coding efficiency improvement of V1, this may not justify the overhead bits of D0.

Fig. 1.  An example of the stereo application scenario

# Proposed method

A high level syntax suggestion for stereo compatibility in 3D-HEVC is demonstrated in Table 1, where related syntax elements are signaled in video parameter set (VPS). DDC tools such as BVSP and DoNBDV can be optional instead of always being mandatory. An encoder can decide whether to enable DDC depending on the application scenario. Moreover, an extractor can determine how to dispatch NALs according to these syntax elements. Equivalently, the proposed syntax elements can also be signalled in sequence parameter set (SPS) as implemented in HTM-6.0, demonstrated by Table 2.

Table 1.  Proposed syntax elements in VPS.

| vps_extension2( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( (layerId ! = 0) && !DepthLayerFlag[ layerId ] ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_res_pred_flag[ layerId ] | u(1) |
| depth_dependence_flag[ layerId ] | u(1) |
| if( depth_dependence_flag[ layerId ] != 0 ){ |  |
| vsp_compensation_flag [ layerId ] | u(1) |
| vsp_dv_refine_flag [ layerId ] | u(1) |
| } |  |
| } |  |
| if ( DepthLayerFlag[ layerId ] ) { |  |
| if ( (layerId ! = 0) |  |
| vsp_compensation_flag [ layerId ] | u(1) |
| … |  |
| } |  |

Table 2.  Proposed syntax elements in SPS.

| sps_extension2( ) { | Descriptor |
| --- | --- |
| if ( ( nuh_layer_i ! = 0 ) && !VpsDepthFlag[ nuh_layer_id ] ){ |  |
| depth_dependence_flag | u(1) |
| if( depth_dependence_flag ) { |  |
| vsp_compensation_flag | u(1) |
| vsp_dv_refine_flag | u(1) |
| } |  |
| } |  |
| else if( nuh_layer_i ! = 0 ) { |  |
| vsp_compensation_flag |  |
| } |  |
| } |  |

# Conclusion

In this proposal, a high level syntax suggestion for stereo compatibility in 3D-HEVC is proposed. With the proposed syntax elements in VPS, BVSP and DoNBDV are optional, and the encoder can choose whether to preserve stereo compatibility.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 3,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1005, January, 2013.

- D. Tian, F. Zou, A. Vetro, “CE1.h: Backward view synthesis prediction using neighbouring blocks,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0152, January, 2013.

- Y.-L. Chang, C.-L. Wu, Y.-P. Tsai, S. Lei, “3D-CE1.h: Depth-oriented neighboring block disparity vector (DoNBDV) with virtual depth retrieval,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C0131, January, 2013.

- HTM-6.0, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.0/

- D. Rusanovskyy, K. Müller, A. Vetro, “Common test conditions of 3DV core experiments,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-C1100, January, 2013.