| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0162r1 |
| --- | --- |

| Title: | CE1.a related: Harmonization of inter-view and view synthesis prediction in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenyi Su, Dmytro Rusanovskyy, Miska M. Hannuksela. | Tel: Email: | +358 7180 08000 dmytro.rusanovskyy@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution proposes harmonization of view synthesis prediction (VSP) and inter-view prediction (IVP) in 3D-AVC. It is proposed to perform motion compensation (MC) for VSP- and IVP-coded blocks through an identical process, namely through the MC modules of H.264/AVC, with the difference in motion vector signaling. For VSP coded blocks, motion information is derived from the depth samples rather than coded in the bitstream. Additionally, it is proposed to perform motion compensation for VSP_SKIP and VSP_DIRECT with 8x8 block size, instead of the current 4x4. The simulation results show that solution for IVP/VSP harmonization provides about 0.1% of dBR gain for coded textures. Being combined with 8x8 block size implementation for VSP_SKIP/VSP_DIRECT, an execution time reduction up-to 25% and 9% on average is achieved with a 0.05% dBR penalty on the coding gain for MVD/synthesized data. It is proposed to adopt presented solution to the software and specification text.

Revision 1 of the document presents specification text reflecting the decisions of JCT-3V on 20th April 2013.

# Introduction

Current design of ME/MC in 3D-ATM consists of following major components:

- Temporal prediction (TP)

- Inter-view prediction (IVP)

  - Generic mode

  - Interview Direct/Skip

- View Synthesis prediction (VSP)

  - Generic mode

  - VSP Direct/ VSP Skip mode

Taking into consideration current VSP implementation, the two components IVP.a and VSP.a (generic modes) can be conceptually merged into a single group with difference in signaling method.

The IVP performs motion search in inter-view reference frame and signals to the decoder {refIdx, dMVx and dMVy}. The dMVx and dMVy components are predicted from the MVP process, which highly likely to result in Disparity(Cb), due to current MVP design. The VSP.a coding mode can be considered a special case of IVP.a with different signaling of motion vectors. The VSP.a performs motion search in VSP frame, which is generated as entire frame in the encoder side. The motion search in VS frame is limited to a single motion vectors (0,0) and thus it effectively results in motion partition selection (4x4,8x8 and so on) only, not in selection of motion vectors. Following the selection of optimal partition, refIdx VSP coded block, is signaled to the decoder and no dMVx and dMVy are signaled.

Decoder, receiving the refIdx and evaluating as belonging to virtual_view, skip decoding of dMVx and dMVY and reset motion vectors to {0,0}. The coded block with such motion information {refIdx_vsp, 0,0} is sent to the VS process for given motion partitioning.

In turn, the view synthesis (VS) is implemented as block-based process independently from motion partitioning.  For every motion partition, VSP performs spatial splitting into non-overlapping blocks size of 4x4 and derives independent motion vectors for each of them as its disparity (max_out_4) of d(Cb). For example, the motion partition 16x16 being coded with VSP, would be split into 16 blocks size of 4x4 and 16 motion vectors to be derived at the decoder side. Such design significantly increases decoding complexity.

# Proposed technique

## IVP/VSP harmonization

We propose to perform motion compensation (MC) for IVP and VSP coded blocks identically, refereeing to a singled refIdx, with difference on the motion vector signaling.

For every macroblock or motion partitioning coded with IVP/VSP prediction the flag b_vsp_flag is signaled following the reference index refIdx. If the flag value is zero, residual motion vectors are coded in the bitstream and decoding/MC process for this partition is identical to the specified in H.264/AVC. If the flag is equal to 1, motion vectors are not signaled through the bitstream, instead they are derived from depth map samples associated with current motion partitioning. The disparity derivation method is identical to what is specified in 3D-AVC spec. However, entire motion partition is predicted with a single disparity value, which effectively imposes variable block size VSP implementation. The motion compensation process is identical the one specified in H.264/AVC. See figure 1 for visualization of the concept.

Figure 1. Visualization of the harmonized IVP/VSP prediction

Proposed method is utilized for coding dependent texture views and applied to macroblock and sub-macroblock types of P and B slices except of SKIP/Direct and their VSP equivalents.

## Changes to VSP_Skip/Direct modes

No changes in signaling or coding/decoding process are introduced to PVSP_SKIP and BVSP_DIRECT mb_types. However, the elementary block size of BVSP implementation was increased from 4x4 to 8x8 samples. Thus, total amount of motion vectors derived from disparity is reduced by 4x.

## Extract of slice-header syntax is shown in Table 1.

- Simulation results

Proposed solution for harmonized IVP and VSP was integrated to 3D-AVC v7.0 and executed under the JCT-3V CTC. Simulation results are provided in Table 4 show that up-to 0.15% of dBR for coded texture is achieved with noticeable complexity reduction.

Table 4. Compression efficiency of the proposed scheme compared against the 3D-AVC 7.0 anchor

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Syntheses PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -0.12 | 0.00 | -0.11 | 0.00 | -0.08 | 0.00 |
| S02 | -0.11 | 0.00 | -0.09 | 0.00 | -0.15 | 0.00 |
| S03 | -0.09 | 0.00 | -0.08 | 0.00 | -0.10 | 0.00 |
| S04 | -0.15 | 0.01 | -0.14 | 0.01 | -0.16 | 0.01 |
| S05 | 0.00 | 0.00 | 0.02 | 0.00 | 0.00 | 0.00 |
| S06 | 0.03 | 0.00 | 0.04 | 0.00 | -0.01 | 0.00 |
| S08 | 0.02 | 0.00 | 0.03 | 0.00 | -0.03 | 0.00 |
| Average | -0.06 | 0.00 | -0.04 | 0.00 | -0.08 | 0.00 |

Following this, we evaluated performance of the combination of the harmonized IVP and VSP with increased elementary BVSP block size from 4x4 to 8x8. Simulation results for this scheme are provided in Tables 5 and 6. Proposed scheme provides decoding time reduction up-to 25% with negligible penalty 0.04% of dBR on average.

Table 5. Compression efficiency of the proposed scheme with BVSP size 8x8 compared against the 3D-AVC 7.0 anchor (4x4 BVSP size)

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Syntheses PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -0.04 | 0.00 | -0.04 | 0.00 | -0.06 | 0.00 |
| S02 | -0.02 | 0.00 | 0.00 | 0.00 | -0.09 | 0.00 |
| S03 | 0.38 | -0.01 | 0.36 | -0.01 | 0.30 | -0.01 |
| S04 | -0.24 | 0.01 | -0.21 | 0.01 | -0.28 | 0.01 |
| S05 | 0.12 | -0.01 | 0.10 | 0.00 | 0.16 | -0.01 |
| S06 | 0.24 | -0.01 | 0.23 | -0.01 | 0.25 | -0.01 |
| S08 | 0.02 | 0.00 | 0.03 | 0.00 | 0.01 | 0.00 |
| Average | 0.06 | 0.00 | 0.07 | 0.00 | 0.04 | 0.00 |

Table 6. Complexity estimates for the proposed scheme comparing against the 3D-AVC 7.0 anchor.

|  | Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- | --- |
|  | Encoder Time, % | Decoder Time, % | Rendering Time, % |
| S01 | 1.05 | 0.75 | N/A |
| S02 | 1.06 | 1.00 | N/A |
| S03 | 1.04 | 0.89 | N/A |
| S04 | 1.05 | 0.77 | N/A |
| S05 | 1.07 | 1.00 | N/A |
| S06 | 1.06 | 1.11 | N/A |
| S08 | 1.06 | 0.88 | N/A |
| Average | 1.05 | 0.91 | N/A |

# Proposed specification text

## Changes related to IVP/VSP harmonization

All occurrences of “synthetic reference component” are replaced by “inter-view reference component” throughout the 3D-AVC specification.

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| if ( nal_unit_type = = 21 && ( slice_type != I && slice_type != SI )) { |  |  |
| if( DepthFlag ) |  |  |
| depth_weighted_pred_flag | 2 | u(1) |
| else if( avc_3d_extension_flag && ViewCompOrder( 0, view_idx ) > ViewCompOrder( 1, view_idx ) ) { |  |  |
| dmvp_flag | 2 | u(1) |
| if( seq_view_synthesis_flag ) |  |  |
| slice_vsp_flag | 2 | u(1) |
| } |  |  |
| if ( 3dv_acquisition_idc != 1 && (depth_weighted_pred_flag = = 1 ¦¦ dmvp_flag = = 1) ) |  |  |
| dps_id | 2 | ue(v) |
| } |  |  |
| } |  |  |
| … |  |  |

slice_vsp_flag together with bvsp_flag[ mbPartIdx ] specify, when ref_idx_lX[ mbPartIdx ] (with X equal to 0 or 1) refers to an inter-view reference picture, which motion vector derivation process specified in subclause J.8.3.1 is in use.

- J.7.3.5.1	Macroblock prediction syntax

| mb_pred( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| } else if( MbPartPredMode( mb_type, 0 ) != Direct ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L1 && mb_alc_flag = = 0 ) { |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| if( VspRefL0Flag[ mbPartIdx ] && slice_vsp_flag ) |  |  |
| bvsp_flag_l0[ mbPartIdx ] | 2 | u(1) ¦ ae(v) |
| } |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) { |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| if( VspRefL1Flag[ mbPartIdx ] && slice_vsp_flag ) |  |  |
| bvsp_flag_l1[ mbPartIdx ] | 2 | u(1) ¦ ae(v) |
| } |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode ( mb_type, mbPartIdx ) != Pred_L1 && ( !VspRefL0Flag[ mbPartIdx ] ¦ ¦ !bvsp_flag_l0[ mbPartIdx ] ) ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 && ( !VspRefL1Flag[ mbPartIdx ] ¦ ¦ !bvsp_flag_l1[ mbPartIdx ] ) ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |

- J.7.3.5.2	Sub-macroblock prediction syntax

| sub_mb_pred( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| sub_mb_type[ mbPartIdx ] | 2 | ue(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && mb_type != P_8x8ref0 && sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 && mb_alc_flag = = 0 ) { |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| if( VspRefL0Flag[ mbPartIdx ] && slice_vsp_flag ) |  |  |
| bvsp_flag_l0[ mbPartIdx ] | 2 | u(1) ¦ ae(v) |
| } |  |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 ) { |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| if( VspRefL1Flag[ mbPartIdx ] && slice_vsp_flag ) |  |  |
| bvsp_flag_l1[ mbPartIdx ] | 2 | u(1) ¦ ae(v) |
| } |  |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 && ( !VspRefL0Flag[ mbPartIdx ] ¦ ¦ !bvsp_flag_l0[ mbPartIdx ] ) ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ subMbPartIdx ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 && ( !VspRefL1Flag[ mbPartIdx ] ¦ ¦ !bvsp_flag_l1[ mbPartIdx ] ) ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ subMbPartIdx ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |

- J.7.4.5.1	Macroblock prediction syntax

…

bvsp_flag_lX[ mbPartIdx ] (X being equal to 0 or 1) equal to 0 specifies, when ref_idx_lX[ mbPartIdx ] (with X equal to 0 or 1) refers to an inter-view reference picture, which motion vector derivation process specified in subclause J.8.3.1 is in use. When bvsp_flag_lX[ mbPartIdx ] is not present and VspRefLXFlag[ mbPartIdx ] is equal to 0, bvsp_flag_lX[ mbPartIdx ] is inferred to be equal to 0. When bvsp_flag_lX[ mbPartIdx ] is not present and VspRefLXFlag[ mbPartIdx ] is equal to 1, bvsp_flag_lX[ mbPartIdx ] is inferred to be equal to slice_vsp_flag.

- J.8.3.1	Derivation process for motion vector components and reference indices

…

If VspRefLXFlag[ mbPartIdx ] is equal to 0 or both VspRefLXFlag[ mbPartIdx ] is equal to 1 and bvsp_flag[ mbPartIdx ] is equal to 0, the following applies:

–	When predFlagLX is equal to 1 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the derivation process for luma motion vector prediction in subclause Error: Reference source not found is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX.

–	When predFlagLX is equal to 1 and either DepthFlag is equal to 1 or dmvp_flag is equal to 0, the derivation process for luma motion vector prediction in subclause 8.4.1.3 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX.

–	The luma motion vectors are derived by

mvLX[ 0 ] = mvpLX[ 0 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 0 ]	(J-XXX)

mvLX[ 1 ] = mvpLX[ 1 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 1 ]	(J-XXX)

Otherwise (VspRefLXFlag[ mbPartIdx ] is equal to 1 and bvsp_flag[ mbPartIdx ] is equal to 1), subclause J.8.3.1.8.1 is invoked with DepthCurrPic, the location of the top-left sample of macroblock partition mbPartIdx and the width and height of of macroblock partition mbPartIdx (assigned to partWidth and partHeight, respectively) as inputs and the output assigned to mvLX., the following applies:

–	The luma motion vectors are derived by

mvLX[ 0 ] = 0		(J-XXX)

mvLX[ 1 ] = 0		(J-XXX)

…

- J.8.3.2	Decoding process for Inter or view synthesis prediction samples

Inputs to this process are:

–	a macroblock partition mbPartIdx,

–	a sub-macroblock partition subMbPartIdx,

–	variables specifying partition width and height for luma and chroma (if available), partWidth, partHeight, partWidthC (if available) and partHeightC (if available),

–	luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0 chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1,

–	variables for weighted prediction logWDC, w0C, w1C, o0C, o1C with C being replaced by L and, when ChromaArrayType is not equal to 0, Cb and Cr.

Outputs of this process are the prediction samples predPart, which are a (partWidth)x(partHeight) array predPartL of prediction luma samples, and when ChromaArrayType is not equal to 0 two (partWidthC)x(partHeightC) arrays predPartCb, predPartCr of prediction chroma samples, one for each of the chroma components Cb and Cr.

Let predPartL0L and predPartL1L be (partWidth)x(partHeight) arrays of predicted luma sample values and when ChromaArrayType is not equal to 0 predPartL0Cb, predPartL1Cb, predPartL0Cr, and predPartL1Cr be (partWidthC)x(partHeightC) arrays of predicted chroma sample values.

For LX being replaced by either L0 or L1 in the variables predFlagLX, RefPicListX, refIdxLX, refPicLX, predPartLX, the following is specified.

When predFlagLX is equal to 1, the following applies:

–	If refIdxLX indicates a synthetic reference component, the array predPartLXL and when ChromaArrayType is not equal to 0 the arrays predPartLXCb and predPartLXCr are derived by invoking the process specified in subclause  with the current partition specified by mbPartIdx\subMbPartIdx, the motion vectors mvLX, mvCLX (if available), the reference arrays with refPicLXL, refPicLXCb (if available), and refPicLXCr (if available), and currDepthPic being the decoded sample array of the depth view component of the view component pair for which the texture view component is the current texture view component given as input.

–	Otherwise, the following applies:

–	The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and when ChromaArrayType is not equal to 0 two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.4.2.1 with refIdxLX and RefPicListX given as input.

–	The array predPartLXL and when ChromaArrayType is not equal to 0 the arrays predPartLXCb and predPartLXCr are derived by invoking the process specified in subclause 8.4.2.2 with the current partition specified by mbPartIdx\subMbPartIdx, the motion vectors mvLX, mvCLX (if available), and the reference arrays with refPicLXL, refPicLXCb (if available), and refPicLXCr (if available) given as input.

For C being replaced by L, Cb (if available), or Cr (if available), the array predPartC of the prediction samples of component C is derived by invoking the process specified in subclause 8.4.2.3 with the current partition specified by mbPartIdx and subMbPartIdx, the prediction utilization flags predFlagL0 and predFlagL1, the arrays predPartL0C and predPartL1C, and the variables for weighted prediction logWDC, w0C, w1C, o0C, o1C given as input.

- J.8.3.2.1	Decoding process for view synthesis prediction samples

Inputs of this process are:

–	the current partition of the current texture view component given by its partition index mbPartIdx and its sub-macroblock partition index subMbPartIdx

–	the width and height partWidth, partHeight of this partition in luma-sample units,

–	a luma motion vector mvLX given in quarter-luma-sample units,

–	when ChromaArrayType is not equal to 0, a chroma motion vector mvCLX with a precision of one-(4*SubWidthC)-th chroma-sample units horizontally and one-(4*SubHeightC)-th chroma-sample units vertically,

–	the selected reference picture sample arrays refPicLXL, and when ChromaArrayType is not equal to 0, refPicLXCb, and refPicLXCr, and

–	a decoded depth view component currDepthPic.

Outputs of this process are:

–	a (partWidth)x(partHeight) array predPartLXL of prediction luma sample values,

–	when ChromaArrayType is not equal to 0, two (partWidthC)x(partHeightC) arrays predPartLXCb, and predPartLXCr of prediction chroma sample values.

Let ( xAL, yAL ) be the location given in full-sample units of the upper-left luma sample of the current partition given by mbPartIdx\subMbPartIdx relative to the upper-left luma sample location of the given two-dimensional array of luma samples.

Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in quarter-sample units. These variables are used only inside this subclause for specifying general fractional-sample locations inside the reference sample arrays refPicLXL, refPicLXCb (if available), and refPicLXCr (if available).

The variables currIndex and refIndex are derived by:

currIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of refPicLXL )

For each luma sample location (0 <= xL < partWidth, 0 <= yL < partHeight) inside the prediction luma sample array predPartLXL, the corresponding prediction luma sample value predPartLXL[ xL, yL ] is derived as specified by the following ordered steps:

- The variables vspBlockSize, dX, dY, shiftX, shiftY, and maxDepth are derived by: [Ed. (MH): Fix style/indents in the following.]

vspBlockSize = 4 >> reduced_resolution_flag
dX = xAL >> reduced_resolution_flag
dY = yAL >> reduced_resolution_flag
shiftX = ( (xL >> 2) << 2 ) >> reduced_resolution_flag
shiftY = ( (yL >> 2) << 2 ) >> reduced_resolution_flag
maxDepth = INT_MIN
for( j = 0; j < vspBlockSize; j+=( vspBlockSize – 1 ) )
	for( i = 0; i < vspBlockSize; i+=( vspBlockSize – 1 ) )
		if( depthPic[ dX + shiftX + i, dY + shiftY + j ] > maxDepth ) 
			maxDepth = depthPic[ dX + shiftX + i, dY + shiftY + j ]

- The variable disparityValue is derived by:

disparityValue = Disparity( maxDepth, currIndex, refIndex )

- Variables xIntL, yIntL, xFracL, and yFracL are derived by:

xIntL = xAL + xL + ( disparityValue >> 2 )
yIntL = yAL + yL

xFracL = disparityValue & 3
yFracL = 0

- The prediction luma sample value predPartLXL[ xL, yL ] is derived by invoking the process specified in subclause 8.4.2.2.1 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLXL given as input.

When ChromaArrayType is not equal to 0, the following applies.

Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in one-(4*SubWidthC)-th chroma-sample units horizontally and one-(4*SubHeightC)-th chroma-sample units vertically. These variables are used only inside this subclause for specifying general fractional-sample locations inside the reference sample arrays refPicLXCb, and refPicLXCr.

For each chroma sample location (0 <= xC < partWidthC, 0 <= yC < partHeightC) inside the prediction chroma sample arrays predPartLXCb and predPartLXCr, the corresponding prediction chroma sample values predPartLXCb[ xC, yC ] and predPartLXCr[ xC, yC ] are derived as specified by the following ordered steps:

- The variables vspBlockSize, dX, dY, shiftX, shiftY, and maxDepth are derived by: [Ed. (MH): Fix style/indents in the following.]

dX = xAL >> reduced_resolution_flag
dY = yAL >> reduced_resolution_flag
if( ChromaArrayType  = =  1  | |  ChromaArrayType  = =  2 ) {
	vspBlockSize = ( 4 >> 1) >> reduced_resolution_flag
	shiftX = ( ( xC >> 1 ) << 1 ) >> reduced_resolution_flag
	shiftY = ( ( yC >> 2 ) << 2 ) >> reduced_resolution_flag
} else {
	vspBlockSize = 4 >> reduced_resolution_flag
	shiftX = ( ( xC >> 2 ) << 2 ) >> reduced_resolution_flag
	shiftY = ( ( yC >> 2 ) << 2 ) >> reduced_resolution_flag
}
maxDepth = INT_MIN
for( j = 0; j < vspBlockSize; j += ( vspBlockSize -1) )
	for( i = 0; i < vspBlockSize; i += ( vspBlockSize -1 ) )
		if( depthPic[ dX + shiftX + i, dY + shiftY +j ] > maxDepth ) 
			maxDepth = depthPic[ dX + shiftX + i, dY + shiftY + j ]

- The variable disparityValue is derived by:

disparityValue = Disparity( maxDepth, currIndex, refIndex )

- The variable yFracC is derived by:

yFracC = 0

- Depending on ChromaArrayType, the variables xIntC, yIntC, and xFracC are derived as follows:

–	If ChromaArrayType is equal to 1,

xIntC = ( xAL / SubWidthC ) + ( disparityValue >> 3 ) + xC
yIntC = ( yAL / SubHeightC ) + yC
xFracC = ( disparityValue & 7 )

–	Otherwise, if ChromaArrayType is equal to 2,

xIntC = ( xAL / SubWidthC ) + ( disparityValue >> 3 ) + xC
yIntC = ( yAL / SubHeightC ) + yC
xFracC = ( disparityValue & 7 )

–	Otherwise (ChromaArrayType is equal to 3),

xIntC = xAL + ( disparityValue >> 2 ) + xC
yIntC = yAL + yC
xFracC = ( disparityValue & 3 )

- Depending on ChromaArrayType, the following applies:

–	If ChromaArrayType is not equal to 3, the following applies:

–	The prediction sample value predPartLXCb[ xC, yC ] is derived by invoking the process specified in subclause 8.4.2.2.2 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCb given as input.

–	The prediction sample value predPartLXCr[ xC, yC ] is derived by invoking the process specified in subclause 8.4.2.2.2 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCr given as input.

–	Otherwise (ChromaArrayType is equal to 3), the following applies:

–	The prediction sample value predPartLXCb[ xC, yC ] is derived by invoking the process specified in subclause 8.4.2.2.1 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCb given as input.

–	The prediction sample value predPartLXCr[ xC, yC ] is derived by invoking the process specified in subclause 8.4.2.2.1 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCr given as input.

## Specification text removals due to removal of the synthetic reference picture concept

A synthetic reference picture is not included in the reference picture list. Hence, ref_pic_list_3dvc_modification( ) syntax and semantics as well as the related decoding process are removed.

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| if( avc_3d_extension_flag && slice_header_prediction_flag != 0 ) { |  |  |
| pre_slice_header_src | 2 | u(2) |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| pre_ref_lists_src | 2 | u(2) |
| if ( !pre_ref_lists_src ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| if( nal_unit_type = = 20 ¦ ¦ ( nal_unit_type = = 21 && DepthFlag ) ) |  |  |
| ref_pic_list_mvc_modification( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| else if( nal_unit_type = = 21 && DepthFlag = = 0 ) |  |  |
| ref_pic_list_3dvc_modification( ) | 2 |  |
| } |  |  |
| } |  |  |
| … |  |  |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| if( nal_unit_type = = 20 ¦ ¦ ( nal_unit_type = = 21 && DepthFlag ) ) |  |  |
| ref_pic_list_mvc_modification( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| else if( nal_unit_type = = 21 && DepthFlag = = 0 ) |  |  |
| ref_pic_list_3dvc_modification( ) |  |  |
| else |  |  |
| ref_pic_list_modification( ) | 2 |  |
| … |  |  |
| } |  |  |

- J.8.1.1	Modification process for reference picture lists

Input to this process is reference picture list RefPicList0 and, when decoding a B slice, also reference picture list RefPicList1.

Outputs of this process are a possibly modified reference picture list RefPicList0 and, when decoding a B slice, also a possibly modified reference picture list RefPicList1.

When ref_pic_list_modification_flag_l0 is equal to 1, the following ordered steps are specified:

- Let refIdxL0 be an index into the reference picture list RefPicList0. It is initially set equal to 0.

- The corresponding syntax elements modification_of_pic_nums_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies:

–	If modification_of_pic_nums_idc is equal to 0 or equal to 1, the process specified in subclause Error: Reference source not found is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise, if modification_of_pic_nums_idc is equal to 2, the process specified in subclause Error: Reference source not found is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise, if modification_of_pic_nums_idc is equal to 4 or equal to 5, the process specified in subclause  is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise, if modification_of_pic_nums_idc is equal to 6, the process specified in subclause  is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise (modification_of_pic_nums_idc is equal to 3), the modification process for reference picture list RefPicList0 is finished.

When ref_pic_list_modification_flag_l1 is equal to 1, the following ordered steps are specified:

- Let refIdxL1 be an index into the reference picture list RefPicList1. It is initially set equal to 0.

- The corresponding syntax elements modification_of_pic_nums_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies:

–	If modification_of_pic_nums_idc is equal to 0 or equal to 1, the process specified in subclause Error: Reference source not found is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise, if modification_of_pic_nums_idc is equal to 2, the process specified in subclause Error: Reference source not found is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise, if modification_of_pic_nums_idc is equal to 4 or equal to 5, the process specified in subclause  is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise, if modification_of_pic_nums_idc is equal to 6, the process specified in subclause  is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise (modification_of_pic_nums_idc is equal to 3), the modification process for reference picture list RefPicList1 is finished.

- J.8.1.1.1	Modification process for reference picture lists for synthetic reference components

Inputs to this process are an index refIdxLX and a reference picture list RefPicListX (with X being 0 or 1).

Outputs of this process are an incremented index refIdxLX and a modified reference picture list RefPicListX.

The following procedure is conducted to place a synthetic reference component with VSP reference index equal to vsp_ref_idx into the index position refIdxLX, shift the position of any other remaining pictures to later in the list, and increment the value of refIdxLX:

for( cIdx = num_ref_idx_lX_active_minus1 + 1; cIdx > refIdxLX; cIdx− − )
	RefPicListX[ cIdx ] = RefPicListX[ cIdx − 1]
RefPicListX[ refIdxLX++ ] = synthetic reference component with VSP reference index equal to vsp_ref_idx

…

- J.7.3.3.1.2	Reference picture list 3DVC modification syntax

| ref_pic_list_3dvc_modification( ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type % 5 != 2 && slice_type % 5 != 4 ) { |  |  |
| ref_pic_list_modification_flag_l0 | 2 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |  |
| do { |  |  |
| modification_of_pic_nums_idc | 2 | ue(v) |
| if( modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| else if ( modification_of_pic_nums_idc = = 4 ¦ ¦ modification_of_pic_nums_idc = = 5 ) |  |  |
| abs_diff_view_idx_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 6 ) |  |  |
| vsp_ref_idx | 2 | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| if( slice_type % 5 = = 1 ) { |  |  |
| ref_pic_list_modification_flag_l1 | 2 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |  |
| do { |  |  |
| modification_of_pic_nums_idc | 2 | ue(v) |
| if( modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| else if ( modification_of_pic_nums_idc = = 4 ¦ ¦ modification_of_pic_nums_idc = = 5 ) |  |  |
| abs_diff_view_idx_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 6 ) |  |  |
| vsp_ref_idx | 2 | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| } |  |  |

- J.7.4.3.1.2	Reference picture list 3DVC modification semantics

The semantics specified in subclause I.7.4.3.1.1 apply with the following additions.

The following additional table entry is inserted in Table H-1.

| modification_of_pic_nums_idc | Modification specified |
| --- | --- |
| 6 | vsp_ref_idx is present and corresponds to a VSP reference index |

modification_of_pic_nums_idc shall not be equal to 6, if seq_view_synthesis_flag is equal to 0.

- J.8.1	3DVC decoding process for reference picture lists construction

The specifications of subclause I.8.1 apply with the following changes and additions:

–	Modification process for reference picture lists is specified in subclause  instead of subclause I.8.1.2.

…

- 3D Enhanced High profile

…

–	Reference picture list 0 for any coded slice shall contain at most one synthetic reference component. Reference picture list 1 for any coded slice shall contain at most one synthetic reference component that shall be the same as the synthetic reference picture included in reference picture list 0 for the same coded slice, if any. The synthetic reference component in reference picture list 0 or 1 of a coded slice of a view component shall be the same as the synthetic reference component in reference picture list 0 or 1, respectively, of any other coded slice of the same view component.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).