| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0164 |
| --- | --- |

| Title: | 3D-AVC: Removal of texture to depth resolution ratio restrictions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy, Payman Aflaki, Miska M. Hannuksela | Tel: Email: | +358 7180 08000 dmytro.rusanovskyy@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

The current 3D-AVC specification imposes restrictions on spatial resolution ratios between coded texture and depth: only two ratios are supported, 1:1 (texture and depth are coded at equal spatial resolution) and 1:0.5 (depth data is presented at half spatial resolution in horizontal and vertical directions). It is argued that these restrictions should be relaxed to make 3D-AVC applicable to a broader range of applications. This contribution asserts that MVD data with an arbitrary depth-to-texture resolution can be coded with 3D-AVC without significant changes to the codec design and proposes changes to the specification text.

# Introduction

The current 3D-AVC specification imposes restrictions on spatial resolution ratios between coded texture and depth: only two ratios are supported, 1:1 (texture and depth are coded at equal spatial resolution) and 1:0.5 (depth data is presented at half spatial resolution in horizontal and vertical directions). These restrictions have been included into the specification text at the time when 3D-AVC included in-loop upsampling of the depth map for purposes of Forward VSP and D-MVP in order to reduce computational complexity. However, the current design of the 3D-AVC does not require in-loop up-sampling of the depth map and therefore the mentioned restrictions are not justified any longer and should be relaxed.

# Proposed solution

3D-AVC specifies DMVP and VSP coding tools that applied for coding of texture samples of dependent views with help of associated depth map samples. For these purposes, selected depth map samples are converted to disparity vectors and utilized by DMVP and VSP.

Disparity value D has a linear relationship with depth map value d as equation (1) shows:

D = ( d * DisparityScale + DisparityOffset << BitDepth ) + ( 1 << ( log2Div – 1 ) ) ) >> log2Div	   (1)

where d is a depth sample value e.

DisparityScale is a scale factor, DisparityOffset is an offset value, BitDepth is equal to 8 and log2Div is a shift parameter that depends on the required accuracy of the disparity vectors. DisparityScale and DisparityOffset are transmitted within a bitstream with conventional variable length coding. For every pair of views (source view and target view) utilized in joint coding an independent set of parameters DisparityScale and DisparityOffset are transmitted.

The “disparity from depth derivation” method specified in 3D-AVC for purposes of VSP and DMVP coding modes is specified as follows.

The disparity for the currently coded texture block Cb is derived from the block of depth map data d(Cb) associated with currently coded texture block Cb. Depth map samples located at four corners (A: top-left, B: top-right, C: bottom-left and D: bottom-right) of d(Cb) are compared against each other and maximal depth map value among them is converted to disparity value, as specified in equation (1). Assuming related spatial coordinates, depth sample d selected for disparity conversion selected as following:

A = d(Cb)0,0

B = d(Cb)0,size_h

A = d(Cb) size_v,0

B = d(Cb)size_v,size_h

d = max(A,B,C,D).								(3)

where size_v and size_h are the spatial dimensions of block Cb.

In the case of reduced resolution depth map (1:0.5), spatial coordinates of texture’s block corners are downscaled by mixed_res flag to meet depth map resolution:

A = d(Cb)0,0

B = d(Cb)0,size_h>>mixed_res

A = d(Cb) size_v>> mixed_res,0

B = d(Cb)size_v>> mixed_res,size_h>> mixed_res

d = max(A,B,C,D).								(4)

As it seen from equation (4), utilized disparity derivation method can be adopted to arbitrary depth to texture resolution ratios without significant changes to the specification text.

In our proposal, we introduce two variables that are determined from dimensions of texture and depth pictures

ratioX = (int) widthT/widthD;

ratioY = (int) heightT/heightD; 								(5)

and the spatial coordinates utilized for scaling are derived as follows:

A = d(Cb)0,0

B = d(Cb)0,size_h/ratioX

A = d(Cb) size_v/ratioY,0,

B = d(Cb)size_v/ratioY,size_h/ratioX

d = max(A,B,C,D).								(6)

# Proposed specification text

There are two options for indicating the resolution of depth view component being different than that for texture view components:

- Extend the current reduced_resolution_flag signaling in seq_parameter_set_3dvc_extension( ) with explicit indication of the resolution and cropping window of the depth view components.

- Remove reduced_resolution_flag from seq_parameter_set_3dvc_extension( ). Use one active SPS for 3D-AVC texture views and another active SPS for 3D-AVC depth views. Indicate the resolution with the existing syntax elements in the SPS. Most of the syntax elements of the active SPSs are required to be identical.

The first option is preferred as it requires fewer SPSs to be coded and activated.

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| reduced_resolution_flag | 0 | u(1) |
| if( reduced_resolution_flag ) { |  |  |
| depth_pic_width_in_mbs_minus1 | 0 | ue(v) |
| depth_pic_height_in_map_units_minus1 | 0 | ue(v) |
| } |  |  |
| … |  |  |

reduced_resolution_flag equal to 1 specifies that the depth view components of a view component pair have a lower spatial resolution than the luma component of the texture view component of the same view component pair. reduced_resolution_flag equal to 0 specifies that when both depth view components and texture view components are present, they have the same spatial resolution.

The values of pic_width_in_mbs_minus1 and pic_height_in_map_units_minus1 are inferred to be equal to depth_pic_width_in_mbs_minus1 and depth_pic_height_in_map_units_minus1 for depth views for which this 3DVC sequence parameter set is an active view 3DVC sequence parameter set.

Variables RatioX and RatioY are determined from spatial dimensions of the texture picture by dividing them by spatial dimensions of the depth map picture.

RatioX = ( pic_width_in_mbs_minus1 + 1 ) / ( depth_pic_width_in_mbs_minus1 + 1 )
RatioY = ( pic_height_in_map_units_minus1 + 1 ) / ( depth_pic_height_in_map_units_minus1 + 1 )

J.8.3.2.1	Decoding process for view synthesis prediction samples

For each luma sample location (0 <= xL < partWidth, 0 <= yL < partHeight) inside the prediction luma sample array predPartLXL, the corresponding prediction luma sample value predPartLXL[ xL, yL ] is derived as specified by the following ordered steps:

The variables vspBlockSize, dX, dY, shiftX, shiftY, and maxDepth are derived by: [Ed. (MH): Fix style/indents in the following.]

vspBlockSizeX = 4/ RatioX
vspBlockSizeY = 4 / RatioY

dX = ( xAL + xL ) / RatioX
dY = ( yAL + yL ) / RatioY

D1 = depthPic[ dX, dY ]
D2 = depthPic[ dX, dY + vspBlockSizeY ]
D3 = depthPic[ dX + vspBlockSizeY, dY ]
D4 = depthPic[ dX + vspBlockSizeX, dY + vspBlockSizeY ]

maxDepth = max(D1,D2,D3,D4)

vspBlockSize = 4 >> reduced_resolution_flag
dX = xAL >> reduced_resolution_flag
dY = yAL >> reduced_resolution_flag
shiftX = ( (xL >> 2) << 2 ) >> reduced_resolution_flag
shiftY = ( (yL >> 2) << 2 ) >> reduced_resolution_flag
maxDepth = INT_MIN
for( j = 0; j < vspBlockSize; j+=( vspBlockSize – 1 ) )
	for( i = 0; i < vspBlockSize; i+=( vspBlockSize – 1 ) )
		if( depthPic[ dX + shiftX + i, dY + shiftY + j ] > maxDepth ) 
			maxDepth = depthPic[ dX + shiftX + i, dY + shiftY + j ]

## IVMP-related discussion

The changes proposed in this contribution do not cover IVMP. Considering that IVMP is enabled with sequence parameter set, bitstream compliance can be achieved by encoder side operations on enabling/disabling IMVP.

# Simulation results

The proposed technique was integrated to the 3D-ATM v7.0 and tested under the CTC. To validate the performance of the proposed technique at different depth map resolutions, the original depth map data was downsampled to several resolutions. At the decoder, the output depth map was upsampled to the original resolution at the integrated post-processing stage and utilized for view synthesis. A list of tested resolutions is shown in Table 1.

During the conducted experiments, the encoder options to disable VSD and IVMP were selected, otherwise the CTC was kept intact. The simulation results for the conducted experiments are provided in Tables 2-4. The anchor, utilized for comparison was produced by 3D-ATM v7.0 executed under the CTC with disabled VSO.

Table 1. Tested MVD data configuration (Texture full resolution)

| Experiment | Depth/Texture resolution ratio (vertical x horizontal) |
| --- | --- |
| Anchor | 0.5x0.5 |
| Scheme 1 | 0.25x1.0 |
| Scheme 2 | 0.25x0.5 |
| Scheme 3 | 0.25x0.25 |

Table 2. Compression efficiency of the Scheme 1 compared against the Anchor (VSD=OFF)

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Syntheses PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 0.08 | 0.00 | 1.29 | -0.04 | -1.03 | 0.04 |
| S02 | 0.11 | 0.00 | 0.61 | -0.02 | 0.96 | -0.03 |
| S03 | 0.03 | 0.00 | 1.12 | -0.04 | -7.15 | 0.24 |
| S04 | -0.08 | 0.00 | 1.08 | -0.04 | -0.12 | 0.00 |
| S05 | 0.01 | 0.00 | 1.29 | -0.06 | 0.91 | -0.04 |
| S06 | -0.01 | 0.00 | 1.87 | -0.09 | 1.69 | -0.07 |
| S08 | 0.00 | 0.00 | 0.80 | -0.03 | 1.71 | -0.06 |
| Average | 0.02 | 0.00 | 1.15 | -0.04 | -0.43 | 0.01 |

Table 3. Compression efficiency of the Scheme 2 compared against the Anchor (VSD=OFF)

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Syntheses PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 0.14 | -0.01 | -3.89 | 0.13 | -2.08 | 0.07 |
| S02 | 0.43 | -0.01 | -4.32 | 0.13 | -2.72 | 0.09 |
| S03 | 0.37 | -0.01 | -1.98 | 0.07 | -0.36 | 0.01 |
| S04 | 0.54 | -0.02 | -3.70 | 0.14 | -1.22 | 0.04 |
| S05 | 0.34 | -0.02 | -9.02 | 0.43 | -6.87 | 0.30 |
| S06 | 0.18 | -0.01 | -7.28 | 0.36 | -5.41 | 0.25 |
| S08 | 0.10 | 0.00 | -4.99 | 0.22 | 0.05 | 0.00 |
| Average | 0.30 | -0.01 | -5.03 | 0.21 | -2.66 | 0.11 |

Table 4. Compression efficiency of the Scheme 3 compared against the Anchor (VSD=OFF)

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Syntheses PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 0.30 | -0.01 | -6.76 | 0.22 | 1.79 | -0.07 |
| S02 | 0.79 | -0.02 | -7.01 | 0.22 | -2.16 | 0.07 |
| S03 | 0.85 | -0.03 | -3.18 | 0.11 | 22.47 | -0.65 |
| S04 | 1.19 | -0.04 | -5.80 | 0.22 | 2.37 | -0.10 |
| S05 | 0.70 | -0.03 | -15.40 | 0.78 | -8.72 | 0.38 |
| S06 | 0.59 | -0.03 | -12.10 | 0.62 | -6.94 | 0.32 |
| S08 | 0.16 | -0.01 | -7.35 | 0.32 | 3.94 | -0.15 |
| Average | 0.65 | -0.02 | -8.23 | 0.36 | 1.82 | -0.03 |

# Discussion

It should be noted, that simulation results presented in tables 2-4 do not provide us with information on the MVD coding configuration that is the most efficient in the compression efficiency sense.

A compression efficiency study conducted with a complete 3D-ATM v7.0 anchor (VSO=ON), provided in Table 5, shows that the current CTC conditions provide a better compression efficiency than any of the tested schemes. Obviously the tested scheme in Table 5 is penalized by the absence of VSO. Nevertheless, until the VSO is modified to support various depth resolutions, it is recommended to preserve the current CTC without changes.

Table 5. Compression efficiency of the Scheme 2 (VSO=OFF) compared against the Anchor (VSD=ON)

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Syntheses PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -0.26 | 0.01 | -0.55 | 0.02 | 3.55 | -0.12 |
| S02 | -0.03 | 0.00 | -1.17 | 0.04 | 1.23 | -0.04 |
| S03 | -0.34 | 0.01 | -3.45 | 0.12 | 4.21 | -0.13 |
| S04 | -0.56 | 0.02 | -1.73 | 0.06 | 1.39 | -0.05 |
| S05 | -0.20 | 0.01 | 2.80 | -0.13 | 4.29 | -0.18 |
| S06 | -0.24 | 0.01 | 1.18 | -0.06 | 2.27 | -0.10 |
| S08 | 0.08 | 0.00 | -0.27 | 0.01 | 5.79 | -0.20 |
| Average | -0.22 | 0.01 | -0.46 | 0.01 | 3.25 | -0.12 |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).