| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0165 |
| --- | --- |

| Title: | 3D-CE1.h: VSP Complexity Analysis and Constrained VSP (CVSP) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou, Dong Tian Anthony Vetro, Huifang Sun 201 Broadway, 8th Floor Cambridge, MA 02139 | Tel: Email: | +1-617-621-7556 {fzou, dong, avetro, hsun}@merl.com |
| Source: | Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

This contribution presents a complexity comparison between traditional block-based motion compensation (MC) and view synthesis prediction (VSP) based compensation with different configurations. Constraints on the VSP are proposed to reduce the number of fetched pixels for VSP compensation to maintain comparable complexity with traditional block-based MC. Simulations show that the proposed scheme could achieve bitrate savings under common test conditions: 0.2% BDBR for video PSNR/video bitrate, 0.2% for video PSNR/total bitrate and 0.2% for synthesis PSNR/total bitrate.

# Introduction

During the previous JCT3V meeting, a view synthesis prediction (VSP) scheme [1] was adopted by incorporating a VSP candidate into the merge candidate list and applying 4x4 block based compensation for a VSP prediction block (PB) in 3D-HEVC. In the 4x4 block based VSP compensation, a disparity vector is obtained from a corresponding 4x4 depth block. However, the block based VSP reduces the prediction accuracy compared with 1x1 pixel based VSP compensation, where the per pixel disparity vector is obtained from the per-pixel depth map.

Complexity concerns were raised regarding the 1x1 pixel based VSP compensation during that meeting, but it remained a general concern without in-depth study. To better clarify the complexity from pixel based VSP compensation, a thorough complexity analysis is conducted to compare the traditional block based MC and pixel based VSP compensation.

# Complexity analysis on motion/VSP compensation

In terms of software implementation, VSP compensation does not have a significant impact on run-time [1]. However, general concerns were raised about the implementation in hardware, especially for the 1x1 configuration. In this contribution, a complexity study is performed to compare traditional motion compensation (MC) and VSP compensation (VSP MC).

Several assumptions are made herein to reflect typical implementation practices for hardware.

- The reference frame buffer is assumed to be stored in an off-chip memory, referred as DPB buffer hereinafter;

- Quarter pixel accuracy is applied for traditional block based motion compensation with separable horizontal and vertical interpolation filter of 8 tap for half pixel position, and of 7 tap for quarter pixel position in HEVC, as described in Annex A for easier reference. The same quarter pixel accuracy is applied for VSP compensations as well;

- Prediction block (PB) size is represented as MxN (M and N stand for the width and the height of the prediction block.);

- The interpolation process is applied on an on-chip Processing Elements (PE), using the on-chip memory with data fetched from off-chip DPB buffer.

## High level analysis – Four factors to be evaluated

There are many factors that need be evaluated with respect to hardware implementation complexity/cost. For example, the number of calculations (addition, subtraction, multiplication, division, etc.), memory size, memory access bandwidth, and so on.

In this sub-section, we study the involved complexity issue for traditional MC, 4x4 VSP MC, 1x1 VSP MC. Before presenting the low level analysis, we first investigate the complexities from four aspects, which will guide us for in-depth low level analysis in the next sub-section.

Table 1 shows the results with a general MxN prediction block, and Table 2, 3, and 4 show the results with 4x4, 8x8 and 8x4 prediction blocks, respectively.

- Factor A. DPB pixel fetching.

In order to generate a predictor block, corresponding reference pixels need be fetched from DPB. The number of DPB pixels to be fetched would impact the memory bandwidth and computation complexity.

For traditional MC, since an 8-tap interpolation to generate the fractional position is the worst case for data fetching, the number of fetched DPB pixels is counted assuming all interpolation is performed using 8-tap interpolations. As we know, additional four (4) neighboring integer pixels are needed on each side of the prediction block both horizontally and vertically, therefore, the total number of fetched pixels from DPB would be (M+8)x(N+8).

For 4x4 VSP MC, the prediction block of MxN is split into MxN/(4x4) sub-blocks with a 4x4 size. Each 4x4 needs to fetch 4x(4+8) pixels from DPB, considering there is no vertical interpolations. Therefore, the total number of DPB pixels fetched is 4x(4+8)xMxN/(4x4).

For 1x1 VSP MC, each pixel need to fetch 8 pixels from DPB in the horizontal direction, so the total number is MxNx8.

- Factor B. DPB addressing calculation.

In order to fetch the DPB pixels, the address in the DPB buffer need be calculated which involves some arithmetic calculations. Line buffer access is assumed when possible.

For traditional block based MC, the number of addressing calculation is equal to the height N of the block.

For 4x4 VSP MC, the block of MxN is split into MxN/(4x4) sub-blocks of size 4x4. Each 4x4 needs 4 lines’ addressing calculation. And there are Nx(M/4) line addressing calculations for the block of MxN.

For 1x1 VSP MC, each pixel incurs an addressing calculation. For a block of MxN, there are totally MxN addressing calculations.

- Factor C. Maximum total number of interpolation.

The total number of interpolations in the worst case would have the most impact on memory requirements and computation complexity.

For traditional MC, the worst case happens for the sub pixel located at j00 as shown in Fig. 2. If data re-use is not considered, (8+1)xMxN interpolations are incurred. If vertically interpolated data can be re-used, the number of interpolation can be reduced to (N+8)xM+NxM = (2N+8)xM, but it may lead to more complex circuit design in order to save the computation power.

For 4x4 VSP MC, the prediction block MxN is split into MxN/(4x4) sub-blocks of size 4x4. Each 4x4 need to apply 4x4 = 16 interpolations, as there is no vertical interpolation. Therefore, 4x4xMxN/(4x4) = MxN interpolations are incurred.

For 1x1 VSP MC, the total number of interpolation is MxN.

- Factor D. On-chip pixel accessing.

The maximum number of reference pixels to be read out from on-chip memory to generate a single sub pixel in the worst case would also impact the memory bandwidth, and thus needs to be considered.

For traditional MC, the worst case happens for sub-pixel j00, which needs to load 8x8 integer pixels from on-chip memory for interpolation.

For either 4x4 or 1x1 VSP MC, as there is no vertical interpolation, the worst case is to load 8 integer pixels from on-chip memory for interpolation.

## Low level analysis about arithmetic calculations

From the high level analysis in subsection 2.1, it is noted that the major computations involved are interpolations and addressing calculations for the motion compensations under study. In this sub-section, we conduct a low level analysis on the arithmetic calculations involved within an interpolation and address calculation.

For an 8-tap interpolation procedure, it involves six (6) multiplications, four (4) additions, four (4) subtractions, and one (1) shifting. See equation (1-2) in Annex on how the numbers are counted.

Address calculations need be measured with respect to different compensation processes.

In traditional MC, a motion vector is needed to calculate an address in the DPB buffer. If the buffer is designed as a 2D array, two (2) additions are required to move the buffer pointer along each dimension.

For VSP MC a depth value is needed to calculate an address in the DPB buffer. Converting a depth value to disparity is currently implemented through a look up table, which requires one (1) addition. Moreover, assuming the DPB address of the collocated pixel is known, one more addition is required to horizontally shift the buffer pointer. Hence, two (2) additions in total are required for each address calculation for VSP MC.

In Table 5, the calculations involved in a single interpolation and a single address calculation for an 8x4 block are tabulated based on the above analysis.

## Overall comparison of decoder complexity

Finally, we compare the four different MC processes on an MxN prediction block. Table 2, 3 and 4 show the results on a 4x4 block, an 8x8 block, and 8x4 block respectively, and we mainly discuss 8x4 case (Table 4) here.

It can be observed that 1x1 VSP MC is inferior to traditional MC regarding Factors A and B, and superior regarding Factors C and D.

For address calculations (Factor B), the inferiority of 1x1 VSP comes from the intrinsic feature that each pixel may have a different disparity and hence result in irregular addressing when performing VSP MC for a whole prediction block. The irregular addressing required by 1x1 VSP compensation cannot be avoided; however, we believe it is not too difficult to be implemented.

It is further noted that 1x1 VSP MC would require much more pixel fetching from DPB (Factor A) compared to traditional MC. For example, traditional MC needs to fetch 192 pixels, while 1x1 VSP MC needs to fetch 256 pixels. Fortunately, the pixel fetching for an object in the scene still may be well overlapped for 1x1 VSP MC, as the neighboring reference pixels are likely to be close to each other. Therefore, it is possible to utilize the fact and design a method to avoid the any duplicated fetching so as to reduce the number of pixels fetched from DPB. A constrained VSP will be proposed in Section 3 on this issue.

To make a final complexity evaluation of each MC method, we further make a side-by-side comparison on a 64x64 block with 8x4 PB partitions, which is the worst case.

Firstly, the factors from Table 4 are multiplied (except Factor D) by 128 and shown in Table 6, as there are 128 8x4 PB partitions in a 64x64 block. Secondly, the factors from Table 5 are also multiplied by 128 and shown in Table 7.

From Table 7, it is observed that interpolation is the major source of computation complexity for all motion compensation methods. Therefore, it can be easily concluded that 1x1 VSP MC involves much less arithmetic calculations.

Regarding the on-chip memory bandwidth, the compensation methods could be compared based on the number of required interpolations, Factor C in Table 6. It could be concluded that 1x1 VSP MC requires much less bandwidth than traditional MC.

Regarding the off-chip memory bandwidth, the number of fetched DPB pixels is a dominant factor. In this aspect, 1x1 VSP MC requires higher bandwidth than traditional MC. However, the defect would be overcome with the proposed CVSP in section 3.

So far, we can conclude that 1x1 VSP compensation is comparable to traditional MC.

## Overall comparison of encoder complexity

In principle, the complexity analysis for decoder would apply for encoder as well. Additionally, 1x1 VSP prediction does not require any additional estimation at encoder while motion estimation is required for regular motion compensation. In this sense, the 1x1 VSP MC incurs even less complexity compared with traditional MV.

Table 1. Complexity comparison of different MC processes on an MxN block

| Decoder Side Analysis | Traditional MC | 4x4 VSP MC | 1x1 VSP MC | Proposed CVSP MC (1x1) |
| --- | --- | --- | --- | --- |
| A: Number of fetched pixels from DPB | (M+8)x(N+8) | 4x(4+8)xMxN/(4x4) | MxNx8 | (M+8+α)xN= (M+8)x(N+8), α is the maximum disparity constraint |
| B: Addressing calculation | N (line buffer read) | Nx(M/4) (line buffer read) | MxN | N (line buffer read) |
| C: Maximum total number of interpolation | (8+1)xMxN (not consider data reuse) (N+8)xM+NxM =(2N+8)xM (consider data reuse) | MxN, | MxN | MxN (interpolation used only for horizontal direction) |
| D: Number of read out pixels | 8x8=64 | 8 | 8 | 8 |

Table 2. Complexity comparison of different MC processes on a 4x4 block

| Decoder Side Analysis | Traditional MC | 4x4 VSP MC | 1x1 VSP MC | Proposed CVSP MC (1x1) |
| --- | --- | --- | --- | --- |
| A: Number of fetched pixels from DPB | 144 | 48 | 128 | 144 |
| B: Addressing calculation | 4 (line buffer read) | 4 (line buffer read) | 16 | 4 (line buffer read) |
| C: Maximum total number of interpolation | 144 (not consider data reuse) 64 (consider vertical interpolated intermediate fractional data reuse) | 16 | 16 | 16 (interpolation used only for horizontal direction) |
| D: Number of read out pixels | 64 | 8 | 8 | 8 |

Table 3. Complexity comparison of different MC processes on an 8x8 block

| Decoder Side Analysis | Traditional MC | 4x4 VSP MC | 1x1 VSP MC | Proposed CVSP MC (1x1) |
| --- | --- | --- | --- | --- |
| A: Number of fetched pixels from DPB | 256 | 192 | 512 | 256 |
| B: Addressing calculation | 8 (line buffer read) | 16 (line buffer read) | 64 | 8 (line buffer read) |
| C: Maximum total number of interpolation | 576 (not consider data reuse) 192 (consider vertical interpolated intermediate fractional data reuse) | 64 | 64 | 64 (interpolation used only for horizontal direction) |
| D: Number of read out pixels | 64 | 8 | 8 | 8 |

Table 4. Complexity comparison of different MC processes on an 8x4 block

| Decoder Side Analysis | Traditional MC | 4x4 VSP MC | 1x1 VSP MC | Proposed CVSP MC (1x1) |
| --- | --- | --- | --- | --- |
| A: Number of fetched pixels from DPB | 192 | 96 | 256 | 192 |
| B: Addressing calculation | 4 (line buffer read) | 8 (line buffer read) | 32 | 4 (line buffer read) |
| C: Maximum total number of interpolation | 288 (not consider data reuse) 128 (consider vertical interpolated intermediate fractional data reuse) | 32 | 32 | 32 (interpolation used only for horizontal direction) |
| D: Number of read out pixels | 64 | 8 | 8 | 8 |

Table 5. Arithmetic calculations involved on an 8x4 block

|  |  | Traditional MC | 4x4 VSP MC | 1x1 VSP MC | Proposed CVSP MC (1x1) |
| --- | --- | --- | --- | --- | --- |
| Interpolation | X | 128*6=768 | 32*6=192 | 32*6=192 | 32*6=192 |
|  | / | 0 | 0 | 0 | 0 |
|  | + | 128*4=512 | 32*4=128 | 32*4=128 | 32*4=128 |
|  | - | 128*4=512 | 32*4=128 | 32*4=128 | 32*4=128 |
|  | >> | 128*1=128 | 32*1=32 | 32*1=32 | 32*1=32 |
| Addressing calculation | X | 0 | 0 | 0 | 0 |
|  | / | 0 | 0 | 0 | 0 |
|  | + | 4*2=8 | 8*2=16 | 32*2=64 | 4*2=8 |
|  | - | 0 | 0 | 0 | 0 |
|  | >> | 0 | 0 | 0 | 0 |

Table 6. Complexity comparison of different MC processes on a 64x64 block with all 8x4 PB partitions

| Decoder Side Analysis | Traditional MC | 4x4 VSP MC | 1x1 VSP MC | Proposed CVSP MC (1x1) |
| --- | --- | --- | --- | --- |
| A: Number of fetched pixels from DPB | 192*128=24576 | 96*128=12288 | 256*128=32768 | 192*128=24576 |
| B: Addressing calculation | 4*128=512 (line buffer read) | 8*128=1024 (line buffer read) | 32*128=4096 | 4*128=512 (line buffer read) |
| C: Maximum total number of interpolation | 288*128=36864 (not consider data reuse) 128*128=16384 (consider vertical interpolated intermediate fractional data reuse) | 32*128=4096 | 32*128=4096 | 32*128=4096 (interpolation used only for horizontal direction) |
| D: Number of read out pixels | 64 | 8 | 8 | 8 |

Table 7. Arithmetic calculations involved on a 64x64 block with all 8x4 PB partitions

|  |  | Traditional MC | 4x4 VSP MC | 1x1 VSP MC | Proposed CVSP MC (1x1) |
| --- | --- | --- | --- | --- | --- |
| Interpolation | X | 768*128=98304 | 192*128=24576 | 192*128=24576 | 192*128=24576 |
|  | / | 0 | 0 | 0 | 0 |
|  | + | 512*128=65536 | 128*128=16384 | 128*128=16384 | 128*128=16384 |
|  | - | 512*128=65536 | 128*128=16384 | 128*128=16384 | 128*128=16384 |
|  | >> | 128*128=16384 | 32*128=4096 | 32*128=4096 | 32*128=4096 |
| Addressing calculation | X | 0 | 0 | 0 | 0 |
|  | / | 0 | 0 | 0 | 0 |
|  | + | 8*128=1024 | 16*128=2048 | 64*128=8192 | 8*128=1024 |
|  | - | 0 | 0 | 0 | 0 |
|  | >> | 0 | 0 | 0 | 0 |

# Proposed constrained VSP prediction (CVSP)

Recall that in subsection 2.3, the analysis shows that the off-chip memory bandwidth of 1x1 VSP MC is higher than traditional MC. To reduce the off-chip memory bandwidth, in this section, a constraint on the reference pixel location is applied for 1x1 VSP MC, referred to as constrained VSP (CVSP) herein.

In particular, it is proposed to apply a clipping function on the reference pixel location derivation in the horizontal direction as follows:

RefPixLocX = CurBlkLocX + clip3(min, max, xTxt+DV);

where CurBlkLocX is the horizontal position of the upper left pixel of the current NxM block. Min and Max are clipping parameters. xTxt is the relative horizontal position in the current NxM block with a value being 0, 1, 2, …, xTxt-1. DV is the per pixel disparity derived from per pixel depth value. The resulting parameter RefPixlLocX is the horizontal position of the reference pixel in the reference picture.

With this constraint, all the reference pixels are constrained in a rectangular region with horizontal position ranging from CurBlkLocX+min to CurBlkLocX+max. And the total number of fetched pixels from DPB is (max-min+1+8)xN=(M+8+α)xN, where α+M=max-min+1. And then we design α, such that (M+8+α)xN is equal to (M+8)x(N+8)  (the number of fetched pixels from DPB in traditional MC). With the proposed constrained VSP (CVSP), highlighted as column in Table 1, the 1x1 CVSP MC shows the same number of pixels fetched from DPB as that of traditional MC.

In the proposed CVSP method, for each PB size, a horizontal range parameter α is stored in a LUT shown in Table 8. And two methods to design max and min in CVSP are discussed.

- Method 1

For the dependent view (left view) with negative disparity relative to the base view, the leftmost reference position RefLeftMostLocX= Min(CurBlkLocX+xTxt+DV) is used as the starting position CurBlkLocX+min for the rectangular region.

For the dependent view (right view) with positive disparity relative to the base view, the rightmost reference position RefRightMostLocX=Max(CurBlkLocX+xTxt+DV)  is used as the ending position CurBlkLocX+max for the rectangular region.

Therefore, for left view, min = RefLeftMostLocX- CurBlkLocX and max= min+ α+M-1.

For right view, max = RefRightMostLocX- CurBlkLocX and min = max- α-M+1.

With the derived min and max, clipping operation is applied during reference pixel access from on-chip memory for interpolation purpose.

- Method  2:

For the dependent view (left view) with negative disparity relative to the base view, the leftmost reference position RefLeftMostLocX= Min(CurBlkLocX+xTxt+DV) is used as the starting position CurBlkLocX+min for the rectangular region.

For the dependent view (right view) with positive disparity relative to the base view, the leftmost reference position RefLeftMostLocX= Min(CurBlkLocX+xTxt+DV) is used as the starting position CurBlkLocX+min for the rectangular region.

Therefore, for left view, min = RefLeftMostLocX- CurBlkLocX and max= min+ α+M-1.

For right view, min = RefLeftMostLocX- CurBlkLocX and max= min+ α+M-1.

With the proposed methods 1 and 2 of CVSP, the number of pixels fetched from DPB is the same as that of the traditional MC, and thus equal off-chip memory bandwidth. As the worst case of MC tabulated in Table 6, CVSP has the same complexity in Factors A, B and D, and less complexity in Factor C compared with traditional MC. In general, CVSP has comparable complexity compared with traditional MC.

From the above study, the following conclusions can be made:

Regarding Factors A, B and D, the proposed CVSP has the same complexity as the traditional MC.

Regarding Factors C, the proposed CVSP has significantly reduced complexity to 25% compared with the traditional MC.

Regarding the arithmetic calculations for interpolation, CVSP has significantly reduced complexity also to 25% compared with the traditional MC.

Regarding the arithmetic calculations for addressing, CVSP has the same complexity as the traditional MC.

Hence, we can make a general conclusion that CVSP has less complexity when compared with the traditional MC in terms of memory bandwidth (both on-chip and off-chip) and arithmetic calculation, and is feasible for hardware implementation.

Table 8. α value for different PB sizes

| M | N | Traditional MC (M+8)x(N+8) | CVSP (M+ α +8)xN | α | α+M |
| --- | --- | --- | --- | --- | --- |
| 4 | 8 | 192 | 192 | 12 | 16 |
| 8 | 4 | 192 | 192 | 32 | 40 |
| 8 | 8 | 256 | 256 | 16 | 24 |
| 8 | 16 | 384 | 384 | 8 | 16 |
| 16 | 8 | 384 | 384 | 24 | 40 |
| 16 | 16 | 576 | 576 | 12 | 28 |
| 16 | 32 | 960 | 960 | 6 | 22 |
| 32 | 16 | 960 | 960 | 20 | 52 |
| 32 | 32 | 1600 | 1600 | 10 | 42 |
| 32 | 64 | 2880 | 2880 | 5 | 37 |
| 64 | 32 | 2880 | 2880 | 18 | 82 |
| 64 | 64 | 5184 | 5184 | 9 | 73 |

# Performance evaluation

Besides the complexity reduction of the proposed CVSP, the RD performance of CVSP is also evaluated. In this section, three experiments are performed following the common test conditions as defined in [2].

The first experiment is to enable 1x1 pixel based VSP in the reference software HTM6.0 [3] by setting the macro MERL_VSP_BLOCKSIZE_C0152 to be 1. From this experiment, the performance of unconstrained 1x1 VSP is evaluated and the results are shown in Table 9. The overall bitrate saving is about 0.2%.

The second experiment is to test the performance of the proposed CVSP with Method 1 and the results are shown in Table 10.

The third experiment is to test the performance of the proposed CVSP with Method 2 and the results are shown in Table 11.

From the above three experiments, we can found that 1x1 VSP has obvious RD improvement for sequences with high depth quality, e.g. Undo_Dancer and GT_Fly. On the other hand, CVSP maintains similar RD improvements as unconstrained 1x1 VSP, while the complexity is reduced as per section 3.

Table 9. Unconstrained 1x1 VSP in the HTM 6.0.

Table 10. Constrained 1x1 VSP with Method 1 based on HTM 6.0.

Table 11. Constrained 1x1 VSP with Method 2 based on HTM 6.0.

# Conclusions

In this contribution, the complexities for different motion compensation methods are evaluated, and we conclude that the proposed constrained 1x1 VSP shows less complexity than a traditional MC, and is feasible for proper hardware implementation. On the other hand, the constrained 1x1 VSP can maintain similar coding performance as an unconstrained 1x1 VSP. Therefore, it is recommended to include the 1x1 CVSP into the reference software, CTC and working draft.

# Patent rights declaration(s)

Mitsubishi Electric Research Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] D. Tian, F. Zou and A. Vetro (MERL), “CE1.h: Backward View Synthesis Prediction using Neighbouring Blocks”, JCT3V-C0152, Geneva, CH, Jan. 2013

[2] D. Rusanovskyy, K. Mueller and A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-C1100, Geneva, CN, Jan. 2013

[3] https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.0

# Annex A

## HEVC 8/7 tap interpolation filter

Fig. 2. Integer and fractional positions for HEVC-based 8/7 tap interpolation filter

- The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 shall be derived by applying the 8-tap filter to the nearest integer position samples:

a0,0 = ( −A−3,0 + 4*A−2,0 − 10*A−1,0 + 58*A0,0 +	 17*A1,0 − 5*A2,0 + A3,0 ) >> shift1	(1-1)

b0,0 = ( −A−3,0 + 4*A−2,0 − 11*A−1,0 + 40*A0,0 +	 40*A1,0 − 11*A2,0 + 4*A3,0 − A4,0 ) >> shift1	(1-2)

c0,0 = ( A−2,0 − 5*A−1,0 + 17*A0,0 +	 58*A1,0 − 10*A2,0 + 4*A3,0 − A4,0 ) >> shift1	(1-3)

d0,0 = ( −A0,−3 + 4*A0,−2 − 10*A0,−1 + 58*A0,0 +	 17*A0,1 − 5*A0,2 + A0,3 ) >> shift1	(1-4)

h0,0 = ( −A0,−3 + 4*A0,−2 − 11*A0,−1 + 40*A0,0 +	 40*A0,1 − 11*A0,2 + 4*A0,3 − A0,4 ) >> shift1	(1-5)

n0,0 = ( A0,−2 − 5*A0,−1 + 17*A0,0 +	 58*A0,1 − 10*A0,2 + 4*A0,3 − A0,4 ) >> shift1	(1-6)

- The samples labelled e0,0, i0,0, p0,0, f0,0, j0,0, q0,0, g0,0, k0,0 and r0,0 shall be derived by applying the 8-tap filter to the samples a0,i, b0,i and c0,i where i = −3..4 in vertical direction:

e0,0 = ( −a0,−3 + 4*a0,−2 − 10*a0,−1 + 58*a0,0 +	 17*a0,1 − 5*a0,2 + a0,3 ) >> shift2	(1-7)

i0,0 = ( −a0,−3 + 4*a0,−2 − 11*a0,−1 + 40*a0,0 +	 40*a0,1 − 11*a0,2 + 4*a0,3 − a0,4 ) >> shift2	(1-8)

p0,0 = ( a0,−2 − 5*a0,−1 + 17*a0,0 +	 58*a0,1 − 10*a0,2 + 4*a0,3 − a0,4 ) >> shift2	(1-9)

f0,0 = ( −b0,−3 + 4*b0,−2 − 10*b0,−1 + 58*b0,0 +	 17*b0,1 − 5*b0,2 + b0,3 ) >> shift2	(1-10)

j0,0 = ( −b0,−3 + 4*b0,−2 − 11*b0,−1 + 40*b0,0 +	 40*b0,1 − 11*b0,2 + 4*b0,3 − b0,4 ) >> shift2	(1-11)

q0,0 = ( b0,−2 − 5*b0,−1 + 17*b0,0 +	 58*b0,1 − 10*b0,2 + 4*b0,3 − b0,4 ) >> shift2	(1-12)

g0,0 = ( −c0,−3 + 4*c0,−2 − 10*c0,−1 + 58*c0,0 +	 17*c0,1 − 5*c0,2 + c0,3 ) >> shift2	(1-13)

k0,0 = ( −c0,−3 + 4*c0,−2 − 11*c0,−1 + 40*c0,0 +	 40*c0,1 − 11*c0,2 + 4*c0,3 − c0,4 ) >> shift2	(1-14)

r0,0 = ( c0,−2 − 5*c0,−1 + 17*c0,0 +	 58*c0,1 − 10*c0,2 + 4*c0,3 − c0,4 ) >> shift2	(1-15)