| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH , 17–23 Jan. 2013 | Document: JCT3V-C1005_spec_d1 |
| --- | --- |

| Title: | 3D-HEVC Test Model 3 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-3V |  |  |
| Purpose: | 3D-HEVC Test Model Description, draft specification |  |  |
| Author(s) or Contact(s): | Gerhard Tech Fraunhofer HHI Krzysztof Wegner Poznan University of Technology Ying Chen Qualcomm Incorporated Sehoon Yea LG Electronics | Email: Email: Email: Email: | gerhard.tech@hhi.fraunhofer.de kwegner@multimedia.edu.pl cheny@qti.qualcomm.com sehoon.yea@lge.com |
| Source: | Editor |  |  |

_____________________________

Abstract

Draft 3 of 3D-HEVC Test Model Description

Ed. Notes (WD3) (changes compare to JCT3V-B1005):

- ----------- Release d1 -----------

- (3Dc-04) Revised text related to edge intra.

- (3DC-GT2) Editorial improvements, small corrections.(among others tickets #21 #22)

- ----------- Release d0 -----------

- Converted to .doc- File

- Split of Test Model text and specification text

- (3DE-05) Alignment with MV-HEVC draft 3.

- (3DE-01) Reordered sub-clauses related to disparity estimation and additional motion candidates.

- (3DN-20) Alignment of JCT3V-C0152 + JCT3V-C0137.

- (3DN-07/JCT3V-C0137) Texture motion vector candidate for depth.

- (3DN-07/JCT3V-C0137) Removal of MPI.

- (3DN-19) Camera parameters

- (3Dn−03) Wedgelet pattern generation process.

- (3Dn-01) Incorporated missing intra-predicted wedgelet partition mode

- (3DN-08/JCT3V-C0138) Removal of parsing dependency for inter-view residual prediction.

- (3DN-18/JCT3V-C0160) QTL disabled for RAP.

- (3DN-17/JCT3V-C0154) Reference sub-sampling for SDC and DMM.

- (3Dc-03) Fix SDC

- (3DN-16/JCT3V-C0096) Removal of DMM 2 from SDC.

- (3DN-15/JCT3V-C0034) Delta DC processing for DMMs.

- (3DN-14/JCT3V-C0044) Signalling of wedgeIdx for DMM3.

- (3DN-02/JCT3V-C0152) View synthesis prediction (without disparity derivation part).

- (3DN-03/JCT3V-C0112) Restricted search of max disparity.

- (3DN-01,02/JCT3V-C0131,JCT3V-C0152) Disparity derivation from depth maps.

- (3Dc-02) Incorporated missing conditions of long/short-term pictures in AMVP (related to JCT3V-B0046).

- (3DN-13/JCT3V-C0116) Inter-view vector scaling for AMVP.

- (3DE-03) Incorporated derivation process for AMVP from base spec.

- (3DN-12/JCT3V-C0115) Signalling of inter-view motion vector scaling.

- (3DE-02) Incorporated TMVP text from base spec.

- (3DN-09/JCT3V-C0047) Alternative reference index for TMVP.

- (3DN-10/JCT3V-C0051) Unification of inter-view candidate derivation.

- (3DE-01) Revised text related to residual prediction.

- (3DN-06/JCT3V-C0129) Vertical component in residual prediction.

- (3DN-05/JCT3V-C0097/JCT3V-C0141) Temporal blocks first in DV derivation.

- (3DC-GT1) Editorial improvements, small corrections.

- (3DN-04/JCT3V-C0135) Restriction on the temporal blocks for memory bandwidth reduction in DV derivation.

- (3Dn-02) Full sample MV accuracy for depth.

- (3DN-11/JCT3V-C0046) Extension of illumination compensation to depth.

- (3Dc-01) Fix Illumination compensation (including ic_flag for skip).

Ed. Notes (WD2) (changes compared to JCT3V-A1005)

- Accepted changes and marked delta to base spec

- (3DC-GT2) Editorial improvements, small corrections

- (3DC-CY) Editorial improvements, small corrections

- (MVS-02/JCT3V-B0046) Treatment of inter-view pictures as long term- reference pictures

- (3DE-11) Revised text related to 3Dn-01

- (3Dn-01/m23639) Results on motion parameter prediction

- (3DE-12) Revised text related residual prediction

- (3DE-10) Revised text Related to Illumination compensation.

- (3DN-01/JCT3V-B0045) Illumination compensation for inter-view prediction.

- (3Dn-02/m24766) Restricted Inter-View Residual Prediction

- (3DE-09) Revised text related to depth intra: Edge Intra

- (3DE-09) Revised text related to depth intra: SDC

- (3DE-09) Revised text related to depth intra: DMMs

- (3DO-01/JCT3V-B0131) Depth distortion metric with a weighted depth fidelity term

- (3DN-12/JCT3V-B0036) Simplified Depth Coding with an optional Depth LUT

- (3DN-13/JCT3V-B0039) Simplified Wedgelet search for DMM modes 1 and 3

- (3DN-03/JCT3V-B0083) Unconstrained motion parameter inheritance

- (3DE-08) Incorporated context tables for SDC

- (3DE-07) Improved MPI text.

- (3DN-02/JCT3V-B0068) Incorporated Depth Quadtree Prediction.

- (3DE-06) Incorporated parsing process, including tables for DMMs.

- (3DE-05) Added missing initialization of invalid motion/disparity parameters

- (3DC-03) Added missing pruning of collocated merge candidate due to number of total candidates.

- (3DE-04) Moved pruning of spatial merge candidate B2 due to number of total candidates.

- (3DE-03) Moved derivation of disparity one level higher in process hierarchy.

- (3DE-02) Inserted "Derivation process for motion vector components and reference indices" from base spec

- (3DC-02) Fixed storage of IvpMvFlagLX and IvpMvDisp.

- (3DN-09-10-11/JCT3V-B0048,B0069,B0086) Modification inter-view merge candidates

- (3DC-01) Fixed derivation of inter-view merge candidates.

- (3DE-01) Revised derivation of disparity from temporal candidates

- (3DN-04/JCT3V-B0047) Improvements for disparity vector derivation)

- (3DN-08/JCT3V-B0136) Support of parallel merge in disparity vector derivation

- (3DN-05/JCT3V-B0135) Modified disparity vector derivation process for memory reduction

- (3DN-04/JCT3V-B0111) Decoupling inter-view candidate for AMVP

- (3DN-07/JCT3V-B0096) Removal of dependency between multiple PUs in a CU for DV-derivation

- (3DC-GT) Small corrections, editorial improvements

Ed. Notes (WD1) ( changes compare to N12744)

- (3D08/JCT3V-A0126) (T,N) Simplified disparity derivation

- (3D16) Moved 3D-tool related flags from SPS to VPS, removal camera parameters

- (3D09/JCT3V-A0049) (N) Inter-view motion prediction modification

- (3D13/JCT3V-A0119) (T) VSO depth fidelity

- (3D07/JCT3V-A0070) (T,N) Region boundary chain coding for depth maps

- (3D06/JCT3V-A0087) (T) RDO selection between Non-Zero Residual and All-Zero Residual Intra

- (3D12) (T) Depth Quadtree Prediction

- (3D15) (N) Fix references

- (3D11) (T,N) Improvement of text of already adopted tools

- (3D10/JCT3V-A0097) (T;N) Disparity vector generation

- (3D02) (N) Removed MV-Part and update to Annex F

- (3D03) (T) Labelling of tools not in CTC/Software. Removal?

- (3D05/JCT3V-A0093) (T) VSO early skip

- (3D04/JCT3V-A0033) (T) VSO model based estimation

- (3D14) (N) Update of low level specification to match HEVC text specification 8(d7)

- (3D01) (N): Removed HEVC text specification

Contents

Abstract	1

Contents	4

List of Figures	5

List of Tables	5

Annex H 	3D High Efficiency Video Coding	7

H.1	Scope	7

H.2	Normative references	7

H.3	Definitions	7

H.4	Abbreviations	7

H.5	Conventions	7

H.6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	8

H.7	Syntax and semantics	8

H.7.1	Method of specifying syntax in tabular form	8

H.7.2	Specification of syntax functions, categories, and descriptors	8

H.7.3	Syntax in tabular form	8

H.7.3.1	NAL unit syntax	8

H.7.3.2	Raw byte sequence payloads and RBSP trailing bits syntax	8

H.7.3.3	Profile, tier and level syntax	10

H.7.3.4	Scaling list data syntax	10

H.7.3.5	Slice header syntax	11

H.7.3.6	Slice data syntax	13

H.7.3.7	Coding tree unit syntax	13

H.7.3.8	Coding quadtree syntax	14

H.7.3.9	Coding unit syntax	14

H.7.3.10	Prediction unit syntax	19

H.7.3.11	Transform tree syntax	19

H.7.3.12	Transform unit syntax	19

H.7.3.13	Residual coding syntax	19

H.7.4	Semantics	20

H.7.4.1	NAL unit semantics	20

H.7.4.2	Raw byte sequence payloads and RBSP trailing bits semantics	20

H.7.4.3	Profile, tier and level semantics	22

H.7.4.4	Operation point semantics	22

H.7.4.5	Slice header semantics	22

H.7.4.6	Slice data semantics	23

H.7.4.7	Coding tree unit semantics	23

H.7.4.8	Coding quadtree semantics	23

H.7.4.9	Coding unit semantics	24

H.7.4.10	Prediction unit semantics	27

H.7.4.11	Transform tree semantics	28

H.7.4.12	Transform coefficient semantics	28

H.7.4.13	Residual coding semantics	28

H.8	Decoding process	29

H.8.1	General decoding process	29

H.8.1.1	Decoding process for a coded picture with nuh_layer_id greater than 0	29

H.8.2	NAL unit decoding process	29

H.8.3	Slice decoding process	29

H.8.3.1	Decoding process for picture order count	29

H.8.3.2	Decoding process for reference picture set	29

H.8.3.3	Decoding process for generating unavailable reference pictures	29

H.8.3.4	Decoding process for reference picture lists construction	29

H.8.3.5	Derivation process for the candidate picture list for disparity vector derivation	29

H.8.3.6	Decoding process for a depth lookup table	30

H.8.3.7	Derivation process for the alternative target reference index for TMVP in merge mode	31

H.8.4	Decoding process for coding units coded in intra prediction mode	31

H.8.4.1	General decoding process for coding units coded in intra prediction mode	31

H.8.4.2	Derivation process for luma intra prediction mode	31

H.8.4.3	Derivation process for chroma intra prediction mode	33

H.8.4.4	Decoding process for intra blocks	33

H.8.5	Decoding process for coding units coded in inter prediction mode	48

H.8.5.1	Inter prediction process	48

H.8.5.2	Decoding process for prediction units in inter prediction mode	48

H.8.5.3	Decoding process for the residual signal of coding units coded in inter prediction mode	77

H.8.5.4	Derivation process for a disparity vector	77

H.8.6	Scaling, transformation and array construction process prior to deblocking filter process	81

H.8.7	In-loop filter process	82

H.9	Parsing process	82

H.9.1	Parsing process for Truncated Unary codes	82

H.9.2	Parsing process for 0-th order Exp-Golomb codes	82

H.9.2.1	Mapping process for signed Exp-Golomb codes	82

H.9.3	CABAC parsing process for slice data	82

H.9.3.1	Initialization process	82

H.9.3.2	Binarization process	86

H.9.3.3	Decoding process flow	89

H.9.3.4	Arithmetic encoding process (informative)	90

H.10	Specification of bitstream subsets	91

H.11	Profiles and levels	91

H.12	Byte stream format	91

H.13	3D-HEVC hypothetical reference decoder	91

H.14	3D-HEVC SEI messages	91

H.15	Video usability information	91

List of Figures

No table of figures entries found.

List of Tables

Table H-1  – Mapping of ScalabiltyId to scalability dimensions	20

Table H-2 – Interpretation of dmm_mode	25

Table H-3 – Interpretation of edge_code[ x0 + i][ y0 + j ][ k ]	26

Table H-4 – Interpretation of sdc_pred_mode	27

Table H-5 – Specification of intra prediction mode and associated names	31

Table H-6 – Specification of refDeltaHor, refDeltaVer  depending on predModeIntra, refAboveFlag, intraPredAng	38

Table H-7 – Specification of wedgeOri depending on refWedgePattern	39

Table H−8 – Specification of resShift	43

Table H−9 – Specification of xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE	44

Table H−10 – Specification of xS, yS, xE, yE	45

Table H−11 Specification of xOff, yOff	46

Table H-12 – Specification of divCoeff depending on psDenomDiv	74

Table H-13 – LUT_constraint_vsp[ nPbW ][ nPbH ]	76

Table H-13 – Association of ctxIdx and syntax elements for each initializationType in the initialization process	83

Table H-14 – Values of variable initValue for dmm_flag ctxIdx	84

Table H-15 – Values of variable initValue for dmm_mode ctxIdx	84

Table H-16 – Values of variable initValue for wedge_full_tab_idx ctxIdx	84

Table H-17– Values of variable initValue for wedge_predtex_tab_idx ctxIdx	84

Table H-18– Values of variable initValue for wedge_predtex_tab_idx ctxIdx	84

Table H-19 – Values of variable initValue for dmm_dc_1_abs and dmm_dc_2_abs ctxIdx	84

Table H-20 – Values of variable initValue for edge_code ctxIdx	84

Table H-21 – Values of variable initValue for ic_flag ctxIdx	84

Table H-22 – Values of variable initValue for mvp_l0_idx and mvp_l1_idx ctxIdx	85

Table H-23 – Values of variable initValue for sdc_flag ctxIdx	85

Table H-24 – Values of variable initValue for sdc_residual_flag ctxIdx	85

Table H-25 – Values of variable initValue for sdc_residual_sign_flag ctxIdx	85

Table H-26 – Values of variable initValue for sdc_residual_abs_minus1 ctxIdx	85

Table H-27 – Values of variable initValue for sdc_pred_mode ctxIdx	85

Table H-28 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset	87

Table H-29 – Values of wedgeFullTabIdxBits[ log2PUSize ]	88

Table H-30 – Values of wedgePredTexTabIdxBits[ log2PUSize ]	88

Table H-31 – Assignment of ctxIdxInc to syntax elements with context coded bins	89

Table H-32 – Specification of ctxIdxInc using left and above syntax elements	90

Table H-34 – Specification of ctxIdxInc using segment index	90

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

3D High Efficiency Video Coding Draft Specification

The specifications made in this section are based on HEVC text specification draft 8 (JCTVC-J1003_d7) and MV-HEVC draft 3 (JCT3V-C1004_d1).

In text blocks copied from JCTVC-J1003_d7 changes are highlighted.

Modifications of HEVC specification:

In Foreword replace paragraph that start with "In this Recommendation | International Standard Annexes":

In this Recommendation | International Standard Annexes A through H contain normative requirements and are an integral part of this Recommendation | International Standard.

In 0.7, add the following paragraph after the paragraph that starts with "Annex F":

Annex H specifies multiview and depth video coding, referred to as 3D High Efficiency Video Coding (3D-HEVC). The reader is referred to Annex H for the entire decoding process for 3D-HEVC, which is specified there with references being made to clauses 1-9 and Annexes A-G.

- Annex H 	3D High Efficiency Video Coding

This annex specifies 3D high efficient video coding, referred to as 3D-HEVC.

- Scope

Bitstreams and decoders conforming to the profile specified in this annex are completely specified in this annex with reference made to clauses 2-9 and Annexes A-G.

- Normative references

The specifications in clause 2 apply.

- Definitions

For the purpose of this annex, the following definitions apply in addition to the definitions in clause F.3 and G.3. These definitions are either not present in clause F.3 and G.3 or replace definitions in clause F.3 and G.3.

- depth view: A sequence of depth view components associated with an identical value of view order index.

- depth view component: A coded representation of the depth of a view in a single access unit.

- texture view: A sequence of texture view components associated with an identical value of view order index.

- texture view component: A coded representation of the texture of a view in a single access unit.

- view component: A coded representation of a view in a single access unit  A view component may contain a depth view component and a texture view component.

[Ed. (GT): Above definitions are not necessary with MV-HEVC Draft 3]

- Abbreviations

The specification in clause 4 apply.

- Conventions

The specification in clause 5 apply.

- Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specification in clause 6 apply.

- Syntax and semantics

This clause specifies syntax and semantics for coded video sequences that conform to one or more of the profiles specified in this annex.

  - Method of specifying syntax in tabular form

The specifications in subclause 7.1 apply.

  - Specification of syntax functions, categories, and descriptors

The specifications in subclause 7.2 apply.

  - Syntax in tabular form

    - NAL unit syntax

The specifications in subclause G.7.3.1 and all its subclauses apply.

    - Raw byte sequence payloads and RBSP trailing bits syntax

      - Video parameter set RBSP

The specifications in subclause G.7.3.2.1 apply.

[ Ed. (GT): Inclusion of potential VPS extension 3 needs to be specified. ]

        - Video parameter set extension syntax

The specifications in subclause G.7.3.2.1.1 apply.

        - Video parameter set extension 2 syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| VpsDepthFlag[ layerId ] = layerId % 2 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V DepthLayerFlag[ layerId ] should be derived from dimension_id to be aligned with basic idea of MV-HEVC Draft 3. ] |  |
| ViewId[ layerId ] = layerId >> 1 [ Ed. (GT): For consistency with 3D-HEVC Draft 2 only. After confirmation by JCT-3V ViewId[ layerId ] should be derived as specified in MV-HEVC Draft 3. ] |  |
| view_id[ layerId ] [ Ed. (GT): This syntax element is different from ViewId[ layerId ]. See comment in MV-HEVC Draft 3. Renaming/alignment is required. ] | u(8) |
| if ( (layerId ! = 0) && !DepthLayerFlag[ layerId ] ) { |  |
| iv_mv_pred_flag[ layerId ] | u(1) |
| iv_res_pred_flag[ layerId ] | u(1) |
| } |  |
| if ( DepthLayerFlag[ layerId ] ) { |  |
| vps_dmm_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) { |  |
| dlt_depth_value[ layerId ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

      - Sequence parameter set RBSP syntax

The specifications in subclause G.7.3.2.2 apply.

[ Ed. (GT): Inclusion of a potential further SPS extension needs to be specified. ]

        - Sequence parameter set extension syntax

The specifications in subclause G.7.3.2.2.1 apply.

        - Sequence parameter set extension 2 syntax

[ Ed. (GT). As decided camera parameters are specified as done in software (where one SPS is signalled per layer and DepthFlag and ViewId are derived directly from nuh_layer_id). When ViewId[ nuh_layer_id ] (and potentially also VpsDepthFlag[ nuh_layer_id ]) are derived from VPS extension syntax elements in MV-HEVC draft 3, a parsing dependency from VPS occurs here. This needs further discussion. ]

| sps_extension2( ) { | Descriptor |
| --- | --- |
| if( !VpsDepthFlag[ nuh_layer_id ] ) { [Ed. (GT): Parsing dependency from VPS when VpsDepthFlag is derived from dimension_id.] |  |
| cp_precision | ue(v) |
| cp_in_slice_header_flag | u(1) |
| if( !cp_in_slice_header_flag ) { |  |
| for ( i = 0; i < ViewId[ nuh_layer_id ]; i++ ) { [Ed. (GT): Parsing dependency from VPS when ViewId is derived from dimension_id. .] |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

      - Picture parameter set RBSP syntax

The specifications in subclause G.7.3.2.3 apply.

      - Supplemental enhancement information RBSP syntax

The specifications in subclause G.7.3.2.4 apply.

      - Access unit delimiter RBSP syntax

The specifications in subclause G.7.3.2.5 apply.

      - End of sequence RBSP syntax

The specifications in subclause G.7.3.2.6 apply.

      - End of bitstream RBSP syntax

The specifications in subclause G.7.3.2.7 apply.

      - Filler data RBSP syntax

The specifications in subclause G.7.3.2.8 apply.

      - Slice layer RBSP syntax

The specifications in subclause G.7.3.2.9 apply.

      - RBSP slice trailing bits syntax

The specifications in subclause G.7.3.2.10 apply.

      - RBSP trailing bits syntax

The specifications in subclause G.7.3.2.11 apply.

      - Byte alignment syntax

The specifications in subclause G.7.3.2.12 apply.

    - Profile, tier and level syntax

The specifications in subclause G.7.3.3 apply.

    - Scaling list data syntax

The specifications in subclause G.7.3.4 apply.

    - Slice header syntax

[Ed. (GT): Slice header syntax and following syntax structures have not yet been updated to HEVC version 1.]

      - General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| else if ( layer_id ) |  |
| slice_ic_enable_flag [ Ed.(GT): Should be moved to extension.. ] | u(1) |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pic_slice_level_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| slice_header_extension |  |
| slice_header_extension2_flag | u(1) |
| if ( slice_header_extension2_flag ) { |  |
| slice_header_extension2_length | ue(v) |
| for( i = 0; i < slice_header_extension2_length; i++) |  |
| slice_header_extension2_data_byte[ i ] | u(8) |
| } |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

        - Slice header extension syntax

| slice_header_extension( ) { | Descriptor |
| --- | --- |
| if( cp_in_slice_header_flag ) { |  |
| for ( i = 0; i < ViewIdx; i++ ) { |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |

      - Short-term reference picture set syntax

The specifications in subclause 7.3.5.2 apply.

      - Reference picture list modification syntax

The specifications in subclause 7.3.5.3 apply.

      - Prediction weight table syntax

The specifications in subclause 7.3.5.4 apply.

    - Slice data syntax

The specifications in subclause 7.3.6 apply.

    - Coding tree unit syntax

The specifications in subclause 7.3.7 apply.

    - Coding quadtree syntax

| coding_quadtree( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples && log2CbSize > Log2MinCbSize && NumPCMBlock = = 0 && !predSplitCuFlag ) |  |
| split_cu_flag[ x0 ][ y0 ] | ae(v) |
| if( ( cu_qp_delta_enabled_flag && log2CbSize >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDelta = 0 |  |
| } |  |
| if( split_cu_flag[ x0 ][ y0 ]) { |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| coding_quadtree( x0, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_quadtree( x1, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x0, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x1, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| } else { |  |
| if( NumPCMBlock = = 0 ) |  |
| coding_unit( x0, y0, log2CbSize , ctDepth) |  |
| else |  |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } |  |
| } |  |

    - Coding unit syntax

      - General Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode = = MODE_INTRA && DepthFlag ) |  |
| sdc_flag[ x0 ][ y0 ] [Ed. (GT): In software there seems to be a dependency of position of sdc_flag and slice type.] | ae(v) |
| if( sdc_flag[ x0 ][ y0 ] ) { |  |
| sdc_pred_mode | ae(v) |
| if( sdc_pred_mode = = 1 ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( PredMode[ x0 ][ y0 ] ! = MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) && !predPartModeFlag) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| if ( vps_dmm_flag[ nuh_layer_id ] && log2CbSize <= Log2MaxDmmCbSize ) { |  |
| log2PbSize = log2CbSize − ( PartMode = = PART_NxN ? 1 : 0 ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| dmm_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if (dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| dmm_mode[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULL ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULLDELTA ) ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| wedge_predtex_tab_idx[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| dmm_delta_end_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + j ] ) |  |
| dmm_delta_end_abs_minus1[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + j ] ) |  |
| dmm_delta_end_sign_flag[ x0 ][ y0 ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| DmmDeltaFlag[ x0 + i ][ y0 + j ] = ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_WFULLDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_WPREDDIRDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_WPREDTEXDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_CPREDTEXDELTA ) ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0 + j ] ) |  |
| dmm_dc_1_abs[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_1_abs[ x0 + i ][ y0 + j ] != 0 ) |  |
| dmm_dc_1_sign_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] ) |  |
| dmm_dc_2_abs[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_2_abs[ x0 + i ][ y0 + j ] != 0 ) |  |
| dmm_dc_2_sign_flag[ x0 + i ][ y0 + j ] | ae(v) |
| } else { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else { |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| if( rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] = = 31 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) <= 5 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) >= 2 ) { |  |
| edge_intra_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if( edge_intra_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_start_left_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_start_position[ x0 + i ][ y0+ j ] | ae(v) |
| num_edge_codes_minus1[ x0 + i ][ y0+ j ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ x0 + i ][ y0 + j ][ k ] | ae(v) |
| edge_dc_flag[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_dc_1_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_1_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_1_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_dc_2_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_2_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_2_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag && !sdc_flag) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| if( sdc_flag ) { |  |
| sdcNumSegments = ( sdc_pred_mode = = 1 ) ? 2 : 1 |  |
| for ( i = 0; i < sdcNumSegments ; i++) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i | ae(v) |
| if( sdc_residual_flag [ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

[Ed. (GT): DMM syntax differs from syntax used in SW, since intra prediction syntax elements moved from PU to CU in HEVC text specification draft 8]

      - PCM sample syntax

The specifications in subclause 7.3.9.2 apply.

    - Prediction unit syntax

      - General prediction unit syntax

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_idxflag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_idxflag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

      - Motion vector difference coding syntax

The specifications in subclause G.7.3.10.2 apply.

    - Transform tree syntax

The specifications in subclause G.7.3.11 apply.

    - Transform unit syntax

The specifications in subclause G.7.3.12 apply.

    - Residual coding syntax

The specifications in subclause G.7.3.13 apply.

  - Semantics

    - NAL unit semantics

      - General NAL unit semantics

The specifications in subclause G.7.4.1.1  apply.

      - NAL unit header semantics

The specification in subclause G.7.4.1.2 apply with the following modifications and additions.

      - Encapsulation of an SODB within an RBSP (informative)

The specifications in subclause G.7.4.1.3 apply.

      - Order of NAL units and association to coded pictures, access units, and video sequences

The specifications in subclause G.7.4.1.4 apply.

    - Raw byte sequence payloads and RBSP trailing bits semantics

      - Video parameter set RBSP semantics

The specifications in subclause G.7.4.2.1 apply, with the following modifications and additions:

vps_extension2_flag equal to 0 specifies that no vps_extension2( ) syntax structure is present in the VPS RBSP syntax structure. vps_extension_flag equal to 1 specifies that the vps_extension2( ) syntax structure is present in the VPS RBSP syntax structure. When vps_max_layers_minus1 is greater than 0, vps_extension_flag2 shall be equal to 1.

vps_extension3_flag equal to 0 specifies that no vps_extension3_data_flag syntax elements are present in the VPS RBSP syntax structure. vps_extension2_flag shall be equal to 1 in bitstreams conforming to Annex H of this Recommendation | International Standard. The value of 1 for vps_extension3_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for vps_extension3_flag in a VPS NAL unit.

        - Video parameter set extension 1 semantics

The specifications in subclause G.7.4.2.1.1 apply, with the following modifications and additions:

[ Ed.(GT): After approval by JCT-3V following modifications might be applied to align 3D-HEVC with MV-HEVC Draft 3.

- Table F-1 is replaced by Table H-1.

Table H-1  – Mapping of ScalabiltyId to scalability dimensions

| scalability_mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | multiview | ViewId |
| 1 | depth | VpsDepthFlag |
| 2-15 | Reserved |  |

- The following is appended to the semantics of dimension_id:

for (i = 0; i <= vps_max_layers_minus1; i++) 
	VpsDepthFlag[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 1 ]

[Ed.(GT): Consider renaming of ViewId to VpsViewIdx such that the variable ViewIdx = VpsViewIdx[ nuh_layer_id ] can be used in decoding process of slice segment data. ]

].

        - Video parameter set extension 2 semantics

view_id[ layerId ] specifies the view identifier of the layer wither layer_id equal to layerId.

[Ed. (GT): This syntax element is different from ViewId[ layerId ] and can have arbitrary values. See comment in MV-HEVC Draft 3. Renaming/alignment is required. ]

iv_mv_pred_flag[ layerId ] indicates whether inter-view motion parameter prediction is used in the decoding process of the layer with layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 0 specifies that inter-view motion parameter prediction is not used for the layer with layer_id equal to layerId. iv_mv_pred_flag[ layerId ] equal to 1 specifies that inter-view motion parameter prediction may be used for the layer with layer_id equal to layerId.

When iv_mv_pred_flag[ layerId ] is not present, its value shall be inferred to be equal to 0.

iv_res_pred_flag[ layerId ] indicates whether inter-view residual prediction is used in the decoding process of the layer with layer_id equal to layerId. iv_res_pred_flag[ layerId ] equal to 0 specifies that inter-view residual prediction is not used for the layer with layer_id equal to layerId. iv_res_pred_flag[ layerId ] equal to 1 specifies that inter-view residual prediction may be used for the layer with layer_id equal to layerId. When iv_res_pred_flag[ layerId ] is not present, its value shall be inferred to be equal to 0.

vps_dmm_flag[ layerId ] equal to 1 specifies that depth map modelling modes may be used in the decoding process of the layer with layer_id equal to layerId. vps_dmm_flag[ layerId ] equal to 0 specifies that depth map modelling modes are not used in the decoding process of the layer with layer_id equal to layerId. When not present vps_dmm_flag[ layerId ] shall be inferred to be equal to 0.

lim_qt_pred_flag[ layerId ] equal to 1 specifies that prediction of a limited quadtree is used for depth view components for depth view components with layer_id equal to layerId.. lim_qt_pred_flag[ layerId ] equal to 0 specifies that prediction of a limited quadtree is not used for the depth view components with layer_id equal to layerId. When lim_qt_pred_flag[ layerId ] is not present, it shall be inferred to be equal to 0.

dlt_flag[ layerId ] equal to 1 specifies that depth lookup table is used and that residual values for simplified depth coded coding units are to be interpreted as indices of the depth lookup table for depth view components with layer_id equal to layerId. dlt_flag[ layerId ] equal to 0 specifies that depth lookup table is not used and residual values for simplified depth coded coding units are not to be interpreted as indices for depth view components with layer_id equal to layerId. When dlt_flat[ layerId ] is not present, it shall be inferred to be equal to 0.

num_depth_values_in_dlt[ layerId ] specifies the number of different depth values and the number of elements in the depth lookup table for depth view components of the current layer with layer_id equal to layerId.

dlt_depth_value[ layerId ][ j ] specifies the j-th entry in the depth lookup table for depth view components with layer_id equal to layerId.

[Ed. (GT):  In the HTM-6.0 depth tool related flags are signalled in the SPS.]

iv_mv_scaling_flag equal to 1 specifies that motion vectors used for inter-view prediction may be scaled based on view_id[ layerId ] values. iv_mv_scaling_flag equal to 0 specifies that motion vectors used for inter-view prediction are not scaled based on view_id[ layerId ] values.

      - Sequence parameter set RBSP semantics

The specifications in subclause G.7.4.2.2 apply, with the following modifications and additions:

sps_extension2_flag equal to 0 specifies that no sps_extension2( ) syntax structure is present in the SPS RBSP syntax structure. sps_extension2_flag equal to 1 specifies that the sps_extension2( ) syntax structure is present in the SPS RBSP syntax structure. When vps_max_layers_minus1 is greater than 0, sps_extension2_flag shall be equal to 1.

sps_extension3_flag equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. sps_extension3_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for sps_extension3_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all sps_extension_data_flag syntax elements that follow the value 1 for sps_extension3_flag in an SPS NAL unit.

        - Sequence parameter set extension 2 semantics

cp_precision specifies the precision of cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ]. The value of cp_precision shall be in the range of 0 to 5, inclusive.

cp_in_slice_header_flag equal to 0 specifies that the syntax elements of cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] are present in the sequence parameter set. A value of camera_parameter_in_slice_header_flag equal to 1 specifies that the cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] syntax elements are present in the slice segment header. When not present cp_in_slice_header_flag is inferred to be equal to 0.

cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] specify conversion parameters for converting a depth value to a disparity value.

      - Picture parameter set RBSP semantics

The specifications in subclause G.7.4.2.3 apply.

      - Supplemental enhancement information RBSP semantics

The specifications in subclause G.7.4.2.4 apply.

      - Access unit delimiter RBSP semantics

The specifications in subclause G.7.4.2.5 apply.

      - End of sequence RBSP semantics

The specifications in subclause G.7.4.2.6 apply.

      - End of bitstream RBSP semantics

The specifications in subclause G.7.4.2.7 apply.

      - Filler data RBSP semantics

The specifications in subclause G.7.4.2.8 apply.

      - Slice layer RBSP semantics

The specifications in subclause G.7.4.2.9 apply.

      - RBSP slice trailing bits semantics

The specifications in subclause G.7.4.2.10 apply.

      - RBSP trailing bits semantics

The specifications in subclause G.7.4.2.11 apply.

      - Byte alignment semantics

The specifications in subclause G.7.4.2.12 apply.

    - Profile, tier and level semantics

The specifications in subclause G.7.4.3 apply.

    - Operation point semantics

The specifications in subclause G.7.4.4 apply.

    - Slice header semantics

      - General slice header semantics

The specification in subclause G.7.4.5.1 apply with the following modifications and additions.

The variable DepthFlag is set equal to VpsDepthFlag[ nuh_layer_id ] and the variable ViewIdx is set equal to ViewId[ nuh_layer_id ].

five_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5. The maximum number of merging MVP candidates, MaxNumMergeCand is computed as

- MaxNumMergeCand = 5 − five_minus_max_num_merge_cand + iv_mv_pred_flag[ nuh_layer_id ]	(H-1)

The value of five_minus_max_num_merge_cand shall be limited such that MaxNumMergeCand is in the range of 0 to (5 + iv_mv_pred_flag[ nuh_layer_id ]), inclusive.

slice_ic_enable_flag equal to 1 specifies illumination compensation is enabled for the current slice. slice_ic_enable_flag equal to 0 specifies that illumination compensation is disabled for the current slice, When not present, slice_ic_enable_flag is inferred to be equal to 0.

slice_header_extension2_flag equal to 0 specifies that no slice header extension 2 syntax elements are present slice_header_extension2_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for slice_header_extension2_present_flag is reserved for future use by ITU-T | ISO/IEC.

slice_header_extension2_length specifies the length of the slice segment header extension 2 data in bytes, not including the bits used for signalling slice_header_extension2_length itself. The value of slice_header_extension_length shall be in the range of 0 to 256, inclusive.

slice_header_extension2_data_byte may have any value. Decoders shall ignore the value of slice_header_extension_data_byte. Its value does not affect decoder conformance to profiles specified in this version of this Specification.

        - Slice header extension semantics

cp_scale[ i ], cp_off[ i ], cp_inv_scale_plus_scale[ i ], and cp_inv_off_plus_off[ i ] specify conversion parameters for converting a depth value to a disparity value.

The array DepthToDisparityB[ i ][ d ] specifying the disparity between the current view and the view with ViewIdx equal i corresponding to the depth value d in the view with ViewIdx equal to i and the array DepthToDisparityF[ i ][ d ] specifying the disparity between the view with ViewIdx equal i and the current view corresponding to the depth value d in the current view is derived as specified in the following:

- The variable log2Div is set equal to BitDepthY − 1 + cp_precision.

- For d in range of 0 to ( ( 1 << BitDepthY )− 1), inclusive, the following applies:

- For i in the range of 0 to ViewIdx − 1, the following applies:

- offset = ( cp_off[ i ] << BitDepthY ) + ( ( 1 << log2Div ) >> 1 )	(H-2)

- scale = cp_scale[ i ]		(H-3)

- DepthToDisparityB[ i ][ d ] = ( scale * d + offset ) >> log2Div	(H-4)

- invOffset = ( ( cp_inv_off_plus_off[ i ] − cp_off[ i ] ) << BitDepthY ) + ( ( 1 << log2Div ) >> 1 )	(H-5)

- invScale = ( cp_inv_scale_plus_scale[ i ] − cp_scale[ i ] )	(H-6)

- DepthToDisparityF[ i ][ d ] = ( invScale * d + invOffset ) >> log2Div 	(H-7)

      - Short-term reference picture set semantics

The specifications in subclause 7.4.5.2 apply.

      - Reference picture list modification semantics

The specifications in subclause 7.4.5.3 apply.

      - Weighted prediction parameters semantics

The specifications in subclause 7.4.5.4 apply.

    - Slice data semantics

The specifications in subclause 7.4.6 apply.

    - Coding tree unit semantics

The specifications in subclause 7.4.7 apply, with the following modifications and additions.

The array TextureCtDepth is set equal to the array CtDepth of the view component with view order index equal to ViewIdx of the current slice and DepthFlag equal to 0.

The array TexturePartMode is set equal to the array CtPartMode of the view component with view order index equal to ViewIdx of the current slice and DepthFlag equal to 0.

    - Coding quadtree semantics

The specifications in subclause 7.4.8 apply.

[Ed. (GT): Numbering of syntax and semantics in JCTVC-J1003_d7 do not match from subclause 7.4.8 on. However, the "correct" numbering of syntax is used here assuming that JCTVC-J1003_d7 would match.]

The variable predSplitCuFlag specifying whether the split_cu_flag is predicted by inter-component prediction is derived as specified in the following.

- If slice_type is not equal to I, RapPicFlag is equal to 0, and lim_qt_pred_flag[ nuh_layer_id ] is equal to 1, predSplitCuFlag is set equal to ( TextureCtDepth[ x0 ][ y0 ] <= ctDepth )

- Otherwise (slice_type is equal to I or RapPicFlag is equal to 1 or lim_qt_pred_flag[ nuh_layer_id ] is equal to 0), predSplitCuFlag is set equal to 0.

split_cu_flag[ x0 ][ y0 ] specifies whether a coding unit is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When split_cu_flag[ x0 ][ y0 ] is not present the following applies.

- If log2CbSize is greater than Log2MinCbSizeY and predSplitCuFlag is equal to 0 the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- Otherwise (log2CbSize is equal to Log2MinCbSizeY or predSplitCuFlag is equal to 1), the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 0.

    - Coding unit semantics

      - General coding unit semantics

The specification in subclause F.7.4.9.1 apply with the following modifications and additions.

- If slice_type is not equal to I, RapPicFlag is equal to 0 and lim_qt_pred_flag[ nuh_layer_id ] is equal to 1, the variable predPartModeFlag specifying whether part_mode is predicted by inter-component prediction is derived as follows.

predPartModeFlag = ( TextureCtDepth[ x0 ][ y0 ] = = ctDepth ) && ( TexturePartMode[ x0 ][ y0 ] ! = PART_NxN )	(H-8)

- Otherwise (slice_type is equal to I or RapPicFlag is equal to 1 or lim_qt_pred_flag[ nuh_layer_id ] is equal to 0), predPartModeFlag is set equal to 0.

pred_mode_flag equal to 0 specifies that the current coding unit is coded in inter prediction mode. pred_mode_flag equal to 1 specifies that the current coding unit is coded in intra prediction mode. The variable PredMode[ x ][ y ] is derived as follows for x = x0..x0 + nCbS − 1 and y = y0..y0 + nCbS − 1.

- If pred_mode_flag is equal to 0,

- PredMode[ x ][ y ] is set to MODE_INTER.

- Otherwise (pred_mode_flag is equal to 1),

- PredMode[ x ][ y ] is set to MODE_INTRA.

When pred_mode_flag is not present, the variable PredMode[ x ][ y ] is derived as follows for x = x0..x0 + nCbS − 1 and y = y0..y0 + nCbS − 1.

- If slice_type is equal to I,

- PredMode[ x ][ y ] is inferred to be equal to MODE_INTRA

- Otherwise (slice_type is equal to P or B),

- , if skip_flag[ x0 ][ y0 ] is equal to 1,

    - PredMode[ x ][ y ] is inferred to be equal to MODE_SKIP

part_mode specifies partitioning mode of the current coding unit. The semantics of part_mode depend on PredMode[ x0 ][ y0 ]. The variables PartMode and IntraSplitFlag are derived from the value of part_mode as defined in Table 7-10.

The value of part_mode is restricted as follows.

- If PredMode[ x0 ][ y0 ] is equal to MODE_INTRA, part_mode shall be equal to 0 or 1.

- Otherwise (PredMode[ x0 ][ y0 ] is equal to MODE_INTER), the following applies

- If log2CbSize is greater than Log2MinCbSize and amp_enabled_flag is equal to 1, part_mode shall be in the range of 0 to 2, inclusive and in the range of 4 to 7, inclusive.

- Otherwise, if log2CbSize is greater than Log2MinCbSize and amp_enabled_flag is equal to 0, part_mode shall be in the range of 0 to 2, inclusive.

- Otherwise, if log2CbSize is equal to 3, the value of part_mode shall be in the range of 0 to 2, inclusive.

- Otherwise (log2CbSize is greater than 3), the value of part_mode shall be in the range of 0 to 3, inclusive.

When part_mode is not present, the variables PartMode and IntraSplitFlag are derived as follows.

- PartMode is inferred to be equal to PART_2Nx2N,

- IntraSplitFlag is inferred to be equal to 0.

The variable Log2MaxDmmCbSize is set equal to 5.

dmm_flag[ x0 + i][ y0 + j ] equal to 0 specifies that depth map model modes are not used. dmm_flag[ x0 + i][ y0 + j ] equal to 1 specifies that depth map model modes are used that the dmm_mode syntax element is present. When the dmm_flag[ x0 + i][ y0 + j ] is not present, its value should be inferred to be equal to 0.

dmm_mode[ x0 + i][ y0 + j ] shall be one of the values shown in Table H-2. dmm_mode is used to derive the intra prediction mode when the dmm_flag[ x0 + i][ y0 + j ] is equal to 1.

Table H-2 – Interpretation of dmm_mode

| dmm_mode | Method identifier |
| --- | --- |
| 0 | MODE_DMM_WFULL |
| 1 | MODE_DMM_WFULLDELTA |
| 2 | MODE_DMM_WPREDTEX |
| 3 | MODE_DMM_WPREDTEXDELTA |
| 4 | MODE_DMM_CPREDTEX |
| 5 | MODE_DMM_CPREDTEXDELTA |
| 6 | MODE_DMM_WPREDDIR |
| 7 | MODE_DMM_WPREDDIRDELTA |

wedge_full_tab_idx[ x0 + i][ y0 + j ] specifies the index of the wedgelet pattern in the corresponding pattern list when dmm_mode[ x0 + i][ y0 + j ] is equal to MODE_DMM_WFULL or equal to MODE_DMM_WFULLDELTA.

wedge_predtex_tab_idx[ x0 + i][ y0 + j ] specifies the index of the wedgelet pattern when dmm_mode[ x0 + i][ y0 + j ] is equal to MODE_DMM_WPREDTEX or equal to MODE_DMM_WPREDTEXDELTA.

dmm_delta_end_flag[ x0 + i][ y0 + j ] equal to 0 specifies that dmm_delta_end_abs_minus1[ x0 + i][ y0 + j ] and dmm_delta_end_sign[ x0 + i][ y0 + j ] syntax elements are not present. dmm_delta_end_flag equal to 1 specifies that dmm_delta_end_abs_minus1[ x0 + i][ y0 + j ] and dmm_delta_end_sign[ x0 + i][ y0 + j ] syntax elements are present.

dmm_delta_end_abs_minus1[ x0 + i][ y0 + j ] and dmm_delta_end_sign_flag[ x0 + i][ y0 + j ] are used to derive DmmDeltaEnd[ x0 + i][ y0 + j ] as follows:

- DmmDeltaEnd[ x0 + i][ y0 + j ] = 
( 1 − 2 *dmm_delta_end_sign_flag[ x0 + i][ y0 + j ] ) *	( dmm_delta_end_abs_minus1[ x0 + i][ y0 + j ] + 1)	(H-9)

dmm_dc_0_abs[ x0 + i][ y0 + j ], dmm_dc_0_sign_flag[ x0 + i][ y0 + j ], dmm_dc_1_abs[ x0 + i][ y0 + j ], dmm_dc_1_sign_flag[ x0 + i][ y0 + j ] are used to derive DcOffsetP0[ x0 + i][ y0 + j ] and DcOffsetP1[ x0 + i][ y0 + j ] values as follows:

- DcOffsetP0[ x0 + i][ y0 + j ]  = 
( 1 − 2 *dmm_dc_0_sign_flag[ x0 + i][ y0 + j ] ) * dmm_dc_0_abs[ x0 + i][ y0 + j ]	(H-10)
DcOffsetP1[ x0 + i][ y0 + j ]  = 
( 1 − 2 *dmm_dc_1_sign_flag[ x0 + i][ y0 + j ] ) * dmm_dc_1_abs[ x0 + i][ y0 + j ]	(H-11)

edge_intra_flag[ x0 + i][ y0 + j ] equal to 0 specifies that region boundary chain coding is not used. edge_intra_flag[ x0 + i][ y0 + j ] equal to 1 specifies that region boundary chain coding is used.

edge_start_left_flag[ x0 + i][ y0 + j ] equal to 0 specifies that the start point of region boundary chain coding is located on the top row boundary of the current block. edge_start_left_flag[ x0 + i][ y0 + j ] equal to 1 specifies that the start point of region boundary chain coding is located on the left column boundary of the current block.

edge_start_position[ x0 + i][ y0 + j ] specifies the column position of the start point of region boundary chain coding when edge_start_left_flag[ x0 + i][ y0 + j ] is equal to 0 and specifies the row position of the start point of region boundary chain coding when edge_start_left_flag[ x0 + i][ y0 + j ] is equal to 1.

num_edge_codes_minus1[ x0 + i][ y0 + j ] +1 specifies the number of edges within the current block.

edge_code[ x0 + i][ y0 + j ][ k ] shall be one of the values shown in Table H-3. edge_code[ x0 + i][ y0 + j ][ k ] is used to derive the edge direction of the k-th edge when edge_intra_flag[ x0 + i][ y0 + j ] is equal to 1.

Table H-3 – Interpretation of edge_code[ x0 + i][ y0 + j ][ k ]

| edge_code[ x0 + i][ y0 + j ][ k ] | edge direction |
| --- | --- |
| 0 | 0° |
| 1 | 45° |
| 2 | −45° |
| 3 | 90° |
| 4 | −90° |
| 5 | 135° |
| 6 | −135° |

edge_dc_flag[ x0 + i][ y0 + j ] equal to 0 specifies that de-quantized offset is not used. edge_dc_flag[ x0 + i][ y0 + j ] equal to 1 specifies that de-quantized offset is used.

edge_dc_0_abs[ x0 + i][ y0 + j ], edge_dc_0_sign_flag[ x0 + i][ y0 + j ], edge_dc_1_abs[ x0 + i][ y0 + j ], edge_dc_1_sign_flag[ x0 + i][ y0 + j ] are used to derive DcOffsetP0[ x0 + i][ y0 + j ] and DcOffsetP1[ x0 + i][ y0 + j ] values as follows.

- DcOffsetP0[ x0 + i][ y0 + j ] = 
( 1 − 2 *edge_dc_0_sign_flag[ x0 + i][ y0 + j ]) * edge_dc_0_abs[ x0 + i][ y0 + j ]	(H-12)
DcOffsetP1[ x0 + i][ y0 + j ] = 
( 1 − 2 *edge_dc_1_sign_flag[ x0 + i][ y0 + j ]) * edge_dc_1_abs[ x0 + i][ y0 + j ]	(H-13)

The variables x1 and y1 specifying luma locations relative to the top left sample of the current picture are derived as specified in the following.

- x1 = x0 + 		
( ( PartMode = = PART_NxN | | PartMode = = PART_Nx2N) ? ( nCbs / 2) : 0 ) + 	
( ( PartMode = = PART_nLx2N ) ? ( nCbs / 4) : 0 ) + 		
( ( PartMode = = PART_nRx2N ) ? ( 3* nCbs / 4) : 0 ) 		(H-14)

- y1 = y0 + 		
( ( PartMode = = PART_NxN | | PartMode = = PART_2NxN) ? ( nCbs / 2)  : 0 ) + 	
( ( PartMode = = PART_2NxnU ) ? ( nCbs / 4) : 0 ) + 		
( ( PartMode = = PART_2NxnD ) ? ( 3* nCbs / 4) : 0 ) 		(H-15)

With X being replaced by 0 and 1, the variable refViewIdxLX[ x ][ y ] is set equal to the view order index of RefPicListLX[ RefIdxLX[ x ][ y ] ]..

The variable anyTempRefPicFlag specifies whether one or more prediction units of the current coding unit utilize a temporal reference picture. The variable anyTempRefPicFlag is initially set equal to 0. When PredMode[ x0 ][ y0 ] is not equal to MODE_INTRA the following applies for X being replaced by 0 and 1, and Y being equal to 1−X.

- anyTempRefPicFlag = anyTempRefPicFlag | |  	
(inter_pred_idc[ x0 ][ y0 ] ! = Pred_LY && refViewIdxLX[ x0 ][ y0 ] = = ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y1 ] ! = Pred_LY && refViewIdxLX[ x0 ][ y1 ] = = ViewIdx)  | |	
(inter_pred_idc[ x1 ][ y0 ] ! = Pred_LY && refViewIdxLX[ x1 ][ y0 ] = = ViewIdx)  | |	 
(inter_pred_idc[ x1 ][ y1 ] ! = Pred_LY && refViewIdxLX[ x1 ][ y1 ] = = ViewIdx) 	  (H-16)

The variable anyIvRefPicFlag specifies whether one or more prediction units of the current coding unit utilize an inter-view reference picture. The variable anyIvRefPicFlag is initially set equal to 0. When PredMode[ x0 ][ y0 ] is not equal to MODE_INTRA the following applies for X being replaced by 0 and 1, and Y being equal to 1−X.

- If DepthFlag is equal to 0, the following applies:

- anyIvRefPicFlag = anyIvRefPicFlag | |  	
(inter_pred_idc[ x0 ][ y0 ] ! = Pred_LY && refViewIdxLX[ x0 ][ y0 ] ! = ViewIdx)  | |	 
(inter_pred_idc[ x0 ][ y1 ] ! = Pred_LY && refViewIdxLX[ x0 ][ y1 ] ! = ViewIdx)  | |	
(inter_pred_idc[ x1 ][ y0 ] ! = Pred_LY && refViewIdxLX[ x1 ][ y0 ] ! = ViewIdx)  | |	 
(inter_pred_idc[ x1 ][ y1 ] ! = Pred_LY && refViewIdxLX[ x1 ][ y1 ] ! = ViewIdx) 	  (H-17)

- Otherwise ( DepthFlag is not equal to 0 ), the following applies:

for ( m = 0; m < nCbs; m += ( 1  <<  Log2MinTrafoSize ) ) 
	for ( n = 0; n < nCbs; n += ( 1  <<  Log2MinTrafoSize ) ) 
		anyIvRefPicFlag = anyIvRefPicFlag | |  
		(inter_pred_idc[ x0 + m ][ y0+n ] ! = Pred_LY && refViewIdxLX[ x0+ m ][ y0+ n ] ! = ViewIdx)

ic_flag equal to 1 specifies illumination compensation is used for the current coding unit. ic_flag equal to 0 specifies illumination compensation is not used for the current coding unit. When not present, ic_flag is inferred to be equal to 0.

The variable icEnableFlag specifying whether ic_flag is present in the bitstream is derived as

- icEnableFlag = slice_ic_enable_flag && anyIvRefPicFlag	(H-18)

sdc_flag equal to 0 specifies that simplified depth coding is not used. sdc_flag equal to 1 specifies that simplified depth coding is used. When sdc_flag is not present, it is inferred to be equal to 0.

sdc_pred_mode shall be one of the values shown in Table H-4. sdc_pred_mode specifies the intra prediction mode used for simplified depth coding.

Table H-4 – Interpretation of sdc_pred_mode

| sdc_pred_mode | Associated Intra Prediction Mode |
| --- | --- |
| 0 | Intra_DC |
| 1 | Intra_DepthPartition( 35 ) |
| 2 | Intra_Planar |

sdc_residual_flag[ x0 ][ y0 ][ i ] equal to 0 specifies that the residual is zero for segment i. sdc_residual_flag equal to 1 specifies that the residual is non-zero and the sdc_residual_sign_flag and sdc_residual_abs[ i ] syntax elements are present for segment i.

sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] and sdc_residual_sign_flag[ x0 ][ y0 ][ i ] are used to derive SdcResidual[ x0 ][ y0 ][ i ] for segment i as follows:

- SdcResidual[ x0 ][ y0 ][ i ] = 
( 1 − 2 * sdc_residual_sign_flag[ x0 ][ y0 ][ i ] ) * ( sdc_residual_mag_minus1[ x0 ][ y0 ][ i ] + 1) 	(H-19)

When DepthFlag is equal to 0, for use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = x0..x0 + (1 << log2cbSize) − 1, y = y0..y0 + (1 << log2cbSize) − 1:

- CtDepth[ x ][ y ] = ctDepth		(H-20)
CtPartMode[ x ][ y ] = PartMode		(H-21)
CbSize[ x ][ y ] = ( 1 << log2cbSize )		(H-22)
CbPosX[ x ][ y ] = x0		(H-23)
CbPosY[ x ][ y ] = y0		(H-24)

      - PCM sample semantics

The specifications in subclause 7.4.9.2 apply.

    - Prediction unit semantics

      - General prediction unit semantics

The specifications in subclause 7.4.10.1 apply, apply with the following modifications and additions

mvp_l0_idx[ x0 ][ y0 ] specifies the motion vector predictor index of list 0 where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When mvp_l0_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

mvp_l1_idx[ x0 ][ y0 ] has the same semantics as mvp_l0_idx, with l0 and list 0 replaced by l1 and list 1, respectively.

merge_idx[ x0 ][ y0 ] specifies the merging candidate index of the merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When merge_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

      - Motion vector difference coding semantics

The specifications in subclause 7.4.10.2 apply.

    - Transform tree semantics

The specifications in subclause 7.4.11 apply.

    - Transform coefficient semantics

The specifications in subclause 7.4.12 apply.

    - Residual coding semantics

The specifications in subclause 7.4.13 apply.

- Decoding process

  - General decoding process

The specifications in subclause F.8.1 apply with the following modification.

- All invocations of the process specified in subclause G.8.1 are replaced with invocations of the process specified in subclause H.13.1.1.

    - Decoding process for a coded picture with nuh_layer_id greater than 0

The decoding process operates as follows for the current picture CurrPic:

  - The decoding of NAL units is specified in subclause 8.2.

  - The following steps specify the decoding processes using syntax elements in the slice segment layer and above:

    - Prior to decoding the first slice of the current picture, subclause G.8.1.1 is invoked.

    - At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause G.8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

    - When iv_mv_pred_flag[ nuh_layer_id ] is equal to 1 or iv_res_pred_flag[ nuh_layer_id ] is equal to 1, the decoding process for candidate picture list for disparity vector derivation in subclause H.13.3.7 is invoked at the beginning of the decoding process for each P or B slice [Ed. (GT): VSP should be added here as condition.]

    - At the beginning of the decoding process for each P or B slice, the derivation process for the alternative target reference index for TMVP in merge mode as specified in subclause H.13.3.7 is invoked.

    - When dlt_flag[ nuh_layer_id ] is equal to 1, the decoding process for the depth lookup table in subclause H.13.3.6 is invoked at the beginning of the decoding process of first slice.

    - After the decoding of the last slice of the current picture, the marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause G.8.1.2 is invoked.

  - The following ordered steps specify the decoding processes using syntax elements in the coding tree unit layer and above.

    - The processes in subclauses H.13.4, H.13.5, H.13.6, and H.13.7 are invoked for the decoding processes using syntax elements in the coding tree unit layer and above.

  - NAL unit decoding process

( Void )

  - Slice decoding process

( Void ).

    - Decoding process for picture order count

( Void )

    - Decoding process for reference picture set

( Void )

    - Decoding process for generating unavailable reference pictures

( Void )

    - Decoding process for reference picture lists construction

( Void )

    - Derivation process for the candidate picture list for disparity vector derivation

[Ed. (GT): This algorithm is different from the algorithm in software. However, it seems to be equivalent. ]

The variable NumDdvCandPics is set equal to 0 and the candidate picture list DdvCandPicList with a number of NumDdvCandPics elements is constructed as follows.

When slice_temporal_mvp_enable_flag is equal to 1 the following ordered steps apply:

- DdvCandPicList[ 0 ] is set equal to RefPicListX[ collocated_ref_idx ], with X equal to ( 1 – collocated_from_l0_flag ), and NumDdvCandPics  is set equal to 1.

- The variable lowestTemporalIDRefs is set equal to 7.

- NumDdvCandPics, DdvCandPicList[ 1 ] and lowestTemporalIDRefs are derived as specified in the following.

for ( dir = 0; dir < 2 ; dir++) {
	X = dir ? collocated_from_l0_flag : (1- collocated_from_l0_flag)
	for ( i =0 ; i <= num_ref_idx_lX_default_active_minus1; i++) {
		if (	 ViewIdx of the current view component is equal to ViewIdx of RefPicListX[ i ] 
			&& (X = = collocated_from_l0_flag || i != collocated_ref_idx ) 
			&& ( NumDdvCandPics ! = 2)) {
			if (RefPicListX[ i ] is a random access view component) 	{
				DdvCandPicsList[ 1 ] = RefPicListX[ i ]
				NumDdvCandPics =2
			}
			else if ( lowestTemporalIDRefs > TemporalID of RefPicListX[ i ])
				lowestTemporalIDRefs = TemporalID of RefPicListX[ i ]
		}
	}
}

- When NumDdvCandPics is equal to 1, the following applies.

pocDistance =255
for ( dir = 0; dir < 2 ; dir++) {
	X = dir ? collocated_from_l0_flag : (1 – collocated_from_l0_flag)
	for ( i =0 ; i <= num_ref_idx_lX_default_active_minus1; i++) {
		if (	ViewIdx of the current view component is equal to ViewIdx of RefPicListX[ i ]
			&& (X = = collocated_from_l0_flag || i != collocated_ref_idx )
			&& TemporalID of RefPicListX[ i ] = = lowestTemporalIDRefs 
			&& (Abs( PicOrderCntVal – PicOrderCnt( RefPicListX[ i ] ) ) < pocDistance) ){
			pocDistance = Abs( PicOrderCntVal – PicOrderCnt( RefPicListX[ i ] )
			Z = X
			idx = i
		}
 	}
}
if (pocDistance < 255) {
	DdvCandPicsList[ 1 ] = RefPicListZ[ idx ]
	NumDdvCandPics =2
}

    - Decoding process for a depth lookup table

This process is only invoked when dlt_flag[ nuh_layer_id ] is equal to 1.

The list elements Idx2DepthValue[ i ] specifying the depth value of the i-th index in the lookup table with i ranging from 0 to  num_depth_values_in_dlt[ nuh_layer_id ] – 1, inclusive is derived as follows.

- For i = 0..num_depth_values_in_dlt[ nuh_layer_id ] – 1 the elements in Idx2DepthValue are derived as follows.

- Idx2DepthValue[ i ] is set equal to dlt_depth_value[ nuh_layer_id ][ i ]

The list elements DepthValue2Idx[ d ] specifying the index of depth values d in the lookup table with d ranging from 0 to BitDepthY – 1, inclusive are derived as specified in the following.

for (d = 0; d < BitDepthY; d++) {
		idxLower = 0 
		for ( iL = 1, foundFlag = 0; iL < num_depth_values_in_dlt[ nuh_layer_id ] && !foundFlag; iL++ ) 
			if ( Idx2DepthValue[ iL ] > d ) {
				idxLower = iL – 1
				foundFlag = 1
			}
		idxUpper = num_depth_values_in_dlt[ nuh_layer_id ] – 1
		for ( iU = num_depth_values_in_dlt[ nuh_layer_id ] – 2, foundFlag = 0; iU >= 0 && !foundFlag; iU++ ) 
			if ( Idx2DepthValue[ iU ] < d ) {
				idxUpper = iU + 1
				foundFlag = 1
			}
		if ( Abs( d – Idx2DepthValue[ idxLower ] ) < Abs ( d – Idx2DepthValue[ idxUpper ] ) ) 
			DepthValue2Idx[ d ] = idxLower
		else
			DepthValue2Idx[ d ] = idxUpper

    - Derivation process for the alternative target reference index for TMVP in merge mode

This process is invoked when the current slice is a P or B slice.

The variables AltRefIdxL0 and AltRefIdxL1 are set equal to –1 and the following applies for X in the range of 0 to 1, inclusive:

- When X is equal to 0 or the current slice is a B slice the following applies:

zeroIdxLtFlag = RefPicListX[ 0 ] is a short-term reference picture ? 0 : 1
for( i = 1; i <= num_ref_idx_lX_active_minus1 && AltRefIdxLX = = –1; i++)
	if ( ( zeroIdxLtFlag && RefPicListX[ i ] is a short-term reference picture) | |
		( !zeroIdxLtFlag && RefPicListX[ i ] is a long-term reference picture) ) 
		AltRefIdxLX = i

  - Decoding process for coding units coded in intra prediction mode

    - General decoding process for coding units coded in intra prediction mode

The specifications in subclause 8.4.1 apply with the following modification:

–	All invocations of the process specified in subclause 8.4.2 are replaced with invocations of the process specified in subclause H.13.4.2

–	All invocations of the process specified in subclause 8.4.4.1 are replaced with invocations of the process specified in subclause H.13.4.4

    - Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PbSize specifying the size of the current luma prediction block.

Table H-5 specifies the value for the intra prediction mode and the associated names.

Table H-5 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_DC |
| 2..34 | Intra_Angular |
| 35...42 | Intra_DepthPartition (used only for depth) |
| Otherwise (43, 44) | Intra_Chain (used only for depth) |

[Ed. (GT): Since the Intra_FromLuma mode has been removed in the HEVC text spec used as base for this document, the dmm mode and intra chain numbers are here decremented by 1 compared to SW.]

IntraPredMode[ xB ][ yB ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1.

- If sdc_flag[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is derived as follows.

- If sdc_pred_mode[ xB ][ yB ] is equal to 0, IntraPredMode[ xB ][ yB ] is set equal to 1.

- Otherwise, if sdc_pred_mode[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is set equal to 35.

- Otherwise, if sdc_pred_mode [ xB ][ yB ] is equal to 2, IntraPredMode[ xB ][ yB ] is set equal to 0.

- Otherwise, if dmm_flag[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is derived as follows:

- IntraPredMode[ xB ][ yB ] = dmm_mode[ xB ][ yB ] + 35	(H-25)

- Otherwise (dmm_flag[ xB ][ yB ] is equal to 0 and sdc_flag[ xB ][ yB ] is equal to 0), IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

  - The neighbouring locations ( xBA, yBA ) and ( xBB, yBB ) are set equal to ( xB−1, yB ) and ( xB, yB−1 ), respectively.

  - For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

    - The availability derivation process for a block in z-scan order as specified in subclause 6.4.1 is invoked with the location ( xCurr, yCurr ) set equal to ( xB, yB ) and the neighbouring location ( xN, yN ) set equal to ( xBN, yBN ) as the input and the output is assigned to availableN.

    - The candidate intra prediction mode candIntraPredModeN is derived as follows.

      - If availableN is equal to FALSE, candIntraPredModeN is set equal to Intra_DC.

      - Otherwise, if PredMode[ xBN ][ yBN ] is not equal to MODE_INTRA, candIntraPredModeN is set equal to Intra_DC,

      - Otherwise, if N is equal to B and yB−1 is less than (( yB >> Log2CtbSizeY ) << Log2CtbSizeY), intraPredModeB is set equal to Intra_DC.

      - Otherwise, candIntraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ].

  - The candModeList[ x ] with x=0..2 is derived as follows:

    - If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

      - If candIntraPredModeA is less than 2 (either Intra_Planar or Intra_DC), candModeList[ x ] with x=0..2 is derived as:

          - candModeList[0] = Intra_Planar		(H-26)
candModeList[1] = Intra_DC		(H-27)
candModeList[2] = Intra_Angular (26)	(H-28)

      - Otherwise, candModeList[ x ] with x=0..2 is derived as:

          - candModeList[0] = candIntraPredModeA	(H-29)
candModeList[1] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(H-30)
candModeList[2] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(H-31)

    - Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

      - candModeList[0] and candModeList[1] are derived as follows:

          - candModeList[0] = candIntraPredModeA	(H-32)
candModeList[1] = candIntraPredModeB	(H-33)

      - If none of candModeList[0] and candModeList[1] is equal to Intra_Planar, candModeList[2] is set equal to Intra_Planar,

      - Otherwise, if none of candModeList[0] and candModeList[1] is equal to Intra_DC, candModeList[2] is set equal to Intra_DC,

      - Otherwise, candModeList[2] is set equal to Intra_Angular (26).

  - IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

    - If prev_intra_luma_pred_flag[ xB ][ yB ] is equal to 1, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[ mpm_idx ].

    - Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

- The array candModeList[x], x=0..2 is modified as the following ordered steps:

  - When candModeList[0] is greater than candModeList[1], both values are swapped as follows.

( candModeList[0], candModeList[1] ) = Swap( candModeList[0], candModeList[1] )	(H-34)

  - When candModeList[0] is greater than candModeList[2], both values are swapped as follows.

( candModeList[0], candModeList[2] ) = Swap( candModeList[0], candModeList[2] )	(H-35)

  - When candModeList[1] is greater than candModeList[2], both values are swapped as follows.

( candModeList[1], candModeList[2] ) = Swap( candModeList[1], candModeList[2] )	(H-36)

- IntraPredMode[xB][yB] is derived as the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_luma_pred_mode[ xB ][ yB ]

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is greater than or equal to candModeList[ 2 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

    - Derivation process for chroma intra prediction mode

The specifications in subclause 8.4.3 apply.

    - Decoding process for intra blocks

      - General decoding process for intra blocks

Inputs to this process are:

- a sample location ( xB0, yB0 ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable log2TrafoSize specifying the size of the current transform block,

- a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

- a variable intraPredMode specifying the intra prediction mode,

- a variable cIdx specifying the chroma component of the current block.

Output of this process is:

- a modified reconstructed picture before deblocking filtering.

The variable splitFlag is derived as follows:

- If cIdx is equal to 0, splitFlag is set equal to split_transform_flag[ xB0 ][ yB0 ][ trafoDepth ].

- Otherwise, if all of the following conditions are true, splitFlag is set equal to 1.

- cIdx is greater than 0

- split_transform_flag[ xB0 << 1 ][ yB0 << 1 ][ trafoDepth ] is equal to 1

- log2TrafoSize is greater than 2

- Otherwise, splitFlag is set equal to 0.

Depending splitFlag the following applies:

- If splitFlag is equal to 1, the following ordered steps apply:

  - The variables xB1 and yB1 are derived as follows.

    - The variable xB1 is set equal to xB0 + ( ( 1 << log2TrafoSize ) >> 1 ).

    - The variable yB1 is set equal to yB0 + ( ( 1 << log2TrafoSize ) >> 1 ).

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xB0, yB0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode intraPredMode, and the variable cIdx as the inputs and the output is a modified reconstructed picture before deblocking filtering.

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xB1, yB0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode intraPredMode, and the variable cIdx as the inputs and the output is a modified reconstructed picture before deblocking filtering.

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xB0, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode intraPredMode, and the variable cIdx as the inputs and the output is a modified reconstructed picture before deblocking filtering.

  - The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xB1, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode intraPredMode, and the variable cIdx as the inputs and the output is a modified reconstructed picture before deblocking filtering.

- Otherwise, ( splitFlag is equal to 0 ), the following ordered steps apply:

  - The variable nT is set equal to 1 << log2TrafoSize

  - The general intra sample prediction process as specified in subclause G.8.4.4.2.1 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, the transform block size nT and the variable cIdx as the inputs and the output is a (nT)x(nT) array predSamples.

  - Depending on sdc_flag, the following applies:

    - If the sdc_flag is equal to 0, following applies:

      - The scaling and transformation process as specified in subclause 8.6.2 is invoked with the location ( xB0, yB0 ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nT as the inputs and the output is a (nT)x(nT) array resSamples.

      - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the transform block location ( xB0, yB0 ), the transform block size nT, the variable cIdx, the (nT)x(nT) array predSamples, and the (nT)x(nT) array resSamples as the inputs.

    - Otherwise ( sdc_flag is equal to 1) the following ordered steps apply:

      - The depth value reconstruction process as specified in subclause H.13.4.4.3 is invoked with the location ( xB0, yB0 ), the transform size trafoSize set equal to nT, the (nT)x(nT) array predSamples, and the intra prediction mode intraPredMode, as the inputs and the output is a (nT)x(nT) array resSamples.

  - For x in the range of 0 to nT − 1 and y in the range of 0 to nT − 1, the following applies:

    - When cIdx is equal to 0, ResSamplesL[ xB0 + x ][ yB0 + y ] is set equal to 0.

    - When cIdx is equal to 1, ResSamplesCb[ xB0 + x ][ yB0 + y ] is set equal to 0.

    - When cIdx is equal to 2, ResSamplesCr[ xB0 + x ][ yB0 + y ] is set equal to 0.

      - Intra sample prediction

        - General intra sample prediction

The specification in subclause 8.4.4.2.1 with the following paragraphs added to the end of subclause apply:

- Otherwise, if intraPredMode is equal to Intra_DepthPartition(35,36), the corresponding intra prediction mode specified in subclause H.13.4.4.2.7 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, the sample array p and the transform block size nT as the inputs and the output are the predicted sample array predSamples.

- Otherwise, if intraPredMode is equal to Intra_DepthPartition(37,38), the corresponding intra prediction mode specified in subclause  is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, the sample array p and the transform block size nT as the inputs and the output are the predicted sample array predSamples.

- Otherwise, if intraPredMode is equal to Intra_DepthPartition(39,40), the corresponding intra prediction mode specified in subclause H.13.4.4.2.9 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, with the sample array p and the transform block size nT as the inputs and the output are the predicted sample array predSamples.

- Otherwise, if intraPredMode is equal to Intra_DepthPartition(41,42), the corresponding intra prediction mode specified in subclause H.13.4.4.2.10 is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, with the sample array p and the transform block size nT  as the inputs and the output are the predicted sample array predSamples.

- Otherwise, if intraPredMode is equal to Intra_Chain(43,44), the corresponding intra prediction mode specified in subclause  is invoked with the location ( xB0, yB0 ), the intra prediction mode intraPredMode, with the sample array p and the transform block size nT  as the inputs and the output are the predicted sample array predSamples.

        - Reference sample substitution process for intra sample prediction

The specifications in subclause 8.4.4.2.2 apply.

        - Filtering process of neighbouring samples

The specifications in subclause 8.4.4.2.3 apply.

        - Specification of Intra_Planar (0) prediction mode

The specifications in subclause 8.4.4.2.4 apply.

        - Specification of Intra_DC (1) prediction mode

The specifications in subclause 8.4.4.2.5 apply.

        - Specification of Intra_Angular (2..34) prediction mode

The specifications in subclause 8.4.4.2.6 apply.

        - Specification of Intra_DepthPartition (35, 36) prediction mode

Inputs to this process are:

- a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable intraPredMode specifying the intra prediction mode,

- the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

- a variable nT specifying the transform size,

Output of this process is:

- predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 35 or 36.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified by the following ordered steps:

  - The variable wedgePattern[ x ][ y ] with x, y =0..nT−1, specifying a binary partition pattern is derived as.

      - wedgePattern = WedgePatternTable[ Log2( nT) ][ wedge_full_tab_idx[ xB ][ yB ] ]	(H-37)

  - The depth partition value derivation and assignment process as specified in subclause H.13.4.4.2.12 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern [ xB ][ yB ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(36) ), intraChainFlag set equal to 0, and the DC Offsets DcOffsetP0[ xB ][ yB ], and DcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

  - For x, y = 0..nT − 1, inclusive the following applies:

    - WedgeIdx[ xB +x ][ yB +y] is set equal to wedge_full_tab_idx[ xB ][ yB ].

        - Specification of Intra_DepthPartition (37, 38) prediction mode

Inputs to this process are:

- a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable intraPredMode specifying the intra prediction mode,

- the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

- a variable nT specifying the transform size,

Output of this process is:

- predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 37 or 38.

Let textureComp be the texture view component with view order index equal to ViewIdx and DepthFlag equal to 0. The variable texturePredMode is set equal to the PredMode[ xB ][ yB ] of textureComp. The variable textureIntraPredMode is set equal to IntraPredMode[ xB ][ yB ] of the textureComp.

[Ed. (GT) Consider storing texturePredMode and textureIntraPredMode explicitly.]

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived by the following ordered steps.

  - Depending on texturePredMode and textureIntraPredMode, the list candWedgeIndList specifying indices of candidate wedgelets is derived as specified in the following:

    - If texturePredMode is equal to MODE_INTRA and textureIntraPredMode is in the range of 2 to 34, the following applies.

        - candWedgeIndList = TextModePredWedgeIndTable[ Log2( nT ) ][ texturePredMode ]	(H-38)

    - Otherwise ( texturePredMode is not equal to MODE_INTRA or textureIntraPredMode is not in the range of 2 to 34), the following applies.

        - candWedgeIndList = CoarseWedgeIndTable[ Log2( nT ) ]	(H-39)

  - The variable wedgeIdx is derived as:

  - wedgeIdx = candWedgeIndList[ wedge_predtex_tab_idx[ xB ][ yB ]]	(H-40)

  - The variable wedgePattern [ x ][ y ] with x, y =0..nT−1, specifying a binary partition pattern is derived as.

      - wedgePattern = WedgePatternTable[ Log2( nT) ][ wedgeIdx]	(H-41)

  - The depth partition value derivation and assignment process as specified in subclause H.13.4.4.2.12 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(38) ), intraChainFlag set equal to 0, and the DC Offsets DcOffsetP0[ xB ][ yB ], and DcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

        - Specification of Intra_DepthPartition (39, 40) prediction mode

Inputs to this process are:

- a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable intraPredMode specifying the intra prediction mode,

- the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

- a variable nT specifying the transform size,

Output of this process is:

- predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 39 or 40.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified by the following ordered steps:

  - The variable recTextPic is set equal to the array of the reconstructed luma picture samples of the view component with view order index equal to ViewIdx and DepthFlag equal to 0.

  - The variable textThresh specifying a threshold for the segmentation of recTextPic is derived as specified in the following.

    - The variable sumTextPicVals is set equal to 0.

    - For x = 0..nT−1 the following applies

      - For y = 0..nT−1 the following applies

          - sumTextPicVals += recTextPic[ xB + x ][ yB + y ]	(H-42)

    - The variable textThresh is set equal to sumTextPicVals / ( nT * nT )

  - The variable wedgeletPattern[ x ][ y ] with x, y =0..nT−1 specifying a binary partition pattern is derived as specified in the following.

    - For x = 0..nT−1 the following applies

      - For y = 0..nT−1 the following applies

          - wedgeletPattern[ x ][ y ] = ( recTextPic[ xB + x ][ yB + y ] > textThresh )	(H-43)

  - The depth partition value derivation and assignment process as specified in subclause H.13.4.4.2.12 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgeletPattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(40) ), intraChainFlag set equal to 0, and the DC Offsets DcOffsetP0[ xB ][ yB ], and DcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

        - Specification of Intra_DepthPartition (41, 42) prediction mode

Inputs to this process are:

- a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable intraPredMode specifying the intra prediction mode,

- the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

- a variable nT specifying the transform size,

Output of this process is:

- predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 41 or 42.

Sample location ( xRef, yRef ) specifies the top-left sample of a reference block relative to the top-left sample of the current picture.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified by the following ordered steps:

  - The flag refAvailableFlag specifying whether a neighbouring block is available for prediction is set equal to 0.

  - The luma location ( xRefA, yRefA ) is set equal ( xB, yB − 1 ) and the luma location ( xRefL, yRefL ) is set equal to the ( xB − 1, yB ).

  - For N being replace by A and L, the following applies:

    - The availability derivation process for a block in z-scan order as specified in subclause 6.4.1 is invoked with the location of the current block ( xB, yB ) and the location ( xRefN, yRefN) as the input and the output is assigned to flag availableFlagN.

    - When refAvailableFlag is equal to 0, availableFlagN is equal to 1 and PredMode[ xRefN ][ yRefN ] is equal to MODE_INTRA and IntraPredMode[ xRefN ][ yRefN ] is in the range of 35 to 38 or 41 to 42, the following applies:

      - The flag refAboveFlag is set equal to ( N = = A )

      - The variable refWedgeIdx is set to the Wedgelet pattern index value WedgeIdx[ xRefN ][ yRefN ].

      - The Wedgelet direction derivation process as specified in subclause H.13.4.4.2.10.1 is invoked with sample location ( xB, yB ), the transform size nT, the flag refAboveFlag and the reference Wedgelet index refWedgeIdx as inputs and the output is assigned to refAvailableFlag, refEndPos, refDeltaHor and refDeltaVer.

  - When refAvailableFlag is equal to 0, the following applies:.

    - The reference partition end position refEndPos and the flag refAboveFlag are derived as specified in the following:

maxSlopeAbove = 0 
			posMaxSlopeAbove = 0
			maxSlopeLeft = 0
			posMaxSlopeLeft = 0
			for( k = 0; k < nT − 1; k++ ) {
				if( Abs( p[ k+1 ][ −1 ] − p[ k ][ −1 ] ) > maxSlopeAbove ) {
					maxSlopeAbove = Abs( p[ k+1 ][ −1 ] − p[ k ][ −1 ] )
					posMaxSlopeAbove = Min( k+1, nT−2 )
				}
				if( Abs( p[ −1 ][ k+1 ] - p[ −1 ][ k ] ) > maxSlopeLeft ) {
					maxSlopeLeft = Abs( p[ −1 ][ k+1 ] - p[ −1 ][ k ] )
					posMaxSlopeLeft = Min( k+1, nT−2 )
				}
			}
			refEndPos = ( maxSlopeAbove > maxSlopeLeft ) ? posMaxSlopeAbove : posMaxSlopeLeft
			refAboveFlag = ( maxSlopeAbove > maxSlopeLeft ) ? 1 : 0

    - When availableFlagA is equal to 1 or availableFlagL is equal to 1, the following applies:

      - The luma position ( x, y ) is set equal to ( availableFlagA ? ( xRefA , yRefA ) : ( xRefL , yRefL ).

      - When PredMode[ x ][ y ] is equal to MODE_INTRA and IntraPredMode[ x ][ y ] is in the range of 2 to 34, the following applies:

      - The flag refAvailableFlag is set equal to 1.

      - The variable predModeIntra is set equal to IntraPredMode[ x ][ y ].

      - Depending on predModeIntra, the variable intraPredAng is derived as specified in 8-4.

      - Depending on predModeIntra, refAboveFlag and intraPredAng, the variables refDeltaHor and refDeltaVer are derived as specified in Table H-6.

Table H-6 – Specification of refDeltaHor, refDeltaVer 
depending on predModeIntra, refAboveFlag, intraPredAng

| predModeIntra < 18 | refAboveFlag | refDeltaHor | refDeltaVer |
| --- | --- | --- | --- |
| 1 | 1 | ( intraPredAng > 0 ) ? −32 : 32 | Abs( intraPredAng ) |
|  | 0 | 32 | − intraPredAng |
| 0 | 1 | − intraPredAng | 32 |
|  | 0 | Abs( intraPredAng ) | ( intraPredAng>0 ) ? −32 : 32 |

  - The variable resShift specifying a shift for wedgelet sampling, the variable wedgeOri specifying a wedgelet orientation. and the xS, yS, xE and yE specifying the horizontal and vertical position of the start and end of the wedgelet partition line are derived as specified in the following:

    - The variable resShift is set as specified in Table H−8 with log2BlkSize equal to Log2( nT ).

    - If refAvailableFlag is equal to 1, the wedgelet direction prediction process specified in subclause H.13.4.4.2.10.2 is invoked with sample location ( xB, yB ), the transform block size nT, the reference block direction flag refAboveFlag, reference partition end position refEndPos, and reference partition horizontal and vertical slope refDeltaHor and refDeltaVer as inputs and the output is assigned to wedgeOri, xS, yS, xE and yE.

    - Otherwise (refAvailableFlag is not equal to 1), the variables wedgeOri, xS, yS, xE and yE are set equal to 0.

  - The Wedgelet pattern generation process as specified in subclause H.13.4.4.2.13.1 is invoked with the transform size nT, the variable, resShift, the Wedgelet orientation identifier wedgeOri, the line start horizontal position xS, the line start vertical position yS, the line end horizontal position xE and the line end vertical position yE as inputs and the output is assigned to wedgePattern[ x ][ y ]. [ Ed. (PM) Software uses wedgelet lookup list for partition derivation. (#2) ]

  - The depth partition value derivation and assignment process as specified in subclause H.13.4.4.2.12 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern wedgePattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(42) ), intraChainFlag set equal to 0, and the DC Offsets DcOffsetP0[ xB ][ yB ], and DcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

  - The Wedgelet pattern index WedgeIdx[ xB ][ yB ] is derived as specified in the following:

    - For k = 0..NumWedgePattern[ Log2( nT ) ] − 1, inclusive,  the following applies:

      - The flags patIdenticalFlag and patInvIdenticalFlag are set equal to 1.

      - For x, y = 0..nT − 1, inclusive the following applies.

      - If wedgePattern[ x ][ y ] is not equal to WedgePatternTable[ Log2( nT ) ][ k ][ x ][ y ], patIdenticalFlag is set to 0.

      - Otherwise ( wedgePattern[ x ][ y ] is equal to WedgePatternTable[ Log2( nT ) ][ k ][ x ][ y ] ), patInvIdenticalFlag is set to 0.

      - When patIdenticalFlag is equal to 1 or patInvIdenticalFlag is equal to 1, the following applies

      - For x, y = 0..nT − 1, inclusive the following applies:

      - WedgeIdx[ xB +x ][ yB +y] is set equal to k.

          - Wedgelet direction derivation process

Inputs to this process are:

- a sample location ( xN, yN ) specifying a position in the neighbouring block relative to the top-left sample of the current picture,

- a variable nT specifying the transform block size,

- a flag refAboveFlag specifying whether the above or left neighbouring block is used for prediction,

- a variable refWedgeIdx specifying the wedgelet pattern list index of the reference block.

Outputs of this process are:

- a flag refValidFlag specifying whether prediction from the reference block is possible or not,

- a variable refEndPos specifying the end position of the reference block partition,

- variables refDeltaHor and refDeltaVer specifying the horizontal and vertical slope of the reference block partition direction.

- The flag refValidFlag and the variables refOffset, refEndPos, refDeltaHor and refDeltaVer are set equal to 0.

- The luma location ( xN, yN ) is set equal to ( refAboveFlag ? ( xB, yB − 1 ) : ( xB − 1, yB ) ).

- The variable refBlkSize is set equal to ( CbSize[ xN ][ yN ] >> ( ( CtPartMode[ xN ][ yN ] = = PART_NxN ) ? 1 :0 ) ) and the variable refOffset is derived as specified in the following: .

- xRef = CbPosX[ xN ][ yN ] + ( ( xB – CbPosX[ xN ][ yN ] ) / refBlkSize ) * refBlkSize	(H-44)
yRef = CbPosY[ xN ][ yN ] + ( ( yB – CbPosY[ xN ][ yN ] ) / refBlkSize ) * refBlkSize	(H-45)
refOffset = refAboveFlag ? Max( 0, xB – xRef ) : Max( 0, yB – yRef )	(H-46)

The array refWedgePattern[ x ][ y ] of size (refBlkSize)x(refBlkSize), specifying a binary partition pattern is derived as follows:

- refWedgePattern = WedgePatternTable[ Log2( refBlkSize ) ][ refWedgeIdx ]	(H-47)

Depending of refWedgePattern the value of the variable wedgeOri specifying a wedgelet orientation is derived as specified in Table H-7. [Ed. (PM): Software uses wedgelet lookup list for deriving orientation.]

Table H-7 – Specification of wedgeOri depending on refWedgePattern

| refWedgePattern[ 0 ][ 0 ] | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| refWedgePattern[ refBlkSize –1 ][ 0 ] | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |
| refWedgePattern[ 0 ][ refBlkSize –1 ] | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 |
| refWedgePattern[ refBlkSize –1 ][ refBlkSize –1 ] | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| wedgeOri | 0 | 1 | 2 | 3 | 4 |  | 5 |  |

- Depending refBlkSize and wedgeOri, the variables xPosS, yPosS, xPosE, yPosE, xIncS, yIncE, xIncE, yIncE are derived as specified in Table H−9, with wBlkSize being equal to refBlkSize.

- The values of xRefS, yRefS, and xRefE, yRefE are derived as specified in the following:

- [Ed. (PM): Software uses wedgelet lookup list for deriving start and end position.(#2)]

- For C being replaced by S and E, the following applies:

for( k = 0; k < refBlkSize − 1; k++ ) {
		x0 = xPosC + k * xIncC 
		y0 = yPosC + k * yIncC 
		x1 = xPosC + (k + 1)* xIncC 
		y1 = yPosC + (k + 1)* yIncC 
		if( refWedgePattern[ x0 ][ y0 ] ! = refWedgePattern[ x1 ][ y1 ] ) {
			xRefC = ( refWedgePattern[ x0 ][ y0 ] = = 1 ) ? x0 : x1
			yRefC = ( refWedgePattern[ x0 ][ y0 ] = = 1 ) ? y0 : y1
		}
	}

- The  value of refValidFlag, refEndPos, refDeltaHor and refDeltaVer are set equal to 0 and the following applies:

if( ( refAboveFlag && ( wedgeOri = = 2 | | wedgeOri = = 3 | | wedgeOri = = 4) ) | | 
	( !refAboveFlag && ( wedgeOri = = 1 | | wedgeOri = = 2 | | wedgeOri = = 5 ) ) ) {
	if( ( refAboveFlag && 2 = = wedgeOri) | | ( !refAboveFlag && ( 1 = = wedgeOri | | 5 = = wedgeOri ) ) ) {
		( xRefE, xRefS ) = Swap( xRefE, xRefS )
		( yRefE, yRefS ) = Swap( yRefE, yRefS )
	}
	refDeltaHor = xRefE − xRefS
	refDeltaVer = yRefE − yRefS
	refEndPos =  ( refAboveFlag ? xRefE : yRefE ) − refOffset
	if( refEndPos > 0 && refEndPos < ( nT − 1 ) ) {
		refValidFlag = 1
	}
}

          - Wedgelet direction prediction process

Inputs to this process are:

- a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable nT specifying the block size,

- a flag refAboveFlag specifying whether the above or left neighbouring block is used for prediction,

- a variable refEndPos specifying the end position of the reference block partition,

- variables refDeltaHor and refDeltaVer specifying the horizontal and vertical slope of the reference block partition direction.

- Outputs of this process are:

- a variable wedgeOri specifying the orientation identifier of the Wedgelet pattern,

- a variable xS specifying the partition line start horizontal position,

- a variable yS specifying the partition line start vertical position,

- a variable xE specifying the partition line end horizontal position,

- a variable yE specifying the partition line end vertical position.

The variable log2BlkSize is set equal to Log2( nT ).

Depending on log2BlkSize, the variable resShift specifying the re-sampling of the wedgelet partition for the current block is derived as specified in Table H−8.

The variable blkMax specifies the maximum value for partition line start and end positions in the resampled domain is derived as follows:

- blkMax = ( ( resShift >= 0 ) ? ( nT << resShift ) : ( nT >> resShift ) ) – 1	(H-48)

The variable scRefEndPos specifying the reference end position in the resampled domain is derived as follows:

- scRefEndPos = ( resShift >= 0 ) ? ( refEndPos << resShift ) : ( refEndPos >> resShift )	(H-49)

The variables deltaM, deltaN and offsetEndPos are derived as specified in the following:

- deltaM = refAboveFlag ? refDeltaHor : refDeltaVer		(H-50)
deltaN  = refAboveFlag ? refDeltaVer : refDeltaHor		(H-51)
offsetEndPos = DmmDeltaEnd[ xB ][ yB ] * ( refAboveFlag ? 1 : –1 )	(H-52)

The variables mS and nS are derived as specified in the following:

nS = 0
mS = scRefEndPos
if( abs( deltaM ) >= abs( deltaN ) && deltaM ! = 0 ) 
	mS + = ( deltaM > 0 ) ? 1 : –1

Depending on deltaN, the variables wedgeOri, mE and nE are derived as specified in the following:

- When deltaN is equal to 0, the following applies:

mE = ( deltaM < 0 ) ? 0 : blkMax
	nE = Min( Max( offsetEndPos * ( ( deltaM < 0 ) ? 1 : –1 ), 0 ), blkMax )
	wedgeOri = ( deltaM < 0 ) ? 0 : ( refAboveFlag ? 1 : 3 )

- When deltaN is greater than 0, the following applies:

mVirtualEnd =  mS + Round( ( blkMax * deltaM )  deltaN )
	if( mVirtualEnd < 0 ) {
		iNe = Round( ( –mS * deltaN )  deltaM ) + offsetEndPos
		nE = Clip3( 0, blkMax, iNe )
		mE = ( iNe > blkMax ) ? Min( ( iNe – blkMax ), blkMax ) : 0
		wedgeOri = ( iNe > blkMax ) ? ( refAboveFlag ? 4 : 5 ) : 0
	} else if( mVirtualEnd > blkMax ) {
		iNe = Round( ( ( blkMax – mS ) * deltaN )  deltaM ) – offsetEndPos
		nE = Clip3( 0, blkMax, iNe )
		mE = ( iNe > blkMax ) ? Max( ( blkMax – ( iNe – blkMax ) ), 0 ) : blkMax
		wedgeOri = ( iNe > blkMax ) ? ( refAboveFlag ? 4 : 5 ) : ( ( refAboveFlag ) ? 1 : 3 )
	} else { 
		iMe = mVirtualEnd + offsetEndPos
		mE = Clip3( 0, blkMax, iMe )
		if( iMe < 0 ) {
			nE = Max( ( blkMax + iMe ), 0 )
			wedgeOri = 0 
		}else if( iMe > blkMax ) {
			nE = Max( ( blkMax – ( iMe – blkMax ) ), 0 )
			wedgeOri = ( refAboveFlag ? 1 : 3 )
		}else {
			nE = blkMax
			wedgeOri = ( refAboveFlag ? 4 : 5 )
		}
	}

Depending on refAboveFlag and wedgeOri, the variables mS, nS, mE and nE are modified as specified in the following:

if( ( refAboveFlag && wedgeOri = = 1 ) | | ( !refAboveFlag && ( wedgeOri = = 0 | | wedgeOri = = 5 ) ) ) {
	( mE, mS ) = Swap( mE, mS )
	( nE, nS ) = Swap( nE, nS )
}

Depending on refAboveFlag, the variables yS, yS, yE and yE are derived as specified in the following:

- ( xS, yS ) = refAboveFlag ? ( mS, nS ) : ( nS, mS )		(H-53)
( xE, yE ) = refAboveFlag ? ( mE, nE ) : ( nE, mE )		(H-54)

        - Specification of Intra_Chain (43, 44) prediction mode

Inputs to this process are:

- a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable intraPredMode specifying the intra prediction mode,

- the neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

- a variable nT specifying the transform size,

Output of this process is:

- predicted samples predSamples[ x ][ y ], with x, y = 0..nT−1.

This intra prediction mode is invoked when intraPredMode is equal to 43 or 44.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as specified in the following ordered steps.

- [Ed. (GT) The process of edge reconstruction (items 1 - 3) need to be specified more in detail. In current state the specification is not precise enough. (#3)]

  - The start point of the chain in the current prediction unit is derived from edge_start_left_flag[ xB ][ yB ] and edge_start_position[ xB ][ yB ].

- If edge_start_left_flag[ xB ][ yB ] is equal to 1, the start point is set as s[ 0 ][ yS ] where yS is equal to edge_start_position[ xB ][ yB ].

- Otherwise, the start point is set as s[ xS ][ 0 ] where xS is equal to edge_start_position[ xB ][ yB ].

  - The number of edges and the direction of each edge are derived from num_edge_codes_minus1[ xB ][ yB ] and edge_code[ xB ][ yB ][ k ], for k = 0.. num_edge_codes_minus1[ xB ][ yB ]. The direction of the edge is derived from edge_code as shown in Table H-3.

  - The region boundary generated by connecting each edge separates the predicted samples predSamples[ x ][ y ], with x, y = 0..nT−1 into two regions: the region rA that covers the top-left pixel (x0, y0) and the region rB that covers the remaining region.

For x, y = 0..nT−1 the binary pattern edgePattern[ x ][ y ] is derived as follows:

- If (x, y) is covered by rA, edgePattern[ x ][ y ] =  0.

- Otherwise, edgePattern[ x ][ y ] = 1.

  - The depth partition value derivation and assignment process as specified in subclause H.13.4.4.2.12 is invoked with the neighbouring samples p[ x ][ y ], the binary pattern edgePattern[ x ][ y ], the transform size nT, the dcOffsetAvailFlag set equal to ( intraPredMode = = Intra_DepthPartition(44) ), intraChainFlag set equal to 1, and the DC offsets DcOffsetP0[ xB ][ yB ], and DcOffsetP1[ xB ][ yB ] as inputs and the output is assigned to predSamples[ x ][ y ].

        - Depth partition value derivation and assignment process

Inputs to this process are:

- neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

- an binary array partitionPattern[ x ][ y ], with x, y =0..nT−1, specifying a partitioning of the prediction block in a partition 0 and a partition 1.

- a variable nT specifying the transform size,

- a flag dcOffsetAvailFlag, specifying whether DC Offset values are available

- a flag intraChainFlag, specifying whether the current  intra prediction mode is equal to Intra_Chain(43,44)

- the variables dcOffsetP0 and dcOffsetP1, specifying the DC offsets for the block partitions

Output of this process is:

- predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

The variable log2SubSample is set equal to ( !intraChainFlag && ( nT < 32 ) ) 0 : 1.

For X being replaced by 0 and 1, the values of the prediction samples predSamples[ x][ y ] are derived as specified by the following ordered steps:

  - The variable sumNeigh specifying the sum of the neighbouring samples depth values of partition X is set equal to 0 and the variable numNeigh specifying the number of the neighbouring samples of partition X is set equal to 0 and the following applies.

    - For x = 0..( nT >> log2SubSample ) − 1, inclusive the following applies:

      - When partitionPattern[ x << log2SubSample ][ 0 ] is equal to X, the following applies:

          - sumNeigh += p[ x<< log2SubSample ][ −1 ]	(H-55)
numNeigh += 1		(H-56)

    - For y = 0..( nT  >> log2SubSample )− 1,inclusive the following applies:

      - When partitionPattern[ 0 ][ y << log2SubSample ] is equal to X, the following applies:

          - sumNeigh += p[ −1 ][ y << log2SubSample ]	(H-57) 
numNeigh += 1		(H-58)

  - The variables predDcVal specifying the predicted constant partition values for partition X is derived as follows.

      - predDcVal = ( numNeigh ! = 0 ) ? ( sumNeigh / numNeigh ) : ( 1 << ( BitDepthY  − 1 ) )	(H-59)

  - The variable dcOffset specifying the DC offset is derived as follows.

    - If dcOffsetAvailFlag is equal to 1, the following applies. :

        - dcOffsetScale = intraChainFlag ? Clip3( 1, ( 1 << BitDepthY ) − 1, Round( 2 (QP’Y /10)- 2 ) ) : 1 	(H-60)

        - dcOffset = dcOffset * dcOffsetScale		(H-61)

    - Otherwise ( dcOffsetAvailFlag is equal to 0), dcOffset is set equal to 0.

  - The predicted sample values predSamples are derived as follows for x = 0..nT−1 and for y = 0..nT−1.

    - When partitionPattern[ x ][ y ] is equal to X, the following applies.

        - predSamples[ x ][ y ] = predDcVal + dcOffset	(H-62)

        - Specification of tables WedgePatternTable, CoarseWedgeIndTable, TextModePredWedgeIndTable.

Note − Tables and values resulting from the processes specified in the following are independent of any information contained in the bitstream. Therefore the derivation process described in this subclause can be carried out once as part of the initialization of the decoding process. Alternatively, the tables and values can be stored within the decoder (read-only) memory as fixed lookup tables, such that the derivation process described in this section does not need to be implemented in the decoder at all.

The list WedgePatternTable[ log2BlkSize ] of binary partition patterns of size (1 << log2BlkSize )x(1 << log2BlkSize), the variable NumWedgePattern[ log2BlkSize ] specifying the number of binary partition patterns in list WedgePatternTable[ log2BlkSize ], the list CoarseWedgeIndTable[ log2BlkSize ] of indices to the list WedgePatternTable[ log2BlkSize ] and the variable  NumCoarseWedgeInd[ log2BlkSize ] specifying the number of elements in list CoarseWedgeIndTable[ log2BlkSize ] are derived as specified in the following:

- For log2BlkSize ranging from 2 to Log2MaxDmmCbSize, inclusive, the following applies:

- Depending on log2BlkSize, the variable resShift is derived as specified in Table H−8.

Table H−8 – Specification of resShift

| log2BlkSize | resShift |
| --- | --- |
| 2,3 | 1 |
| 4 | 0 |
| Otherwise (5… Log2MaxDmmCbSize) | −1 |

- The variable wBlkSize is set equal to ( 1 << ( log2BlkSize + resShift ) )

- For wedgeOri in the range of 0 to 5, inclusive, the following ordered steps apply.

    - Depending on wedgeOri the variables xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE are derived as specified in Table H−9.

Table H−9 – Specification of xPosS, yPosS, xPosE, yPosE, xIncS, yIncS, xIncE, yIncE

| wedgeOri | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| xPosS | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 | wBlkSize − 1 |
| yPosS | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | 0 | 0 |
| xPosE | 0 | wBlkSize − 1 | wBlkSize− 1 | 0 | 0 | 0 |
| yPosE | 0 | 0 | wBlkSize − 1 | wBlkSize − 1 | wBlkSize − 1 | 0 |
| xIncS | 1 | 0 | −1 | 0 | 1 | 0 |
| yIncS | 0 | 1 | 0 | −1 | 0 | 1 |
| xIncE | 0 | −1 | 0 | 1 | 1 | 0 |
| yIncE | 1 | 0 | −1 | 0 | 0 | 1 |

    - For m in the range of 0 to wBlkSize − 1, inclusive, the following applies:

      - For n in the range of 0 to wBlkSize − 1, inclusive,  the following applies:

      - The Wedgelet pattern generation process as specified in subclause H.13.4.4.2.13.1 is invoked with patternSize being equal to ( 1 << log2BlkSize ), the variable resShift, variable wedgeOri, xS being equal to ( xPosS + m * xIncS ), yS being equal to ( yPosS + m * yIncS ), xE being equal to ( xPosE + n * xIncE ) and yE being equal to ( yPosE + n * yIncE ) as inputs and the output is the binary array curWedgePattern.

      - The flag isCoarseFlag specifying whether wedgeletPattern is part of the coarse subset or not is set equal to ( (m % 2 ) = = 0 && ( n % 2 ) = = 0 ).

      - The wedgelet pattern list insertion process as specified in subclause H.13.4.4.2.13.2 is invoked with log2BlkSize, the binary partition pattern curWedgePattern and isCoarseFlag as inputs.

- The variable TextModePredWedgeIndTable[ log2BlkSize ][ texturePredMode ] specifies a list of indices to the table WedgePatternTable[ log2BlkSize ]. NumTextModePredWedgeInd[ log2BlkSize ][ texturePredMode ] specifies the number of elements of the list TextModePredWedgeIndTable[ log2BlkSize ][ texturePredMode ]. For log2BlkSize ranging from 2 to Log2MaxDmmCbSize, inclusive, TextModePredWedgeIndTable[ log2BlkSize ][ texturePredMode ] and NumTextModePredWedgeInd[ log2BlkSize ][ texturePredMode ] are derived as specified in the following.

- [Ed. (GT) To be specified.(#8)]

          - Wedgelet pattern generation process

Inputs to this process are:

- a variable patternSize specifying the binary partition pattern size,

- a resolution shift value resShift specifying the precision of the wedgelet partition start and end positions relative to patternSize,

- a variable wedgeOri specifying the orientation identifier of the wedgelet pattern,

- a variable xS specifying the partition line start horizontal position,

- a variable yS specifying the partition line start vertical position,

- a variable xE specifying the partition line end horizontal position,

- a variable yE specifying the partition line end vertical position.

Output of this process is:

- binary array wedgePattern[ x ][ y ] of size ( patternSize )x( patternSize )

- The variable curSize specifying the size of the current partition pattern is derived as follows:.

- curSize = ( resShift = = 1 ) ? ( patternSize  << 1 ) : patternSize	(H-63)

- When resShift is equal to −1 variables xS, yS, xE and yE are modified as specified in Table H−10.

Table H−10 – Specification of xS, yS, xE, yE

| wedgeOri | xS | yS | xE | yE |
| --- | --- | --- | --- | --- |
| 0 | xS << 1 | yS << 1 | xE << 1 | yE << 1 |
| 1 | curSize−1 | yS << 1 | xE << 1 | yE << 1 |
| 2 | xS << 1 | curSize−1 | curSize−1 | yE << 1 |
| 3 | xS << 1 | yS << 1 | xE << 1 | curSize−1 |
| 4 | xS << 1 | yS << 1 | xE << 1 | curSize−1 |
| 5 | curSize−1 | yS << 1 | xE << 1 | yE << 1 |

- The values of variable curPattern[ x ][ y ], are derived as specified by the following ordered steps.

  - For x, y = 0..curSize − 1, curPattern[ x ][ y ] is set equal to 0.

  - The samples of the array curPattern that form a line between ( xS, yS ) and ( xE, yE ) are set equal to 1 as specified in the following: [Ed: (PM): Software uses floating point arithmetic. (#1)]

x0 = xS
		y0 = yS
		x1 = xE
		y1 = yE
		if( abs( yE − yS ) > abs( xE − xS ) ) {
			( x0, y0 ) = Swap( x0, y0 )
			( x1, y1 ) = Swap( x1, y1 )
		}
		if( x0 > x1 ) {
			( x0, x1 ) = Swap( x0, x1 )
			( y0, y1 ) = Swap( y0, y1 )
		}
		sumErr = 0
		posY = y0
		for( posX = x0; posX <= x1; posX +  + ) {
			if( abs( yE − yS ) > abs( xE − xS ) )  
				curPattern[ posY ][ posX ] = 1 
			else 
				curPattern[ posX ][ posY ] = 1 
			sumErr + = ( abs( y1 − y0 ) << 1 )
			if( sumErr >= ( x1 − x0 ) ) {
				posY + = ( y0 < y1 ) ? 1 : −1
				sumErr −= ( x1 − x0 ) << 1
			}
		}

  - The samples of curPattern belonging to the smaller partition are set equal to 1 as specified in the following:

if( wedgeOri = = 0 ) 
			for( iX = 0; iX < xS; iX ++ )  
				for( iY = 0; curPattern[ iX ][ iY ] = = 0; iY++ )  
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri = = 1 ) 
			for( iY = 0; iY < yS; iY ++ )  
				for( iX = curSize − 1; curPattern[ iX ][ iY ] = = 0 ; iX−− )
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri = = 2 ) 
			for( iX = curSize − 1; iX > xS; iX−− ) 
				for( iY = curSize − 1; curPattern[ iX ][ iY ] = = 0 ; iY−−)
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri = = 3 ) 
			for( iY = curSize − 1; iY > yS; iY−− )  
				for( iX = 0; curPattern[ iX ][ iY ] = = 0 ; iX ++) 
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri = = 4 ) && ( ( xS + xE ) < curSize ) ) 
			for( iY = 0; iY < curSize; iY ++ ) 
				for( iX = 0; curPattern[ iX ][ iY ] = = 0 ; iX ++) 
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri = = 4 )
		 	for( iY = 0; iY < curSize; iY ++ )  
				for( iX = curSize − 1; curPattern[ iX ][ iY ] = = 0 ; iX−−) 
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri = = 5 )  && ( ( yS + yE ) < curSize ) )  
			for( iX = 0; iX < curSize; iX ++ )  
				for( iY = 0; curPattern[ iX ][ iY ] = = 0 ;iY ++ )  
					curPattern[ iX ][ iY ] = 1
		else if( wedgeOri = = 5 ) 
			for( iX = 0; iX < curSize; iX ++ )  
				for( iY = curSize − 1; curPattern[ iX ][ iY ] = = 0 ; iY−−)  
					curPattern[ iX ][ iY ] = 1

  - The binary partition pattern wedgePattern[ x ][ y ], with x, y = 0..patternSize − 1, is derived as specified in the following.

    - If resShift is equal to 1, the following applies.

      - Depending on wedgeOri, the variables xOff and yOff are set as specified in Table H−11.

Table H−11 Specification of xOff, yOff

| wedgeOri | ( xS + xE ) < curSize | xOff | yOff |
| --- | --- | --- | --- |
| 0 |  | 0 | 0 |
| 1 |  | 1 | 0 |
| 2 |  | 1 | 1 |
| 3 |  | 0 | 1 |
| 4 | 0 | 1 | 0 |
| 4 | 1 | 0 | 0 |
| 5 | 0 | 0 | 1 |
| 5 | 1 | 0 | 0 |

      - For x, y = 0 ..patternSize − 1 the following applies:

        - wedgePattern[ x ][ y ] = curPattern[ ( x << 1 ) + xOff ][ ( y << 1 ) + yOff ]	(H-64)

    - Otherwise ( resShift is not equal to 1 ), wedgePattern is set equal to curPattern.

          - Wedgelet pattern list insertion process

Inputs to this process are:

- a variable log2BlkSize specifying the binary partition pattern size as ( 1 << log2BlkSize ),

- binary partition pattern wedgePattern[ x ][ y ], with x, y =0..( 1 << log2BlkSize )−1,

- a flag isCoarseFlag specifying whether wedgeletPattern is part of the coarse subset or not.

- The variable isValidFlag specifying whether the binary partition pattern wedgePattern is added to the list WedgePatternTable[ log2BlkSize ] not is set 	equal to 0.

- The value of isValidFlag is derived as specified by the following ordered steps.

  - For x, y = 0..( 1 << log2BlkSize ) − 1 the following applies.

    - When wedgePattern[ x ][ y ] is not equal to wedgePattern[ 0 ][ 0 ] the flag isValidFlag is set to 1.

  - For k = 0..NumWedgePattern[ log2BlkSize ] − 1 the following applies.

    - The flag patIdenticalFlag is set equal to 1.

    - For x, y =0..( 1 << log2BlkSize )−1 the following applies.

      - When wedgePattern[ x ][ y ] is not equal to WedgePatternTable[ log2BlkSize ][ k ][ x ][ y ], patIdenticalFlag is set to 0.

    - When patIdenticalFlag is equal to 1, isValidFlag is set to 0.

  - For k = 0..NumWedgePattern[ log2BlkSize ] − 1 the following applies.

    - The flag patInvIdenticalFlag is set to 1.

    - For x, y =0..( 1 << log2BlkSize ) − 1 the following applies.

      - When wedgePattern[ x ][ y ] is equal to WedgePatternTable[ log2BlkSize ][ k ][ x ][ y ], patInvIdenticalFlag is set to 0.

    - When patInvIdenticalFlag is equal to 1, isValidFlag is set to 0.

- When isValidFlag is equal to 1, the following applies.

- The pattern WedgePatternTable[ log2BlkSize ][ NumWedgePattern[ log2BlkSize ] ] is set equal to wedgePattern.

- The value of NumWedgePattern[ log2BlkSize ] is increased by one.

- When isCoarseFlag is equal to 1 the following applies.

    - The pattern CoarseWedgeIndTable[ log2BlkSize ][ NumCoarseWedgeInd[ log2BlkSize ] ] is set equal to NumWedgePattern[ log2BlkSize ] − 1.

    - The value of NumCoarseWedgeInd[ log2BlkSize ] is increased by one.

      - Depth value reconstruction process

Inputs to this process are:

- a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable nT specifying the prediction size

- predicted samples predSamples[ x ][ y ], with x, y =0..nT−1

- a variable intraPredMode specifying the prediction mode of the current prediction block

Output of this process is:

- reconstructed depth value samples resSamples[ x ][ y ], with x, y = −1..2*nT−1.

The values of the prediction samples predSamples[ x ][ y ], are derived as specified by the following ordered steps.

  - Depending on intraPredMode the array wedgePattern[ x ][ y ] with x, y =0..nT−1 specifying the binary segmentation pattern is derived as follows.

    - If intraPredMode is equal to Intra_DepthPartition( 35 ), the following applies.

        - wedgePattern = WedgePatternTable[ Log2( nT) ][ wedge_full_tab_idx[ xB ][ yB ] ]

    - Otherwise ( intraPredMode is not equal to Intra_DepthPartition( 35 ) ), the following applies.

      - For x, y = 0..nT−1 wedgePattern[ x ][ y ] is set equal to 0.

  - The variable log2SubSample is set equal to ( nT < 32 ) ? 0 : 1.

  - For p in the range of 0 to 1, inclusive,  the variable dcPred[ p ] is derived as specified in the following:

sumPred = 0
		numPred = 0. 
		for( x = 0; x < ( nT >> log2SubSample ); x++ ) { 
			xS  = x << log2SubSample  
			for ( y = 0; y < ( nT >> log2SubSample ) ;y++ ) { 
				yS = y << log2SubSample .
				if ( p = = wedgePattern[ xS ][ yS ] ) {
					sumPred += predSamples[ xS ][ yS ]
					numPred += 1
				}
			}
		dcPred[ p ] = ( numPred > 0 ) ? ( sumPred / numPred ) : 0

[ Ed. (GT): Is this averaging necessary for DMM or DC mode? ]

  - For x, y = 0..nT−1, the reconstructed depth value samples resSamples[ x ][ y ] are derived as specified in the following.

    - If dlt_flag[ nuh_layer_id ] is equal to 0, the following applies:

        - resSamples[ x ][ y ] = dcPred[ wedgePattern[ x ][ y ] ] +	
	SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]

    - Otherwise ( dlt_flag[ nuh_layer_id ] is equal to 1 ), the following applies.

        - dltIdxPred = DepthValue2Idx[ dcPred[ wedgePattern[ x ][ y ] ] ] 	(H-65)
dltIdxResi = SdcResidual[ xB ][ yB ][ wedgePattern[ x ][ y ] ]	(H-66)
resSamples[ x ][ y ] = Idx2DepthValue[ dltIdxPred + dltIdxResi ]	(H-67)

  - Decoding process for coding units coded in inter prediction mode

The specifications in subclause 8.5 apply with the following modification:

–	All invocations of the process specified in subclause 8.5.1 are replaced with invocations of the process specified in subclause H.13.5.1.

    - Inter prediction process

The specifications in subclause 8.5.1 apply with the following modification:

–	All invocations of the process specified in subclause 8.5.2 are replaced with invocations of the process specified in subclause H.13.5.2.

    - Decoding process for prediction units in inter prediction mode

The specifications in subclause 8.5.2 apply with the following modification:

–	All invocations of the process specified in subclause 8.5.2.1 are replaced with invocations of the process specified in subclause H.13.5.2.1.

–	All invocations of the process specified in subclause 8.5.2.2 are replaced with invocations of the process specified in subclause H.13.5.4.

      - Derivation process for motion vector components and reference indices

Input to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xB, yB ) of the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- luma motion vectors mvL0 and mvL1

- chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Let ( xP, yP ) specify the top-left sample location of the current luma prediction block relative to the top-left luma sample of the current picture where xP = xC + xB and yP = yC + yB.

Let the variable currPic and ListX be the current picture and RefPicListX (with X being 0 or 1) of the current picture, respectively.

The function LongTermRefPic( picX, refIdx, LX ), with X being either 0 or 1, is defined as follows. If the picture with index refIdx from reference picture list LX of the picture picX was marked as "used for long term reference" at the time when picX was the current picture, LongTermRefPic( picX, refIdx, LX ) is equal to 1; otherwise LongTermRefPic( picX, refIdx, LX ) is equal to 0.

The flag vspModeFlag is set equal to 0, the variable refViewIdx is set equal to −1 and the flag availableFlagIvMC is set equal to 0.

For X being replaced by 0 and 1, the variable ivpMvFlagLX and the variable ivpMvDispLX are derived as:

- ivpMvFlagLX = 0		(H-68)
ivpMvDispLX[ 0 ] = 0		(H-69)
ivpMvDispLX[ 1 ] = 0		(H-70)

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1 as well as prefFlagL0 and prefFlagL1, the following applies.

- If PredMode[ xC ][ yC ] is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause H.13.5.2.1.1 is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPbW, nPbH and the partition index partIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, the disparity vectors ivpMvDispL0 and ivpMvDispL1, the flag vspModeFlag, the variable refViewIdx and the flag availableFlagIvMC.  .

- Otherwise, if PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in subclause H.13.5.2.1.1 is invoked with the luma location ( xC, yC ), luma location ( xP, yP ), variables nCS, nPbW and nPbH and the partition index partIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, the disparity vectors ivpMvDispL0 and ivpMvDispL1, the flag vspModeFlag, the variable refViewIdx and the flag availableFlagIvMC. .

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX and in Pred_LX and in the syntax elements ref_idx_lX and MvdLX, the following applies.

    - The variables refIdxLX and predFlagLX are derived as follows.

      - If inter_pred_idc[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

          - refIdxLX = ref_idx_lX[ xP ][ yP ]		(H-71)
predFlagLX = 1		(H-72)

      - Otherwise, the variables refIdxLX and predFlagLX are specified by

          - refIdxLX = −1		(H-73)
predFlagLX = 0		(H-74)

    - The variable mvdLX is derived as follows.

        - mvdLX[ 0 ] = MvdLX[ xP ][ yP ][ 0 ]		(H-75)
mvdLX[ 1 ] = MvdLX[ xP ][ yP ][ 1 ]		(H-76)

    - When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in subclause H.13.5.2.1.5 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), variables nPbW and nPbH, refIdxLX, and the partition index partIdx as the inputs and the output being mvpLX.

    - When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

        - uLX[ 0 ] = (mvpLX[ 0 ] + mvdLX[ 0 ] + 216) % 216	(H-77)
mvLX[ 0 ] = ( uLX[ 0 ] >= 215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ]	(H-78)
uLX[ 1 ] = (mvpLX[ 1 ] + mvdLX[ 1 ] + 216) % 216	(H-79)
mvLX[ 1 ] = ( uLX[ 1 ] >= 215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(H-80)

        - NOTE – The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −215 to 215 − 1.

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause 8.5.2.1.8 is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

If iv_res_pred_flag[ nuh_layer_id ] is equal to 1, availableFlagIvMC is equal to 1, and PartMode[ xP ][ yP ] is equal to PART_2Nx2N, ResPredFlag[ xP ][ yP ] is set equal to 0, otherwise ( iv_res_pred_flag[ nuh_layer_id ] is equal to 0 or availableFlagIvMC is equal to 0 or PartMode[ xP ][ yP ] is not equal to PART_2Nx2N ), ResPredFlag[ xP ][ yP ] is set equal to 0.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xP.. ( xP + nPbW − 1 ), y = yP..( yP + nPbH− 1 ) (with X being either 0 or 1):

- IvpMvFlagLX[ x ][ y ] = ivpMvFlagLX		(H-81)
IvpMvDispLX[ x ][ y ] = ivpMvDispLX		(H-82)
VspModeFlag[ x ][ y ] = vspModeFlag		(H-83)
RefViewIdx[ x ][ y ] = refViewIdx		(H-84)

[Ed.(GT): The complete derivation process for inter-view motion vectors candidates has the problem (in software as well as in this draft), that it is assumed that disparity vectors are related to the base view. However, in the draft and the software there are some loops looking for an arbitrary non-base view inter-view reference picture for inter-view motion vector prediction in the reference picture lists. This works for CTC, since other inter-view reference pictures apart from the base view are not allowed. However, to enable a more general approach, this should be discussed and possibly made more general in SW and text. (#5) ]

[Ed.(CY): To fix the above mentioned problem, the derived disparity vector should be associated with a reference view index.]

        - Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode[ xC ][ yC ] is equal to MODE_SKIP or PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- the luma motion vectors mvL0 and mvL1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1,

- the disparity vectors ivpMvDispL0 and ivpMvDispL1.

- the flag vspModeFlag, specifying, whether the current PU is coded using view synthesis prediction,

- the variable refViewIdx specifying a reference view for VSP,

- the flag availableFlagIvMC, specifying whether the IvMc candidate is available.

The variables singleMCLFlag is derived as follows.

- If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

- Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPbW and nPbH are set equal to nCS.

NOTE – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the disparity vectors ivpMvDispL0 and ivpMvDispL1 are derived as specified by the following ordered steps:

  - The derivation process for spatial merge ing candidates from neighboring prediction unit partitions as specified in subclause G.8.5.2.1.2 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the variable singleMCLFlag, the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

  - The reference index for temporal merging candidate refIdxLX (with X being 0 or 1) is set equal to 0.

  - The derivation process for temporal luma motion vector prediction in subclause H.13.5.2.1.7 is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, refIdxLX, and mergeFlag being equal to 1 as the inputs and with the output being the availability flag availableFlagLXCol, the temporal motion vector mvLXCol, and the reference index refIdxLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

      - availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(H-85) 
predFlagLXCol = availableFlagLXCol		(H-86)

  - Depending on iv_mv_pred_flag[ nuh_layer_id ], the following applies.

    - If multi_view_mv_pred_flag is equal to 0, the flags availableFlagIvMC and availableFlagIvDC are set equal to 0.

    - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause H.13.5.2.1.9 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, and the partition index partIdx, as the inputs and the output is assigned to the view order index of the reference view refViewIdx, the availability flags availableFlagIvMC and availableFlagIvDC, the reference indices refIdxLXIvMC and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC and predFlagLXIvDC, and the motion vectors mvLXIvMC and mvLXIvDC (with X being 0 or 1, respectively), and the disparity vector mvDisp.

  - Depending on view_synthesis_pred_flag, the following applies.

  - [ Ed. (GT): The variable view_synthesis_pred_flag is missing in software and HLS. ]

    - If view_synthesis_pred_flag is equal to 0, the flag availableFlagVSP is set equal to 0 and the variable refViewIdx is set equal to −1.

    - Otherwise (view_synthesis_pred_flag is equal to 1), the derivation process for a view synthesis prediction merge candidate as specified in subclause H.13.5.2.1.12 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, and the partition index partIdx as input and the outputs are the availability flag availableFlagVSP, the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list utilization flags predFlagL0VSP and predFlagL1VSP, the motion vectors mvL0VSP and mvL1VSP, and reference view index refViewIdx.

  - Depending on DepthFlag, the following applies.

    - If DepthFlag is equal to 0, the variable availableFlagT is set equal to 0.

    - Otherwise ( DepthFlag is equal to 1), the derivation process for the texture merging candidate as specified in subclause H.13.5.2.1.13 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH as the inputs and the outputs are the flag availableFlagT, the prediction utilization flags predFlagL0T and predFlagL1T, the reference indices refIdxL0T and refIdxL1T, and the motion vectors mvL0T and mvL1T.

  - The merge candidate list, mergeCandList, is constructed as specified by the following ordered steps.

  - The variable numMergeCand is set equal to 0.

  - The variable vspCandInListFlag is set equal to 0.

  - When availableFlagT is equal to 1, the insertion process for a candidate from a potentially view synthesis predicted neighbour as specified in subclause H.13.5.2.1.1.1 is invoked with the list mergeCandList, the variable numMergeCand, the variable vspCandInListFlag, the candidate position indicator N being equal to T, the luma location ( xP, yP ), and the variables nPbW and nPbH as the inputs and the outputs are the list mergeCandList, the variable numMergeCand and the flag vspCandInListFlag.

  - When availableFlagIvMC is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC and the variable numMergeCand is increased by 1.

  - When availableFlagA1 is equal to 1, the following applies:

      - When the following condition is true,

      - availableFlagT = = 0 && availableFlagIvMC  = = 0,

        - or one or more of the following conditions are true, with N being replaced by T and IvMC:

        - availableFlagN = = 1 && predFlagLXN ! = predFlagLXA1, (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && mvLXN ! = mvLXA1 (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && refIdxLXN ! = refIdxLXA1 (with X being replaced by 0 and 1),

the insertion process for a candidate from a potentially view synthesis predicted neighbour block as specified in subclause H.13.5.2.1.1.1 is invoked with the list mergeCandList, the variable numMergeCand, the variable vspCandInListFlag, the candidate position indicator N being equal to A1, the luma location ( xP, yP ), and the variables nPbW and nPbH as the inputs and the outputs are the list mergeCandList, the variable numMergeCand and the flag vspCandInListFlag.

  - When availableFlagB1 is equal to 1, the following applies:

      - When the following condition is true ,

      - availableFlagT = = 0 && availableFlagIvMC = = 0,

or one or more of the following conditions is true, with N being replaced by T and IvMC:

- availableFlagN = = 1 && predFlagLXN ! = predFlagLXB1, (with X being replaced by 0 and 1),

- availableFlagN = = 1 && mvLXN ! = mvLXB1 (with X being replaced by 0 and 1),

- availableFlagN = = 1 && refIdxLXN ! = refIdxLXB1 (with X being replaced by 0 and 1),

the insertion process for a candidate from a potentially view synthesis predicted neighbour block as specified in subclause H.13.5.2.1.1.1 is invoked with the list mergeCandList, the variable numMergeCand, the variable vspCandInListFlag, the candidate position indicator N being equal to B1, the luma location ( xP, yP ), and the variables nPbW and nPbH as the inputs and the outputs are the list mergeCandList, the variable numMergeCand and the flag vspCandInListFlag.

  - When availableFlagB0 is equal to 1, the insertion process for a candidate from a potentially view synthesis predicted neighbour block as specified in subclause H.13.5.2.1.1.1 is invoked with the list mergeCandList, the variable numMergeCand, the variable vspCandInListFlag, the candidate position indicator N being equal to B0, the luma location ( xP, yP ), and the variables nPbW and nPbH as the inputs and the outputs are the list mergeCandList, the variable numMergeCand and the flag vspCandInListFlag.

  - When availableFlagIvDC is equal to 1, and one or more of the following conditions is true,

      - availableFlagA1 = = 0,

      - predFlagLXA1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

      - mvLXA1 ! = mvLXIvDC (with X being replaced by 0 and 1),

      - refIdxLXA1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

and one or more of the following conditions is true,

      - availableFlagB1 = = 0,

      - predFlagLXB1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

      - mvLXB1 ! = mvLXIvDC (with X being replaced by 0 and 1),

      - refIdxLXB1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to IvDC and the variable numMergeCand is increased by 1.

  - When availableFlagA0 is equal to 1, the insertion process for a candidate from a potentially view synthesis predicted neighbour block as specified in subclause H.13.5.2.1.1.1 is invoked with the list mergeCandList, the variable numMergeCand, the variable vspCandInListFlag, the candidate position indicator N being equal to A0, the luma location ( xP, yP ), and the variables nPbW and nPbH as the inputs and the outputs are the list mergeCandList, the variable numMergeCand and the flag vspCandInListFlag.

  - When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the insertion process for a candidate from a potentially view synthesis predicted neighbour block as specified in subclause H.13.5.2.1.1.1 is invoked with the list mergeCandList, the variable numMergeCand, the variable vspCandInListFlag, the candidate position indicator N being equal to B2, the luma location ( xP, yP ), and the variables nPbW and nPbH as the inputs and the outputs are the list mergeCandList, the variable numMergeCand and the flag vspCandInListFlag.

  - When availableFlagVSP is equal to 1, numMergeCand is less than 4 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag and vspCandInListFlag is 0, vspCandInListFlag is set equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to VSP and the variable numMergeCand is increased by 1.

  - When availableFlagCol is equal to 1 and numMergeCand is less than 5 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to Col and the variable numMergeCand is increased by 1.

- A1, if availableFlagA1 is equal to 1

- B1, if availableFlagB1 is equal to 1

- B0, if availableFlagB0 is equal to 1

- A0, if availableFlagA0 is equal to 1

- B2, if availableFlagB2 is equal to 1

- Col, if availableFlagCol is equal to 1

  - The variable numMergeCand and numOrigMergeCand are is set equal to numMergeCand to the number of merging candidates in the mergeCandList.

  - When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause 8.5.2.1.3 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for zero motion vector merging candidates specified in subclause 8.5.2.1.4 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

  - The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

      - mvLX[ 0 ] = mvLXN[ 0 ]		(H-87)

      - mvLX[ 1 ] = mvLXN[ 1 ]		(H-88)

      - refIdxLX = refIdxLXN		(H-89)

      - predFlagLX = predFlagLXN		(H-90)

  - When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nPbW + nPbH ) is equal to 12, the following applies.

      - refIdxL1 = −1		(H-91)

      - predFlagL1 = 0		(H-92)

  - The variable vspModeFlag is set equal to ( mergeCandList[ merge_idx[ xP][ yP ] ] = = VSP ).

[Ed. (GT): In software VspModeFlag[ x ][ y ] is used in several places (deblocking filter, AMVP) this is not yet reflected in the draft.]

  - The disparity availability flag ivpMvFlagLX and the disparity vector ivpMvDispLX are derived as follows (with X being replace by 0 or 1).

    - If all of the following conditions are true, ivpMvFlagLX is set equal to 1 and ivpMvDispLX is set equal to mvDisp [Ed. (GT) There is some redundancy in draft and software since ivpMvDispLX is derived for each list, although it is always equal for both lists.(#7) ]

      - availableFlagIvMC = = 1

      - merge_idx[ xP][ yP ] = = 0

      - predFlagLXIvMC = = 1

- [Ed. (GT): PredMode[ xC ][ yC ] = = MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

    - Otherwise, ivpMvFlagLX is set equal to 0 and both components of ivpMvDispLX are set equal to 0.

          - Insertion process for a candidate from a potentially view synthesis predicted neighbour block

Inputs of this process are

- a merge candidate list mergeCandList,

- a variable numMergeCand specifying the number of elements in mergeCandList,

- a candidate position indicator N

- a variable vspCandInListFlag specifying whether a VSP candidate is present in the list.

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height of the luma prediction block.

Outputs of this process are

- a potentially modified merge candidate list mergeCandList,

- a potentially modified variable numMergeCand specifying the number of elements in mergeCandList,

- a potentially modified variable vspCandInListFlag specifying whether a VSP candidate is present in the list.

The variable nbIsVspFlag is derived as specified in the following:

- If N is equal to A1 , the variable nbIsVspFlag is set equal to VspModeFlag[ xP − 1 ][ yP + nPbH − 1 ],

- Otherwise, if N is equal to B1 , nbIsVspFlag is set equal to VspModeFlag[ xP + nPbW − 1 ][ yP − 1 ],

- Otherwise, if N is equal to B0 , the variable nbIsVspFlag is set equal to VspModeFlag[ xP + nPbW ][ yP − 1 ],

- Otherwise, if N is equal to A0 , the variable nbIsVspFlag is set equal to VspModeFlag[ xP − 1 ][ yP + nPbH ],

- Otherwise, if N is equal to B2 , the variable nbIsVspFlag is set equal to VspModeFlag[ xP − 1 ][ yP − 1 ],

- Otherwise, ( N is equal to T ), the following applies:

- Let textPic be the picture with PicOrderCntVal and ViewIdx equal to PicOrderCnt and ViewIdx of the current picture and DepthFlag being equal to 0

- The variable textVspModeFlag is set equal to VspModeFlag of picture textPic.

- The variable nbIsVspFlag is set equal to textVspModeFlag[ xP + (nPbW <<  1) ][ yP + (nPbH <<  1) ]

- The variables mergeCandList, numMergeCand and vspCandInListFlag are modified as specified in the following:

- If nbIsVspFlag is equal to 0, the entry mergeCandList[ numMergeCand ] is set equal to N and the variable numMergeCand is increased by 1.

- Otherwise, ( nbIsVspFlag is equal to 1 ), when vspCandInListFlag is equal to 0, the following applies:

- The variable vspCandInListFlag is set equal to 1.

- The entry mergeCandList[ numMergeCand ] is set equal to VSP

- The variable numMergeCand is set equal to (numMergeCand + 1)

        - Derivation process for spatial merge candidates

The specifications in subclause 8.5.2.1.2 apply, with the following changes:

The following paragraph

- N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

is removed.

        - Derivation process for combined bi-predictive merging candidates

The specifications in subclause 8.5.2.1.3 apply.

        - Derivation process for zero motion vector merging candidates

The specifications in subclause 8.5.2.1.4 apply.

        - Derivation process for luma motion vector prediction

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- the reference index refIdxLX (with X being equal to 0 or 1) specifying a reference picture in the reference picture list RefPicListLX.

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Output of this process is

- the motion vector predictor mvpLX (with X being equal to 0 or 1).

The motion vector predictor mvpLX is derived as specified in the following.

- If mvp_lX_idx[ xP ][ yP ] is not equal to 0 or iv_mv_pred_flag[ nuh_layer_id ] is equal to 0, the derivation process for a non-inter-view luma motion vector prediction as specified in subclause H.13.5.2.1.5.1 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the size of the current luma coding block nCS, the width and the height of the luma prediction block nPbW and nPbH, the reference index refIdxLX (with X being 0 or 1, respectively), the partition index partIdx, and the motion vector predictor index mvp_lX_idx[ xP ][ yP ] − iv_mv_pred_flag[ nuh_layer_id ] as the inputs and the output is the motion vector predictor mvpLX (with X being equal to 0 or 1)

- Otherwise (if mvp_lX_idx[ xP ][ yP ] is equal to 0 and iv_mv_pred_flag[ nuh_layer_id ] is equal to 1) the following applies.

- The derivation process for the inter-view motion vector predictor candidate as specified in subclause H.13.5.2.1.5.2 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPbW and nPbH, the partition index partIdx, the reference index refIdxLX as the inputs and the outputs are the view order index of the reference, refViewIdx, the flag availableFlagLXInterView and the motion vector mvLXInterView (with X being equal to 0 or 1).

- The motion vector predictor mvpLX is derived as follows.

    - If availableFlagLXInterView is equal to 1, the motion vector predictor mvpLX is set equal to mvLXInterView.

    - Otherwise (if availableFlagLXInterView is equal to 0), the following applies;

        - mvpLX[ 0 ] = 0		(H-93)
mvpLX[ 1 ] = 0		(H-94)

          - Derivation process for a non-inter-view luma motion vector prediction

The specifications in subclause 8.5.2.1.5 apply with the following additional input parameter.

- a motion vector candidate index mvpIdxLX (with X being equal to 0 or 1)

and the following changes.

- mvp_lX_flag[ xP][ yP ] is replaced by mvpIdxLX.

- All invocations of the process specified in subclause 8.5.2.1.6 are replaced with invocations of the process specified in subclause H.13.5.2.1.6

- All invocations of the process specified in subclause 8.5.2.1.7 are replaced with invocations of the process specified in subclause H.13.5.2.1.7, with the input variable mergeFlag being equal to 0.

          - Derivation process for the inter-view motion vector predictor candidate

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

- a reference index refIdxLX (with X being equal to 0 or 1) specifying a reference picture in the reference picture list RefPicListLX.

Outputs of this process are:

- a view order index refViewIdx specifying a reference view,

- a flag availableFlagLXInterView specifying whether the inter-view motion vector candidate is available,

- a motion vector candidate mvLXInterView (if availableFlagLXInterView is equal to 1).

The derivation process for a disparity vector as specified in subclause H.13.5.4 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, the partition index partIdx and the variable deriveFromDepthFlag being equal to 1, as the inputs and the view order index refViewIdx, a flag availableDV and a disparity vector mvDisp as the outputs. The variable refPicViewIdx is set equal to the variable ViewIdx of the RefPicListLX[ refIdxLX ].

[Ed. (GT): refViewIdx might be used here to further restrict the reference views]

Depending on the value of refPicViewIdx, the following applies:

- If refPicViewIdx is not equal to ViewIdx, the derivation process for a disparity inter-view motion vector candidate as specified in subclause H.13.5.2.1.11 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the prediction list indication being equal to X, the view identifier refViewIdx, the disparity vector mvDisp, the flag mergeFlag being equal to 0, and the reference index refIdxLX, as the inputs and the outputs are the flag availableFlagLXInterView, the motion vector candidate mvLXInterView, and the reference index refIdxLX.

- Otherwise (refPicViewIdx is equal to ViewIdx), the derivation process for a temporal inter-view motion vector candidate as specified in subclause H.13.5.2.1.10 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the prediction list indication being equal to X, the reference view identifier refViewIdx, the disparity vector mvDisp, the flag mergeFlag being equal to 0, and the reference index refIdxLX, as the inputs and the outputs are the flag availableFlagLXInterView and the motion vector candidate mvLXInterView.

[Ed. Note (CY): consider adding definition for inter-view disparity motion vector and disparity vector.]

        - Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1),

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A, or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The function ViewId( picX ) is specified as follows:

- ViewId( picX ) = view_id of the picture picX		(H-95)

The variable currViewId is set equal to view_id[ nuh_layer_id ]..

If RefPicListX[ refIdxLX ] is a long-term reference picture, the variable currRefIsLtFlag is set equal to 1, otherwise ( RefPicListX[ refIdxLX ] is not a long-term reference picture ), currRefIsLtFlag is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

  - The sample location (xA0, yA0) is set equal to (xP − 1, yP + nPbH) and the sample location (xA1, yA1) is set equal to (xA0, yA0 − 1).

  - The availability flag availableFlagLXA is set equal to 0 and the both components of mvLXA are set equal to 0.

  - The availability derivation process for a prediction block as specified in subclause 6.4.2 is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xN, yN ) set equal to ( xA0, yA0 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableA0.

  - The availability derivation process for a prediction block as specified in subclause 6.4.2 is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xN, yN ) set equal to ( xA1, yA1 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableA1.

  - When availableA0 or availableA1 is equal to TRUE, the variable isScaledFlagLX is set equal to 1.

  - For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ), the following applies repeatedly until availableFlagLXA is equal to 1:

    - When availableAk is equal to TRUE, PredMode[ xAk][ yAk ] is not equal to MODE_INTRA and availableFlagLXA is equal to 0, the following applies.

      - If, PredFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the following assignments are made.

          - mvLXA = MvLX[ xAk ][ yAk ]		(H-96)
refIdxA = RefIdxLX[ xAk ][ yAk ]		(H-97)

      - Otherwise, the following applies:

      - If RefPicListY[ refIdxLY[ xAk ][ yAk ] is a long-term reference picture, the variable nbRefIsLtFlag is set equal to 1, otherwise ( RefPicListY[ refIdxLY[ xAk ][ yAk ] is not a long-term reference picture ), nbRefIsLtFlag is set equal to 0.

      - When PredFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListY[ refIdxLY[ xAk ][ yAk ] ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] ) and currRefIsLtFlag is equal to nbRefIsLtFlag, availableFlagLXA is set equal to 1 and the following assignments are made.
[Ed. (GT): Test of ViewIdx might be required here. Moreover there seems to be some redundant code in software trying to add this candidate again (with additional ViewId test) in function xAddMVPCandOrder. (#9)]

            - mvLXA = MvLY[ xAk ][ yAk ]		(H-98)
refIdxA = RefIdxLY[ xAk ][ yAk ]	(H-99)

  - When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ), the following applies repeatedly until availableFlagLXA is equal to 1:

    - When availableAk is equal to TRUE, PredMode[ xAk][ yAk ] is not equal to MODE_INTRA and availableFlagLXA is equal to 0, the following applies.

      - If PredFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1 and the following assignments are made.

          - mvLXA = MvLX[ xAk ][ yAk ]		(H-100)
refIdxA = RefIdxLX[ xAk ][ yAk ]		(H-101)
refPicListA = RefPicListX		(H-102)

      - Otherwise, if PredFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1 and the following assignments are made.

          - mvLXA = MvLY[ xAk ][ yAk ]		(H-103)
refIdxA = RefIdxLY[ xAk ][ yAk ]		(H-104)
refPicListA = RefPicListY		(H-105)

    - If RefPicListA[ refIdxA ] is a long-term reference picture, the variable nbRefIsLtFlag is set equal to 1, otherwise ( RefPicListA[ refIdxA ] is not a long-term reference picture ), nbRefIsLtFlag is set equal to 0.

    - When none of the following conditions is true,

    - availableFlagLXA is equal to 0,

    - nbRefIsLtFlag is not equal to currRefIsLtFlag,

    - currRefIsLtFlag is equal to 0 and PicOrderCnt( refPicListA[ refIdxA ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] )

    - currRefIsLtFlag is equal to 1, and ViewId( refPicListA[ refIdxA ] ) is equal to ViewId( refPicListX[ refIdxX ] ).

    - currRefIsLtFlag is equal to 1 and ( ( ViewIdx != 0 ) && iv_mv_scaling_flag ) is equal to 0.

      - mvLXA is derived as specified below.

        - tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td		(H-106)

        - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(H-107)

        - mvLXA = Clip3( −32768, 32767, Sign2( distScaleFactor * mvLXA ) * 	
	( (Abs( distScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(H-108)

where td and tb are derived as

- If RefPicListX[ refIdxLX ] is short-term reference picture,

            - td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( refPicListA[ refIdxA ] ) )	(H-109)

            - tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ) )	(H-110)

            - Otherwise ( RefPicListX[ refIdxLX ] is not a short-term reference picture)

            - td = Clip3( −128, 127, currViewId – ViewId( refPicListA[ refIdxA ] ) )	(H-111)

            - tb = Clip3( −128, 127, currViewId – ViewId( RefPicListX[ refIdxLX ] ) )	(H-112)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

  - Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPbW, xB1 = xB0− 1, xB2 = xP − 1 and yBk = yP − 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and its extended line.

  - When yP−1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), the following applies.

      - xB0 = (xB0>>3)<<3) + ((xB0>>3)&1)*7		(H-113)
xB1 = (xB1>>3)<<3) + ((xB1>>3)&1)*7		(H-114)
xB2 = (xB2>>3)<<3) + ((xB2>>3)&1)*7		(H-115)

  - The availability flag availableFlagLXB is set equal to 0 and the both components of mvLXB are set equal to 0.

  - For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPbW, xB1 = xB0 − 1, and xB2 =  xP − 1, the following applies repeatedly until availableFlagLXB is equal to 1:

    - The availability derivation process for a prediction block as specified in subclause 6.4.2 is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xN, yN ) set equal to ( xBk, yBk ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableBk.

    - When availableBk is equal to TRUE and availableFlagLXB is equal to 0, the following applies.

      - If PredFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the following assignments are made.

          - mvLXB = MvLX[ xBk ][ yBk ]		(H-116)
refIdxB = RefIdxLX[ xBk ][ yBk ]		(H-117)

        - the following applies:

        - If RefPicListY[ refIdxLY[ xAk ][ yAk ] is a long-term reference picture, the variable nbRefIsLtFlag is set equal to 1, otherwise ( RefPicListY[ refIdxLY[ xAk ][ yAk ] is not a long-term reference picture ), nbRefIsLtFlag is set equal to 0.

        - When, if PredFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListY[ refIdxLY[ xBk ][ yBk ] ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] ) and currRefIsLtFlag is equal to nbRefIsLtFlag, availableFlagLXB is set equal to 1 and the following assignments are made.

            - mvLXB = MvLY[ xBk ][ yBk ]		(H-118)
refIdxB = RefIdxLY[ xBk ][ yBk ]		(H-119)
[Ed. (GT): Test of ViewId might be required here. Moreover there seems to be some redundant code in software trying to add this candidate again (with additional ViewId test) in function xAddMVPCandOrder. (#9)]

  - When isScaledFlagLX is equal to 0 and availableFlagLXB is equal to 1, availableFlagLXA is set equal to 1 and the following assignments are made.

      - mvLXA = mvLXB		(H-120)
refIdxA = refIdxLXB		(H-121)

  - When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPbW, xB1 = xB0 − 1, and xB2 = xP − 1, the following applies repeatedly until availableFlagLXB is equal to 1:

    - The availability derivation process for a prediction block as specified in subclause 6.4.2 is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xN, yN ) set equal to ( xBk, yBk ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableBk.

    - When availableBk is equal to TRUE and availableFlagLXB is equal to 0, the following applies.

      - If PredFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1 and the following assignments are made.

          - mvLXB = MvLX[ xBk ][ yBk ]		(H-122)
refIdxB = RefIdxLX[ xBk ][ yBk ]		(H-123)
refPicListB = RefPicListX		(H-124)

      - Otherwise, if PredFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1 and the following assignments are made.

          - mvLXB = MvLY[ xBk ][ yBk ]		(H-125)
refIdxB = RefIdxLY[ xBk ][ yBk ]		(H-126)
refPicListB = RefPicListY		(H-127)

    - If RefPicListB[ refIdxB ] is a long-term reference picture, the variable nbIsLtFlag is set equal to 1, otherwise ( RefPicListB[ refIdxB ] is not a long-term reference picture ), nbIsLtFlag is set equal to 0.

    - When none of the following conditions is true,

    - availableFlagLXB is equal to 0,

    - nbRefIsLtFlag is not equal to currRefIsLtFlag,

    - currRefIsLtFlag is equal to 0 and PicOrderCnt( refPicListB[ refIdxB ] ) is equal to PicOrderCnt( RefPicListX[ refIdxLX ] )

    - currRefIsLtFlag is equal to 1, and ViewId( refPicListB[ refIdxB ] ) is equal to ViewId( refPicListX[ refIdxX ] ).

    - currRefIsLtFlag is equal to 1 and ( ( ViewIdx != 0 ) && iv_mv_scaling_flag ) is equal to 0.

      - mvLXB is derived as specified below.

        - tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td		(H-128)

        - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(H-129)

        - mvLXB =Clip3( −32768, 32767, Sign2( distScaleFactor * mvLXB ) * 
	( (Abs( distScaleFactor * mvLXB ) + 127 ) >> 8 ) )	(H-130)

      - where td and tb are derived as

      - If RefPicListX[ refIdxLX ] is short-term reference picture,

            - td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( refPicListB[ refIdxB ] ) )	(H-131)

            - tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ) )	(H-132)

            - Otherwise ( RefPicListX[ refIdxLX ] is not a short-term reference picture)

            - td = Clip3( −128, 127, currViewId – ViewId( refPicListA[ refIdxB ] ) )	(H-133)

            - tb = Clip3( −128, 127, currViewId – ViewId( RefPicListX[ refIdxLX ] ) )	(H-134)

        - Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a reference index refIdxLX (with X being 0 or 1),

- a flag mergeFlag specifying whether a merge candidate is derived.

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol,

- the reference index refIdxLX.

The function ViewId( picX ) is specified as follows:

- ViewId( picX ) = view_id of the picture picX		(H-135)

The function RefViewId( picX, refIdx, LX ) returns the view_id value of the reference picture with index refIdx from reference picture list LX of the picture picX and is specified as follows.

- RefViewId( picX, refIdx, LX ) = ViewId( RefPicListX[ refIdx ] of the picture picX)	(H-136)

- [Ed. (GT). The functions specified above might be specified in the scope view_id is parsed ].

The function RefPicOrderCnt( picX, refIdx, LX ) returns the picture order count PicOrderCntVal of the reference picture with index refIdx from reference picture list LX of the picture picX and is specified as follows.

- RefPicOrderCnt( picX, refIdx, LX ) = PicOrderCnt(RefPicListX[ refIdx ] of the picture picX)	(H-137)

Depending on the values of slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the picture that contains the collocated partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList1[ collocated_ref_idx ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P), the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList0[ collocated_ref_idx ].

Variable colPb and its position ( xPCol, yPCol ) are derived in the following ordered steps:

  - The variable colPb is derived as follows

      - yPRb = yP + nPbH		(H-138)

    - If ( yP >> Log2CtbSizeY ) is equal to ( yPRb >> Log2CtbSizeY ), the horizontal component of the right-bottom luma location of the current luma prediction block is defined by

        - xPRb = xP + nPbW		(H-139)

- and the variable colPb is set as the luma prediction block covering the modified location given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

    - Otherwise ( ( yP >> Log2CtbSizeY ) is not equal to ( yPRb >> Log2CtbSizeY ) ), colPb is marked as "unavailable".

  - When colPb is coded in an intra prediction mode or colPb is marked as "unavailable", the following applies.

    - Central luma location of the current prediction block is defined by

        - xPCtr = ( xP + ( nPbW >> 1 ) )		(H-140)

        - yPCtr = ( yP + ( nPbH >> 1 ) )		(H-141)

    - The variable colPb is set as the luma prediction block covering the modified location given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

  - ( xPCol, yPCol ) is set equal to the top-left sample of the colPb relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If one or more of the following conditions are true, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

    - colPb is coded in an intra prediction mode.

    - colPb is marked as "unavailable".

    - slice_temporal_mvp_enable_flag is equal to 0.

    - Otherwise, the following ordered steps apply.

    - The motion vector mvCol, the reference index refIdxCol, and the reference list identifier listCol are derived as follows.

      - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to MvL1[ xPCol ][ yPCol ], RefIdxL1[ xPCol ][ yPCol ], and L1, respectively. 
[Ed. (GT): Issue in base spec (JCTVC-J1003_d7: PredFlagL0,MvL1 and RefIdxL1 of which picture?]

      - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

      - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to MvL0[ xPCol ][ yPCol ], RefIdxL0[ xPCol ][ yPCol ], and L0, respectively.

      - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

      - If PicOrderCnt( pic ) of every picture pic in every reference picture lists is less than or equal to PicOrderCntVal, mvCol, refIdxCol, and listCol are set equal to MvLX[ xPCol ][ yPCol ], RefIdxLX[ xPCol ][ yPCol ] and LX, respectively with X being the value of X this process is invoked for.

      - Otherwise (PicOrderCnt( pic ) of at least one picture pic in at least one reference picture list is greater than PicOrderCntVal, mvCol, refIdxCol and listCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxLN[ xPCol ][ yPCol ] and LN, respectively with N being the value of collocated_from_l0_flag.

      - The variables curIvFlag and colIvFlag, specifying whether inter-view prediction is utilized for the current and collocated PU are derived as:

        - curIvFlag = RefPicListX[ refIdxLX ] is a long term reference picture ? 1 : 0	(H-142)

        - colIvFlag = LongTermRefPic( colPic, refIdxCol, listCol ) 	(H-143)

        - When curIvFlag is not equal to colIvFlag, mergeFlag is equal to 1 and AltRefIdxLX is not equal to −1, the variables refIdxCol and colIvFlag are modified as follows:

        - refIdxLX = AltRefIdxLX		(H-144)

- curIvFlag = RefPicListX[ refIdxLX ] is a long term reference picture ? 1 : 0	(H-145)

- The motion vector mvLXCol is modified as follows.

      - If curIvFlag is not equal to colIvFlag, availableFlagLXCol is set equal to 0.

      - Otherwise, ( curIvFlag is equal to colIvFlag), availableFlagLXCol is set equal to 1 and the following applies.

      - The variables distCur and distCol specifying a POC or ViewId difference are derived as follows.

      - If curIvFlag is equal to 0 or ( ( ViewIdx != 0 ) && iv_mv_scaling_flag ) is equal to 0, the following applies.

              - distCur = PicOrderCntVal − PicOrderCnt( RefPicListX[ refIdxLX ] )	(H-146)

              - distCol = PicOrderCnt( colPic ) − RefPicOrderCnt( colPic, refIdxCol, listCol )	(H-147)

              - Otherwise, ( curIvFlag is equal to 1 and ( ( ViewIdx != 0 ) && iv_mv_scaling_flag ) is equal to 1), the following applies.

              - distCur = view_id[ nuh_layer_id ] − ViewId( RefPicListX[ refIdxLX ] )	(H-148)

              - distCol = ViewId( colPic ) − ViewId( colPic, refIdxCol, listCol )	(H-149)

              - The motion vector mvLXCol is derived as follows:

              - If distCur is equal to distCol, mvLXCol is set equal to mvCol.

              - Otherwise, ( distCur is not equal to distCol ),

              - mvLXCol is derived as scaled version of the motion vector mvCol as specified in the following:

                - td = Clip3( −128, 127, distCol )	(H-150)

                - tb = Clip3( −128, 127, distCur )	(H-151)

                - tx = ( 16384 + ( Abs( td ) >>1 ) ) / td	(H-152)

                - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(H-153)

                - mvLXCol =  Clip3( −32768, 32767, Sign2( distScaleFactor * mvCol ) *
( (Abs( distScaleFactor * mvCol ) + 127 ) >> 8 ) )	(H-154)

        - Derivation process for chroma motion vectors

The specifications in subclause 8.5.2.1.8 apply.

        - Derivation process for inter-view merge candidates

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with X being 0 or 1, respectively)

- a view order index refViewIdx specifying a reference view.

- the availability flags availableFlagIvMC and availableFlagIvDC specifying whether the inter-view merge candidates are available,

- the reference indices refIdxLXIvMC and refIdxLXIvDC,

- the prediction list utilization flags predFlagLXIvMC and predFlagLXIvDC,

- the motion vectors mvLXIvMC and mvLXIvDC,

- the disparity vector mvDisp.

The derivation process for a disparity vector as specified in subclause H.13.5.4 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, the partition index partIdx, the variable deriveFromDepthFlag being equal to 1, as the inputs and the outputs are and the view order index of the reference view refViewIdx, the flag availableDV and the disparity vector mvDisp.

The temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

    - The derivation process for a temporal inter-view motion vector candidate as specified in subclause H.13.5.2.1.10 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the prediction list indication X , the view identifier refViewIdx, the disparity vector mvDisp, the flag mergeFlag being equal to 1, and the reference index refIdxLX being equal to −1, and as the inputs and the outputs are the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

  - The availability flag availableFlagIvMC, and the prediction utilization flags predFlagL0IvMC and predFlagL1IvMC are derived by

      - availableFlagIvMC = availableFlagL0IvMC | | availableFlagL1IvMC	(H-155)
predFlagL0IvMC = availableFlagL0IvMC		(H-156)
predFlagL1IvMC = availableFlagL1IvMC		(H-157)

The disparity inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

    - The derivation process for the disparity inter-view motion vector candidate as specified in subclause H.13.5.2.1.11 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the view identifier refViewIdx, the disparity vector mvDisp, the prediction list indication X, the flag mergeFlag being equal to 1, and the reference index refIdxLX being equal to −1, as the inputs and the outputs are the flag availableFlagLXIvDC, the motion vector mvLXIvDC, and the reference index refIdxLXIvDC.

  - The availability flag availableFlagIvDC, and the prediction utilization flags predFlagL0IvDC and predFlagL1IvDC are derived by

      - availableFlagIvDC = availableFlagL0IvDC | | availableFlagL1IvDC 	(H-158)
predFlagL0IvDC = availableFlagL0IvDC		(H-159)
predFlagL1IvDC = availableFlagL1IvDC		(H-160)

        - Derivation process for a temporal inter-view motion vector candidate

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a prediction list indication X,

- a reference view index refViewIdx.

- a disparity vector mvDisp,

- a flag mergeFlag specifying whether a merge candidate is derived,

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX.

Outputs of this process are:

- a flag availableFlagLXInterView specifying whether the temporal inter-view motion vector candidate is available,

- a temporal inter-view motion vector candidate mvLXInterView (if availableFlagLXInterView is equal to 1),

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX,

The flag availableFlagLXInterView is set equal to 0, both components of mvLXInterView are set equal to 0.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

The reference layer luma location ( xRef, yRef ) is derived by

- xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 ) + ( ( mvDisp[ 0 ] + 2 ) >> 2 ) )	(H-161)
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 ) >> 1 ) + ( ( mvDisp[ 1 ] + 2 ) >> 2 ))	(H-162)

Let refCU be the coding unit that covers the luma location ( xRef, yRef ) in the view component with ViewIdx equal to refViewIdx.

[Ed. (GT:) In software refCU is the coding unit that covers the luma location ( xRef, yRef ) in the view component that closest (in terms of ViewIdx) to the current view component and that is included in one of the reference lists of the current view component.(#6)]

[Ed. (CY): What is implemented in the software has no impact on the final results in common test condition, so it is preferred to have the current text and software changes may be desirable. ]

When the variable PredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following ordered steps apply, for Y in the range of X to (1 – X), inclusive:

  - The variable refPredFlagLY is set equal to the prediction utilization flag predFlagLY of the prediction unit refPU.

  - The variable refRefIdxLY, is set equal to the reference index refIdxLY of  the prediction unit refPU.

  - The variable refMvLY is set equal to the motion vector mvLY of the prediction unit refPU.

  - The variable refRefPicListLY, is set equal to the reference picture list RefPicListLY of the prediction unit refCU.

  - When refPredFlagLY is equal to 1, the following applies for each i from ( mergeFlag ? 0 : refIdxLX ) to ( mergeFlag ? num_ref_idx_lX_active_minus1 : refIdxLX), inclusive:

    - When availableFlagLXInterView is equal to 0, and the picture order count of the picture refRefPicListLY[ refRefIdxLY ] is equal to the picture order count of the picture RefPicListLX[ i ], the flag availableFlagLXInterView is set equal to 1 and the following applies.

      - The motion vector mvLXInterView is derived by:

        - mvLXInterView[ 0 ] = refMvLY[ 0 ]		(H-163)
mvLXInterView[ 1 ] = refMvLY[ 1 ]		(H-164)

      - When mergeFlag is equal to 1, the reference index refIdxLX is derived by:

        - refIdxLX = i		(H-165)

        - Derivation process for a disparity inter-view motion vector candidate

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a prediction list indication X,

- a  reference view index refViewIdx,

- a disparity vector mvDisp,

- a flag mergeFlag specifying whether a merge candidate is derived,

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX.

Outputs of this process are:

- a flag availableFlagLXInterView specifying whether the disparity inter-view motion vector candidate is available,

- a disparity inter-view motion vector candidate mvLXInterView (if availableFlagLXInterView is equal to 1),

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX.

[GT (Ed.) This derivation process might be changed to use refViewIdx to restrict the reference view to be the base view or the view mvDisp is related to. ]

The flag availableFlagLXInterView is set equal to 0, both components of mvLXInterView are set equal to 0.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

For each i from ( mergeFlag ? 0 : refIdxLX ) to ( mergeFlag ? num_ref_idx_lX_active_minus1 : refIdxLX), inclusive, the following applies:

- When PicOrderCntVal of the picture RefPicListX[ i ] is equal to the PicOrderCntVal or the current picture and availableFlagLXInterView is equal to 0, availableFlagLXInterView is set equal to 1 and the following applies:

    - The motion vector mvLXInterView is derived by

        - mvLXInterView[ 0 ] = mvDisp[ 0 ]		(H-166)
mvLXInterView[ 1 ] = 0		(H-167)

    - When mergeFlag is equal to 1, the reference index refIdxLX is derived by

        - refIdxLX = i		(H-168)

        - Derivation process for a view synthesis prediction merge candidate

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- a view order index refViewIdx specifying a reference view,

- the availability flag availableFlagVSP whether the VSP merge candidate is available,

- the reference indices refIdxL0VSP and refIdxL1VSP ,

- the prediction list utilization flags predFlagL0VSP and predFlagL1VSP,

- the motion vectors mvL0VSP and mvL1VSP.

The derivation process for a disparity vector as specified in subclause H.13.5.4 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, the partition index partIdx and the flag deriveFromDepthFlag being equal to 1 as the inputs and the outputs are the flag availableDV, the view order index of the reference view refViewIdx and the disparity vector mvDisp.

The variable availableFlagVSP is set equal to 1 and for X in the range of 0 to 1, inclusive, the following applies:

- mvLXVSP = ( X = = 0 ) ? mvDisp : ( 0, 0 )		(H-169)
refIdxLXVSP = −1 		(H-170)
predFlagLXVSP = ( 1 − X )		(H-171)

        - Derivation process for the texture merging candidate

This process is not invoked when DepthFlag is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are:

- a flag availableFlagT specifying whether the texture merging candidate is available,

- the prediction utilization flags predFlagL0T and predFlagL1T,

- the reference indices refIdxL0T and refIdxL1T (when availableFlagT is equal to 1),

- the motion vectors mvL0T and mvL1T (when  availableFlagT is equal to 1).

The variable availableFlagT is set equal to 0. The variables predFlagL0T and predFlagL1T are set equal to 0. The variables refIdxL0T and refIdxL1T are set equal to −1. Both components of the motion vectors mvL0T and mvL1T are set equal to 0.

The texture luma location ( xRef, yRef ) is derived by:

- xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 ) )	(H-172)

- yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 ) >> 1 ) )	(H-173)

- [ Ed. (GT): Is clipping necessary? ]

Let textPic be the picture with PicOrderCntVal and ViewIdx equal to PicOrderCnt and ViewIdx of the current picture and DepthFlag being equal to 0 and let textPU the PU at covering the position ( xRef, yRef ) in textPic.

For X in the range of 0 to 1, inclusive, the following applies:

[Ed. (GT): It should be verified that the picture with refIdxLX is also a reference picture for depth. (#10)]

  - The variable textPredFlagLX is set equal to PredFlagLX of textPic. The variable textRefIdxLX is set equal to RefIdxLX of textPic. The variable textMvLX is set equal to the MvLX of textPic.

  - When X is equal to 0 or the current slice is a B slice, the following applies:

    - When textPredFlagLX[ xRef ][ yRef ] is equal to 1, the following applies:

      - mvLXT[ 0 ] = ( textMvLX[ xRef ][ yRef ][ 0 ] + 2 ) >> 2	(H-174)

      - mvLXT[ 1 ] = ( textMvLX[ xRef ][ yRef ][ 1 ] + 2 ) >> 2	(H-175)

      - refIdxLX = textRefIdxLX[ xRef ][ yRef ]		(H-176)

      - predFlagLXT = 1		(H-177)

      - availableFlagT = 1		(H-178)

      - Decoding process for inter prediction samples

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	a variable nCS specifying the size of the current luma coding block,

–	variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

–	luma motion vectors mvL0 and mvL1,

–	chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1.

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma residual samples for the component Cr, where nCSC is derived as specified below.

The variable nCSL is set equal to nCS and the variable nCSC is set equal to nCS >> 1.

- If VspModeFlag[ xC + xB ][ yC + yB ] is equal to 0, the following ordered steps apply:

  - Let predSamplesL0L and predSamplesL1L be (nPbW)x(nPbH) arrays of predicted luma sample values and predSampleL0Cb, predSampleL1Cb, predSampleL0Cr, and predSampleL1Cr be (nPbW/2)x(nPbH/2) arrays of predicted chroma sample values.

  - For X being replaced by either 0 or 1, the following applies:

    - When predFlagLX is equal to 1, the following applies.

      - The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdxLX as input.

      - Depending on DepthFlag, the following applies:

      - If DepthFlag is equal to 0, the arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the fractional sample interpolation process specified in subclause 8.5.2.2.2 with the luma locations ( xC, yC ), ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr given as input.

      - Otherwise, (DepthFlag is equal to 1), arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr are derived by invoking the full sample interpolation process specified in subclause H.13.5.2.2.4 with the luma locations ( xC, yC ), ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, the motion vectors mvLX, mvCLX, and the reference arrays with refPicLXL, refPicLXCb and refPicLXCr given as input.

  - Depending on ic_flag, the arrays predSamplesL ,predSampleCb, and predSampleCr are derived as specified in the following:

    - If ic_flag is equal to 0, the following applies.

      - The array predSampleL of the prediction samples of luma component is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the luma location ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, and the sample arrays predSamplesL0L and predSamplesL1L as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1 and cIdx equal to 0 given as input.

      - The array predSampleCb of the prediction samples of component Cb is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCb set equal to nPbW/2, nPbHCb set equal to nPbH/2, and the sample arrays predSamplesL0Cb and predSamplesL1Cb as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 1 given as input.

      - The array predSampleCr of the prediction samples of component Cr is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCr set equal to nPbW/2, nPbHCr set equal to nPbH/2, and the sample arrays predSamplesL0Cr and predSamplesL1Cr as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 2 given as input.

    - Otherwise ( ic_flag is equal to 1), the following applies.

      - The array predSampleL of the prediction samples of luma component is derived by invoking the illumination compensated sample prediction process specified in subclause H.13.5.2.2.4, with the luma location ( xC, yC ), the size of the current luma coding block nCS, the luma location ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, and the sample arrays predSamplesL0L and predSamplesL1L as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvL0, mvL1 and cIdx equal to 0 given as input.

      - The array predSampleCb of the prediction samples of component Cb is derived by invoking the illumination compensated sample prediction process specified in subclause H.13.5.2.2.4, with the luma location ( xC, yC ), the size of the current luma coding block nCS, with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCb set equal to nPbW/2, nPbHCb set equal to nPbH/2, and the sample arrays predSamplesL0Cb and predSamplesL1Cb as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvCL0, mvCL1, and cIdx equal to 1 given as input.

      - The array predSampleCr of the prediction samples of component Cr is derived by invoking the illumination compensated sample prediction process specified in subclause H.13.5.2.2.4, with the luma location ( xC, yC ), the size of the current luma coding block nCS, with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCr set equal to nPbW/2, nPbHCr set equal to nPbH/2, and the sample arrays predSamplesL0Cr and predSamplesL1Cr as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvCL0, mvCL1, and cIdx equal to 2 given as input.

- Otherwise, ( VspModeFlag[ xC + xB ][ yC + yB ] is equal to 0 ), the following applies:

- The arrays predSamplesL ,predSampleCb, and predSampleCr are derived by invoking the view synthesis prediction process as specified in subclause H.13.5.2.2.7, with the luma locations ( xC, yC ), ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, and the motion vector mvL0 as the inputs and the outputs are the sample arrays predSamplesLXL, predSamplesLXCb, and predSamplesLXCr.

When ResPredFlag[ xC + xB ][ yC + yB ] is equal to 1, the inter-view residual prediction process as specified in subclause H.13.5.2.2.6 is invoked with the luma locations ( xC, yC ) and ( xC + xB, yC + yB ), the size of the current luma coding block nCS, the variables nPSW and nPSH, and the arrays predSamplesL, predSamplesCb, and predSamplesCr as the inputs and the outputs are modified versions of the arrays predSamplesL, predSamplesCb, and predSamplesCr.

        - Reference picture selection process

The specifications in subclause 8.5.2.2.1 apply.

        - Fractional sample interpolation process

The specifications in subclause 8.5.2.2.2 apply.

        - Weighted sample prediction process

The specifications in subclause 8.5.2.2.3 apply.

        - Full sample interpolation process

Inputs to this process are:

- a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

- a luma location ( xB, yB ) specifying the top-left sample of the current luma prediction block relative to the top left sample of the current luma coding block,

- the width and height of the prediction block, nPbW and nPbH, in luma-sample units,

- a luma motion vector mvLX given in quarter-luma-sample units,

- a chroma motion vector mvCLX given in eighth-chroma-sample units,

- the reference picture sample arrays refPicLXL, refPicLXCb, and refPicLXCr.

Outputs of this process are:

- a (nPbW)x(nPbH) array predSampleLXL of prediction luma sample values,

- two (nPbW/2)x(nPbH/2) arrays predSampleLXCb, and predSampleLXCr of prediction chroma sample values.

The location ( xP, yP ) given in full-sample units of the upper-left luma samples of the current prediction block relative to the upper-left luma sample location of the given reference sample arrays is derived by

- xP = xC + xB		(H-179)
yP = yC + yB		(H-180)

Let ( xIntL, yIntL ) be a luma location given in full-sample units specifying sample locations inside the reference sample arrays refPicLXL.

For each luma sample location ( xL = 0..( nPbW − 1 ), yL = 0..( nPbH−1 ) ) inside the prediction luma sample array predSampleLXL, the corresponding prediction luma sample value predSampleLXL[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, are derived as specified in the following:

- xIntL = xP + mvLX[ 0 ] + xL		(H-181)
yIntL = yP + mvLX[ 1 ] + yL		(H-182)

- The prediction luma sample value predSampleLXL[ xL ][ yL ] is derived as specified in the following:

- predSampleLXL[ xL ][ yL ] = refPicLXL[ xIntL ][ yIntL ]		(H-183)

For each chroma sample location ( xC = 0..( nPbW/2−1), yC = 0..( nPbH/2−1 ) ) inside the prediction chroma sample arrays predSampleLXCb and predSampleLXCr, the corresponding prediction chroma sample values predSampleLXCb[ xC ][ yC ] and predSampleLXCr[ xC ][ yC ] are set to be equal to ( 1  <<  ( BitDepthC − 1 ) ).

[Ed. (GT): In current software and draft chroma planes are also present for depth. A general discussion is needed to specify how chroma planes are handled. (#12)]

        - Illumination compensated sample prediction process

Inputs to this process are:

- a location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left sample of the current picture,

- the size of current luma coding block nCS,

- a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

- the width and height of this prediction block, nPbW and nPbH,

- two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1,

- prediction list utilization flags, predFlagL0 and predFlagL1,

- reference indices, refIdxL0 and refIdxL1,

- motion vector mvL0 and mvL1

- colour component index, cIdx,

Outputs of this process are:

- the (nPbW)x(nPbH) array predSamples of prediction sample values.

Variables shift1, shift2, offset1 and offset2 are derived as follows.

- The variable shift1 is set equal to 14 − bitDepth and the variable shift2 is set equal to 15 − bitDepth,

- The variable offset1 is derived as follows.

- If shift1 is greater than 0, offset1 set equal to 1 << ( shift1 − 1 ).

- Otherwise (shift1 is equal to 0), offset1 is set equal to 0.

- The variable offset2 is set equal to 1 << ( shift2 − 1 ).

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to 1 or 2), bitDepth is set equal to BitDepthC.

The derivation process for illumination compensation mode availability and parameters as specified in subclause H.13.5.2.2.5.1 is invoked with the luma location ( xC, yC ), the size of the current luma coding block nCS, prediction list utilization flags, predFlagL0 and predFlagL1, reference indices refIdxL0 and refIdxL1, motion vectors mvL0 and mvL1, the bit depth of samples, bitDepth, a variable cIdx specifying colour component index as the inputs and the outputs are the flags puIcFlagL0 and puIcFlagL1, the variables icWeightL0 and icWeightL1 specifying weights for illumination compensation, the variables icOffsetL0 and icOffsetL1 specifying offsets for illumination compensation, the variables icShiftL0 and icShiftL1 specifying bit shifts for illumination compensation.

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x ][ y ] with x = 0..(nPbW)−1 and y = 0..(nPbH)−1 are derived as follows.

- If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

- clipPredVal= Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL0[ x ][ y ] + offset1 ) >> shift1 )	(H-184)

- predSamples[ x ][ y ] = !puIcFlagL0 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL0 ) >> icShiftL0 ) + icOffsetL0 ) 	(H-185)

- Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

- clipPredVal = Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL1[ x ][ y ] + offset1 ) >> shift1 )	(H-186)

- predSamples[ x ][ y ] = !puIcFlagL1 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL1 ) >> icShiftL1 ) + icOffsetL1 ) 	(H-187)

- Otherwise,

- predVal0 = !puIcFlagL0 ? predSamplesL0[ x ][ y ] : 
( ( predSamplesL0[ x ][ y ] * icWeightL0 ) >> icShiftL0  ) + ( icOffsetL0 << shift1 ) ) 	(H-188)

- predVal1 = !puIcFlagL1 ? predSamplesL1[ x ][ y ] : 
( ( predSamplesL1[ x ][ y ] * icWeightL1 ) >> icShiftL1 ) + ( icOffsetL1 << shift1 ) ) 	(H-189)

- predSamples[ x ][ y ] = 
Clip3( 0, ( 1 << bitDepth ) − 1, ( predVal0 + predVal1 + offset2 ) >> shift2 )	(H-190)

          - Derivation process for illumination compensation mode availability and parameters

Inputs to this process are:

- a luma location ( xCl, yCl ) specifying the top-left sample of the current coding block relative to the top left sample of the current picture,

- the size of the current luma coding block, nCSl,

- prediction list utilization flags, predFlagL0 and predFlagL1,

- reference indices refIdxL0 and refIdxL1,

- motion vectors mvL0 and mvL1

- a bit depth of samples, bitDepth.

- a variable cIdx specifying colour component index.

Outputs of this process are:

- flags puIcFlagL0 and puIcFlagL1 specifying whether illumination compensation is enabled.

- variables icWeightL0 and icWeightL1 specifying weights for illumination compensation

- variables icOffsetL0 and icOffsetL1 specifying offsets for illumination compensation

- variables icShiftL0 and icShiftL1 specifying bit shifts for illumination compensation.

The variables puIcFlagL0 and PuIcFlagL1 are set equal to 0, the variables icWeightL0 and icWeightL1 are set equal to 1, the variables icOffsetL0 and icOffsetL1 are set equal to 0 and the variables icShiftL0 and icShiftL1 are set equal to 0.

The variables nCS specifying the current luma or chroma coding block size, and the location ( xC, yC ) specifying the top left sample of the current luma or chroma coding block is derived as follows.

- nCS = ( cIdx = = 0 ) ? nCSl : nCSl / 2 		(H-191)

- ( xC, yC ) =  ( cIdx = = 0 ) ? ( xC, yC ) : ( xCl /2 , yCl / 2 ) 	(H-192)

The variable availFlagCurAboveRow specifying the availability of above neighbouring row samples is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4.1 with the location ( xCurr, yCurr ) set equal to ( xCl, yCl ) and the neighbouring location ( xN, yN ) set equal to ( xCl, yCl−1 ) as the input and the output is assigned to availFlagCurAboveRow.

The variable availFlagCurLeftCol specifying the availability of left neighbouring column samples is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4.1 with the location ( xCurr, yCurr ) set equal to ( xCl, yCl ) and the neighbouring location ( xN, yN ) set equal to ( xCl−1, yCl ) as the input and the output is assigned to availFlagCurLeftCol.

[Ed. (GT) The availability derivation is specified as performed in the software. However, the check of the availability of left and above PU might not be sufficient to guarantee the availability of the whole left column or above row. A check of availability similar to that used for intra prediction might be a better solution. ]

When availFlagCurAboveRow is equal to 0 and availFlagCurLeftCol is equal to 0 the whole derivation process of this subclause terminates.

For X being replaced by 0 and 1, when predFlagLX is equal to 1, the variable puIcFlagLX is derived by the following ordered steps.

  - The variable refPicLX specifying the reference picture from reference picture list X is set equal to RefPicListX[ refIdxLX ] .

  - If the view order index of the picture refPicLX is not equal to ViewIdx, the variable puIvPredFlagLX specifying whether inter-view prediction from List X is utilized is set equal to 1, otherwise (predFlagLX is equal to 0 or the view order index of the picture RefPicListX[ refIdxLX ]  is equal to ViewIdx ), puIvPredFlagLX is set equal to 0.

  - If puIvPredFlagLX is equal to 0, the variable puIcFlagLX is set equal to 0, otherwise (puIvPredFlagLX is equal to 1 ) the following applies.

    - The luma location (xRLX, yRLX) specifying the top-left sample of the reference block in refPicLX is derived as

        - xRLX = xC + ( mvLX[ 0 ] >> (2 + (cIdx ? 1 : 0)) )	(H-193)

        - yRLX = yC + ( mvLX[ 1 ] >> (2 + (cIdx ? 1 : 0)) ) 	(H-194)

    - The variable availFlagAboveRowLX specifying whether the above neighbouring row samples of the current block and the reference block are available is derived as specified in the following.

        - availFlagAboveRowLX = ( yRLX > 0 ) && availFlagCurAboveRow 	 (H-195)

    - The variable availFlagLeftColLX specifying whether the left neighbouring column samples of the current block and the reference block are available is derived as specified in the following.

        - availFlagLeftColLX = ( xRLX > 0 ) && availFlagCurLeftCol	(H-196)

    - The variable puIcFlagLX is derived as follows.

        - puIcFlagLX = availFlagAboveRowLX | | availFlagLeftColLX 	 (H-197)

Depending on the colour component cIdx, the variable curRecSamples specifying the reconstructed picture samples of the current picture is derived as

- curRecSamples = ( !cidx ) ? RecSamplesL : ( ( icdx == 1 ) ? RecSamplesCb : RecSamplesCr) 	(H-198)

- [Ed. (GT). The reconstructed samples before deblocking filter RecSamplesL, RecSamplesCb and RecSamplesCr as used above although not explicitly defined. However, they should be defined in the base spec. ]

- For X being replaced by 0 and 1, when puIcFlagLX is equal to 1, the variables icWeightLX, icOffsetLX and icShiftLX are derived by the following ordered steps.

  - Depending on the colour component cIdx, the variable refRecSamples specifying the reconstructed picture samples of the reference picture is derived as specified in the following.

    - If cIdx is equal to 0, refRecSamples is set equal to reconstructed picture sample array SL of picture refPicLX.

    - Otherwise, if cIdx is equal to 1, refRecSamples is set equal to the reconstructed chroma sample array SCb of picture refPicLX.

    - Otherwise (cIdx is equal to 2), refRecSamples is set equal to the reconstructed chroma sample array SCr of picture refPicLX.

  - The lists curNeighSampleListLX and refNeighSampleListLX specifying the neighbouring samples in the current picture and the reference picture are derived as specified in the following.

    - The variable numNeighSamplesLX specifying the number of elements in curNeighSampleListLX and in refNeighSampleLX is set equal to 0.

    - The variable leftNeighOffLX specifying the offset of the left neighbouring samples in curNeighSampleListLX and refNeighSampleLX is derived as

        - leftNeighOffLX = availFlagAboveRowLX ? 0 : nCS 	(H-199)

    - For i ranging from 0 to nCS−1, inclusive the following applies.

      - When availFlagAboveRowLX is equal to 1 the following applies.

          - curNeighSampleListLX[ i ] = curRecSamples[ xC + i][ yC − 1 ] 	(H-200)

          - refNeighSampleListLX[ i ] = refRecSamples[ xRLX + i ][ yRLX − 1 ]	(H-201)

          - numNeighSamplesLX += 1		(H-202)

      - When availFlagLeftColLX is equal to 1 the following applies

          - curNeighSampleListLX[ i + leftNeighOffLX ] = curRecSamples[ xC − 1][ yC + i ] 	(H-203)

          - refNeighSampleListLX[ i + leftNeighOffLX ] = refRecSamples[ xRLX − 1][ yRLX + i ] 	(H-204)

          - numNeighSamplesLX += 1		(H-205)

  - The derivation process for illumination compensation parameters as specified in subclause H.13.5.2.2.5.2 is invoked, with the list of neighbouring samples in the current picture curNeighSampleList, the list of neighbouring samples in the reference picture refNeighSample list, the number of neighbouring samples numNeighSamlesLX and the size of the current luma coding block nCSl as inputs and the illumination parameters icWeightLX, icOffsetLX and icShiftLX as outputs.

          - Derivation process for illumination compensation parameters

Inputs to this process are:

- a list curSampleList specifying the current samples, ,

- a list refSampleList specifying the reference samples,

- a variable numSamples specifying the number of elements in curSampleList and refSampleList.

- a bit depth of samples, bitDepth.

- the size of the current luma coding block nCSl

Outputs of this process are:

- a variable icWeight specifying a weight for illumination compensation,

- a variable icOffset specifying a offset for illumination compensation,

- a variable icShift specifying a bit shift for illumination compensation.

The variable icWeight is set equal to 1, the variable icOffset is set equal to 0, and the variable icShift is set equal to 0.

The variables sumRef, sumCur, sumRefSquare and sumProdRefCur are set equal to 0 and the following applies for i ranging from 0 to numSamples − 1, inclusive.

- sumRef += refSampleList[ i ]		(H-206)

- sumCur += curSampleList[ i ]		(H-207)

- sumRefSquare += ( refSampleList[ i ] * refSampleList[ i ] )	(H-208)

- sumProdRefCur += ( refSampleList[ i ] * curSampleList[ i ] ) 	(H-209)

The variables precShift and precOffset specifying the bit shift and offset needed to restrict precision to 16 bit are derived as

- precShift = Max( 0, bitDepth + Log2( nCSl) − 14 )		(H-210) 
precOffset =  1 << ( precShift − 1 )		(H-211)

The variables psSumRef, psSumCur, psSumRefSquare and psSumProdRefCur are derived as

- psSumRef = ( precShift > 0 ) ? (sumRef + precOffset) >> precShift: sumRef	(H-212)

- psSumCur = ( precShift > 0 ) ? (sumCur + precOffset ) >> precShift : sumCur	(H-213)

- psSumRefSquare = ( precShift > 0 ) ? (sumRefSquare + precOffset ) >> precShift : sumRefSquare	(H-214)

- psSumProdRefCur = ( precShift > 0 ) ? (sumProdRefCur + precOffset ) >> precShift : sumProdRefCur	(H-215)

The variable avgShift and avgOffset specifying the bit shift and offset needed for averaging are derived as

- avgShift = Ceil( Log2( numSamples ) ) − precShift		(H-216)

- avgOffset = 1 << ( avgShift − 1 )		(H-217)

When avgShift is equal to 0 the whole derivation process specified in this subclause terminates..

The variables numerDiv and denomDiv specifying numerator and denominator of a following divisions are derived as.

- numerDiv= ( psSumProdRefCur << avgShift ) – psSumRef * psSumCur	(H-218)

- denomDiv= ( psSumRefSquare << avgShift ) – psSumRef * psSumRef	(H-219)

The variables psShiftNumer and psShiftDenom specifying the bit shifts to restrict the precision of numerDiv and denomDiv to 15 bit and 6 bit, respectively are derived as

- psShiftNumer = Max( 0, Floor( Log2( Abs( numerDiv ) ) ) − 14)	(H-220)

- psShiftDenom = Max( 0, Floor( Log2( Abs( denomDiv ) ) ) − 5)	(H-221)

The variables psNumerDiv and psDenomDiv are derived as

- psNumerDiv = numerDiv >> psShiftNumer 		(H-222)

- psDenomDiv = denomDiv >> psShiftDenom 		(H-223)

The variable psIcWeight specifying the shifted weight for illumination compensation is derived as specified in the following.

- If psDenomDiv is greater than 0, the following applies,

    - The value of variable divCoeff  is derived from Table H-12 depending on psDenomDiv.

    - The value of psIcWeight is derived as

        - psIcWeight = psNumerDiv * divCoeff 		(H-224)

        - Otherwise( psDenomDiv is less or equal to 0), psIcWeight is set equal to 0.

The variable icShift specifying a bit shift for illumination compensation is set equal to 13.

The variable invPsShift specifying the number of bits needed to shift psIcWeight back to a range of 16 bit precision is derived as

- invPsShift = psShiftDenom – psShiftNumer + 15 – icShift	(H-225)

The variable invPsIcWeight specifying a weight for illumination compensation with 16 bit precision is derived as specified in the following

- If invPsShift is less than 0, the following applies:

      - invPsIcWeight = Clip3( psIcWeight << ( Abs( invPsShift ) ), −215, 215 − 1 )	(H-226)

      - Otherwise, ( invPsIcWeight is greater than or equal to 0), the following applies:

      - invPsIcWeight = Clip3( psIcWeight >> ( Abs( invPsShift ) ), −215, 215 − 1 )	(H-227)

The variable icWeight specifying a weight for illumination compensation with 7 bit precision is derived as specified in the following:

- If invPsIcWeight is greater than or equal to −26 and less than 26, the following applies.

      - icWeight = invPsIcWeight 		(H-228)

      - Otherwise, ( invPsIcWeight is less than −26 or greater than or equal to 26 ), the following applies.

      - decIcShift = Max( 0, Floor(Log2( Abs( icWeight ) ) − 5 ) )	(H-229)

      - [Ed (GT): In software a function counting leading zero ones is utilized to derive decIcShift. Does this match with draft text?]

      - icWeight = invPsIcWeight >> decIcShift		(H-230)

      - icShift −= decIcShift		(H-231)

The variable icOffset specifying an offset for illumination compensation is derived as:

- icOffset = ( psSumCur – ( ( icWeight*psSumRef ) >>  icShift ) + avgOffset ) >>  avgShift	(H-232)

Table H-12 – Specification of divCoeff depending on psDenomDiv

| psDenomDiv | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| divCoeff | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 | 2521 |
| psDenomDiv | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| divCoeff | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 | 1260 |
| psDenomDiv | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| divCoeff | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 | 840 |
| psDenomDiv | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 |
| divCoeff | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 | 630 |
| psDenomDiv | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 |  |
| divCoeff | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 | 512 |  |

        - Inter-view residual prediction process

The process is only invoked if res_pred_flag is equal to 1.

Inputs to this process are:

- a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,prediction list utilization flags, predFlagL0 and predFlagL1,

- a (nPSW)x(nPSH) array predSamplesL of luma prediction samples,

- two (nPSW / 2)x(nPSH / 2) arrays predSamplesCb and predSamplesCr of chroma prediction samples.

Output of this process are:

- a modified version of the (nPSW)x(nPSH) array predSamplesL,

- a modified versions of the (nPSW / 2)x(nPSH / 2) arrays predSamplesCb and predSamplesCr.

The derivation process for a disparity vector as specified in subclause H.13.5.4 is invoked with the luma locations ( xC, yC ) and ( xP, yP ), the coding block size nCS, the variables nPSW and nPSH, the partition index partIdx and the variable deriveFromDepthFlag being equal to 0, as the inputs and the outputs are the view order index refViewIdx, the flag availableDV and the disparity vector mvDisp.[Ed. (GT) partIdx is missing as input to this subclause.].

[Ed. (GT). In software refViewIdx is set equal to 0. Might be better to reuse disparity and refViewIdx from inter-view motion vector prediction. ].

Let refResSamplesL be the (PicWidthInSamplesL)x(PicHeightInSamplesL) array of luma residual samples ResSamplesL: of the view component with ViewIdx equal to refViewIdx. Let refResSamplesCb and refResSamplesCr be the (PicWidthInSamplesL / 2)x(PicHeightInSamplesL / 2) arrays of Cb and Cr residual samples ResSamplesCb and ResSamplesCr, respectively, for inter-coded coding units for the view component with ViewIdx equal to refViewIdx.

When the flag availableDV is equal to 0 the whole decoding process of this sub-clause terminates.

For y proceeding over the values 0..(nPSH – 1) and x proceeding over the values 0..(nPSW – 1), the following ordered steps apply.

  - The variables xR0, xR1, yR0, yR1 and w0, w1, w2, and w3 are derived by

      - xR0 = Clip3( 0, PicWidthInSamplesL – 1, xP + x + (mvDisp[ 0 ] >> 2 ) )	(H-233)
xR1 = Clip3( 0, PicWidthInSamplesL – 1, xP + x + (mvDisp[ 0 ] >> 2 ) + 1 )	(H-234)
yR0 = Clip3( 0, PicHeightInSamplesL – 1, yP + y + (mvDisp[ 1 ] >> 2 ) )	(H-235)
yR1 = Clip3( 0, PicHeightInSamplesL – 1, yP + y + (mvDisp[ 1 ] >> 2 ) + 1 )	(H-236)
w0 = 4 – mvDisp[ 0 ] + ( ( mvDisp[ 0 ] >> 2 ) << 2 )	(H-237)
w1 = mvDisp[ 0 ] − ( ( mvDisp[ 0 ] >> 2 ) << 2 )	(H-238)
w2 = 4 – mvDisp[ 1 ] + ( ( mvDisp[ 1 ] >> 2 ) << 2 )	(H-239)
w3 = mvDisp[ 1 ] − ( ( mvDisp[ 1 ] >> 2 ) << 2 )	(H-240)

  - The sample predSamplesL[ x ][ y ] is modified by

      - deltaL1 = ( w0 * refResSamplesL[ xR0 ][ yR0 ] + w1 * refResSamplesL[ xR1 ][ yR0 ] + 4 ) >> 3	(H-241)
deltaL2 = ( w0 * refResSamplesL[ xR0 ][ yR1 ] + w1 * refResSamplesL[ xR1 ][ yR1 ] + 4 ) >> 3	(H-242)
deltaL = ( w2 * deltaL1 + w3 * deltaL2 + 4 ) >> 3	(H-243)
predSamplesL[ x ][ y ] = predSamplesL[ x ][ y ] + deltaL	(H-244)

For y proceeding over the values 0..(nPSH / 2 – 1) and x proceeding over the values 0..(nPSW / 2 – 1), the following ordered steps are specified:

  - The variables xR0, xR1, yR0, yR1 and w0, w1, w2, and w3 are derived by

      - xR0 = Clip3( 0, PicWidthInSamplesL / 2 – 1, xP / 2 + x + (mvDisp[ 0 ] >> 3 ) )	(H-245)
xR1 = Clip3( 0, PicWidthInSamplesL / 2 – 1, xP / 2 + x + (mvDisp[ 0 ] >> 3 ) + 1 )	(H-246)
yR0 = Clip3( 0, PicHeightInSamplesL / 2 – 1, yP / 2 + y + (mvDisp[ 1 ] >> 3 ) )	(H-247)
yR1 = Clip3( 0, PicHeightInSamplesL / 2 – 1, yP / 2 + y + (mvDisp[ 1 ] >> 3 ) + 1 )	(H-248)
w0 = 8 – mvDisp[ 0 ] + ( (mvDisp[ 0 ] >> 3 ) << 3 )	(H-249)
w1 = mvDisp[ 0 ] − ( (mvDisp[ 0 ] >> 3 ) << 3 )	(H-250)
w2 = 8 – mvDisp[ 1 ] + ( (mvDisp[ 1 ] >> 3 ) << 3 )	(H-251)
w3 = mvDisp[ 1 ] − ( (mvDisp[ 1 ] >> 3 ) << 3 )	(H-252)

  - The sample predSamplesCb[ x ][ y ] is modified by

      - deltaCb1 = ( w0 * refResSamplesCb[ xR0 ][ yR0 ] + w1 * refResSamplesCb[ xR1 ][ yR0 ] + 8 ) >> 4	(H-253)
deltaCb2 = ( w0 * refResSamplesCb[ xR0 ][ yR1 ] + w1 * refResSamplesCb[ xR1 ][ yR1 ] + 8 ) >> 4	(H-254)
deltaCb  = ( w2 * deltaCb1 + w3 * deltaCb2 + 8 ) >> 4	(H-255)
predSamplesCb[ x ][ y ] = predSamplesCb[ x ][ y ] + deltaCb 	(H-256)

  - The sample predSamplesCr[ x ][ y ] is modified by

      - deltaCr1 = ( w0 * refResSamplesCr[ xR0 ][ yR0 ] + w1 * refResSamplesCr[ xR1 ][ yR0 ] + 8 ) >> 4	(H-257)
deltaCr2 = ( w0 * refResSamplesCr[ xR0 ][ yR1 ] + w1 * refResSamplesCr[ xR1 ][ yR1 ] + 8 ) >> 4	(H-258)
deltaCr = ( w2 * deltaCr1+ w3 * deltaCr2) + 8 ) >> 4	(H-259)
predSamplesCr[ x ][ y ] = predSamplesCr[ x ][ y ] + deltaCr 	(H-260)

        - View synthesis prediction process

Inputs to this process are:

- a location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left sample of the current picture,

- a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

- the width and height of this prediction block, nPbW and nPbH,

- a motion vector mvL0

Outputs of this process are:

–	an array predSamplesL of luma prediction samples,

–	an array predSamplesCb of chroma prediction samples for the component Cb

–	an array predSamplesCr of chroma prediction samples for the component Cr

The location ( xP, yP ) given in full-sample units of the upper-left luma samples of the current prediction block relative to the upper-left luma sample location of the current picture is derived by:

- xP = xC + xB		(H-261)
yP = yC + yB		(H-262)

The variable refViewIdx is set equal to RefViewIdx[ xP ][ yP ].

The variable refIdx is set equal to – 1 and the following applies:

- For X in the range of 0 to 1, inclusive, the following applies:

- For i in the range of 0 to NumRefPicsLX – 1, inclusive, the following applies:

    - When refIdx is equal to – 1, ViewIdx of picture RefPicListX[ i ] is equal to refViewIdx, the variable Y is set equal to X, and refIdx is set equal to i.

[Ed. (GT): In software always the base-view with current POC is used, regardless of being in a reference list or not. ]

The reference picture consisting of an ordered two-dimensional array refPicL of luma samples and two ordered two-dimensional arrays refPicCb and refPicCr of chroma samples is derived by invoking the process specified in subclause 8.5.2.2.1 with refIdx as input.

The derivation process for disparity sample array as specified in section H.13.5.4.3 is invoked with the luma location ( xP, yP ), the disparity vector mvDisp being equal to mvL0, the variable refViewIdx, the variables nPSW and nPSH, the variable nSubBlkW being equal 4 the variable nSubBlkW being equal to 4, the look-up table LUT_constraint_VSP storing constraint disparity parameter and the flag restMaxSearchFlag being equal to 0, as the inputs, and the output is the array disparitySamples of size (nPSW)x(nPSH).

Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in quarter-sample units.

Let cParam be the constraint disparity parameter given by LUT_constraint_vsp[ nPbW ][ nPbH ].

Two variables min and max for constraint disparity range are set to 0 and the following applies

If disparitySamples[ 0 ][ 0 ] is negative

- For each luma sample location ( xL = 0..nPbW−1, yL = 0..nPbH−1 ) inside the prediction luma sample array predSamplesL, find the minimum value min =  xL+ disparitySamples[ xL ][ yL ] and max is set equal to min+cParam+nPbW-1

Otherwise,

- For each luma sample location ( xL = 0..nPbW−1, yL = 0..nPbH−1 ) inside the prediction luma sample array predSamplesL, find the maximum value max =  xL+ disparitySamples[ xL ][ yL ] and min is set equal to max-cParam-nPbW+1

For each luma sample location ( xL = 0..nPbW−1, yL = 0..nPbH−1 ) inside the prediction luma sample array predSamplesL, the corresponding prediction luma sample value predSamplesL[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL, and yFracL are derived by

- xIntL = xP + xL+ clip3(min, max, disparitySamples[ xL ][ yL ])		(H-263)
yIntL = yP + yL		(H-264)

- if ( (xL+ disparitySamples[ xL ][ yL ]) < min || (xL+ disparitySamples[ xL ][ yL ]) > max)

- xFracL = 0		(H-265)
yFracL = 0		(H-266)

- else

- xFracL = disparitySamples[ xL ][ yL ] & 3		(H-267)
yFracL = 0		(H-268)

- The prediction luma sample value predSamplesL[ xL][ yL ] is derived by invoking the process specified in subclause 8.5.2.2.2.1 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicL given as input.

Table H-13 – LUT_constraint_vsp[ nPbW ][ nPbH ]

| nPSW | nPbH | cParam |
| --- | --- | --- |
| 4 | 8 | 12 |
| 8 | 4 | 32 |
| 8 | 8 | 16 |
| 8 | 16 | 8 |
| 16 | 8 | 24 |
| 16 | 16 | 12 |
| 16 | 32 | 6 |
| 32 | 16 | 20 |
| 32 | 32 | 10 |
| 32 | 64 | 5 |
| 64 | 32 | 18 |
| 64 | 64 | 9 |

[Ed. (GT): As for inter prediction the treatment of colour planes for depth needs to be discussed. In software colour planes are set to 128 in VSP process. (#12)]

Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in one-eighth sample units.

For each chroma sample location ( xC = 0..nPbW/2−1, yC = 0..nPbH/2−1 ) inside the prediction chroma sample arrays predSamplesCb and predSamplesCr, the corresponding prediction chroma sample values predSampleLXCb[ xC ][ yC ] and predSamplesCr[ xC ][ yC ] are derived as follows:

- The variables xIntC, yIntC, xFracC, and yFracC are derived by

- xIntC = ( xP / 2 ) + xC + disparitySamples[ xC << 1 ][ yC << 1 ]	(H-269)
yIntC = ( yP / 2 ) + yC		(H-270)

- xFracC = disparitySamples[ xC << 1][ yC << 1 ] & 7		(H-271)
yFracC = 0		(H-272)

- The prediction sample value predSamplesCb[ xC ][ yC ] is derived by invoking the process specified in subclause 8.5.2.2.2.2 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicCb given as input.

- The prediction sample value predSamplesCr[ xC ][ yC ] is derived by invoking the process specified in subclause 8.5.2.2.2.2 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicCr given as input.

    - Decoding process for the residual signal of coding units coded in inter prediction mode

The specifications in subclause 8.5.3 apply, with the following addition at the end of the subclause:

For x in the range of 0 to nCSL − 1 and y in the range of 0 to nCSL − 1, the following applies:

- ResSamplesL[ xC + x ][ yC + y ] is set equal to resSamplesL[ x ][ y ].

For x in the range of 0 to nCSC − 1 and y in the range of 0 to nCSC − 1, the following applies:

- ResSamplesCb[ xC /2 + x ][ yC /2 + x] is set equal to resSamplesCb[ x ][ y ].

- ResSamplesCr[ xC /2 + x ][ yC /2 + x ] is set equal to resSamplesCr[ x ][ y ].

    - Derivation process for a disparity vector

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

- a flag deriveFromDepthFlag specifying whether the disparity vector is derived form a depth component

- Outputs of this process are:

- a view order index refViewIdx specifying a reference view.

- a flag availableDV specifying whether the disparity vector is available,

- a disparity vector mvDisp.

The flag availableDV is set equal to 0, and both components of the disparity vector mvDisp are set equal to 0.

The variable checkParallelMergeFlag is derived as follows.

- If one or more of the following conditions are true, checkParallelMergeFlag is set equal to 1.

- PredMode[ xC ][ yC ] is equal to MODE_SKIP

- PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1.

- Otherwise, checkParallelMergeFlag is set equal to 0.

The derivation process for a disparity vector from temporal neighbour block as specified in H.13.5.4.1 is invoked with the luma location ( xP, yP ), variables nPSW and nPSH as inputs, and flag availableDV, and disparity vector mvDisp outputs.

When availableDV is equal to 0, for each N being A1, B1, B0, A0, or B2, and ( xN, yN ) being ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW − 1,  yP − 1 ), ( xP + nPSW,  yP − 1 ), ( xP − 1,  yP + nPSH  ), or ( xP − 1,  yP − 1 ) the following ordered steps apply.

  - When yP−1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), the following applies.

        - xB0 = ((xB0>>3)<<3) + ((xB0>>3)&1)*7		(H-273)
xB1 = ((xB1>>3)<<3) + ((xB1>>3)&1)*7		(H-274)
xB2 = ((xB2>>3)<<3) + ((xB2>>3)&1)*7		(H-275)

  - The availability derivation process for a prediction block as specified in subclause 6.4.2 is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the partition index partIdx, and the luma location ( xN, yN ) as inputs and the output is assigned to the prediction block availability flag availableN.

  - When all of the following conditions are true, availableA1 is set equal to 0.

    - N is equal to A1

    - PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N

    - partIdx is equal to 1

  - When all of the following conditions are true, availableB1 is set equal to 0.

    - N is equal to B1

    - PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD

    - partIdx is equal to 1

  - When all of the following conditions are true, availableN is set equal to 0.

    - checkParallelMergeFlag is equal to 1

    - (xP >> (log2_parallel_merge_level_minus2 + 2) ) is equal to
(xN >> (log2_parallel_merge_level_minus2 + 2) )

    - (yP >> (log2_parallel_merge_level_minus2 + 2) ) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2) ).

  - The flag availableIvpMvSearchFlagN is set equal to availableN.

  - When one of the following conditions is true, availableIvpMvSearchFlagN is set equal to 0.

    - N is equal to B0 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

    - N is equal to B1 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

    - N is equal to B2 and (( yN >> Log2CtbSizeY ) << Log2CtbSizeY) is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY)

  - The flags availableFlagIvpMvN0 and availableFlagIvpMvN1 are set equal to 0.

  - For each X from 0 to 1, the following applies:

    - When availableDV is equal to 0, availableN is equal to 1 and PredFlagLX[ xN ][ yN ] is equal to 1, the following applies:

      - If RefIdxLX[ xN ][ yN ] is greater than or equal to 0, the following applies:

      - If RefPicListX[ RefIdxLX[ xN ][ yN ] ] is an inter-view reference picture, the following applies:

            - refViewIdx = ViewIdx( RefPicListX[ RefIdxLX[ xN ][ yN ] ] )	(H-276)
mvDisp = MvLXN[ xN ][ yN ]		(H-277)
availableDV = 1		(H-278)

            - Otherwise (RefPicListX[ RefIdxLX[ xN ][ yN ] ] is not an inter-view reference picture), the following applies:

            - When availableIvpMvSearchFlagN is equal to 1 and PredMode[ xN ][ yN ] is equal to MODE_SKIP and IvpMvFlagLX[ xN ][ yN ] is equal to 1, the following applies:

              - ivpMvDispLXN = IvpMvDispLX[ xN ][ yN ]	(H-279)
availableFlagIvpMvLXN = 1		(H-280)

      - Otherwise ( RefIdxLX[ xN ][ yN ] is less than 0 ), the following applies:

      - When VspModeFlag[ xN ][ yN ] is equal to 1, the following applies:

            - refViewIdx = RefViewIdx[ xN ][ yN ]	(H-281)
mvDisp = MvLXN[ xN ][ yN ]		(H-282)
mvDisp[ 1 ] = 0		(H-283)
availableDV = 1		(H-284)

            - [Ed. (GT): In software refViewIdx is set equal to 0. (#19)]

When availableDV is equal to 0 for each N being A1, B1, B0, A0, and B2, the following applies.

- For each X from 0 to 1, the following applies.

    - When availableDV is equal to 0 and availableFlagIvpMvLXN is equal to 1, the following applies:

        - mvDisp = ivpMvDispLXN		(H-285)
availableDV = 1		(H-286)

When availableDV is equal to 1 and deriveFromDepthFlag is equal to 1, the following ordered steps apply:

  - The derivation process for disparity sample array as specified in subclause H.13.5.4.3 is invoked with the luma locations xP, yP, the disparity vector mvDisp, the view identifier refViewIdx, the variables nPSW, nPSH, the variable nSubBlkW equal to nPSW, the variable nSubBlkH equal to nPSH, and the flag restMaxSearchFlag being equal to 1 as the inputs, and the output is the array disparitySamples of size (nPSWDs)x(nPSHDs).

  - The horizontal component of the disparity vector mvDisp[ 0 ] is set equal to disparitySamples[ 0 ][ 0 ].

[Ed. (GT): The disparity vector derivation has the problem related to refViewIdx as described in the editor's comment in subclause H.13.5.2.1 and needs further discussion. (#5) ]

      - Derivation process for a disparity vector from temporal neighbour blocks

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH

Outputs of this process are

- the disparity vector mvDisp,

- the availability flag availableFlag.

- The luma location ( xPRb , yPRb) specifying the position of the right bottom luma prediction block of the current luma prediction block is derived as follows:

- yPRb = yP + nPbH		(H-287)

- xPRb = xP + nPbW		(H-288)

The flag availableRb specifying the availability of the right bottom luma prediction block is derived as follows:

- If all of the following conditions are true, availableRb is set equal to 1.

- xPRb is less than pic_width_in_luma_samples,

- yPRb is less than pic_height_in_luma_samples,

- ( yP  >>  Log2CtbSizeY ) is equal to ( yPRb  >>  Log2CtbSizeY )

- Otherwise, availableRb is set equal to 0.

The luma location ( xCtr , yCtr) specifying the centre position of the current luma prediction block is derived as follows:

- xPCtr = xP + ( nPbW >> 1 )		(H-289)

- yPCtr = yP + ( nPbH >> 1 )		(H-290)

The flag availableFlag is set equal to 0, and both components of mvDisp are set equal to 0.

For i from 0 to NumDdvCandPics, the following ordered steps apply and the whole decoding process of this sub-clause terminates once availableFlag is set to 1.

  - When availableRb is equal to 1, the derivation process for a disparity vector in a block of a candidate picture as specified in subclause H.13.5.4.2 is invoked with candidate picture DdvCandPicsList[ i ], luma location ( xPRb , yPRb ) as inputs, and the flag availableFlag and the disparity vector mvDisp as outputs.

  - The derivation process for a disparity vector in a block of a candidate picture as specified in subclause H.13.5.4.2 is invoked with candidate picture DdvCandPicsList[ i ], luma location (xPCtr, yPCtr) as inputs, and the flag availableFlag, the disparity vector mvDisp as outputs.

      - Derivation process for a disparity vector in a block of a candidate picture

Inputs to this process are:

–	a candidate picture candPic,

–	a luma location ( xPCand , yPCand ) relative to the top-left luma sample of the candidate picture.

Outputs of this process are:

–	a flag availableDV specifying whether the disparity vector is available,

–	a disparity vector mvDisp.

The motion vector mvDisp and the availability flag availableDV are derived as specified by the following ordered steps and the whole decoding process of this sub-clause terminates once availableDV is set equal to 1.

  - The variable colPu is set as the prediction unit in candPic covering the position ( ( xPCand >>4) <<4 ,  (yPCand >>4) <<4 ).

  - The position ( xPCol, yPCol ) is set equal to the position of the top-left sample of colPu relative to the top-left luma sample of the candPic.

  - If slice_type is equal to B, the variable dir is set equal to collocated_from_l0_flag, otherwise, dir is set equal to 1 – collocated_from_l0_flag.

  - The flag availableDV is set equal to 0, and both components of mvDisp are set equal to 0.

  - For each X from dir to 1 – dir, inclusive, the following applies:

    - The variable candPicRefPicList is set equal to the reference picture list RefPicListX of candPic.

    - The variable candPredFlag is set equal to the prediction list utilization flag PredFlagLX of candPic.

    - The variable candRefIdx is set equal to the reference indices RefIdxLX of candPic.

    - The variable candMV is set equal to the motion vectors mvLX of candPic.

    - When the ViewIdx of candPicRefPicList[ candRefIdx[ xPCol ][ yPCol ] ] is not equal to the ViewIdx of candPic and candPredFlag[ xPCol ][ yPCol ] is equal to 1 and , the following applies:

        - mvDisp = candMV[ xPCol ][ yPCol ]		(H-291)
availableDV = 1		(H-292)

      - Derivation process for a disparity sample array

- Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a disparity vector mvDisp,

- a view identifier refViewIdx specifying a reference view,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit.

- variable nSubBlkW and nSubBlkW specifying the conversion precision of the corresponding depth samples,

- a flag restMaxSearchFlag specifying whether the search for the maximum disparity is restricted.

Outputs of this process are:

- a (nPSW)x(nPSH) array disparitySamples of disparities values.

Let refDepPels be an array of reconstructed depth samples of the depth view component with ViewIdx equal to refViewIdx. The luma location (xTL, yTL) of top-left luma sample of refDepPels is derived by

- xTL = Clip3( 0, pic_width_in_luma_samples – nPSW, xP + ( mvDisp[ 0 ] >> 2 ) )	(H-293)
yTL = Clip3( 0, pic_height_in_luma_samples – nPSH, yP + ( mvDisp[ 1 ] >> 2 ) )	(H-294)

The array disparitySamples of size (nPSW)x(nPSH) is derived as specified in the following:

- For sBy in the range of 0 to ( ( nPSH / nSubBlkH) –1 ), inclusive, the following applies:

- For sBx in the range of 0 to ( ( nPSW / nSubBlkW) –1 ), inclusive, the following applies:

    - The variable maxDep is set equal to –1 and modified as specified in the following.

      - If restMaxSearchFlag is equal to 0, the following applies:

for ( yOff = 0; yOff < nSubBlkH; yOff++ )
					for ( xOff = 0; xOff < nSubBlkW; xOff++ ) {
						x = xTL + sBx * nSubBlkW + xOff
						y = yTL + sBy * nSubBlkH + yOff
						maxDep = Max( maxDep, refDepPels[ x ][ y ] )
					}

      - Otherwise ( restMaxSearchFlag is equal to 1), the following applies:

x = xTL + sBx * nSubBlkW
				y = yTL + sBy * nSubBlkH
				maxDep = Max( maxDep, refDepPels[ x ][ y ] )
				maxDep = Max( maxDep, refDepPels[ x ][ y + nSubBlkH – 1] )
				maxDep = Max( maxDep, refDepPels[ x + nSubBlkW – 1][ y ] )
				maxDep = Max( maxDep, refDepPels[ x + nSubBlkW – 1][ y + nSubBlkH – 1] )
				maxDep = Max( maxDep, refDepPels[ x + nSubBlkW / 2 ][ y + nSubBlkH / 2 ] )

          - [Ed. (GT): Currently restMaxSearchFlag is only equal to 1, when nSubBlkW is equal to nPSW and when nSubBlkH is equal to nPSH ]

    - The values of the array depthSamples are derived as specified in the following:

for ( yOff = 0; yOff < nSubBlkH; yOff++ )
				for( xOff = 0; xOff < nSubBlkW; xOff++ ) {
					x = sBx * nSubBlkW + xOff 
					y = sBy * nSubBlkH + yOff 
					disparitySamples[ x ][ y ] = DepthToDisparityB[ refViewIdx ][ maxDep ]
				}

  - Scaling, transformation and array construction process prior to deblocking filter process

The specifications in subclause 8.6 apply.

  - In-loop filter process

The specifications in subclause 8.7 apply.

- Parsing process

The specifications in clause 9 apply with the following modifications

–	All references to the process specified in subclause 7.3 are replaced with references to the process specified in subclause H.12.3 .

–	All invocations of the process specified in subclause 9.1 are replaced with invocations of the process specified in subclause H.14.1.

  - Parsing process for Truncated Unary codes

The specifications in subclause 9.1 apply with the following modifications.

–	All references to the process specified in subclause 7.3 are replaced with references to the process specified in subclause H.12.3.

  - Parsing process for 0-th order Exp-Golomb codes

The specifications in subclause 9.2 apply with the following modifications .

–	All references to the process specified in subclause 7.3 are replaced with references to the process specified in subclause H.12.3.

–	All invocations of the process specified in subclause 9.2.1 are replaced with invocations of the process specified in subclause H.14.2.1.

    - Mapping process for signed Exp-Golomb codes

The specifications in subclause 9.2.1 apply with the following modifications.

–	All references to the process specified in subclause 9.2 are replaced with references to the process specified in subclause H.14.2

  - CABAC parsing process for slice data

The specifications in subclause 9.3 apply with the following modifications.

–	All references to the process specified in subclauses 7.3.6 to 7.3.13 are replaced with references to the process specified in subclauses H.12.3.6 to H.12.3.13

–	All invocations of the process specified in subclause 9.3.1 are replaced with invocations of the process specified in subclause H.14.3.1.

–	All invocations of the process specified in subclause 9.3.1.2 are replaced with invocations of the process specified in subclause H.14.3.1.2.

–	All invocations of the process specified in subclause 9.3.1.3 are replaced with invocations of the process specified in subclause H.14.3.1.3.

–	All invocations of the process specified in subclause 9.3.1.4 are replaced with invocations of the process specified in subclause H.14.3.1.4.

–	All invocations of the process specified in subclause 9.3.2 are replaced with invocations of the process specified in subclause H.14.3.2.

    - Initialization process

The specifications in subclause 9.3.1 apply with the following modifications.

–	All references to the process specified in subclauses 7.3.6 to 7.3.13 are replaced with references to the process specified in subclauses H.12.3.6 to H.12.3.13

–	All invocations of the process specified in subclause 9.3.1.1 are replaced with invocations of the process specified in subclause H.14.3.1.1.

–	All invocations of the process specified in subclause 9.3.1.2 are replaced with invocations of the process specified in subclause H.14.3.1.2.

      - Initialization process for context variables

The specifications in subclause 9.3.1.1 apply with the following modifications.

–	All references to the process specified in subclauses 7.3.6 to 7.3.13 are replaced with references to the process specified in subclauses H.12.3.6 to H.12.3.13

–	Table H-14 is appended to the end of Table 9-4.

–	Table H-15 to Table H-28 are appended to the end of the subclause.

Table H-14 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | dmm_flag | Table H-15 | 0 | 1 | 2 |
|  | dmm_mode | Table H-16 | 0 | 1 | 2 |
|  | wedge_full_tab_idx | Table H-17 | 0 | 1 | 2 |
|  | wedge_predtex_tab_idx | Table H-18 | 0 | 1 | 2 |
|  | dmm_delta_end_flag dmm_delta_end_abs_minus1 | Table H-19 | 0 | 1 | 2 |
|  | dmm_dc_1_abs dmm_dc_2_abs | Table H-20 | 0 | 1 | 2 |
|  | edge_code | Table H-21 | 0 | 1 | 2 |
|  | ic_flag | Table H-22 |  | 0 | 1 |
|  | mvp_l0_idx mpv_l1_idx | Table H-23 |  | 0..1 | 2..3 |
|  | sdc_flag | Table H-24 | 0..2 | 3..5 | 6..8 |
|  | sdc_residual_flag | Table H-25 | 0..1 | 2..3 | 4..5 |
|  | sdc_residual_sign_flag | Table H-26 | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 | Table H-27 | 0..19 | 20..39 | 40..59 |
|  | sdc_pred_mode | Table H-28 | 0..2 | 3..5 | 6..8 |

[Ed (GT). In software one table with different offsets is used for wedge_full_tab_idx, dmm_delta_end_flag, dmm_delta_end_abs_minus1, dmm_dc_1_abs and dmm_dc_2_abs ].

[Ed (FJ). In software the tables for sdc_residual_flag, sdc_sign_flag, sdc_residual_data and sdc_pred_mode are larger, but many entries are not used. Therefore, only used entries are defined here. Needs software fix. ].

Table H-15 – Values of variable initValue for dmm_flag ctxIdx

| Initialization variable | dmm_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-16 – Values of variable initValue for dmm_mode ctxIdx

| Initialization variable | dmm_mode |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-17 – Values of variable initValue for wedge_full_tab_idx ctxIdx

| Initialization variable | wedge_full_tab_idx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-18– Values of variable initValue for wedge_predtex_tab_idx ctxIdx

| Initialization variable | wedge_predtex_tab_idx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-19– Values of variable initValue for wedge_predtex_tab_idx ctxIdx

| Initialization variable | dmm_delta_end_flag dmm_delta_end_abs_minus1 |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-20 – Values of variable initValue for dmm_dc_1_abs and dmm_dc_2_abs ctxIdx

| Initialization variable | dmm_dc_1_abs dmm_dc_2_abs |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-21 – Values of variable initValue for edge_code ctxIdx

| Initialization variable | edge_code |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table H-22 – Values of variable initValue for ic_flag ctxIdx

| Initialization variable | ic_flag |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | 197 | 197 |

Table H-23 – Values of variable initValue for mvp_l0_idx and mvp_l1_idx ctxIdx

| Initialization variable | mvp_l0_idx and mvp_l1_idx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| initValue | 168 | 168 | 154 | 154 |

Table H-24 – Values of variable initValue for sdc_flag ctxIdx

| Initialization variable | sdc_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 0 | 0 | 64 | 0 | 0 | 0 | 64 | 0 | 0 |

Table H-25 – Values of variable initValue for sdc_residual_flag ctxIdx

| Initialization variable | sdc_residual_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 251 | 255 | 0 | 0 | 56 | 0 |

Table H-26 – Values of variable initValue for sdc_residual_sign_flag ctxIdx

| Initialization variable | sdc_residual_sign_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 255 | 255 | 0 | 252 | 55 | 0 |

Table H-27 – Values of variable initValue for sdc_residual_abs_minus1 ctxIdx

| Initialization variable | sdc_residual_abs_minus1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| initValue | 255 | 255 | 64 | 0 | 2 | 0 | 64 | 0 | 6 | 0 |
|  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| initValue | 67 | 0 | 8 | 0 | 61 | 0 | 7 | 0 | 47 | 0 |
|  | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| initValue | 243 | 255 | 12 | 0 | 14 | 0 | 33 | 0 | 243 | 255 |
|  | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| initValue | 2 | 0 | 66 | 0 | 0 | 0 | 63 | 0 | 1 | 0 |
|  | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| initValue | 12 | 0 | 50 | 0 | 14 | 0 | 27 | 0 | 0 | 0 |
|  | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| initValue | 239 | 255 | 14 | 9 | 27 | 0 | 0 | 0 | 239 | 255 |

Table H-28 – Values of variable initValue for sdc_pred_mode ctxIdx

| Initialization variable | sdc_pred_mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 9 | 255 | 70 | 0 | 9 | 255 | 70 | 0 | 9 |

      - Memorization process for context variables

The specifications in subclause 9.3.1.2 apply with the following modifications

–	All references to the process specified in subclauses 7.3.6 to 7.3.13 are replaced with references to the process specified in subclauses H.12.3.6 to H.12.3.13

      - Synchronization process for context variables

The specifications in subclause 9.3.1.3 apply with the following modifications

–	All references to the process specified in subclauses 7.3.6 to 7.3.13 are replaced with references to the process specified in subclauses H.12.3.6 to H.12.3.13

      - Initialization process for the arithmetic decoding engine

The specifications in subclause 9.3.1.4 apply.

    - Binarization process

The specifications in subclause 9.3.2 apply with the following modifications.

–	Table H-29 is appended to the end of Table 9-37.

Table H-29 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| dmm_flag | 0 | FL, cMax = 1 | 0 | Table H-15 | 0 |
|  | 1 |  | 0 | Table H-15 | 1 |
|  | 2 |  | 0 | Table H-15 | 2 |
| dmm_mode | 0 | FL, cMax = 7 | 0 | Table H-16 | 0 |
|  | 1 |  | 0 | Table H-16 | 1 |
|  | 2 |  | 0 | Table H-16 | 2 |
| wedge_full_tab_idx | 0 | FL, cMax = wedgeFullTabIdxBits[ log2PbSize ] (defined in Table H-30) | 0 | Table H-17 | 0 |
|  | 1 |  | 0 | Table H-17 | 1 |
|  | 2 |  | 0 | Table H-17 | 2 |
| wedge_predtex_tab_idx | 0 | FL, cMax = wedgePredTexTabIdxBits[ log2PbSize ] (defined in Table H-31) | 0 | Table H-18 | 0 |
|  | 1 |  | 0 | Table H-18 | 1 |
|  | 2 |  | 0 | Table H-18 | 2 |
| dmm_delta_end_flag | 0 | FL, cMax = 1 | 0 | Table H-19 | 0 |
|  | 1 |  | 0 | Table H-19 | 1 |
|  | 2 |  | 0 | Table H-19 | 2 |
| dmm_delta_end_abs_minus1 | 0 | FL, cMax = 3 | 0 | Table H-19 | 0 |
|  | 1 |  | 0 | Table H-19 | 1 |
|  | 2 |  | 0 | Table H-19 | 2 |
| dmm_delta_end_sign_flag | na | FL, cMax = 1 | na | na | na, (Bypass) |
| dmm_dc_1_abs dmm_dc_2_abs | 0 | UEG0 [Ed. (GT) To be specified] | 0 | Table H-20 | 0 |
|  | 1 |  | 0 | Table H-20 | 1 |
|  | 2 |  | 0 | Table H-20 | 2 |
| dmm_dc_1_sign_flag dmm_dc_2_sign_flag | na | FL, cMax = 1 | na | na | na, (Bypass) |
| edge_code | 0 | TU, cMax = 6 | 0 | Table H-21 | 0 |
|  | 1 |  | 0 | Table H-21 | 1 |
|  | 2 |  | 0 | Table H-21 | 2 |
| ic_flag | 1 | FL, cMax = 1 | 0 | Table H-22 | 0 |
|  | 2 |  | 0 | Table H-22 | 1 |
| mvp_l0_idx mvp_l1_idx | 1 | FL, cMax = 1 + iv_mv_pred_flag[ nuh_layer_id ] | 1 | Table H-23 | 0 |
|  | 2 |  | 1 | Table H-23 | 2 |
| sdc_flag | 0 | FL, cMax = 1 | 0 | Table H-24 | 0 |
|  | 1 |  | 0 | Table H-24 | 3 |
|  | 2 |  | 0 | Table H-24 | 6 |
| sdc_residual_flag | 0 | FL, cMax = 1 | 0 | Table H-25 | 0 |
|  | 1 |  | 0 | Table H-25 | 2 |
|  | 2 |  | 0 | Table H-25 | 4 |
| sdc_residual_sign_flag | 0 | FL, cMax = 1 | 0 | Table H-26 | 0 |
|  | 1 |  | 0 | Table H-26 | 2 |
|  | 2 |  | 0 | Table H-26 | 4 |
| sdc_residual_abs_minus1 | 0 | FL, cMax = dltFlag ? Floor( Log2( num_depth_values_in_dlt[ nuh_layer_id ] ) : BitDepthY | 9 | Table H-27 | 0 |
|  | 1 |  | 9 | Table H-27 | 20 |
|  | 2 |  | 9 | Table H-27 | 40 |
| sdc_pred_mode | 0 | U [Ed. (GT): in implementation 0 and 1 are switched. ] [Ed. (GT): Could be TU, cMax = 3] | 0 | Table H-28 | 0 |
|  | 1 |  | 0 | Table H-28 | 3 |
|  | 2 |  | 0 | Table H-28 | 6 |

Table H-30 – Values of wedgeFullTabIdxBits[ log2PUSize ]

| Initialization variable | wedgeFullTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 | 6 |
| Value | 7 | 10 | 11 | 11 | 13 |

Table H-31 – Values of wedgePredTexTabIdxBits[ log2PUSize ]

| Initialization variable | wedgePredTexTabIdxBits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| log2PbSize | 2 | 3 | 4 | 5 | 6 |
| Value | 6 | 9 | 9 | 9 | 0 |

      - Unary (U) binarization process

The specifications in subclause 9.3.2.1 apply.

      - Truncated unary (TU) binarization process

The specifications in subclause 9.3.2.2 apply.

      - Truncated Rice (TR) binarization process

The specifications in subclause 9.3.2.3 apply.

      - k-th order Exp-Golomb (EGk) binarization process

The specifications in subclause 9.3.2.4 apply.

      - Fixed-length (FL) binarization process

The specifications in subclause 9.3.2.5 apply.

      - Binarization process for part_mode

The specifications in subclause 9.3.2.6 apply.

      - Binarization process for coeff_abs_level_remaining

The specifications in subclause 9.3.2.7 apply.

      - Binarization process for intra_chroma_pred_mode

The specifications in subclause 9.3.2.8 apply.

      - Binarization process for inter_pred_idc

The specifications in subclause 9.3.2.9 apply.

    - Decoding process flow

The specifications in subclause 9.3.3 apply with the following modifications.

–	All references to the process specified in subclause 9.3.2 are replaced with references to the process specified in subclause  H.14.3.2.

      - Derivation process for ctxIdx

The specifications in subclause 9.3.3.1 apply with the following modifications.

–	Table H-32 is appended to the end of Table 9-37.

Table H-32 – Assignment of ctxIdxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| dmm_flag | 0 | na | na | na | na | na |
| dmm_mode | 0 | 0 | 0 | 0 | 0 | 0 |
| wedge_full_tab_idx | 0 | 0 | 0 | 0 | 0 | 0 |
| dmm_delta_end_flag | 0 | na | na | na | na | na |
| dmm_delta_end_abs_minus1 | 0 | 0 | 0 | 0 | 0 | 0 |
| dmm_dc_1_abs dmm_dc_2_abs | 0 | 0 | 0 | 0 | 0 | 0 |
| edge_code | 0 | 0 | 0 | 0 | 0 | 0 |
| res_pred_flag | 0 | na | na | na | na | na |
| ic_flag | 0 | na | na | na | na | na |
| mvp_l0_idx, mvp_l1_idx | 0 | 1 | na | na | na | na |
| sdc_flag | 0,1,2 (subclause G.9.3.3.1.1) | na | na | na | na | na |
| sdc_residual_flag | 0,1 (subclause G.9.3.3.1.7) | na | na | na | na | na |
| sdc_residual_sign_flag | 0,1 (subclause G.9.3.3.1.7) | na | na | na | na | na |
| sdc_residual_abs_minus1 | 0,10 (subclause H.14.3.3.1.7) |  |  |  |  |  |
| sdc_pred_mode | 0 | 1 | 2 | na | na | na |

        - Derivation process of ctxIdxInc using left and above syntax elements

The specifications in subclause 9.3.3.1.1 apply with the following modifications.

–	Table H-33 is appended to the end of Table 9-38.

Table H-33 – Specification of ctxIdxInc using left and above syntax elements

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| sdc_flag | sdc_flag [ xL ][ yL ] | sdc_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |

        - .Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

The specifications in subclause 9.3.3.1.2 apply.

        - Derivation process of ctxIdxInc for the syntax element coded_sub_block_flag

The specifications in subclause 9.3.3.1.3 apply.

        - Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

The specifications in subclause 9.3.3.1.4 apply.

        - Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

The specifications in subclause 9.3.3.1.5 apply.

        - Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

The specifications in subclause 9.3.3.1.6 apply.

        - Derivation process of ctxIdxInc using segment index

Input to this process is the luma location ( xC, yC ) specifying the top-left luma sample of the current luma coding block relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

If IntraPredMode[ xC ][ yC ] is not equal to 0 and IntraPredMode[ xC ][ yC ] is not equal to 1, ctxIdxInc depends on the SdcSegmentIndex = 0..1, as specified in Table H-34.

Otherwise, the ctxIdxInc is equal to 0.

Table H-34 – Specification of ctxIdxInc using segment index

| Syntax element | IntraPredMode | SdcSegmentIndex | ctxIdxInc |
| --- | --- | --- | --- |
| sdc_residual_flag | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 1 |
| sdc_residual_sign_flag | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 1 |
| sdc_residual_abs_minus1 | IntraPredMode[ xC ][ yC ] == 0 ¦¦ IntraPredMode[ xC ][ yC ] == 1 | 0 | 0 |
|  | IntraPredMode[ xC ][ yC ] != 0 && IntraPredMode[ xC ][ yC ] != 1 | 0 | 0 |
|  |  | 1 | 10 |

      - Arithmetic decoding process

The specifications in subclause 9.3.3.2 apply with the following modifications.

–	All references to the process specified in subclause 9.3.3.1 are replaced with references to the process specified in subclause  H.14.3.1.1.

    - Arithmetic encoding process (informative)

The specifications in subclause 9.3.4 apply with the following modifications.

- All references to the process specified in subclause 9.3.3.2 are replaced with references to the process specified in subclause  H.14.3.3.1.7.

- Specification of bitstream subsets

The specifications in clause 10 apply.

- Profiles and levels

The specifications in Annex A apply, with the following modification:

TBD

- Byte stream format

The specifications in Annex B apply.

- 3D-HEVC hypothetical reference decoder

The specifications in Annex F.13 apply.

- 3D-HEVC SEI messages

The specifications in Annex D apply, with the following modification :

TBD

- Video usability information

The specifications in Annex E apply.