| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0166 |
| --- | --- |

| Title: | 3D-CE1.h: On reference view selection in NBDV and VSP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou, Dong Tian Anthony Vetro, Huifang Sun 8th floor, 201 Broadway, Cambridge, MA 02139 | Tel: Email: | +1-617-621-7556 {fzou, tian, avetro, hsun}@merl.com |
| Source: | Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

In the current 3D-HEVC working draft, there lacks a way to deal with B view case for NBDV derivation and VSP prediction. With the proposed technique, the reference view used in NBDV function would not be fixed to the base view (with refViewIdx = 0), and thus the derivation of disparity vector from the NBDV function for a B view is more general. On the other hand, the reference view used for VSP prediction is not fixed as the base view (with refViewIdx = 0), but is selectable and the selection of the reference view and prediction direction is determined by the availabilities of interview reference pictures in both RefPicList0 and RefPicList1 when a prediction block is chosen as VSP in a B view picture. The proposed fixes show an average bitrate saving 3.1% on a B view, or 0.3% for video PSNR/video bitrate, 0.2% for video PSNR/total bitrate and 0.1% for synthesis PSNR/total bitrate under the IBP test condition.

# Introduction

In the 3rd JCT3V meeting, VSP was adopted into 3D-HEVC by incorporating a VSP candidate in the merge candidate list and applying 4x4 sub-block based compensation for VSP prediction block (PB). However, in the current design of NBDV function and VSP prediction of the working draft, the base view (with refViewIdx = 0) is always selected as reference view and only uni-direction VSP prediction is supported, partly because the inter-view prediction specified in CTC is PIP. More specifically, only one synthesis predictor is generated always from the base view in RefPicList0 regardless of the picture type. In case of PIP interview test condition, uni-direction VSP prediction is sufficient as there is only one reference view available in the reference picture buffer. However, in case of IBP interview test condition, there are up to 2 interview reference pictures available for the B view. Therefore, it is possible to generate two synthetic predictors, one from each reference view, and have them averaged in a bi-prediction manner.

# NBDV fix for depth block fetching

In the current reference software HTM 6.0 and working draft, during NBDV refinement based on a depth block, the interview reference picture is assumed from the base view with refViewIdx = 0. This is true in the PIP test case, as the only available reference view is the base view with refViewIdx = 0. However, it may not always hold in an IBP interview test case, as the derived disparity vector may point to an interview reference picture with refViewIdx = 1. At this time, a depth block from refViewIdx = 1 should be fetched. Therefore, the depth block fetching should depend on the refViewIdx of the interview reference picture that is associated with the disparity vector from the neighboring block.

In particular, if a disparity vector from a neighboring block refers to an interview reference picture with refViewIdx = X, the depth block is proposed to be accessed from depth picture of the reference view with refViewIdx = X, for X = 0,1.

# Uni-VSP and Bi-VSP

In this section, on top of the NBDV fix in Section 2, a bi-direction VSP prediction is proposed for a B view picture by inspecting the availability of the interview reference pictures in the reference picture lists. When there are different interview reference pictures in RefPicList0 and RefPicList1, bi-direction VSP is applied, that is, the two VSP predictors will be generated from each reference list and weighted averaged and the average result serves as a final VSP predictor. Otherwise uni-direction VSP is applied, and the prediction direction is determined based on the reference list that has the interview reference picture derived in NBDV. In other words, uni-VSP or bi-direction VSP is determined based on the picture type and the availability of reference views in the reference picture lists, which is elaborated as follows.

When a picture is coded as B picture and the current block is signaled as VSP predicted, the following steps are applied to determine the prediction direction of VSP.

- Obtain the view index refViewIdxNBDV of the derived disparity vector from NBDV;

- Obtain the reference picture list RefPicListNBDV (either RefPicList0 or RefPicList1) that is associated with the reference picture with view index refViewIdxNBDV;

- Check the availability of an interview reference picture with view index refViewIdx!= refViewIdxNBDV in the reference picture list other than RefPicListNBDV;

  - If such a different interview reference picture is found, bi-direction VSP is applied. The depth block from view index refViewIdxNBDV is used as the current block’s depth information (in case of texture-first coding order), and the two different interview reference pictures (each from one reference picture list) are accessed via backward warping process and further weighted to achieve the final backward VSP predictor;

  - Otherwise, uni-direction VSP is applied with RefPicListNBDV as the reference picture list for prediction.

When a picture is coded as P picture and the current prediction block is using VSP, uni-direction VSP is applied.

- Obtain the view index refViewIdxNBDV of the derived disparity vector from NBDV;

- Obtain the reference picture list RefPicListNBDV (either RefPicList0 or RefPicList1) that is associated with the reference picture with view index refViewIdxNBDV;

- Uni-direction VSP is applied with RefPicListNBDV as the reference picture list for prediction.

# Simulation results

The proposed NBDV fix and bi-VSP are implemented on top of the reference software HTM 6.0 [2] using IBP test configuration provided within the reference software package.

In the first experiment, only the fix of NBDV for depth block fetching is enabled and tested. The results are shown in Table 1. As the modification does not affect the base view (I view) and the P view, the results for video 0 and 1 column are also zeros, while for B view (video 2) a minor bitrate saving 0.2% on average is observed.

In the second experiment, the bi-VSP is further turned on and the simulation results are shown in Table 2.  Again, as the modification does not affect the base view (I view) and the P view, the results for video 0 and 1 match those from anchor coding. For video 2, i.e. the B view, an average bitrate saving of 3.1% BDBR is achieved.

In total, the proposed solution achieves 0.3% BDBR for video PSNR/video bitrate, 0.2% for video PSNR/total bitrate and 0.1% for synthesis PSNR/total bitrate under the IBP test condition.

Table 1. NBDV updates for depth block fetching

Table 2. Bi-direction VSP based on HTM6.0.

# Conclusions

In this contribution, we propose to fix the problem in the current working draft that the base view is always selected as reference view in NBDV derivation and VSP prediction. The fix makes the NBDV function and VSP prediction more general, especially for a B view in IBP interview test case, and applicable to the IPP (other than PIP in CTC) interview test case. There is an average bitrate saving of 3.1% for the B view, though the overall bitrate saving is more limited mainly due to the sequences with low depth quality. Therefore, it is recommended to include the fixes on NBDV and VSP prediction into the reference software, CTC and working draft.

# Patent rights declaration(s)

Mitsubishi Electric Research Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] D. Tian, F. Zou and A. Vetro (MERL), “CE1.h: Backward View Synthesis Prediction using Neighbouring Blocks”, JCT3V-C0152, Geneva, CH, Jan. 2013

[2] https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.0