| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0167 |
| --- | --- |

| Title: | Multiview and multiview-video-plus-depth support in SHM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Hallapuro, Y. Yan (USTC), D. Rusanovskyy, M. M. Hannuksela (Nokia). | Email: | dmytro.rusanovskyy@nokia.com miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

In the JCTVC-L/JCT3V-C meetings, the specifications of MV-HEVC and SHVC were aligned to use a common basis. In this contribution we present a software model (SM) that implements the MV-HEVC specification on top of the SHM v1.0 software and hence is asserted to provide functionality for collaborative development on aligned specifications for spatial and multi-view extensions of HEVC. In addition to quality/spatial scalability and multiview coding, the proposed SM can be used for Multiview Video plus Depth (MVD) data, mixed resolution MVD data (where the depth views have a different spatial resolution than the texture views) and asymmetric multi-view video data (where a base view has a lower spatial resolution than non-base views). All these configurations of SM are reportedly still aligned with the MV-HEVC and SHVC specifications.

It is proposed to adopt the presented SM as a Test Model for collaborative development on aligned specifications for spatial, multi-view and MVD extensions of HEVC.

# Introduction

The SHM software v1.0, which was used as a starting point, includes the following properties.

- Spatial scalability can be performed with RefIdx-based approach

In this configuration, inter-layer prediction does not use block level operations.

- Inter-layer prediction can be enabled through two ways:

  - Sample array prediction

  - Motion information prediction

- Enhanced layer utilizes a previously coded layer as reference (base layer)

A visualization of this concept for a three-layer configuration is shown in Figure 1. Black arrows represent prediction direction and assume that both samples and motion information can be predicted.

Figure 1. Visualization of the reference layer selection in SHM 1.0

# Description of the major software changes

## High-level syntax

- VPS and NAL unit header have been updated to follow the latest MV-HEVC and SHVC WD specifications

- Coding of depth is enabled as an additional scalability dimension

## Reference layer selection

- An encoder-side configuration option was added to control the layer which to be utilized as a reference and the number of such reference layers. The introduced changes relate to software only and can be efficiently signaled with the current syntax.

This change allows improved flexibility in the prediction between layers, compared to SHM 1.0. An example of a possible configuration for multi-view video coding is shown in the Figure 2. Black arrows represent prediction direction and assume that both samples and motion information can be predicted.

Figure 2. Visualization of the reference layer selection in proposed SM

## Motion Estimation and Motion Field Mapping

- Encoder was modified to enable ME not restricted to the zero-MV, thus enabling disparity compensation

- Motion vector prediction can be enabled between layers (e.g. for quality and spatial scalability)

  - Inter-component (texture to depth and depth to texture) prediction implemented through the TMVP mode of HEVC. The decoded picture of the reference layer is marked to be used as collocated picture, and thus provides motion information.

- Downscaling and up-scaling of motion information can be enabled

# Proposed architecture and its configurations

The proposed software can be configured to support various use cases of spatial, view and depth-enhanced scalabilities as well as their combinations. In this section few examples of such configurations are described.

- Codec architecture for multi-view video coding (Configuration A) is visualized in Figure 3, 3 coded views is assumed.

Figure 3. Multi-view video coding configuration, C3 case

- Codec architecture for asymmetric multi-view video coding (Configuration B) is visualized in Figure 4. In this configuration, the base layer is coded at reduced resolution compared to the enhanced layerd.

Figure 4. Asymmetric resolution multi-view video coding, C3 case

- Codec architecture for multi-view plus depth video coding (Configuration C) is visualized in Figure 5.

Arrows represent prediction direction, black arrows represent prediction of both samples and motion information, whereas the red arrow represents prediction of motion information only.

Figure 5. Example of coding configuration for MVD coding, C3 case

- Codec architecture for mixed resolution multi-view plus depth video coding (Configuration D) visualized in Figure 6. In this configuration, the depth pictures have a smaller spatial resolution than that of the texture pictures.

Arrows represent prediction direction, black arrows represent prediction of both samples and motion information, whereas the red arrow represents prediction of motion information only. The motion field of the texture views is downscaled to match reduced resolution of the depth map.

Figure 6. Example of coding configuration for MVD coding, C3 case

- Codec architecture for spatially scalable stereo (Configuration E) is visualized in Figure 5.

In this configuration, jointly coded stereo pair forms a stereo base layer for enhanced stereo pair. Arrows represent prediction direction, black arrows represent prediction of both samples and motion information, whereas the red arrow represents prediction of motion information only.

Figure 7. Example of coding configuration for spatially scalable 2-view coding

# Simulation results

The performance of the proposed Software Model was evaluated on coding of multi-view video data (Configuration A), which is a subset of 3DV test set. The software and experimental framework was configuration to be close to the JCT-3V Common Test Conditions []. Details of the major parameters are specified in the Table 1. Table 2 shows simulation results produced with the SM and compared against the anchor produced with the MV-HEVC configuration of HTM6.0. It should be noted that HTM-6.0 is based on HM-6.0 while SHM-1.0 is based on more recent HM-9.0 software. The difference between the compared models is not big in terms of coding tools, so the results can be considered roughly comparable.

Table 1. Coding configuration of SM for multi-view video coding

| Coding Parameters | Settings |
| --- | --- |
| Multi-view scenario | Three views (C3) |
| Coding Order | T0T1T2 |
| Multi-view resolution ratio | 1.0-1.0-1.0 |
| Inter-view prediction structure | PIP |
| Inter prediction structure | HierarchicalB, GOP8 |
| Intra period | 24 |
| QP settings for texture | 40, 35, 30, 25 |
| Encoder settings | RDO ON, VSO OFF |
| Test sequences and coded views | Texture views specified in CTC |

Table 2. Simulation results produced with proposed SM, compared against the MV-HEVC anchor (HTM5.1)

|  | video 0 | video 1 | video 2 | C3 video |
| --- | --- | --- | --- | --- |
| Balloons | 0.2% | 0.8% | 0.3% | 0.4% |
| Kendo | 0.0% | 0.3% | 1.3% | 0.5% |
| Newspaper | -0.2% | 1.7% | 1.8% | 0.7% |
| GT_Fly | -1.1% | -0.4% | 0.0% | -0.6% |
| Hall2 | -0.8% | 2.7% | 1.3% | 0.6% |
| Street | 0.4% | 4.5% | 4.2% | 1.9% |
| Dancer | -0.2% | 3.1% | 3.0% | 0.8% |
| 1024x768 | 0.0% | 0.9% | 1.2% | 0.6% |
| 1920x1088 | -0.4% | 2.5% | 2.1% | 0.7% |
| Average | -0.2% | 1.8% | 1.7% | 0.6% |

Additionally, SM was configured for coding of Multiview Video plus Depth (MVD) data (Configuration C). Details of the major parameters for such configuration are specified in the Table 3. Table 4 shows simulation results produced with the SM and compared against the 3D-HEVC anchor. It should be noted, that 3D-HEVC anchor utilized encoder side VSO to minimize bitrate of the coded depth map. This tools is currently not available in SM (*), therefore results of view synthesis may not be relevant. As it can be seen from table 3, proposed MV-HEVC implementation with SM provides underperforms about 9% of dBR against the 3D-HEVC anchor.

Table 3 Coding configuration of SM for MVD coding, C3

| Coding Parameters | Settings |
| --- | --- |
| Multi-view scenario | Three views (C3) |
| Coding Order | T0T1T2-D0D1D2 |
| MVD resolution ratio (Texture : Depth) | 1:1 |
| Inter-view prediction structure | PIP |
| Inter prediction structure | HierarchicalB, GOP8 |
| Intra period | 24 |
| QP settings for texture & depth | 40, 35, 30, 25 (as in CTC) |
| Encoder settings | RDO ON, VSO OFF* |
| Test sequences and coded views | Texture of specified in CTC |

Table 4. Simulation results produced with proposed SM, compared against the 3D-HTM anchor, v6.0

|  | video 0 | video 1 | video 2 | video only | MVD total | Synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.2% | 36.2% | 34.1% | 14.6% | 13.3% | 11.4% |
| Kendo | 0.1% | 32.5% | 35.8% | 14.2% | 11.3% | 10.0% |
| Newspaper | -0.2% | 19.5% | 18.4% | 7.6% | 6.4% | 8.6% |
| GT_Fly | -1.1% | 27.8% | 26.0% | 6.9% | 7.4% | 13.2% |
| Hall2 | -0.8% | 20.9% | 21.2% | 8.9% | 8.5% | 13.7% |
| Street | 0.4% | 17.5% | 17.9% | 6.5% | 6.8% | 7.7% |
| Dancer | -0.2% | 24.5% | 24.7% | 7.4% | 7.6% | 34.3% |
| 1024x768 | 0.0% | 29.4% | 29.4% | 12.1% | 10.3% | 10.0% |
| 1920x1088 | -0.4% | 22.7% | 22.5% | 7.4% | 7.6% | 17.2% |
| Average | -0.2% | 25.6% | 25.4% | 9.45% | 8.8% | 14.1% |

In addition to inter-view prediction utilized for coding of MVD data independently to texture and depth, we have tested impact of Inter-Component-Prediction, which was implemented with motion field mapping. The motion vector of the block collocated with Cb was utilized as motion vector predictor instead of temporal candidate. This Configuration C+ICP was compared against the configuration C, where inter-view prediction only was utilized. Table 5 shows that about 1 % of dBR on average for synthesised views can be achieved with enabling the ICP in SM.

Table  5. Coding gain provided by ICP introduced in MVD coding, MVD data coded independently with MV-HEVC.

|  | video 0 | video 1 | video 2 | video only | MVD total | Synthesized |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.6% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.8% |
| Newspaper | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.4% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -1.3% |
| Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | -1.3% |
| Street | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.3% |
| Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -2.7% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.6% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -1.4% |
| Average | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -1.1% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).