| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0168 |
| --- | --- |

| Title: | CE1.h related: Forward Block-based View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yichen Zhang Yin Zhao Lu Yu | Tel: Email: | +86 571 87953107 felixzyc@gmail.com zhaoyin@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This contribution describes the forward block-based view synthesis prediction (FVSP) design for 3D-HEVC. FVSP estimates a reference block in the base view by only using the base view depth map. Then pixels in the reference block are warped to a target block in current coding view to be the prediction block of current coding block. The overall bitrate savings on the coded and synthesized views are -0.2% for XGA sequences and -0.7% for HD sequences compared to HTM-6.0 and the decoding time is 98.1% for all sequences on average. The maximum bitrate savings of texture on dependent left and right views are -6.2% and -6.6%, respectively.

# Introduction

Backward block-based view synthesis prediction (BVSP) for 3D-HEVC was proposed at the 3rd JCT-3V meeting in Geneva [1]. BVSP uses the disparity vector from neighboring block disparity vector (NBDV) to derive a depth block from base view to perform the backward warping operation.

In this contribution, a forward-warping block-based view synthesis method is proposed.

# Proposed FVSP

FVSP estimates a reference blocks in based view which is to be warped to current coding view to generate the view synthesis prediction block (called the target block). The reference blocks estimation procedure (described in 2.1) does not depend on neighboring blocks of current block and only dependents on the depth map of base view. Then reference block is warped to the target block by a low complexity forward-warping method (described in 2.2).

## Reference block estimation

Since the width and location of reference block in base view is not known when coding the current block. So firstly, the reference block is determined by four estimated correspondence of the four corner pixels of current block.

Fig 1. Corner pixels of current PU and the reference block

For a corner pixel (CP) CPj (x, y) (as shown in Fig 1(a)) of current block (of size width*height), 3 correspondence candidates (CC) are estimated:

CC1 = (x – disp(128), y)

CC2 = (x – disp(CC1), y)

CC3 = (x – disp(CC2), y)

where disp(X) is the disparity value of depth of X.

- For correspondence Rj (R0 and R1) of each corner pixel (CP0 and CP1) along LEFT boundary of current block:

Rj=argmaxCCi(CCi|CCi+disp(CCi)≤x)

Then left boundary (LB) of reference block is determine by the LEFT most correspondences:

LB=min(R0,R1)

- For correspondence Rj (R2 and R3) of each corner pixel (CP2 and CP3) along RIGHT boundary of current block:

Rj=argminCCi(CCi|CCi+disp(CCi)≥x)

Right boundary (RB) of reference block is determined by the RIGHT most correspondences:

RB=max(R2,R3)

Then reference block determined as a block lies between LB and RB with same height as current block (as shown in Fig 1(b)).

## Reference block warping

A low complexity forward-warping method [2] is used in this contribution:

- No z-buffering is required during the warping procedure;

- The target block has the same size as current block and the warping area in current coding view is restricted within the target block, i.e., pixels whose warping position is outside of target block are not warped to the current view;

- Interpolation operations are conducted only at most one time for every warped pixel in reference block. This operation is also used in BVSP;

- Interpolation operations are disabled for depth warping (i.e., integer-pixel warping precision).

# Simulation results and discussions

The proposed FVSP was implemented into HTM 6.0. Simulations are conducted according to the common test conditions as defined in [3].

Table 1 shows the performance of the proposed FVSP against HTM6.0. The total BD-rate gains are:

- 0.4% for video PSNR vs. video bitrate

- 0.4% for video PSNR vs. total bitrate

- 0.5% for synthesis PSNR vs. total bitrate

The maximum BD-rate gains for synthesis PSNR vs. total bitrate is 2.2%.

The decoding complexity of FVSP is very close to the anchor.

Table 1. FVSP performance compared to HTM6.0 anchor

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.6% | -0.2% | -0.1% | -0.2% | -0.2% | 93.1% | 98.9% |
| Kendo | 0.0% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 88.4% | 97.0% |
| Newspaper_CC | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | -0.2% | 95.9% | 101.1% |
| GT_Fly | 0.0% | -1.1% | -1.3% | -0.3% | -0.4% | -0.4% | 97.6% | 98.0% |
| Poznan_Hall2 | 0.0% | -0.6% | -0.5% | -0.2% | -0.3% | -0.2% | 135.1% | 98.6% |
| Poznan_Street | 0.0% | -0.5% | -0.2% | -0.1% | -0.1% | -0.2% | 123.2% | 95.2% |
| Undo_Dancer | 0.0% | -6.2% | -6.6% | -1.8% | -1.9% | -2.2% | 93.3% | 98.0% |
| 1024x768 | 0.0% | -0.4% | -0.2% | -0.1% | -0.1% | -0.2% | 92.5% | 99.0% |
| 1920x1088 | 0.0% | -2.1% | -2.1% | -0.6% | -0.7% | -0.7% | 112.3% | 97.4% |
| average | 0.0% | -1.4% | -1.3% | -0.4% | -0.4% | -0.5% | 103.8% | 98.1% |

We also studied the percentage of pixels using VSP modes. Table 2 shows that over all sequences, more pixels in the proposed method are coded with VSP modes due to the fact that forward-warping method generates better synthesis images. Besides, sequences with better quality depth map tend to encode more pixels with FVSP than BVSP. The coding gain for FVSP against BVSP is also correlated with the usage difference (third column of Table 2) between FVSP and anchor.

Table 2. Percentage of pixels using BVSP/FVSP modes

|  | Anchor(BVSP) | FVSP | FVSP − Anchor |
| --- | --- | --- | --- |
| Balloons | 2.16% | 2.57% | 0.41% |
| Kendo | 3.89% | 4.41% | 0.52% |
| Newspaper | 0.77% | 1.08% | 0.31% |
| GT_Fly | 41.70% | 42.62% | 0.92% |
| Poznan_Hall2 | 3.11% | 3.62% | 0.51% |
| Poznan_Street | 4.37% | 4.67% | 0.30% |
| Undo_Dancer | 20.73% | 29.20% | 8.47% |
| Average | 10.96% | 12.59% | 1.63% |

# Conclusions

In this contribution, a forward-warping block-based view synthesis method is proposed. Better block-based synthesis images are generated by forward warping method. It achieves -0.2% gain for XGA sequences, -0.7% gain for HD sequences and -0.5% gain on average. FVSP brings better coding performance with similar decoding complexity compared the BVSP in anchor. It is recommended to adopt the proposed FVSP.

# References

- D. Tian, F. Zou, A. Vetro (MERL), "CE1.h: Backward View Synthesis Prediction Using Neibouring Blocks ", JCT3V-C0152, Geneva, CH, Jan 2013.

- S. Shimizu, S. Sugimoto and H. Kimata, “CE1.a related results on the modification of in-loop view synthesis”, JCT3V-A0023, Stockhom, SE, July 2012.

- D. Rusanovskyy, K. Muller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-C1100, Geneva, CH, Jan 2013.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).