| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0169 |
| --- | --- |

| Title: | CE6.h related: Results on disabling of DMM 4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yunseok Song, Yo-Sung Ho 123 Cheomdangwagi-ro Buk-gu Gwangju, Korea 500-712 | Tel: Email: | +82-62-715-2263 ysong@gist.ac.kr hoyo@gist.ac.kr |
| Source: | Gwangju Institute of Science and Technology (GIST) |  |  |

_____________________________

# Abstract

This document presents results on disabling of DMM 4. This mode was originally designed for predicting contours in the depth map. Tests were conducted to evaluate the efficiency of DMM 4. Simulation results report that disabling of DMM 4 led to 0.2% bitrate reduction in regards to synthesis.

# Introduction

Depth modeling mode (DMM) 4 is for predicting contours of the depth block using its collocated texture luma block (CTLB) as a reference. The average of CTLB pixel values is used a threshold to generate the contour pattern. Figure 1 shows a contour partition of a depth block [1].

# Disabling of DMM 4

In the current 3D-HEVC, the processing depth block size is either 8x8 or 16x16 or 32x32. For a block containing two regions partitioned by contours, its block size must be sufficiently bigger, thus DMM 4 would not show clear benefits in the current scheme. The efficiency of DMM 4 is tested by comparing the results from the anchor and the one with DMM 4 removed.

Figure 1. Contour partition of a depth block

# Simulation Results

Table 1 shows the results under CTC [2]. Disabling of DMM 4 did not affect texture coding performances. In terms of synthesis, 0.2% bitrate reduction was achieved. Encoding and decoding runtimes are similar to those of anchor results since only one mode was concerned.

Table 1. Results under CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.5% | 100.0% |
| Kendo | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 99.7% | 100.2% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 99.4% | 100.5% |
| GT_Fly | 0.0% | 0.2% | 0.2% | 0.1% | 0.1% | -0.3% | 99.5% | 100.7% |
| Poznan_Hall2 | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.3% | 99.8% | 100.1% |
| Poznan_Street | 0.0% | 0.2% | 0.2% | 0.1% | 0.0% | 0.1% | 99.7% | 100.3% |
| Undo_Dancer | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.5% | 99.6% | 99.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 100.2% |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.2% | 99.6% | 100.2% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 99.6% | 100.2% |

# Conclusion

This document reported results of disabling of DMM 4. In the evaluation of DMM 4 disabling in HTM-6.0 under CTC, 0.2% bitrate was achieved for synthesis results.

# Reference

- G. Tech, K. Wegner, Y. Chen, and S. Yea, “3D-HEVC test model 3,” JCT3V-C1005, Geneva, CH, Jan. 2013.

- D. Rusanovskyy, K. Muller, and A. Vetro, “Common test conditions of 3DV core experiments,” JCT3V-C1100, Geneva, CH, Jan. 2013.

# Acknowledgment

This work was supported by the National Research Foundation of Korea (NRF) grant funded by the Korea government (MEST) (No. 2012-0009228).

# Patent rights declaration(s)

Gwangju Institute of Science and Technology (GIST) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).