| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0170 |
| --- | --- |

| Title: | CE2.h related: Enhanced Disparity Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yichen Zhang Yin Zhao Lu Yu | Tel: Email: | +86 571 87953107 felixzyc@gmail.com zhaoyin@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This contribution describes an enhanced disparity vector prediction method which consists of two parts: forward disparity vector (FWDV) derivation and adaptive disparity vector (DV) candidate position adjustment method for 3D-HEVC. FWDV is a DV estimated from depth map of base view and act as a supplementary DV when neighboring block disparity vector (NBDV) method cannot get any DV from neighboring block. The adaptive DV candidate position adjustment method utilizes the prediction type of neighboring block to adjust the position of DV candidate in merge candidate list. The proposed method achieves -0.16% BD-rate gain compared to the HTM6.0 and no decoding time increase is observed.

# Introduction

In the current design of HTM, neighboring block disparity vector (NBDV) is used in motion/disparity vector prediction and inter-view residual prediction (IVRP). NBDV method checks the availability of DV of neighboring blocks in a certain order. Once a DV is found among the neighboring blocks, the checking procedure terminates. When none of the neighboring blocks are inter-view predicted, i.e., no disparity from neighborhood of current block is available.

- In advanced motion vector prediction (AMVP) candidate list and merge candidate list construction process, when number of NBDV is zero, a zero DV is used as supplementary DV and the number of NBDV is set to one;

- In IVRP, when number of NBDV is zero, no supplementary DV is used. So IVRP cannot work under this condition.

In first part of this contribution, a forward disparity vector (FWDV) is proposed. FWDV is a DV estimated from depth map of base view. Its derivation process has no dependency on the availability of DV of neighboring blocks. So FWDV could be always available as long as base view depth map is reconstructed.

The second part of this contribution is adaptive DV candidate position adjustment.

In current design of HTM, the NBDV is put in a fixed position in the merge candidate list. When a block is coded by interview prediction, the cost of transmitting merge index of DV candidate might be a little high. Actually, the correlation of prediction type between the current block and its neighboring blocks is very high. So the position of DV candidate in merge candidate list could be adaptively changed to achieve less cost of transmitting merge index for a DV.

# Proposed Method

The FWDV derivation method is described in section 2.1 and the adaptive DV candidate position adjustment method is described in section 2.2.

## ForWard Disparity Vector (FWDV) Derivation

Figure 1. Derivation process of FWDV

In FWDV derivation, disparity vector of five pixels (four corner pixels and one center pixel, the green pixels in Fig. 1, from P1 to P5) of current block are estimated, and the maximum disparity vector is selected as the FWDV.

Every disparity vector DVi of pixel Pi ( i = 1~5) is estimated as follows:

Let (xi, yi) denote the coordinate of Pi, Crj( j = 1, 2) be the estimated correspondence of Pi.

Cr1 = (xi – Disp(128), yi)

Cr2 = (xi – Disp(Cr1), yi)

DVi = (Disp(Cr2), 0)

where Disp(128) is the disparity value of the medium depth value 128, and Disp(Cri) is the disparity value of depth value of Cri.

## Adaptive Disparity Vector Candidate Position Adjustment Method

Figure 2. Current design of merge candidate list

In the current design of merge candidate list construction, the fixed position of DV candidate is between the candidates from above right and left bottom block (as shown in Figure 2).

We found that when the left or above neighboring block of current block is interview predicted, the current block has a high probability of being interview predicted. So in the proposed method, when the left or above neighboring block is interview predicted, the position of NBDV in the merge candidate list is shifted to the one between candidate from above and above right neighboring blocks (as shown in Figure 3). Otherwise, the position of NBDV stays the same.

Figure 3. DV candidate position adjustment

# Simulation results

The proposed method was implemented into HTM 6.0. Simulations are conducted according to the common test conditions as defined in [1].

Table 1 shows the performance of the proposed FVSP against HTM6.0. The total BD-rate gains are:

- 0.07% for video PSNR vs. video bitrate

- 0.06% for video PSNR vs. total bitrate

- 0.16% for synthesis PSNR vs. total bitrate

The maximum BD-rate gains for synthesis PSNR vs. total bitrate is 0.64%.

The decoding complexity of the proposed method is almost the same to the anchor.

Table 1. Performance of proposed method vs. HTM6.0

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.27% | -0.11% | -0.07% | -0.05% | -0.10% | 86.8% | 100.9% |
| Kendo | 0.00% | -0.08% | -0.15% | -0.06% | -0.05% | -0.01% | 79.9% | 99.8% |
| Newspaper_CC | 0.00% | -0.28% | -0.15% | -0.08% | -0.06% | -0.11% | 86.5% | 102.0% |
| GT_Fly | 0.00% | -0.12% | -0.10% | -0.03% | -0.04% | -0.08% | 93.9% | 99.2% |
| Poznan_Hall2 | 0.00% | -0.21% | -0.41% | -0.16% | -0.13% | -0.19% | 105.6% | 101.6% |
| Poznan_Street | 0.00% | 0.12% | 0.09% | 0.03% | 0.01% | 0.00% | 96.5% | 100.9% |
| Undo_Dancer | 0.00% | -0.20% | -0.41% | -0.09% | -0.10% | -0.64% | 92.6% | 99.0% |
| 1024x768 | 0.00% | -0.21% | -0.14% | -0.07% | -0.05% | -0.07% | 84.4% | 100.9% |
| 1920x1088 | 0.00% | -0.10% | -0.21% | -0.06% | -0.07% | -0.23% | 97.2% | 100.2% |
| average | 0.00% | -0.15% | -0.18% | -0.07% | -0.06% | -0.16% | 91.7% | 100.5% |

The performance of each part of this proposal is also tested.

Table 2 shows the performance of FWDV only simulation and Table 3 shows the performance of adaptive DV candidate position adjustment only simulation.

Table 2. Performance of FWDV vs. HTM6.0

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.15% | -0.20% | -0.07% | -0.05% | -0.11% | 102.8% |  |
| Kendo | 0.00% | 0.03% | -0.03% | 0.00% | 0.02% | 0.05% | 73.1% |  |
| Newspaper_CC | 0.00% | -0.24% | -0.27% | -0.09% | -0.05% | -0.08% | 84.4% |  |
| GT_Fly | 0.00% | 0.05% | 0.10% | 0.01% | 0.01% | -0.04% | 94.0% |  |
| Poznan_Hall2 | 0.00% | -0.24% | -0.45% | -0.16% | -0.15% | -0.11% | 109.9% |  |
| Poznan_Street | 0.00% | 0.30% | -0.16% | 0.02% | 0.01% | 0.01% | 107.9% |  |
| Undo_Dancer | 0.00% | -0.10% | -0.39% | -0.09% | -0.09% | -0.25% | 96.8% |  |
| 1024x768 | 0.00% | -0.12% | -0.17% | -0.05% | -0.02% | -0.05% | 86.8% |  |
| 1920x1088 | 0.00% | 0.00% | -0.22% | -0.05% | -0.05% | -0.10% | 102.2% |  |
| average | 0.00% | -0.05% | -0.20% | -0.05% | -0.04% | -0.08% | 95.6% |  |

Table 3. Performance of DV candidate position adjustment method vs. HTM6.0

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.03% | -0.06% | -0.03% | -0.04% | -0.08% | 95.2% |  |
| Kendo | 0.00% | 0.13% | -0.02% | 0.01% | 0.00% | 0.08% | 80.9% |  |
| Newspaper_CC | 0.00% | -0.02% | 0.10% | 0.03% | 0.04% | -0.02% | 103.7% |  |
| GT_Fly | 0.00% | 0.02% | 0.05% | 0.01% | 0.01% | 0.01% | 95.3% |  |
| Poznan_Hall2 | 0.00% | -0.10% | 0.02% | -0.05% | -0.04% | 0.05% | 132.9% |  |
| Poznan_Street | 0.00% | 0.13% | 0.10% | 0.04% | 0.02% | 0.02% | 99.4% |  |
| Undo_Dancer | 0.00% | -0.05% | -0.02% | -0.01% | -0.01% | -0.48% | 96.4% |  |
| 1024x768 | 0.00% | 0.03% | 0.01% | 0.01% | 0.00% | -0.01% | 93.3% |  |
| 1920x1088 | 0.00% | 0.00% | 0.04% | 0.00% | 0.00% | -0.10% | 106.0% |  |
| average | 0.00% | 0.01% | 0.02% | 0.00% | 0.00% | -0.06% | 100.5% |  |

# Conclusions

In this contribution, an enhanced disparity vector prediction method is proposed. The FWDV provides a supplementary and accurate DV when NBDV get no DV from neighboring blocks. The adaptive DV candidate position adjustment method improves the coding efficiency of merge index of DV. The whole set of improvement achieves -0.16% gain and brings no additional decoding complexity. It is recommended to adopt the proposed method.

# References

- D. Rusanovskyy, K. Muller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, JCT3V-C1100, Geneva, CH, Jan 2013.

# Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).