| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0173 |
| --- | --- |

| Title: | CE6.h related: Removal of division operation for DMM and SDC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng， Yongbing Lin, Jianhua Zheng | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution provides two division free solutions for DMM and SDC. Both test cases at common test condition and All intra condition for DMM and SDC are verified. Experimental results show that the two proposed solutions don't have the impact on the coding performance. Due to the removal of the division operation, the DMM and SDC encoding and decoding process can be simplified.

# Introduction

At current DMM and SDC designs, the DC value is the average of the sample values inside an intra block or inside the prediction samples. The calculation is performed by the division between the summary of sample values and the number of samples which are inside a block for SDC or inside the prediction samples for DMM case. The division are shown as follows:

DMM:

  - The variable sumNeigh specifying the sum of the neighbouring samples depth values of partition X is set equal to 0 and the variable numNeigh specifying the number of the neighbouring samples of partition X is set equal to 0 and the following applies.

    - For x = 0..( nT >> log2SubSample ) − 1, inclusive the following applies:

      - When partitionPattern[ x << log2SubSample ][ 0 ] is equal to X, the following applies:

          - sumNeigh += p[ x<< log2SubSample ][ −1 ]	(Error: Reference source not found-1)
numNeigh += 1		(Error: Reference source not found-2)

    - For y = 0..( nT  >> log2SubSample )− 1,inclusive the following applies:

      - When partitionPattern[ 0 ][ y << log2SubSample ] is equal to X, the following applies:

          - sumNeigh += p[ −1 ][ y << log2SubSample ]	(Error: Reference source not found-3) 
numNeigh += 1		(Error: Reference source not found-4)

  - 2. The variables predDcVal specifying the predicted constant partition values for partition X is derived as follows.

      - predDcVal = ( numNeigh ! = 0 ) ? ( sumNeigh / numNeigh ) : ( 1 << ( BitDepthY  − 1 ) )	(Error: Reference source not found-5)

SDC:

  - For p in the range of 0 to 1, inclusive,  the variable dcPred[ p ] is derived as specified in the following:

sumPred = 0
		numPred = 0. 
		for( x = 0; x < ( nT >> log2SubSample ); x++ ) { 
			xS  = x << log2SubSample  
			for ( y = 0; y < ( nT >> log2SubSample ) ;y++ ) { 
				yS = y << log2SubSample .
				if ( p = = wedgePattern[ xS ][ yS ] ) {
					sumPred += predSamples[ xS ][ yS ]
					numPred += 1
				}
			}
		dcPred[ p ] = ( numPred > 0 ) ? ( sumPred / numPred ) : 0

Because DMM might divide a square block into two non-rectangular partitions, the number of samples inside a block or inside the prediction samples might not be represented as 2N. Therefore the division operation exists in the calculation process of DC value for DMM and SDC, and can’t be converted to the right shift and addition operations.

Compared to other logic unit, divider is implemented with much complex circuit at cost of larger area and longer delay. In hardware design, the division operation can be achieved by the look-up table (LUT) as LUT can store all of results of the division operation. However, since the numerator and denominator vary in the same time at the DC calculation process, it will significantly improve the cost of LUT. The below table summarizes the range of the numerator and denominator, and the size of LUT in DMM and SDC.

Table 1: Cost of look-up table

|  | Numerator (summary of samples value) | Denominator (number of samples) | LUT size |
| --- | --- | --- | --- |
| DMM | 213 | 25 | 218 |
| SDC | 218 | 210 | 228 |

It can be easily concluded that the divider in current DMM and SDC design is expensive in hardware. This contribution provides two solutions that can significant reduce the division operation.

# Removal of division operation

## Solution 1

Considering that the numerator and denominator in DMM and SDC change randomly, the size of LUT is the multiplication of the range of numerator and denominator. To reduce the LUT size, we propose to separate the calculation of the reciprocal of denominator and the calculation of DC value.

At the first step, the reciprocal of denominator is calculated as follow:

Int iX = (C + (numOfPix >> 1)) / numOfPix;

numOfPix is the number of samples (denominator), and C is a constant value. In order to improve the accuracy of iX, the constant value is set to the value which is larger than the maximum of the denominator (numOfPix). Thus, the value of “c” is set to 216 for SDC and 212 for DMM.

Secondly, the DC value is calculated by the multiplication of numerator and iX that has been calculated before.

DC =  Clip( (sumOfPix*iX + shiftBit>>1) >> shiftBit );

sumOfpix is the summary of the sample values (numerator), and shiftBit is set as 16 for SDC and 12 for DMM.

At the above solution, the division operation exists in the first step. Considering that the numerator of the first step is depended on the value of denominator, the LUT size for the storage of division results is reduced from 218 to 25 in DMM, and from 228 to 210 in SDC. The cost of LUT is reduced significantly.

## Solution 2

The solution 1 has simplified the division operation as all of the division results can be stored in two LUTs with the size of 25 for DMM and 10 for SDC. However, the cost of LUTs is still high compared to current HEVC design. In HEVC main profile, the division operation exists in motion vector scaling process, and requires a 28 LUT. The scaling process is show as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-6)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-7)

where td and tb are derived as follows:

td = Clip3( −128, 127, DiffPicOrderCnt( currPic, refPicListA[ refIdxA ] ) )	(8-8)

tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] ) )	(8-9)

Since the value of denominator in motion vector scaling (“td” in HEVC) is ranged from -128 to 127, the division results of the value from 1 to 127 has been stored in motion vector scaling’s LUT. Base on this property, DMM and SDC can reuse the LUT in the motion vector scaling.

For DMM case, the value of denominator is ranged from 1 to 32, the division process for DMM can be changed as follows, and the LUT for DMM can totally reuse motion vector scaling’s LUT.

Int iX    = (0x4000 + (uiNumSmpDC1 >> 1)) / uiNumSmpDC1;

DC1 =  Clip( (sumOfPix*iX + 0x2000) >> 14 );

For SDC case, the value of denominator is ranged from 1 to 1024, partial values are outside the range of the denominator in motion vector scaling. Therefore, the value that larger than 127 is perform by the right shift operation at first so that the value after the right shift can locate in the range of 1 to 127, and LUT for motion vector scaling can still be reused. The division process for SDC is changed as follows:

if( numOfPix == 0 )

{

DC = 0;

}

else if( numOfPix < 128 )

{

Int iX = (0x4000 + (numOfPix >> 1)) / (numOfPix);

DC =  Clip( (iX*sumOfPix + 0x2000) >> 14 );

}

else

{

Int iX = (0x4000 + (numOfPix >> 4)) / (numOfPix >> 3);

DC =  Clip( (iX*sumOfPix + 0x10000) >> 17 );

}

The solution 2 further reduces the LUT size for DMM and SDC. Moreover, the LUT can totally reuses motion vector scaling’s LUT. The cost of the divider in DMM and SDC is considerable reduced.

# Experimental results

The proposed method is implemented on HTM6.0. The coding efficiency is evaluated in random access (CTC) and All-Intra configurations. The table 2 and table 3 show the results of solution1. Table 4 and table 5 show the results of solution 2.

Table 2: Solution 1 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 100.0% | 103.6% | 99.8% |
| Kendo | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 108.5% | 99.7% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.1% | 106.3% | 101.2% |
| GT_Fly | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | -0.1% | 100.1% | 96.8% | 100.7% |
| Poznan_Hall2 | 0.0% | -0.4% | -0.6% | -0.2% | -0.3% | -0.2% | 99.9% | 105.6% | 104.9% |
| Poznan_Street | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 100.0% | 98.2% | 103.1% |
| Undo_Dancer | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.2% | 99.8% | 98.0% | 102.4% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 106.1% | 100.2% |
| 1920x1088 | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | 100.0% | 99.6% | 102.8% |
| average | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 100.0% | 102.4% | 101.7% |

Table 3: Solution 1 (All - intra)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.97% | 102.72% | 99.16% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.92% | 103.10% | 100.90% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.05% | 103.47% | 100.16% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 99.54% | 99.96% | 99.38% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.37% | 102.29% | 100.72% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 99.51% | 100.37% | 98.89% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.29% | 100.63% | 98.25% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.98% | 103.10% | 100.07% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.43% | 100.81% | 99.31% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.66% | 101.79% | 99.64% |

Table 4: Solution 2 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | 100.5% | 100.3% | 101.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 96.8% | 99.9% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.0% | 114.4% | 100.4% |
| GT_Fly | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | 100.6% | 95.2% | 101.0% |
| Poznan_Hall2 | 0.0% | -0.2% | 0.1% | 0.0% | -0.1% | -0.1% | 100.5% | 101.5% | 104.4% |
| Poznan_Street | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 100.0% | 100.3% | 101.2% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 100.1% | 97.1% | 99.4% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 103.9% | 100.4% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 98.5% | 101.5% |
| average | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 100.2% | 100.8% | 101.0% |

Table 5: Solution 2 (All-intra)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.07% | 108.92% | 100.36% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.13% | 103.63% | 103.28% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.75% | 100.90% | 100.95% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | 100.00% | 100.38% | 99.11% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.76% | 100.60% | 100.45% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 99.69% | 98.98% | 99.62% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.42% | 103.36% | 99.64% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.32% | 104.49% | 101.53% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 99.72% | 100.83% | 99.71% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.97% | 102.40% | 100.49% |

# Changes to the description of Test Model

#solution 1

DMM:

  - The variable sumNeigh specifying the sum of the neighbouring samples depth values of partition X is set equal to 0 and the variable numNeigh specifying the number of the neighbouring samples of partition X is set equal to 0 and the following applies.

    - For x = 0..( nT >> log2SubSample ) − 1, inclusive the following applies:

      - When partitionPattern[ x << log2SubSample ][ 0 ] is equal to X, the following applies:

          - sumNeigh += p[ x<< log2SubSample ][ −1 ]	(Error: Reference source not found-10)
numNeigh += 1		(Error: Reference source not found-11)

    - For y = 0..( nT  >> log2SubSample )− 1,inclusive the following applies:

      - When partitionPattern[ 0 ][ y << log2SubSample ] is equal to X, the following applies:

          - sumNeigh += p[ −1 ][ y << log2SubSample ]	(Error: Reference source not found-12) 
numNeigh += 1		(Error: Reference source not found-13)

  - 2. The variables predDcVal specifying the predicted constant partition values for partition X is derived as follows.

      - if ( numNeigh = = 0 ) 
{
  predDcVal =  1 << ( BitDepthY  − 1 ) 
}
else
{
  iX = (0x1000 + (numNeigh >> 1)) / numNeigh
  predDcVal = ( sumNeigh x iX + 0x800 ) >> 12
  predDcVal = Clip (0, 255,  predDcVal) 
}

SDC:

  - For p in the range of 0 to 1, inclusive,  the variable dcPred[ p ] is derived as specified in the following:

sumPred = 0
		numPred = 0. 
		for( x = 0; x < ( nT >> log2SubSample ); x++ ) { 
			xS  = x << log2SubSample  
			for ( y = 0; y < ( nT >> log2SubSample ) ;y++ ) { 
				yS = y << log2SubSample .
				if ( p = = wedgePattern[ xS ][ yS ] ) {
					sumPred += predSamples[ xS ][ yS ]
					numPred += 1
				}
			}
		if (numPred = 0) 
       {
           dcPred[p] = 0

}
       else 
       {
           iX = (0x10000 + numPred>>1) / numPred
           dcPred[p] = Clip (0, 255, (iX x sumPred + 0x8000) >> 16)
       }

#solution 2

DMM:

  - The variable sumNeigh specifying the sum of the neighbouring samples depth values of partition X is set equal to 0 and the variable numNeigh specifying the number of the neighbouring samples of partition X is set equal to 0 and the following applies.

    - For x = 0..( nT >> log2SubSample ) − 1, inclusive the following applies:

      - When partitionPattern[ x << log2SubSample ][ 0 ] is equal to X, the following applies:

          - sumNeigh += p[ x<< log2SubSample ][ −1 ]	(Error: Reference source not found-14)
numNeigh += 1		(Error: Reference source not found-15)

    - For y = 0..( nT  >> log2SubSample )− 1,inclusive the following applies:

      - When partitionPattern[ 0 ][ y << log2SubSample ] is equal to X, the following applies:

          - sumNeigh += p[ −1 ][ y << log2SubSample ]	(Error: Reference source not found-16) 
numNeigh += 1		(Error: Reference source not found-17)

  - 2. The variables predDcVal specifying the predicted constant partition values for partition X is derived as follows.

      - if ( numNeigh = = 0 ) 
{
  predDcVal =  1 << ( BitDepthY  − 1 ) 
}
else
{
  iX = (0x4000 + (numNeigh >> 1)) / numNeigh
  predDcVal = ( sumNeigh x iX + 0x2000 ) >> 14
  predDcVal = Clip (0, 255,  predDcVal) 
}

SDC:

  - For p in the range of 0 to 1, inclusive,  the variable dcPred[ p ] is derived as specified in the following:

sumPred = 0
		numPred = 0. 
		for( x = 0; x < ( nT >> log2SubSample ); x++ ) { 
			xS  = x << log2SubSample  
			for ( y = 0; y < ( nT >> log2SubSample ) ;y++ ) { 
				yS = y << log2SubSample .
				if ( p = = wedgePattern[ xS ][ yS ] ) {
					sumPred += predSamples[ xS ][ yS ]
					numPred += 1
				}
			}
		if (numPred = 0) 
       {
           dcPred[p] = 0

}
       else if (numPred < 128 ) 
       {
           iX = (0x4000 + numPred>>1) / numPred
           dcPred[p] = Clip (0, 255, (iX x sumPred + 0x2000) >> 14)
       }
       else
       {
          iX = (0x4000 + numPred>>4) / (numPred>>3)
          dcPred[p] = Clip (0, 255, (iX x sumPred + 0x10000) >> 17)
       }

# Conclusion

This contribution provides two division removal solutions for DMM and SDC modes. Compare to the anchor, the proposed solutions doesn't have impact on all test cases. Moreover, the DMM and SDC encoding and decoding process are simplified due to the removal of division. Considering the tradeoff between complexity reduction and the impact on the coding performance, it is recommended to incorporate the proposed solution2 into next version of 3D-HEVC Test Model and HTM7.

# Patent rights declaration(s)

Hisilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).