| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0174 |
| --- | --- |

| Title: | CE6.h related: Extension of reference samples sub-sampling to all block-sizes in DMM and SDC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng， Yongbing Lin, Jianhua Zheng | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution suggests applying the reference samples sub-sampling mechanism to all block-sizes in DMM and SDC. Both test cases at common test condition and All intra condition for SDC and SDC + DMM are verified. Experimental results show that the proposed method can achieve 0.0% and 0.0% for AI and CTC in SDC case, and 0.0% and -0.1% for AI and CTC in SDC+DMM case. The decoding time for all cases has slight decrease.

# Introduction

At 3rd JCT-3V meeting, the reference samples sub-sampling for DMM and SDC mechanism has been adopted. The sub-sampling is applied for 32x32 block in DMM, and 32x32 and 64x64 blocks in SDC. The computing complexity for SDC and DMM is reduced without impact on coding performance. To further reduce the computing complexity, this contribution suggests applying reference samples sub-sampling to all block-size in DMM and SDC. To improve the coding performance, encoder optimization is also used.

# Reference samples sub-sampling for all block-sizes in DMM and SDC

With the extension of the suggested sub-sampling method, the reference samples for the intra blocks with the size from 4x4 to 32x32 in DMM, and the intra blocks with the size from 8x8 to 64x64 in SDC will use the 2:1 sub-sampling method.

In SDC case, the DC values for the original samples and the prediction samples in an intra block are calculated. The sdc_residual is derived by the subtraction between the two DC values. Considering that the reconstruction samples might be affected by the reference sub-sampling, the DC value for the original samples is suggested to be calculated without sub-sampling. This encoder side optimization can guarantee that the reconstruction samples for the proposed method in this contribution is still same as before so that the performance is not affected by the sub-sampling. On the other hand, the computing complexity for the decoder can still be reduced as the proposed sub-sampling method is used at the decoder side.

# Experimental results

The proposed method is implemented on HTM6.0. The coding efficiency is evaluated in random access (CTC) and All-Intra configurations. All block-sizes in SDC and DMM use the reference sub-sampling method to calculate the DC value of the prediction samples. The DC value for the original samples is calculated without the reference sub-sampling.

The table 1 and table 2 show the results that only apply the sub-sampling to SDC. Table 3 and table 4 show the results that apply the sub-sampling to both SDC and DMM.

Table 1: sub-sampling for SDC (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 98.7% | 103.4% |
| Kendo | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 100.2% | 97.2% | 102.3% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% | 99.2% | 102.4% | 95.9% |
| GT_Fly | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 99.4% | 88.1% | 95.3% |
| Poznan_Hall2 | 0.0% | 0.2% | -0.2% | 0.0% | 0.0% | 0.0% | 100.2% | 98.1% | 98.2% |
| Poznan_Street | 0.0% | 0.3% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 110.3% | 107.5% |
| Undo_Dancer | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 99.8% | 94.5% | 95.6% |
| 1024x768 | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 99.9% | 99.4% | 100.5% |
| 1920x1088 | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 99.8% | 97.8% | 99.2% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 98.5% | 99.7% |

Table 2: sub-sampling for SDC (All - intra)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 100.2% | 99.3% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.8% | 100.1% | 99.3% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.4% | 97.4% | 100.2% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 98.7% | 99.0% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 102.1% | 100.5% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 103.1% | 99.3% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 104.4% | 99.7% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 99.2% | 99.6% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 102.1% | 99.6% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 100.9% | 99.6% |

Table 3: sub-sampling for SDC and DMM (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% | 100.2% | 98.8% | 102.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 100.2% | 96.2% | 101.6% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.4% | 96.5% | 94.9% |
| GT_Fly | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% | 99.9% | 89.8% | 97.1% |
| Poznan_Hall2 | 0.0% | -0.6% | 0.1% | -0.1% | -0.1% | -0.1% | 100.4% | 97.3% | 97.9% |
| Poznan_Street | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 100.0% | 108.3% | 111.0% |
| Undo_Dancer | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | -0.5% | 99.8% | 94.8% | 97.3% |
| 1024x768 | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 97.2% | 99.5% |
| 1920x1088 | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | -0.1% | 100.0% | 97.5% | 100.8% |
| average | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 100.1% | 97.4% | 100.3% |

Table 4: sub-sampling for SDC and DMM (All-intra)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.0% | 100.0% | 99.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.1% | 97.6% | 99.4% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 100.7% | 101.4% | 100.2% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.5% | 98.3% | 99.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 97.5% | 100.6% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 99.0% | 97.6% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 99.9% | 98.2% | 99.2% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 99.7% | 99.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 98.2% | 99.2% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 98.9% | 99.3% |

Considering that several contributions submitted at this meeting also change the reference samples for DC calculation, we perform some combination tests between the proposed sub-sampling method and the method presented in those contributions. Table 5 and table 6 show the combination results of D0193 and the proposed sub-sampling. The test results show that the combination solution almost does not have impact on the coding performance.

Table 5: sub-sampling for SDC and DMM (CTC, based on D0193 design)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.8% | -0.8% | -0.3% | -0.3% | -0.3% | 100.1% | 99.2% | 100.8% |
| Kendo | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% | 99.4% | 98.7% | 97.2% |
| Newspaper_CC | 0.0% | -0.5% | -0.2% | -0.1% | -0.1% | -0.2% | 98.0% | 92.5% | 96.6% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.5% | 94.5% | 96.5% |
| Poznan_Hall2 | 0.0% | -0.7% | 0.1% | -0.1% | -0.1% | -0.2% | 100.6% | 103.5% | 98.2% |
| Poznan_Street | 0.0% | -0.3% | -0.4% | -0.1% | -0.1% | -0.1% | 100.8% | 96.4% | 98.8% |
| Undo_Dancer | 0.0% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% | 99.7% | 98.4% | 96.7% |
| 1024x768 | 0.0% | -0.5% | -0.3% | -0.1% | -0.1% | -0.2% | 99.2% | 96.8% | 98.2% |
| 1920x1088 | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 100.1% | 98.2% | 97.5% |
| average | 0.0% | -0.3% | -0.1% | -0.1% | -0.1% | -0.1% | 99.7% | 97.6% | 97.8% |

Table 6: sub-sampling for SDC and DMM (All-intra, based on D0193 design)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | -0.01% | 100.0% | 95.1% | 98.3% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | -0.01% | 101.4% | 102.8% | 102.8% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 0.05% | 102.4% | 102.7% | 101.6% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | -0.06% | -0.07% | 99.0% | 92.0% | 93.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 0.05% | 100.3% | 98.8% | 98.9% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | 0.01% | 100.3% | 96.4% | 93.5% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.04% | 100.1% | 99.5% | 101.7% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | 0.01% | 101.3% | 100.2% | 100.9% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | 0.00% | 99.9% | 96.7% | 96.8% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | 0.01% | 100.5% | 98.2% | 98.6% |

# Changes to the description of Test Model

#1

The variable log2SubSample is set equal to ( !intraChainFlag && ( nT < 32 ) ) 0 : 1.

For X being replaced by 0 and 1, the values of the prediction samples predSamples[ x][ y ] are derived as specified by the following ordered steps:

  - The variable sumNeigh specifying the sum of the neighbouring samples depth values of partition X is set equal to 0 and the variable numNeigh specifying the number of the neighbouring samples of partition X is set equal to 0 and the following applies.

    - For x = 0..( nT >> log2SubSample ) − 1, inclusive the following applies:

      - When partitionPattern[ x << log2SubSample ][ 0 ] is equal to X, the following applies:

          - sumNeigh += p[ x<< log2SubSample ][ −1 ]	(Error: Reference source not found-1)
numNeigh += 1		(Error: Reference source not found-2)

    - For y = 0..( nT  >> log2SubSample )− 1,inclusive the following applies:

      - When partitionPattern[ 0 ][ y << log2SubSample ] is equal to X, the following applies:

          - sumNeigh += p[ −1 ][ y << log2SubSample ]	(Error: Reference source not found-3) 
numNeigh += 1		(Error: Reference source not found-4)

#2

The variable log2SubSample is set equal to ( !intraChainFlag && ( nT < 32 ) ) 0 : 1.

For X being replaced by 0 and 1, the values of the prediction samples predSamples[ x][ y ] are derived as specified by the following ordered steps:

  - The variable sumNeigh specifying the sum of the neighbouring samples depth values of partition X is set equal to 0 and the variable numNeigh specifying the number of the neighbouring samples of partition X is set equal to 0 and the following applies.

    - For x = 0..( nT >> log2SubSample ) − 1, inclusive the following applies:

      - When partitionPattern[ x << log2SubSample ][ 0 ] is equal to X, the following applies:

          - sumNeigh += p[ x<< log2SubSample ][ −1 ]	(Error: Reference source not found-5)
numNeigh += 1		(Error: Reference source not found-6)

    - For y = 0..( nT  >> log2SubSample )− 1,inclusive the following applies:

      - When partitionPattern[ 0 ][ y << log2SubSample ] is equal to X, the following applies:

          - sumNeigh += p[ −1 ][ y << log2SubSample ]	(Error: Reference source not found-7) 
numNeigh += 1		(Error: Reference source not found-8)

          - (Error: Reference source not found-9)

# Conclusion

This contribution suggests applying the reference samples sub-sampling to all block-sizes in DMM and SDC modes. With the encoder optimization method, the proposed solution doesn't have impact on all test cases. The computing complexity and decoder run time are also reduced. Therefore, it is recommended to incorporate the proposed sub-sampling solution into next version of 3D-HEVC Test Model and HTM7.

# Patent rights declaration(s)

Hisilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).