| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0176 |
| --- | --- |

| Title: | CE2.h: Enhanced disparity vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jewon Kang Ying Chen Li Zhang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8457 jewonk@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal is a follow-up of JCT3V-C0050. The main idea of this proposal is to use multiple disparity motion vectors conveyed in a texture component for a disparity vector (DV) derivation. The same as in JCT3V-C0050, multiple disparity motion vectors identified from spatial and temporal neighboring blocks and used to calculate a single disparity vector. In addition, a slightly modified method is also presented. The difference between the two methods is related to whether spatial and temporal neighboring blocks are processed in parallel. The coding gain of the proposed method, when evaluated by total bit-rates and the average PSNR of the synthesized views, is -0.1% for both the BVSP on and off cases. The slightly modified method increases the gain for the texture-only coding from 0.1% to 0.2%.

# Introduction

In the current 3D-HEVC [1], a disparity vector (DV) is derived using the NBDV process followed by the Do-NBDV process [2-6]. In the NBDV, a disparity motion vector in spatial and temporal neighboring blocks, or a derived disparity vector (DDV) in spatial neighboring blocks are found. Pre-defined neighboring blocks and temporal reference frame candidates are checked in order. Specifically, two temporal neighboring block positions (i.e. the blocks at the center and the bottom right positions of a co-located PU) in up to two reference pictures, followed by five spatial neighboring block positions in the order of the left, above, above-right, left-below, and above-left, are checked [2-5]. After the NBDV, the obtained disparity motion vector is refined using depth information of a reference view [6]. The refined disparity motion vector becomes a DV, and the derivation process is finished.

The main idea of JCT3V-C0050 [7] is to use multiple disparity motion vectors in the NBDV process rather than to use a single disparity motion vector. In the current design, a disparity motion vector, which could be an outlier in neighboring motion vectors, can be chosen only because the disparity motion vector was found earlier than any other vectors in the checking order. However, the outlier may not serve as a good predictor to locate the corresponding depth block. Thus, in this proposal, we attempt to enhance the accuracy of the disparity motion vector placing the corresponding depth block by considering multiple disparity motion vectors and their improvements by mathematical functions. The enhancement is performed using only motion information transferred from the only texture components. There can be two different manners to handle multiple disparity motion vectors, which will be presented. The DoNBDV is performed one time in the whole DV derivation process because of a complexity reduction.

# Proposal

The overall procedure of the proposed method is summarized in Figure 1, and the procedure is common for the two methods. As shown, the proposed method has two-step procedures, i.e., a searching phase of non-zero disparity motion vectors in spatial and temporal neighboring blocks, and a decision phase applying mathematical functions. If there is no searched disparity motion vector in the first searching phase, the DDV is further checked as in the current design, but the second phase is not applied.

Figure 1: Flow chart of the proposed methods that are common for the Method 1 and Method 2.

## Method 1

### Parallel searching manner of disparity vector candidates

Denote a non-zero disparity motion vector searched in a spatial neighboring position by spatial disparity vector candidates (SDVC), and a non-zero disparity motion vector searched in a temporal neighboring position by temporal disparity vector candidates (TDVC).

The proposed derivation process checks the same pre-defined block positions as the current design, and constructs the SDVC and TDVC lists. Each available candidate is inserted to the corresponding candidate list until the searching phase is finished, if one of the following conditions is true:

- The number of the collected disparity vector candidates exceeds a threshold set equal to 3.

- All pre-defined blocks are checked.

In this way, the TDVC and SDVC are differently managed in the following steps.

### Mathematical function applied to multiple candidates

Simple mathematical functions concerning computational complexity is used to derive a disparity vector by using the constructed SDVC and TDVC lists. In principle, both SDVC and TDVC are considered together as inputs for the mathematical functions to derive the final one. However, if there are insufficient numbers of disparity motion vectors in SDVC (or TDVC), the other vectors in TDVC (or SDVC) is taken account. Meanwhile, based on the total number of the searched disparity vector candidates, the derivation process has some variations in the function. The following scenario is considered.

–	If the total number of the SDVC and the TDVC is more than or equal to three, median function with three inputs is applied where the first two TDVC and one SDVC are used. However, there are not enough TDVC (or SDVC), the available SDVC (or TDVC) are used instead.

–	Otherwise, if the total number is equal to two, a larger one of the first available TDVC and SDVC is chosen. However, if none of the SDVC is available, two TDVC becomes the inputs, and vice versa.

–	Otherwise, the only available one becomes the disparity motion vector to be refined.

In all the procedures, only x-components of the disparity motion vectors are taken into account. That is, the horizontal components of the candidates become the input arguments of the component-wise functions. After one horizontal component with the functions among the candidates is determined, the corresponding vertical component is identified to complete a disparity motion vector.

## Method 2

### Serial searching manner of disparity vector candidates

The main difference between Method 1 is to use a single list to gather disparity motion vectors regardless whether they are from SDVC or TDVC.

The pre-defined block positions are checked in the same order of the current design (temporal block position first), and a list is constructed to contain the disparity motion vectors. Because the searching manner is performed in serial, a disparity motion vector found earlier is inserted to the list first. In a similar manner to the Method 1, the list construction is completed if one of the conditions satisfies:

- The number of the collected disparity vector candidates exceeds a threshold set equal to 3.

- All pre-defined blocks are checked.

### Mathematical function applied to multiple candidates

The same functions as the Method 1 are applied to the disparity motion vectors in the list, but the Method 2 does not consider the type of disparity motion vectors. Thus, it only counts the number of elements in the list, described as follows:

–	If the total number of the disparity motion vectors is more than or equal to three, a median function with three inputs is applied to the first three disparity motion vectors in the list.

–	Otherwise, if the total number is equal to two, a larger one of the two is chosen,

–	Otherwise, the only available one becomes the disparity motion vector to be refined.

# Experimental results

Simulation results of the proposal are shown in Table 1-4. The implementation was based on HTM6.1, and simulations are done under common test conditions [8].

## Common Test Condition (CTC)

As shown in Table 1, the Method 1 provides -0.1% gain in synthesized views, and, as shown in Table 2, the Method 2 provides a small gain only in video 1. The changes in encoding/decoding time are minor.

Table 1: Method 1 VS HTM6.1 (CTC).

Table 2: Method 2 VS HTM6.1 (CTC).

## BVSP and Do-NBDV Off

In this configuration, we turned off the low-level coding tools such as BVSP and Do-NBDV, enabling the depth information for the texture view component coding.

As shown in Table 3 and Table 4, the Method 1 and Method 2, respectively, provide -0.1% gain and -0.2% gain in the texture view component coding.

Table 3: Method 1 VS HTM6.1 (BVSP and DoNBDV Off).

Table 4: Method 2 VS HTM6.1 (BVSP and DoNBDV Off).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 3,” JCT3V-C1005, Jan. 2013.

- L. Zhang, Y. Chen, M. Karczewicz, “3D-CE5.h: Disparity vector generation results,” JCT3V-A0097, Jul. 2012.

- J. Sung, M. Koo, S. Yea, “3D-CE5.h: Simplification of disparity vector derivation for HEVC-based 3D video coding,” JCT3V-A0126, Jul. 2012.

- J. Kang, Y. Chen, L. Zhang, M. Karczewicz, “3D-CE5.h related: Improvements for disparity vector derivation,” JCT3V-B0047, Oct. 2012.

- J. Y. Lee, H. C. Wey, D.S. Park, “3D-CE2.h relate: results on disparity vector derivation,” JCT3V-C0097, Jan. 2013.

- Y.-L. Chang, C.-L. Wu, Y.-P. Tsai, S. Lei, “3D-CE1.h: Depth-oriented neighboring block disparity vector (DoNBDV) with virtual depth retrieval,” JCT3V-C0131, Jan. 2013.

- J. Kang, Y. Chen, L. Zhang, M. Karczewicz, “3D-CE2.h related: Enhanced disparity vector derivation,” JCT3V-C0050, Jan. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-C1100, Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).