| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0177 |
| --- | --- |

| Title: | CE4: Advanced residual prediction for multiview coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Xiang Li Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Currently inter-view residual prediction is enabled in the current HTM design to code the residue of dependent texture views more efficiently. In this proposal, an advanced residual prediction (ARP) is proposed to further improve the coding efficiency of inter-view residual prediction. This proposal is a follow-up of JCT3V-C0049 with the parsing issue removed. In ARP, to ensure high correlation between residues of two views, motion of the current block of picture in current view is applied to the corresponding block in a reference view picture to generate residual in the base view to be used for inter-view residual prediction. Moreover, an adaptive weighting factor is applied to the residue signal so that the prediction error can be further reduced. It is shown that the proposed method achieves compression efficiency gain of 0.8% and 1.3% for coded views in terms of BD rate compared to HTM anchor and HTM with inter-view residual prediction disabled, respectively. To further reduce the memory bandwidth, it is proposed to use fixed reference pictures for all blocks coded with ARP. Compared to the original ARP, the coding loss introduced by such simplification is less than 0.05% for synthesized views. In addition, the results of applying bi-linear filter for non-base texture views in ARP are also reported. It is claimed that analysis shows that the proposed method achieves more than 2.5 times of the coding gain compared to the current inter-view residual prediction while require only insignificant increase of implementation complexity compared to inter-view residual prediction.

# Introduction

In the current design of 3D-HEVC, when inter-view motion prediction is enabled and one prediction unit (PU) selects one merging candidate derived from the motion information of the corresponding block in the reference view, inter-view residual prediction (IVRP) is enabled, i.e., the residue of the corresponding block is used as a predictor of the residue of current. The residue of the corresponding block is added to the motion compensated signal and only the resulting difference signal is transform coded and transmitted. In other words, the residue generated from motion compensation is further predicted by the residue from the base view, before it is transformed, quantized and written in the bitstream.

Although the current IVRP further improves the overall quality of prediction, it has two main drawbacks.

- First, reconstructed residues are used in prediction, which introduces undesired quantization error in prediction and further degrades the performance.

- Second, IVRP is only applied to PUs which selects the merging candidate derived from the reference view. The prohibition of the usage of IVRP restricts the coding gain of IVRP. Meanwhile, when inter-view motion prediction is disabled, IVRP no longer has any effect.

- Third, one additional image shall be allocated to store the residual information of the reference view.

# Proposed method

To further improve the coding efficiency, it is proposed that motion of current view is applied to the corresponding block in a reference view to have a better prediction performance. Moreover, additional weighting factor is introduced to further compensate quality difference between different views.

illustrates the prediction structure of ARP in multiview video coding. As shown in , Dc represents the current block in the current view (view 1), Bc, and Dr denote the representation of Dc in the reference view (view 0) at time Tj and Dc’s temporal prediction from the same view (view 1) at time Ti. VD denotes the motion from Dc to Dr. Since Dc and Bc are actually projections of the same object in two different views, these two blocks should share the same motion information. Therefore, Bc’s temporal prediction Br in view 0 at time Ti can be located from Bc by applying the motion information of VD. The residual of Bc with motion information of VD is then multiplied by a weighting factor and used as the residual predictor for current residual.

Figure 1. Prediction structure of the proposed ARP in 3D-HEVC.

Figure 2. Relationship among current block, reference block and motion compensated block

## Decoding process of ARP

Main procedures of the proposed ARP at the decoder side can be described as follows:

- Obtain a disparity vector as specified in the current 3D-HEVC, pointing to a target reference view. Then, in the picture of the reference view within the same access unit, the corresponding block is located by the disparity vector.

- Re-use the motion information of the current block to derive the motion information for the reference block. Apply motion compensation for the corresponding block based the same motion vector of current block and derived reference picture in the reference view for the reference block, to derive a residue block. The relationship among current block, corresponding block and motion compensated block is shown in Figure 2. The reference picture in the reference view (V0) which has the same POC (Picture Order Count) value as the reference picture of current view (Vm) is selected as the reference picture of the corresponding block.

- Apply the weighting factor to the residue block to get a weighted residue block and add the values of the weighted residue block to the predicted samples.

## Weighting factor

Three weighting factors are used in ARP, i.e., 0, 0.5 and 1. The one leading to minimal rate-distortion cost for the current CU is selected as the final weighting factor and the corresponding weighting index (0, 1 and 2 which correspond to weighting factor 0, 1, and 0.5, respectively) is transmitted in the bitstream at the CU level. All PU predictions in one CU share the same weighting factor. When the weighting factor is equal to 0, ARP is not used for the current CU.

## Removal of parsing issue

Different from JCT3V-C0049 [1], the conditional signaling of weighting factor index which depends on the decoded reference picture information is removed. Therefore, there is no parsing issue in this proposal.

## Further simplifications

## Reference picture selection via motion vector scaling

In JCT3V-C0049, the reference pictures of prediction units coded with non-zero weighting factors may be different from block to block. Therefore, different pictures from the reference view may need to be accessed to generate the motion-compensated block ( i.e., Br in Figure 1 ) of the corresponding block. It is proposed to scale the motion vectors of the current PU towards a fixed picture before performing motion compensation, when the weighting factor is unequal to 0. In this proposal, the fixed picture is defined as the first reference picture of each reference picture list.

## Interpolation filter

In JCT3V-C0049, bi-linear filter is applied during the interpolation process of the corresponding block and its prediction block. While for the prediction block of the current PU in the non-base views, the conventional 8/4-tap filter is applied. It is proposed to always employ bi-linear regardless the block is in base view or non-base view when ARP is applied, if memory bandwidth the PU that uses ARP mode is an critical issue, which is not necessarily the case.

# Compression performance

This section provides simulation results of the proposed ARP in comparison with the 3D-HTM anchors. The platform 3D-HTM 6.0 [2] is utilized and the proposed method is implemented on it. All the simulation tests are performed based on the common test conditions [3]. Two anchors are compared with ARP, i.e., the anchor of 3D-HTM 6.0 and the anchor of 3D-HTM 6.0 with IVRP disabled.

Table 1 provides the results of ARP without scaling compared with 3D-HTM anchor. As shown in Table 1, the overall average bitrate reduction is 0.8%, 0.8% and 0.6% for texture views, coded views, and synthesized views, respectively.

As shown in Table 2, when IVRP is disabled in 3D-HTM, additional coding gain of ARP without scaling could be observed, i.e., 1.3%, 1.2%, 1.1% for texture views, coded views, and synthesized views, respectively.

Table 3 provides the results of ARP with scaling (ARP simp #1) compared with 3D-HTM anchor. As shown in Table 3, the overall average bitrate reduction is 0.7%, 0.7% and 0.6% for texture views, coded views, and synthesized views, respectively.

Table 4 provides the results of ARP with scaling and bilinear both applied (ARP simp #2) compared with 3D-HTM anchor. As shown in Table 3, the overall average bitrate reduction is 0.6%, 0.6% and 0.5% for texture views, coded views, and synthesized views, respectively.

Note for all the results listed below, the decoding time is calculated with one single machine in windows OS. For encoding and rendering times, the data may not be accurate due to the usage of hybrid CPUs.

Table 1: Coding gain of ARP with respect to 3D-HTM anchor for 3-view case (IVRP on)

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -4.3% | -3.8% | -1.4% | -1.3% | -1.2% | 103% | 92% | 94% |
| Kendo | -3.5% | -3.9% | -1.4% | -1.2% | -1.0% | 101% | 91% | 92% |
| Newspapercc | -3.2% | -2.3% | -1.0% | -0.9% | -0.7% | 105% | 88% | 98% |
| GhostTownFly | -0.8% | -0.9% | -0.4% | -0.4% | -0.3% | 103% | 91% | 100% |
| PoznanHall2 | -1.3% | -1.2% | -0.5% | -0.4% | -0.2% | 103% | 88% | 91% |
| PoznanStreet | -3.3% | -3.2% | -0.9% | -0.8% | -0.8% | 101% | 87% | 91% |
| UndoDancer | -1.3% | -1.2% | -0.3% | -0.3% | -0.2% | 106% | 91% | 94% |
| 1024x768 | -3.7% | -3.3% | -1.3% | -1.2% | -1.0% | 103% | 90% | 95% |
| 1920x1088 | -1.6% | -1.6% | -0.5% | -0.5% | -0.4% | 103% | 89% | 94% |
| average | -2.5% | -2.4% | -0.8% | -0.8% | -0.6% | 103% | 90% | 94% |

Table 2: Coding gain of ARP with respect to 3D-HTM anchor for 3-view case (IVRP off)

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -6.3% | -6.0% | -2.3% | -2.2% | -1.9% | 110% | 103% | 102% |
| Kendo | -6.4% | -7.3% | -2.7% | -2.4% | -2.1% | 110% | 102% | 102% |
| Newspapercc | -4.8% | -3.7% | -1.6% | -1.5% | -1.2% | 110% | 101% | 104% |
| GhostTownFly | -0.9% | -1.1% | -0.4% | -0.4% | -0.3% | 106% | 100% | 101% |
| PoznanHall2 | -1.3% | -1.3% | -0.5% | -0.5% | -0.3% | 112% | 100% | 102% |
| PoznanStreet | -3.9% | -4.3% | -1.2% | -1.2% | -1.1% | 111% | 100% | 101% |
| UndoDancer | -2.1% | -2.0% | -0.7% | -0.6% | -0.6% | 111% | 100% | 97% |
| 1024x768 | -5.8% | -5.6% | -2.2% | -2.0% | -1.7% | 110% | 102% | 103% |
| 1920x1088 | -2.1% | -2.2% | -0.7% | -0.7% | -0.6% | 110% | 100% | 100% |
| average | -3.7% | -3.7% | -1.3% | -1.2% | -1.1% | 110% | 101% | 101% |

Table 3: Coding gain of ARP Simp #1 with respect to 3D-HTM anchor for 3-view case (IVRP on)

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -3.7% | -3.2% | -1.2% | -1.2% | -1.0% | 102% | 92% | 95% |
| Kendo | -3.1% | -3.5% | -1.2% | -1.0% | -0.9% | 100% | 92% | 93% |
| Newspapercc | -2.9% | -2.1% | -0.9% | -0.8% | -0.7% | 105% | 89% | 97% |
| GhostTownFly | -0.7% | -0.7% | -0.3% | -0.3% | -0.3% | 103% | 92% | 98% |
| PoznanHall2 | -1.4% | -1.2% | -0.5% | -0.5% | -0.3% | 102% | 89% | 90% |
| PoznanStreet | -3.1% | -3.1% | -0.8% | -0.8% | -0.8% | 101% | 88% | 90% |
| UndoDancer | -1.1% | -1.1% | -0.3% | -0.3% | -0.3% | 105% | 91% | 93% |
| 1024x768 | -3.2% | -2.9% | -1.1% | -1.0% | -0.9% | 103% | 91% | 95% |
| 1920x1088 | -1.6% | -1.5% | -0.5% | -0.5% | -0.4% | 103% | 90% | 93% |
| average | -2.3% | -2.1% | -0.7% | -0.7% | -0.6% | 103% | 90% | 94% |

Table 4: Coding gain of ARP Simp #2 with respect to 3D-HTM anchor for 3-view case (IVRP on)

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -3.2% | -3.0% | -1.0% | -0.9% | -0.8% | 101% | 90% | 95% |
| Kendo | -2.8% | -3.2% | -1.0% | -0.9% | -0.8% | 100% | 90% | 93% |
| Newspapercc | -2.6% | -1.9% | -0.8% | -0.7% | -0.6% | 104% | 88% | 98% |
| GhostTownFly | -0.5% | -0.5% | -0.1% | -0.1% | -0.1% | 102% | 91% | 102% |
| PoznanHall2 | -1.4% | -1.1% | -0.4% | -0.4% | -0.2% | 102% | 89% | 89% |
| PoznanStreet | -2.9% | -2.9% | -0.8% | -0.7% | -0.7% | 101% | 87% | 93% |
| UndoDancer | -1.1% | -1.1% | -0.2% | -0.2% | -0.1% | 105% | 79% | 96% |
| 1024x768 | -2.9% | -2.7% | -0.9% | -0.8% | -0.7% | 102% | 89% | 95% |
| 1920x1088 | -1.5% | -1.4% | -0.4% | -0.4% | -0.3% | 102% | 87% | 95% |
| average | -2.1% | -1.9% | -0.6% | -0.6% | -0.5% | 102% | 88% | 95% |

# Discussions on implementation of ARP

## Hardware implementation implications

In this section, we provide analysis from different aspects on the implementation of the proposed ARP compared with the current HEVC motion compensation and inter-view residual prediction.

### Number of cycles

Video decoder hardware is typically implemented with multiple pipeline stages, each of which performs in budget cycle. The cycle budget depends on targeted hardware speed and different use cases such as spatial resolution and frame rate. Usually the different stages include entropy codec, syntax parser engine, transform, motion compensation and in-loop filtering (e.g., including de-blocking and SAO). The stage that requires the largest number of cycles becomes the bottleneck and shouldn’t be made more complicated, potentially computation increases may be acceptable for other stages which consumes significantly less cycles.

Typically the motion compensation stage takes roughly 60% to 80% of cycle budgets. Therefore, there are extra cycles in the motion compensation stage to accommodate the additional motion compensations for ARP. For the same reason, the fact that more calculations are needed for inter-view residual prediction doesn’t seem to lead to a problem. If these spare 20%-40% cycles are not enough, double calculation capability can be achieved in motion compensation stage by even adding a parallel compensation engine with reasonable hardware cost, as shown in 4.2.2, without surpassing the cycle budget.

### Increase of hardware cost

Motion compensation engine takes about 10% or a little less logic in a typical video decoder system on chip, therefore the increased design logic of motion compensations in ARP might be could be much less than 10% since bi-linear filter is applied instead of 8/4-tap filters. The final hardware cost (silicon area size) may be smaller than this number if floor planning is designed carefully.

Using such a design logic increase to significantly increase the coding performance is acceptable.

### Power consumption

For the ARP mode, the power increase is roughly the same as the design logic increase. However, the ARP mode is not always chosen. Therefore, more intelligently power management scheme such as clock gating may apply to significantly reduce the average power increase, to the same level as existing HEVC design.

### Storage and memory bandwidth

There are several aspects for this category and we provide the analysis one by one.

#### DPB memory

The ARP method utilizes decoded picture that is present in the DPB. Inter-view residual prediction in current 3D-HEVC, however needs to generate a decoded residual picture for the base (reference) view. Therefore, the proposed method decreases the overall memory size.

#### Memory access inside the L2 cache (on-chip memory)

Even not the fastest, accessing the L2 cache is considered as lightweight in hardware implementations. In a video decoder, possible reference regions may be loaded into the L2 cache in advance so that to avoid real-time data fetching beyond L2 cache.

For example, samples in certain regions of reference frames may be dynamically managed in the L2 cache so that it is most likely available when a motion compensation module requires accessing the reference samples.

Therefore, the fetching of external memory into the L2 cache can be done in the background pipeline much earlier to avoid on-the-fly fetching of external memory.

It is always a trade-off for the memory size of the L2 cache and the chances of cache misses, which require on-the-fly fetching external memory from DPB.

The fact of possibly more memory required in L2 may be accommodated by two factors:

- More advanced mechanism to manage which reference picture regions to be pre-fed into the L2 cache;

- Upgraded high-end SoC equipped with more memory in the near future.

If the potential increase of L2 cache to reduce cache misses in ARP considered as a serious issue, the target reference frames used by ARP can be unified to just one frame, by scaling the motion vector, as described in section 2.4.

#### Memory access from external memory

In the original proposed ARP method, since it applies only to 2Nx2N, the worst case is for each 8x8 block, 9*9*3 pixels need to be accessed for each 8x8 group of pixels. Assume the original motion compensation uses bi-directional prediction, the total average number of memory accesses is 450. So in average, each pixel needs accessing (450+9*9*3) / 64 = 10.8 pixels in average.

In the simplified ARP method as described in 2.4.2, since bi-linear filter is always applied, the worst case is for each 8x8 block, 9*9*3 pixels in base view and 9*9*2 pixels in non-base view need to be accessed for each 8x8 group of pixels in the bi-prediction case. So in average, each pixel needs accessing (9*9*2+9*9*3) / 64 = 6.3 pixels in average.

In the inter-view residual prediction method, worst case if for each 8x8 block, 9*9 piles need to be accessed for each 8x8 group of pixels. In this case, in average, each pixel needs accessing (450+9*9) / 64 = 8.3 pixels in average.

Since the additional fetching required by ARP is still not significant when considering the original motion compensation, the memory bandwidth, if purely evaluated by the number of pixels to access for each pixel in a current block, is still in a level comparable to the situation in inter-view residual prediction.

The typical cache scheme and pipeline stage design can tolerate up to 2x amount of data fetches with about 20% overall cycle impact. Considering the extra amount of fetches for ARP over original motion compensation, the data fetch cycle penalty should be acceptable.

## Complexity analysis

As described in the CE4 document [4], the complexity for the worst case should be provided.

The worst case of ARP in terms of memory bandwidth requirement, numbers of multiplications and additions is analyzed based on the template [5] provided by AHG 17. Error: Reference source not found provides a summary.

Table 5 Analysis of the worst case of ARP and IVRP (Anchor is HEVC single view coding)

| Coding Tools | Num. Mul | Num. Adds | Memory Bandwidth |
| --- | --- | --- | --- |
| IVRP | 109% | 106% | 123% |
| ARP without simplifications | 128% | 117% | 169% |
| ARP simp #2 | 47% | 28% | 115% |

As shown in the table, both the original ARP and IVRP have higher complexity than HEVC single view coding. The memory access of the original ARP is increased compared to IVRP since ARP needs to access more reference blocks. When the simplified ARP method is applied, even lower complexity than IVRP (in terms of memory bandwidth and computational complexity) is observed. Here memory bandwidth is calculated assuming 4x2 pattern is used for loading and writing data.

## Summary of the inter-view residual prediction and ARP methods

In Table 6, we provide a comparison the current IVRP and the ARP methods in different aspects to clarify the BD performance gain and implementation complexity and cost. Note that hardware design may trade-off the memory bandwidth increase with the size of L2 cache, so that the number of real-time fetches from the external memory can be minimized with reasonable increase of on-chip memory or advanced mechanism to control the communication between L2 cache and external memory.

We believe the number of calculations less a concern in hardware implementation for the residual prediction related methods. Note that in Table 6, N is the number of temporal pictures used for inter prediction within one view.

Table 6: Summarization of comparisons of IVRP and ARP

| Coding Tools | Coding gain (vs HTM with IVRP off) | Potential memory increase in L2 cache | Memory Bandwidth | Num. Mul | Num. Add |
| --- | --- | --- | --- | --- | --- |
| IVRP | 0.5% | 1 (O(1)) | 123% | 109% | 106% |
| ARP | 1.3% | N (O(n)) | 169% | 128% | 117% |
| ARP simp #1 | 1.3% | 2 (O(1)) | 169% | 128% | 117% |
| ARP simp #2 | 1.1% | 2 (O(1)) | 115% | 47% | 28% |

# References

- L. Zhang, Y. Chen, X. Li, M. Karczewicz, "3D-CE4: Advanced residual prediction for multiview coding," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0049, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

- 3D-HTM version 6.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.0/.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1100, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

- L. Zhang, "Description of Core Experiment 4 (CE4): Residual Prediction," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1104, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

- E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440, Geneva, Switzerland, 14–23 Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).