| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0178 |
| --- | --- |

| Title: | CE5.h: Merge candidates derivation from vector shifting |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vijayaraghavan Thirumalai Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA Thomas Guionnet Christine Guillemot Laurent Guillo | Tel: Email: | 1-858-845-5663 vthiruma@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com +33 299-847-100 thomas.guionnet@inria.fr christine.guillemot@inria.fr laurent.guillo@inria.fr |
| Source: | Qualcomm Incorporated and INRIA, France |  |  |

_____________________________

# Abstract

When inter-view motion prediction is enabled, the current HTM design of the merge candidate list may include an inter-view predicted motion candidate before all the spatial merging candidates. In addition, the disparity vector may also be converted to a disparity motion vector and it is inserted into the list following the above-right spatial candidate. As a harmonized follow-up proposal of JCT3V-C0045 and JCT3V-C0148, it is proposed to add two more candidates. Each candidate is derived with a horizontally shifted disparity vector being used for inter-view motion prediction and if it doesn’t provide an available candidate, then it is derived from a horizontally shifted spatial neighbouring candidate, if it contains a disparity motion vector. Compared to the current HTM design, the proposed method achieves compression efficiency gain of 0.2% in terms of BD rate for both BVSP on and off cases.

# Introduction

In 3D-HEVC, up to six merging candidates are made available for selection when inter-view motion prediction is enabled. When inter-view motion prediction is enabled, the merge candidate list is constructed as follows:

- One candidate derived from inter-view motion prediction, is added to the merge candidate list if available.

As depicted in Figure 1, a corresponding block in the reference view is located based on the disparity vector Error: Reference source not found. The Inter-view Predicted Motion Candidate (IPMC) derived from the corresponding block, if available, is added. Note that IPMC is typically a candidate containing motion vectors referring to temporal reference pictures.

- Spatial merging candidates derived from neighbouring prediction unit (PU) partitions are added to the merge candidate list. This process is similar to that in HEVC, where redundant candidates among the initial available spatial candidates are eliminated from the merging list by comparing motion information of the candidates. In addition, the spatial merging candidates derived from A1 and B1 are compared with IPMC to eliminate the redundancy. Figure 2 depicts the spatial neighbours (A1, B1, B0, A0, B2) which are the potential candidates for the merging list.

- If any of the five spatial neighbouring blocks (A0, A1, B0, B1 or B2) is coded with the BVSP mode, i.e., the maintained flag of the neighbouring block is equal to 1, BVSP merging candidate is treated as the corresponding spatial merging candidate and inserted to the merge candidate list. Note BVSP merging candidate will only be inserted to the merge candidate list once.

- The disparity vector is converted to a disparity motion vector and is further added into the merge candidate list right after the spatial merging candidate from B0 and before the one from A0, if different from the candidates derived from A1 and B1.

- If none of the five spatial neighbouring blocks are coded with the BVSP mode, the BVSP merging candidate is inserted to the merge candidate list just before the temporal merging candidates.

- Other motion vector merging candidates (including temporal/combined bi-predictive/zero motion vector merging candidates) are added to the merge candidate list if the number of valid candidates is less than the maximum number.

Figure 1. Derivation of inter-view predicted motion candidate (IPMC) for the merge mode.

Figure 2. Potential spatial neighbouring candidates for the merge candidate list. Arrows indicate which spatial candidate(s) are to be compared.

The current design may have the following problems that are listed below.

- The disparity vector (DV) may be not accurate enough which results in sub-optimal candidates.

- The disparity motion vector (DMV) of the spatial neighbouring candidates A0, A1, B0, B1 or B2 may be highly correlated with the disparity motion vector of the current coding PU; this is not well exploited in the current design.

Therefore, further coding efficiency improvement can be expected by including more candidates derived from the shifted DV and DMV.

# Proposed Scheme

In JCT3V-C0045 [2], it is proposed that up to two additional candidates with shifted DV may be added. In this proposal, it is proposed to further consider the shifted DMV from the spatial neighbors (in addition to shifted DV), similar as in proposal JCT3V-C0148 [3]. When the inter-view reference derived from the shifted disparity vector is not available, the shifted DMV is used to generate a candidate.

## Generation of the additional candidates derived from shifted DV and/or DMV

Two candidates namely left candidate and right candidate are generated by following the steps described below.

- The disparity vector DV is shifted to derive two shifted disparity vectors, if applicable:

DV is shifted left/right by -4/4 horizontally to derive a LDV/RDV and it is used to derive a left/right IPMC candidate in a reference view.

- If the left/right IPMC is unavailable, a candidate, denoted as Disparity Shifted Motion Vector (DSMV) is derived and set to be the left/right candidate. DSMV is derived as follows.

  - The first available disparity motion vector (DMV) corresponding to the RefPicList0 from the spatial neighboring blocks A1, B1, B0, A0 or B2 is used to derive.

  - If the DMV is available (that is denoted as ‘mv’)

    - DSMV[0] = mv[0], DSMV[1] = mv[1] and DSMV[0][0]-/+=4, reference indices are inherited from the first available candidate (which contains the DMV).

    - When BVSP is enabled, DSMV[0][1] is set to 0.

  - Otherwise, DSMV[0][0] = DV[0]-/+4, DSMV[1][0] = DV[0]-/+4; DSMV[0][1] = 0 and DSMV[1][1]= 0.

## Description of the whole process

To generate a merge candidate list, the following steps are performed in order: (newly introduced steps are highlighted)

- A disparity vector is derived and the corresponding block of current PU based on the disparity vector is identified. IPMC, if derived to be available, is added to the merge candidate list.

- Spatial merging candidates (SMC) are added to the merge candidate list as current design.

  - In addition, during SMC generation process, if a SMC derived from A1 or B1 is the same as the center IPMC in step 1 they are excluded from the merge candidate list.

- Disparity motion vector converted from the disparity vector is added to the merge candidate list right after the SMC derived from B0 and before the SMC derived from A0, if they are different from the A1 and B1.

- The BVSP merging candidate is inserted to the merge candidate list just before the temporal merging candidates, if none of the five spatial candidates are coded with the BVSP mode.

- If a left IPMC was derived to be the left candidate, it is compared with the center IPMC in step 1. If it is not equal to the center IPMC or if the left candidate is a DSMV (when left IPMC is unavailable) the left candidate is inserted.

- Temporal merging candidate is added to the merge candidate list as the current design.

- If a right IPMC was derived to be the right candidate, it is compared with the center IPMC in step 1. If it is not equal to the center IPMC or if the right candidate is a DSMV (when right IPMC is unavailable) the right candidate is inserted.

- Other motion vector merging candidates are added to the merge candidate list if the number of valid candidates is less than the maximum number.

It should be noted that the candidate list size is not changed (still six), once there are enough candidates in the candidate list, no more candidates will be further added.

# Experimental Results

This section provides simulation results of the proposed merge improvement in comparison with the 3DV-HTM anchor. The platform 3D-HTM 6.0 [4] is utilized and the proposed method is implemented on top of it. All the simulation tests are performed based on the common test conditions [5].

The decoding time estimate of both the CTC and the proposed scheme are calculated on a single 64-bit Windows PC with Intel Xeon dual core CPU (2GHz). The encoding and rendering estimates of the proposed scheme are calculated on the Linux cluster; these estimates may not be accurate due to parallel simulations.

The coding performance of the proposed scheme in the BVSP on case is given in Table 1. As shown in Table 1, the overall average bitrate reduction is 0.1%, 0.1%, 0.15% for the decoded texture views, coded and synthesized views, respectively.

Table 1: Coding gain with respect to anchor for 3-view case (proposed scheme vs. CTC)

The proposed scheme is also tested in the BVSP off case by disabling the macro (MERL_VSP_C0152). For a fair comparison, the result of the proposed scheme with BVSP off is compared with the CTC in the BVSP off case. As shown in Table 2, the overall average bitrate reduction is 0.19%, 0.18%, 0.17% for decoded texture views, coded and synthesized views.

In Table 2, the decoding time estimate of both the HTM 6.0 with BVSP off and the proposed scheme are calculated on a single 64-bit Windows PC with Intel Xeon dual core CPU (2GHz).

Table 2: Coding gain with respect to anchor for 3-view case in BVSP off case (proposed scheme vs. HTM 6.0)

# References

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Test Model 3,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1005, 3rd  Meeting: Geneva, CH, 17–23 Jan. 2012

- L. Zhang, Y. Chen, L. He and M. Karczewicz, “3D-CE5.h: 3D-CE5.h: Merge candidates derivation from disparity vector,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0045, 3rd Meeting: Geneva, CH, 17–23 Jan. 2012.

- C. Guillemot and L. Guillo, “3D-CE5.h: Additional merge candidates derived from shifted disparity candidate predictors,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0148, 3rd Meeting: Geneva, CH, 17–23 Jan. 2012.

- 3DV-HTM version 6.0: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.0/ .

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1100, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

INRIA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).