| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0180 |
| --- | --- |

| Title: | CE2.h related: Bug fix on unavailable disparity vector in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jewon Kang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8457 jewonk@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current 3D-HEVC design, a disparity vector provided by the NBDV process may be refined by accessing depth information when BVSP is turned on. However, if the NBDV process returns an unavailable disparity vector, the refining process is disallowed, and the disparity vector of the current block is set to zero. This proposal provides a bug fix to enable to refine the disparity vector though NBDV returns unavailable result, which is achieved by feeding the zero disparity vector into the current refining process. It is reportedly shown that the proposed bug fix provides a -0.1% BD-rate reduction over HTM 6.0.

# Introduction

In the current 3D-HEVC [1], a disparity vector (DV) is derived using the neighboring block disparity vector (NBDV) derivation followed by the depth-oriented NBDV (DoNBDV).

The NBDV process is finished whenever a disparity motion vector or a disparity vector stored in a neighboring block is found. Afterwards, the DoNBDV process starts refining the vector by accessing the depth samples of the base view.

However, if the NBDV process fails to find any disparity motion vector or disparity vector from neighboring blocks, the entire DV derivation process is finished with an unavailable DV as output. In case when the output of NBDV is unavailable, the refining process is not conducted, and the final disparity vector is still set to zero.

# Proposal

It is proposed, when the NBDV process provides unavailable DV, the zero DV is used as an initial vector, and a refined disparity vector is derived in the same way used for available DVs. Therefore, the original disparity vector (set to 0) based on the unavailable DV becomes accurate with the refined disparity vector.

# Experimental results

Simulation results of the proposal are shown in Table 1. The implementation was based on HTM 6.0, and simulations are done under common test conditions [2].

As shown in Table 1, the proposed method provides a coding gain about -0.1% BD-rate reduction.

Table 1: Proposed method VS HTM 6.0 (CTC).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 3,” JCT3V-C1005, Jan. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-C1100, Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).