| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0183 |
| --- | --- |

| Title: | Proposed text for JCT3V-D0183 based on 3D-HEVC Test Model 3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-5776-0696 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document provides the draft text changes of contribution JCT3V-D0183. Draft changes are identified in red texts and newly introduced texts are highlighted in green.

- H.8.4.4.2.12	Depth partition value derivation and assignment process

Inputs to this process are:

- neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

- an binary array partitionPattern[ x ][ y ], with x, y =0..nT−1, specifying a partitioning of the prediction block in a partition 0 and a partition 1.

- a variable nT specifying the transform size,

- a flag dcOffsetAvailFlag, specifying whether DC Offset values are available

- a flag intraChainFlag, specifying whether the current  intra prediction mode is equal to Intra_Chain(43,44)

- the variables dcOffsetP0 and dcOffsetP1, specifying the DC offsets for the block partitions

Output of this process is:

- predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

The variable log2SubSample is set equal to ( !intraChainFlag && ( nT < 32 ) ) 0 : 1.

For X being replaced by 0 and 1, tThe values of the prediction samples predSamples[ x][ y ] are derived as specified by the following ordered steps:

  - bT = (partitionPattern[ 0 ][ 0 ] != partitionPattern[ nT-1 ][ 0 ]) ? 1 : 0				(H-xx)

  - bL = (partitionPattern[ 0 ][ 0 ] != partitionPattern[ 0 ][ nT-1 ]) ? 1 : 0				(H-xx)

- The variable sumNeigh specifying the sum of the neighbouring samples depth values of partition X is set equal to 0 and the variable numNeigh specifying the number of the neighbouring samples of partition X is set equal to 0 and the following applies.

    - For x = 0..( nT >> log2SubSample ) − 1, inclusive the following applies:

      - When partitionPattern[ x << log2SubSample ][ 0 ] is equal to X, the following applies:

          - sumNeigh += p[ x<< log2SubSample ][ −1 ]	(Error: Reference source not found-1)
numNeigh += 1		(Error: Reference source not found-2)

    - For y = 0..( nT  >> log2SubSample )− 1,inclusive the following applies:

      - When partitionPattern[ 0 ][ y << log2SubSample ] is equal to X, the following applies:

          - sumNeigh += p[ −1 ][ y << log2SubSample ]	(Error: Reference source not found-3) 
numNeigh += 1		(Error: Reference source not found-4)

  - The variables predDcVal0 and predDcVal1 specifying the predicted constant partition values for partition X is derived as follows.

      - If bL is equal to bT, the following applies. :

          - dcVal0 = bL ? (p[ −1 ][ nT−1 ] + p[ nT−1 ][ −1 ])>>1 : 1 << ( BitDepthY − 1 )	(H-xx)

          - dcVal1 = (p[ −1 ][ 0 ] + p[ 0 ][ −1 ])>>1	(H-xx)

      - Otherwise, the following applies. :

          - dcVal0 = bL ? p[ −1 ][ nT−1 ] : p[ nT−1 ][ −1 ]	(H-xx)

          - dcVal1 = bL ? p[( nT–1 ) >> 1 ][ −1 ] : p[ −1 ][ ( nT–1 ) >> 1 ]	(H-xx)

      - predDcValX = partitionPattern[ 0 ][ 0 ] ? dcValX : dcValY, for X being equal to 0 and 1 inclusive, and Y being equal to 1-X.

      - predDcVal = ( numNeigh ! = 0 ) ? ( sumNeigh / numNeigh ) : ( 1 << ( BitDepthY  − 1 ) )	(Error: Reference source not found-5)

- For X being replaced by 0 and 1, the following applies.

  - The variable dcOffset specifying the DC offset is derived as follows.

    - If dcOffsetAvailFlag is equal to 1, the following applies. :

        - dcOffsetScale = intraChainFlag ? Clip3( 1, ( 1 << BitDepthY ) − 1, Round( 2 (QP’Y /10)- 2 ) ) : 1 	(Error: Reference source not found-60)

        - dcOffset = dcOffset * dcOffsetScale		(Error: Reference source not found-61)

    - Otherwise ( dcOffsetAvailFlag is equal to 0), dcOffset is set equal to 0.

  - The predicted sample values predSamples are derived as follows for x = 0..nT−1 and for y = 0..nT−1.

    - When partitionPattern[ x ][ y ] is equal to X, the following applies.

        - predSamples[ x ][ y ] = predDcValX + dcOffset	(Error: Reference source not found-62)