| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0184 |
| --- | --- |

| Title: | CE6.h related: Wedgelet modes unification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-5776-0696 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes three changes for the current design related to the wedgelet modes, including DMM 1, DMM 2 and DMM 3, so that the signaling of these modes are in an unified fashion. The major changes are summarized as follows:

- Signaling of DMM 1 as an intra mode;

- For DMM1, further signal a subset index indicating which wedgelet pattern is selected within a pre-defined wedgelet subset identified by the intra mode, similar as in DMM 3.

With proposed method, the wedgelet index signaling of DMM 1 and DMM 3 are unified and the syntax design of DMM modes are made more aligned with normal intra prediction modes of HEVC. It is reported that 0.0% BD-rate on synthesized views is observed with the proposed unification scheme under common test condition. It is also reported that DMM 2 may be removed for better unification of the three wedgelet modes.

# Introduction

In 3D-HEVC, conventional intra coding methods (i.e., Planar, DC and angular intra prediction) and partition-based methods (i.e., DMM, SDC_DMM1 and Chain Coding) are both applied for depth map intra coding. To signal different methods applied for depth intra coding, different schemes are applied and the depth intra mode coding process becomes variant, as illustrated below,

- Conventional intra modes: signaled using the same intra mode coding method in 3D-HEVC

- DMM 1, SDC_DMM1: a wedgelet pattern index is explicitly signalled with up to 11 bins

- DMM 2: the difference to the end position of a predicted wedgelet pattern is explicitly signaled

- DMM 3: a wedgelet subset index is explicitly signaled with up to 9 bits

- Chain coding: signaled as the final intra prediction mode

All DMM modes, no matter from applied with SDC or not, are using the wedgelet partitioning and therefore they are jointly referred as wedgelet modes in this document.

DMM 1 explicitly signals the index to a full wedgelet pattern. DMM 2 enables prediction of the wedgelet pattern cross the blocks, by extending the wedgelet boundary.

In DMM3, each wedgelet pattern is connected to the intra mode with closest directionality, such that a wedgelet subset is identified by the intra mode of the Co-located Texture Luma Block (CTLB), and an index of the subset is explicitly signaled. According to the connections between intra modes and wedgelet patterns, it is possible to re-use the conventional intra mode coding scheme in 3D-HEVC for wedgelet index signalling of DMM1 and SDC_DMM1, such that the wedgelet index signaling and intra mode coding can be unified.

# Proposal

It is proposed to represent the wedgelet index in DMM1 with two parts: one is to signal the intra mode as for normal intra prediction ones in HEVC, the other is an index of a pre-defined wedgelet subset derived by the mapped intra mode, similar as in DMM 3. Therefore, the signalling of all wedgelet modes are unified, two main aspects of the unification are proposed in this contribution as below:

- Due to the limited contribution of DMM2 in HTM-6.0, it is proposed to remove DMM2 and modify the signaling of DMM1, DMM3 and DMM4.

- For wedgelet index signaling in DMM1 and SDC_DMM1, an intra mode is signalled using the conventional intra mode coding scheme, and an index of a pre-defined subset identified by the intra mode is also signaled.

In the second proposed aspect, for signaling the first intra mode part, the same intra mode coding scheme in HEVC is utilized, for signaling the second subset index part, it is proposed to re-use the subset index signaling process in DMM3.

## General coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode = = MODE_INTRA && DepthFlag ) |  |
| sdc_flag[ x0 ][ y0 ] [Ed. (GT): In software there seems to be a dependency of position of sdc_flag and slice type.] | ae(v) |
| if( sdc_flag[ x0 ][ y0 ] ) { |  |
| sdc_pred_mode | ae(v) |
| if( sdc_pred_mode = = 1 ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( !sdc_flag[ x0 ][ y0 ] ¦ ¦ ( sdc_flag[ x0 ][ y0 ] && sdc_pred_mode = = 1 ) { |  |
| if( ( PredMode[ x0 ][ y0 ] ! = MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) && !predPartModeFlag) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| if ( vps_dmm_flag[ nuh_layer_id ] && log2CbSize <= Log2MaxDmmCbSize ) { |  |
| log2PbSize = log2CbSize − ( PartMode = = PART_NxN ? 1 : 0 ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if(!sdc_flag[ x0 ][ y0 ]){ |  |
| dmm_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if (dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| dmm_mode[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULL ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULLDELTA¦ ¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDTEXDELTA ¦ ¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDTEX ) ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + j ] | ae(v) |
| wedge_predtex_tab_idx[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| wedge_predtex_tab_idx[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| dmm_delta_end_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + j ] ) |  |
| dmm_delta_end_abs_minus1[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + j ] ) |  |
| dmm_delta_end_sign_flag[ x0 ][ y0 ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| DmmDeltaFlag[ x0 + i ][ y0 + j ] = ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_WFULLDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_WPREDDIRDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_WPREDTEXDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + j ] = = MODE_DMM_CPREDTEXDELTA ) ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0 + j ] ) |  |
| dmm_dc_1_abs[ x0 + i ][ y0 + j ] | ae(v) |
| … |  |
| } else { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ¦ ¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULL ¦ ¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULLDELTA ) |  |
| dmm1IntraSignalled[ x0 + i ][ y0+ j ] = 1 |  |
| else |  |
| dmm1IntraSignalled[ x0 + i ][ y0+ j ] = 0 |  |
| if( dmm1IntraSignalled[ x0 + i ][ y0+ j ] ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] dmm1IntraSignalled[ x0 + i ][ y0+ j ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else { |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| if( rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] = = 31 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) <= 5 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) >= 2 ) { |  |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( !sdc_flag[ x0 ][ y0 ]) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| … |  |
| } |  |
| } |  |
| … |  |
| } |  |
| } |  |

## General coding unit semantics

dmm_flag[ x0 + i][ y0 + j ] equal to 0 specifies that depth map model modes are not used. dmm_flag[ x0 + i][ y0 + j ] equal to 1 specifies that depth map model modes are used that the dmm_mode syntax element is present. When the dmm_flag[ x0 + i][ y0 + j ] is not present, its value should be inferred to be equal to 0 sdc_flag[ x0 ][ y0 ] ? 1 : 0.

dmm_mode[ x0 + i][ y0 + j ] shall be one of the values shown in Table H-. dmm_mode is used to derive the intra prediction mode when the dmm_flag[ x0 + i][ y0 + j ] is equal to 1. When not present, dmm_mode[ x0 + i ][ y0+ j ] is inferred to be equal to MODE_DMM_WFULL.

Table H-2 – Interpretation of dmm_mode

| dmm_mode | Method identifier |
| --- | --- |
| 0 | MODE_DMM_WFULL |
| 1 | MODE_DMM_WFULLDELTA |
| 2 | MODE_DMM_WPREDTEX |
| 3 | MODE_DMM_WPREDTEXDELTA |
| 4 | MODE_DMM_CPREDTEX |
| 5 | MODE_DMM_CPREDTEXDELTA |
| 6 | MODE_DMM_WPREDDIR |
| 7 | MODE_DMM_WPREDDIRDELTA |

wedge_full_tab_idx[ x0 + i][ y0 + j ] specifies the index of the wedgelet pattern in the corresponding pattern list when dmm_mode[ x0 + i][ y0 + j ] is equal to MODE_DMM_WFULL or equal to MODE_DMM_WFULLDELTA.

wedge_predtex_tab_idx[ x0 + i][ y0 + j ] specifies the index of the wedgelet pattern when dmm_mode[ x0 + i][ y0 + j ] is equal to MODE_DMM_WFULL, MODE_DMM_WFULLDELTA, MODE_DMM_WPREDTEX or equal to MODE_DMM_WPREDTEXDELTA.

dmm_delta_end_flag[ x0 + i][ y0 + j ] equal to 0 specifies that dmm_delta_end_abs_minus1[ x0 + i][ y0 + j ] and dmm_delta_end_sign[ x0 + i][ y0 + j ] syntax elements are not present. dmm_delta_end_flag equal to 1 specifies that dmm_delta_end_abs_minus1[ x0 + i][ y0 + j ] and dmm_delta_end_sign[ x0 + i][ y0 + j ] syntax elements are present.

dmm_delta_end_abs_minus1[ x0 + i][ y0 + j ] and dmm_delta_end_sign_flag[ x0 + i][ y0 + j ] are used to derive DmmDeltaEnd[ x0 + i][ y0 + j ] as follows:

  - DmmDeltaEnd[ x0 + i][ y0 + j ] = 
( 1 − 2 *dmm_delta_end_sign_flag[ x0 + i][ y0 + j ] ) *	( dmm_delta_end_abs_minus1[ x0 + i][ y0 + j ] + 1)	(Error: Reference source not found-1)

## Removal of DMM2

In 3D-HEVC, a syntax element dmm_mode is used to specify which DMM mode is used and whether delta DC is applied. The interpretation of dmm_mode and binary string are described in Table 1.

Table 1: Interpretation and binarization of dmm_mode

| dmm_mode | DMM mode | Delta DC | Binary string |
| --- | --- | --- | --- |
| 0 | DMM 1 | not used | 000 |
| 1 | DMM 1 | used | 100 |
| 2 | DMM 3 | not used | 010 |
| 3 | DMM 3 | used | 110 |
| 4 | DMM 4 | not used | 001 |
| 5 | DMM 4 | used | 101 |
| 6 | DMM 2 | not used | 011 |
| 7 | DMM 2 | used | 111 |

The complexity of DMM2 is relatively high because there are additional computations at both encoder and decoder to predict the wedgelet partitioning pattern using neighboring intra prediction mode or neighboring DMM mode. Furthermore, it is observed that DMM 2 has very limited impact on the coding performance on the latest 3D-HEVC reference software, i.e., HTM-6.0, and it is proposed in this contribution to remove DMM 2 and modify the signaling of dmm_mode accordingly, as described below in Table 2,

Table 2: Proposed binarization of dmm_mode

| dmm_mode | DMM mode | Delta DC | Binary string |
| --- | --- | --- | --- |
| 0 | DMM 1 | not used | 00 |
| 1 | DMM 1 | used | 01 |
| 2 | DMM 3 | not used | 100 |
| 3 | DMM 3 | used | 101 |
| 4 | DMM 4 | not used | 110 |
| 5 | DMM 4 | used | 111 |

# Experimental results

The proposed method is implemented on top of HTM-6.0, and simulations were performed under both “common test condition” [1] and “Intra-only” [2] configurations.

The results are summarized in Table 3 and Table 4. As it is reported, average 0.0% and 0.3% BD-rate on synthesized views are observed for “CTC” and “Intra-only”, respectively.

Table 3: BD rate results for 3-view case under CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 93% | 93% | 87% |
| Kendo | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% | 99% | 101% | 95% |
| Newspapercc | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.3% | 101% | 101% | 99% |
| GhostTownFly | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 100% | 97% | 97% |
| PoznanHall2 | 0.0% | -0.5% | -0.4% | -0.2% | -0.2% | -0.1% | 98% | 91% | 88% |
| PoznanStreet | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 89% | 85% | 82% |
| UndoDancer | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | -0.4% | 100% | 96% | 93% |
| 1024x768 | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.2% | 97% | 99% | 94% |
| 1920x1088 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 97% | 92% | 90% |
| average | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 97% | 95% | 92% |

Table 4: BD rate results for 3-view case under Intra-only

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.5% | 103% | 100% | 103% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.5% | 98% | 97% | 98% |
| Newspapercc | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.6% | 104% | 101% | 100% |
| GhostTownFly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 105% | 103% | 91% |
| PoznanHall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 108% | 95% | 103% |
| PoznanStreet | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 106% | 98% | 93% |
| UndoDancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 100% | 95% | 90% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.5% | 102% | 99% | 100% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 105% | 98% | 94% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | 103% | 99% | 97% |

# References

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-C1100, Geneva, CH, 17–23 Jan. 2013.

- P. Merkle, “Description of Core Experiment 6 (CE6) on Depth Intra Coding,” JCT3V-C1106, Geneva, CH, 17–23 Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).