| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 17–23 Jan. 2013 | Document: JCT3V-C1002 |
| --- | --- |

| Title: | 3D-AVC Draft Text 5 |  |  |
| --- | --- | --- | --- |
| Status: | Draft Output Document of JCT-3V |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Corporation Ying Chen Qualcomm Teruhiko Suzuki Sony Jens-Rainer Ohm RWTH Aachen Gary J. Sullivan Microsoft | Email: | miska.hannuksela@nokia.com cheny@qti.qualcomm.com teruhikos@jp.sony.com ohm@ient.rwth-aachen.de garysull@microsoft.com |
| Source: | Editors |  |  |

_____________________________

- Abstract

This document is the draft 5 of the 3D-AVC specification (“AVC compatible video-plus-depth extension”). The document is based on JCT3V-B1002 with adoptions of the JCT3V-C meeting integrated.

A list of adoptions in the JCT3V-C meeting:

- Missing adoptions from the JCT3V-B meeting and clarifications (as reported in JCT3V-C0002)

  - mb_skip_flag is not necessarily present for each macroblock. Hence, it suggested that the following is added in the semantics of mb_skip_flag: “mb_skip_flag is inferred to be equal to 0 when it is not present”.

  - The parsing process of adaptive luminance compensation was accidentally not integrated into the output document JCT3V-B1002.

- JCT3V-C0130 3D-CE1.a: Inter-view skip/direct mode with sub-partition scheme (4x4 MCP based implementation, Disparity derivation: Max out of 4 corners, Remove sub-MB skip/direct B0054)

- JCT3V-C0133 3D-CE2.a: Results on simplification of the Inter-view candidate derivation

- JCT3V-C0136 3D-CE3.a related: Completed inside-view motion prediction in 3D video coding

- JCT3V-C0140 3D-CE5.a related: Direct MVP derivation with reduced complexity

- JCT3V-C0094 3D-CE7.a Improved Nonlinear Depth Representation (Sub-proposal 1: explicit signaling of NLDR usage in slice header)

- JCT3V-C0054 Simplifications for adaptive luminance compensation in 3D-AVC

- JCT3V-C0168 AHG4: Reduced complexity depth coding in 3D-AVC (Remove JVF from the 3D-AVC draft text)

- JCT3V-C048: AHG8: Draft Text for Signaling of Alternative 3D Format (new SEI)

JCT3V-C1002d3 text includes (with references to the list of adoptions above):

- JCT3V-C0054: Simplifications for adaptive luminance compensation

- Adaptive luminance compensation (ALC) fix: the parsing process is added.

- JCT3V-C0048 editorial improvements.

JCT3V-C1002d2 text includes (with references to the list of adoptions above):

- JCT3V-C0130 3D-CE1.a: Inter-view skip/direct mode with sub-partition scheme (4x4 MCP based implementation, Disparity derivation: Max out of 4 corners, Remove sub-MB skip/direct B0054)

- JCT3V-C0094 3D-CE7.a Improved Nonlinear Depth Representation (Sub-proposal 1: explicit signaling of NLDR usage in slice header). The impact on decoding process has been added.

JCT3V-C1002d1 text includes (with references to the list of adoptions above):

- mb_skip_flag is not necessarily present for each macroblock. Hence, it suggested that the following is added in the semantics of mb_skip_flag: “mb_skip_flag is inferred to be equal to 0 when it is not present”.

- JCT3V-C0133 3D-CE2.a: Results on simplification of the Inter-view candidate derivation

- JCT3V-C0136 3D-CE3.a related: Completed inside-view motion prediction in 3D video coding

- JCT3V-C0140 3D-CE5.a related: Direct MVP derivation with reduced complexity

- JCT3V-C0094 3D-CE7.a Improved Nonlinear Depth Representation (Sub-proposal 1: explicit signaling of NLDR usage in slice header)

- JCT3V-C0168 AHG4: Reduced complexity depth coding in 3D-AVC (Remove JVF from the 3D-AVC draft text)

- JCT3V-C0048 AHG8: Draft Text for Signaling of Alternative 3D Format (new SEI)

In 0.6, add the following paragraph after the paragraph that starts with “An extension of multiview video coding that also supports the inclusion of depth maps”:

A multiview video coding extension with depth information is specified in Annex J. Sub-bitstreams consisting of a texture base view conform to this Specification, sub-bitstreams consisting of multiple texture views may also conform to Annex H of this Specification, and sub-bitstreams consisting of one or more texture views and one or more depth views may also conform to Annex I of this Specification. Enhanced texture view coding that utilizes the associated depth views and decoding processes for depth views are specified for this extension. For temporal bitstream scalability, i.e., the presence of a sub-bitstream with a smaller temporal sampling rate than the bitstream, complete access units are removed from the bitstream when deriving the sub-bitstream. In this case, high-level syntax and inter prediction reference pictures in the bitstream are constructed accordingly. For view bitstream scalability, i.e. the presence of a sub-bitstream with fewer views than the bitstream, NAL units are removed from the bitstream when deriving the sub-bitstream. In this case, inter-view prediction, i.e., the prediction of one view signal by data of another view signal, is typically used for efficient coding.

In 0.7, add the following paragraph after the paragraph that starts with “Annex I specifies”:

Annex J specifies a multiview video coding extension with depth information (3D-AVC). Annex I and J are jointly referred to as 3D Video Coding (3DVC). [Ed.(MH): The use of the abbreviation “3DVC” might be confusing – consider if it is needed.] The reader is referred to Annex J for the decoding process for AVC-compatible multiview extension with depth video, which is specified there with references being made to clauses 2-9 and Annexes A-E and Annexes H-I. Subclause J.10 specifies one profile for AVC-compatible multiview video coding extension with depth information.

In 7.3.1Nal unit syntax, replace the syntax table with:

| nal_unit( NumBytesInNALunit ) { | C | Descriptor |
| --- | --- | --- |
| forbidden_zero_bit | All | f(1) |
| nal_ref_idc | All | u(2) |
| nal_unit_type | All | u(5) |
| NumBytesInRBSP = 0 |  |  |
| nalUnitHeaderBytes = 1 |  |  |
| if( nal_unit_type = = 14 ¦ ¦ nal_unit_type = = 20 ¦ ¦ nal_unit_type = = 21 ) { |  |  |
| if( nal_unit_type ! = 21 ) |  |  |
| svc_extension_flag | All | u(1) |
| else |  |  |
| avc_3d_extension_flag | All | u(1) |
| if( svc_extension_flag ) { |  |  |
| nal_unit_header_svc_extension( ) /* specified in Annex Error: Reference source not found */ | All |  |
| nalUnitHeaderBytes += 3 |  |  |
| } |  |  |
| else if( avc_3d_extension_flag ) { |  |  |
| nal_unit_header_3dvc_extension( ) /* specified in Annex J */ |  |  |
| nalUnitHeaderBytes += 2 |  |  |
| } |  |  |
| else { |  |  |
| nal_unit_header_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | All |  |
| nalUnitHeaderBytes += 3 |  |  |
| } |  |  |
| } |  |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| i += 2 |  |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | All | f(8) |
| } else |  |  |
| rbsp_byte[ NumBytesInRBSP++ ] | All | b(8) |
| } |  |  |
| } |  |  |

In 7.3.2.1.3,Subset sequence parameter set data syntax, replace the syntax table with:

| subset_seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_data( ) | 0 |  |
| if( profile_idc = = 83 ¦ ¦ profile_idc = = 86 ) { |  |  |
| seq_parameter_set_svc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| svc_vui_parameters_present_flag | 0 | u(1) |
| if( svc_vui_parameters_present_flag = = 1 ) |  |  |
| svc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } else if( profile_idc = = 118 ¦ ¦ profile_idc = = 128 ¦ ¦ profile_idc = = 138 ¦ ¦ profile_idc = = 139) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_mvc_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| mvc_vui_parameters_present_flag | 0 | u(1) |
| if( mvc_vui_parameters_present_flag = = 1 ) |  |  |
| mvc_vui_parameters_extension( ) /* specified in Annex Error: Reference source not found */ | 0 |  |
| } |  |  |
| if( profile_idc = = 138 ¦ ¦ profile_idc = = 139 ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 0 | f(1) |
| seq_parameter_set_3dvc_extension( ) /* specified in Annex I */ | 0 |  |
| } |  |  |
| additional_extension3_flag | 0 | u(1) |
| if( additional_extension3_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension3_data_flag | 0 | u(1) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

In 7.3.3,Slice header syntax, replace the syntax table with: [Ed.(MH): As the changes are fairly substantial, consider specifying slice_header_in_avc_3d_extension( ) similarly to slice_header_in_scalable_extension specified in Annex G]

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| if( avc_3d_extension_flag && slice_header_prediction_flag != 0 ) { |  |  |
| pre_slice_header_src | 2 | u(2) |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| pre_ref_lists_src | 2 | u(2) |
| if ( !pre_ref_lists_src ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| if( nal_unit_type = = 20 ¦ ¦ ( nal_unit_type = = 21 && DepthFlag ) ) |  |  |
| ref_pic_list_mvc_modification( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| else if( nal_unit_type = = 21 && DepthFlag = = 0 ) |  |  |
| ref_pic_list_3dvc_modification( ) | 2 |  |
| } |  |  |
| } |  |  |
| if( ( weighted_pred_flag && ( slice_type = = P ¦ ¦ slice_type = = SP ) ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) { |  |  |
| pre_pred_weight_table_src | 2 | u(2) |
| if( !pre_pred_weight_table_src ) |  |  |
| pred_weight_table( ) | 2 |  |
| if(nal_ref_idc != 0 ) { |  |  |
| pre_dec_ref_pic_marking_src | 2 | u(2) |
| if( !pre_dec_ref_pic_marking_src ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| } |  |  |
| slice_qp_delta | 2 | se(v) |
| } else { |  |  |
| if( separate_colour_plane_flag = = 1 ) |  |  |
| colour_plane_id | 2 | u(2) |
| frame_num | 2 | u(v) |
| if( !frame_mbs_only_flag ) { |  |  |
| field_pic_flag | 2 | u(1) |
| if( field_pic_flag ) |  |  |
| bottom_field_flag | 2 | u(1) |
| } |  |  |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| if( pic_order_cnt_type = = 0 ) { |  |  |
| pic_order_cnt_lsb | 2 | u(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt_bottom | 2 | se(v) |
| } |  |  |
| if( pic_order_cnt_type = = 1 && !delta_pic_order_always_zero_flag ) { |  |  |
| delta_pic_order_cnt[ 0 ] | 2 | se(v) |
| if( bottom_field_pic_order_in_frame_present_flag && !field_pic_flag ) |  |  |
| delta_pic_order_cnt[ 1 ] | 2 | se(v) |
| } |  |  |
| if( redundant_pic_cnt_present_flag ) |  |  |
| redundant_pic_cnt | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| direct_spatial_mv_pred_flag | 2 | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = SP ¦ ¦ slice_type = = B ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| if( nal_unit_type = = 20 ¦ ¦ ( nal_unit_type = = 21 && DepthFlag ) ) |  |  |
| ref_pic_list_mvc_modification( ) /* specified in Annex Error: Reference source not found */ | 2 |  |
| else if( nal_unit_type = = 21 && DepthFlag = = 0 ) |  |  |
| ref_pic_list_3dvc_modification( ) |  |  |
| else |  |  |
| ref_pic_list_modification( ) | 2 |  |
| if( ( weighted_pred_flag && ( slice_type = = P ¦ ¦ slice_type = = SP ) ) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |  |
| pred_weight_table( ) | 2 |  |
| if( nal_ref_idc != 0 ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag && slice_type != I && slice_type != SI ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| slice_qp_delta | 2 | se(v) |
| if( slice_type = = SP ¦ ¦ slice_type = = SI ) { |  |  |
| if( slice_type = = SP ) |  |  |
| sp_for_switch_flag | 2 | u(1) |
| slice_qs_delta | 2 | se(v) |
| } |  |  |
| if( deblocking_filter_control_present_flag ) { |  |  |
| disable_deblocking_filter_idc | 2 | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |
| slice_alpha_c0_offset_div2 | 2 | se(v) |
| slice_beta_offset_div2 | 2 | se(v) |
| } |  |  |
| } |  |  |
| if( num_slice_groups_minus1 > 0 && slice_group_map_type >= 3 && slice_group_map_type <= 5) |  |  |
| slice_group_change_cycle | 2 | u(v) |
| if ( nal_unit_type = = 21 && ( slice_type != I && slice_type != SI )) { |  |  |
| if( DepthFlag ) |  |  |
| depth_weighted_pred_flag | 2 | u(1) |
| else if( avc_3d_extension_flag && ViewCompOrder( 0, view_idx ) > ViewCompOrder( 1, view_idx ) ) |  |  |
| dmvp_flag | 2 | u(1) |
| if ( 3dv_acquisition_idc != 1 && (depth_weighted_pred_flag = = 1 ¦¦ dmvp_flag = = 1) ) |  |  |
| dps_id | 2 | ue(v) |
| } |  |  |
| } |  |  |
| if( DepthFlag && profile_idc = = 139 ) |  |  |
| nonlinear_depth_representation_enable_flag | 2 | u(1) |
| } |  |  |

In 7.3.4,Slice data syntax, replace the syntax table with:

| slice_data( ) { | C | Descriptor |
| --- | --- | --- |
| if( entropy_coding_mode_flag ) |  |  |
| while( !byte_aligned( ) ) |  |  |
| cabac_alignment_one_bit | 2 | f(1) |
| CurrMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag ) |  |  |
| moreDataFlag = 1 |  |  |
| prevMbSkipped = 0 |  |  |
| do { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| if( !entropy_coding_mode_flag ) { |  |  |
| mb_skip_run | 2 | ue(v) |
| prevMbSkipped = ( mb_skip_run > 0 ) |  |  |
| for( i=0; i<mb_skip_run; i++ ) |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && mb_skip_run > 0 && VspRefExist ) |  |  |
| mb_skip_type_flag | 2 | u(1) ¦ ae(v) |
| if( mb_skip_run > 0 ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| } else { |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && VspRefExist && leftMbVSSkipped && upMbVSSkipped ) { |  |  |
| mb_vsskip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_vsskip_flag |  |  |
| if( !mb_vsskip_flag ) { |  |  |
| mb_skip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_skip_flag |  |  |
| } |  |  |
| } else { |  |  |
| mb_skip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_skip_flag |  |  |
| if( nal_unit_type = = 21 && !DepthFlag && VspRefExist && !mb_skip_flag ) { |  |  |
| mb_vsskip_flag | 2 | u(1) ¦ ae(v) |
| moreDataFlag = !mb_vsskip_flag |  |  |
| } |  |  |
| } |  |  |
| if(alc_sps_enable_flag && nal_unit_type = = 21 && slice_type = = P && !DepthFlag && !mb_vsskip_flag && mb_skip_flag = = 1 ) |  |  |
| mb_alc_skip_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( moreDataFlag ) { |  |  |
| if( MbaffFrameFlag && ( CurrMbAddr % 2 = = 0 ¦ ¦ ( CurrMbAddr % 2 = = 1 && prevMbSkipped ) ) ) |  |  |
| mb_field_decoding_flag | 2 | u(1) ¦ ae(v) |
| macroblock_layer( ) | 2 ¦ 3 ¦ 4 |  |
| } |  |  |
| if( !entropy_coding_mode_flag ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| else { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| prevMbSkipped = mb_skip_flag ¦ ¦ mb_vsskip_flag |  |  |
| if( MbaffFrameFlag && CurrMbAddr % 2 = = 0 ) |  |  |
| moreDataFlag = 1 |  |  |
| else { |  |  |
| end_of_slice_flag | 2 | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |  |
| } |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| } while( moreDataFlag ) |  |  |
| } |  |  |

In 7.3.5,Macroblock layer syntax, replace the syntax table with:

| macroblock_layer( ) { | C | Descriptor |
| --- | --- | --- |
| if ( IvmpEnabledFlag ) |  |  |
| mb_ivmp_flag |  | u(1) ¦ ae(v) |
| if (!mb_ivmp_flag) { |  |  |
| if( DepthFlag && disp_flag ) |  |  |
| mb_disp_flag |  | u(1) ¦ ae(v) |
| if(!mb_disp_flag){ |  |  |
| mb_type | 2 | ue(v) ¦ ae(v) |
| if( nal_unit_type = = 21 && !DepthFlag && slice_type = = B && direct_spatial_mv_pred_flag && VspRefExist && mb_type = = B_Direct_16x16 ) |  |  |
| mb_direct_type_flag | 2 | u(1) ¦ ae(v) |
| if( alc_sps_enable_flag && nal_unit_type = = 21 && slice_type = = P && !DepthFlag && ( mb_type = = P_L0_16x16 ¦¦ mb_type = = P_L0_L0_16x8 ¦¦ mb_type = = P_L0_L0_8x16 ¦¦ mb_type = = P_8x8 ¦¦ mb_type = = P_8x8ref0 ) ) |  |  |
| mb_alc_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( !mb_disp_flag ) { |  |  |
| if( mb_type = = I_PCM ) { |  |  |
| while( !byte_aligned( ) ) |  |  |
| pcm_alignment_zero_bit | 3 | f(1) |
| for( i = 0; i < 256; i++ ) |  |  |
| pcm_sample_luma[ i ] | 3 | u(v) |
| for( i = 0; i < 2 * MbWidthC * MbHeightC; i++ ) |  |  |
| pcm_sample_chroma[ i ] | 3 | u(v) |
| } else { |  |  |
| noSubMbPartSizeLessThan8x8Flag = 1 |  |  |
| if( mb_type != I_NxN && MbPartPredMode( mb_type, 0 ) != Intra_16x16 && NumMbPart( mb_type ) = = 4 ) { |  |  |
| if( !mb_ivmp_flag ) |  |  |
| sub_mb_pred( mb_type ) | 2 |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 ) { |  |  |
| if( NumSubMbPart( sub_mb_type[ mbPartIdx ] ) > 1 ) |  |  |
| noSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else if( !direct_8x8_inference_flag ) |  |  |
| noSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else { |  |  |
| if( transform_8x8_mode_flag && mb_type = = I_NxN ) |  |  |
| transform_size_8x8_flag | 2 | u(1) ¦ ae(v) |
| if( !mb_ivmp_flag ) |  |  |
| mb_pred( mb_type ) | 2 |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) != Intra_16x16 ) { |  |  |
| coded_block_pattern | 2 | me(v) ¦ ae(v) |
| if( ( CodedBlockPatternLuma > 0 ¦ ¦ mb_alc_flag = = 1 ) && transform_8x8_mode_flag && mb_type != I_NxN && noSubMbPartSizeLessThan8x8Flag && ( mb_type != B_Direct_16x16 ¦ ¦ direct_8x8_inference_flag ) ) |  |  |
| transform_size_8x8_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( CodedBlockPatternLuma > 0 ¦ ¦ CodedBlockPatternChroma > 0 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| mb_qp_delta | 2 | se(v) ¦ ae(v) |
| residual( 0, 15 ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Replace Table 7-1 with:

| nal_unit_type | Content of NAL unit and RBSP syntax structure | C | Annex A NAL unit type class | Annex G and Annex H NAL unit type class | Annex I and Annex J NAL unit type class |
| --- | --- | --- | --- | --- | --- |
| 0 | Unspecified |  | non-VCL | non-VCL | non-VCL |
| 1 | Coded slice of a non-IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | VCL | VCL | VCL |
| 2 | Coded slice data partition A slice_data_partition_a_layer_rbsp( ) | 2 | VCL | not applicable | not applicable |
| 3 | Coded slice data partition B slice_data_partition_b_layer_rbsp( ) | 3 | VCL | not applicable | not applicable |
| 4 | Coded slice data partition C slice_data_partition_c_layer_rbsp( ) | 4 | VCL | not applicable | not applicable |
| 5 | Coded slice of an IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3 | VCL | VCL | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | 5 | non-VCL | non-VCL | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | 0 | non-VCL | non-VCL | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | 1 | non-VCL | non-VCL | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | 6 | non-VCL | non-VCL | non-VCL |
| 10 | End of sequence end_of_seq_rbsp( ) | 7 | non-VCL | non-VCL | non-VCL |
| 11 | End of stream end_of_stream_rbsp( ) | 8 | non-VCL | non-VCL | non-VCL |
| 12 | Filler data filler_data_rbsp( ) | 9 | non-VCL | non-VCL | non-VCL |
| 13 | Sequence parameter set extension seq_parameter_set_extension_rbsp( ) | 10 | non-VCL | non-VCL | non-VCL |
| 14 | Prefix NAL unit prefix_nal_unit_rbsp( ) /* specified in Annex G and Annex H */ | 2 | non-VCL | suffix dependent | suffix dependent |
| 15 | Subset sequence parameter set subset_seq_parameter_set_rbsp( ) /* specified in Annex G, Annex H and Annex I */ | 0 | non-VCL | non-VCL | non-VCL |
| 16 | Depth parameter set | 11 | non-VCL | non-VCL | non-VCL |
| 17..18 | Reserved |  | non-VCL | non-VCL | non-VCL |
| 19 | Coded slice of an auxiliary coded picture without partitioning slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | non-VCL | non-VCL | non-VCL |
| 20 | Coded slice extension slice_layer extension_rbsp( ) | 2, 3, 4 | non-VCL | VCL | VCL |
| 21 | Coded slice extension for a depth view component or a 3DVC texture view component slice_layer_extension_rbsp( ) | 2, 3, 4 | non-VCL | non-VCL | VCL |
| 22..23 | Reserved |  | non-VCL | non-VCL | non-VCL |
| 24..31 | Unspecified |  | non-VCL | non-VCL | non-VCL |

In 7.4.1, Remove the following paragraphs.

reserved_zero_bit shall be equal to 0 for coded video sequences confirming to one or more profiles specified Annex I. Decoders conforming to one or more profiles specified in Annex I shall ignore (remove from the bitstream and discard) NAL units for which nal_unit_type is equal to 21 and reserved_zero_bit is equal to 1

In 7.4.1, add the following paragraphs in the semantics of svc_extension_flag just before the semantics of rbsp_byte[ i ].

When svc_extension_flag is not present, the value of svc_extension_flag is inferred to be equal to 0.

avc_3d_extension_flag indicates for NAL units having nal_unit_type equal to 21 whether a nal_unit_header_mvc_extension( ) or nal_unit_header_3dvc_extension( ) will follow next in the syntax structure.

If avc_3d_extension_flag is not present, the value of avc_3d_extension_flag is inferred to be equal to 0.

The value of DepthFlag is specified as follows.

DepthFlag = ( nal_unit_type  ! =  21 ) ? 0 : ( avc_3d_extension_flag ? depth_flag : 1 )	(7-xx)

The value of avc_3d_extension_flag shall be equal to 0 for coded video sequences conforming to one or more profiles specified in Annex I. Decoders conforming to one or more profiles specified in Annex I shall ignore (remove from the bitstream and discard) NAL units for which nal_unit_type is equal to  21 and for which avc_3d_extension_flag is equal to 1.

In 7.4.3, Slice header semantics, add the following text after the sematics of “pic_parameter_set_id”:

pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src specify if the respective syntax elements are present in the slice header, and, if not, the slice header from which the values of the respective syntax elements are taken as specified in Table J-XX and Table J-YY.

When a syntax element has an inferred value in the slice header from which its value is taken according to Table J-XX and Table J-YY, the syntax element value in the current slice header is equal to this inferred value. When a syntax element is not present and has no inferred value in the slice header from which its value is taken according to Table J-XX and Table J-YY, the syntax element is inferred to be absent in the current slice header.

pre_slice_header_src shall not be equal to 0.

When ViewCompOrder( DepthFlag, view_idx ) is smaller than ViewCompOrder( !DepthFlag, view_idx ), pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src shall not be equal to 2

- Table J-XX – Respective syntax elements for pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src

| Prediction indication syntax element | Respective syntax elements |
| --- | --- |
| pre_slice_header_src | colour_plane_id, frame_num, field_pic_flag, bottom_field_flag, idr_pic_id, pic_order_cnt_lsb, delta_pic_order_cnt_bottom, delta_pic_order_cnt[ 0 ], delta_pic_order_cnt[ 1 ], redundant_pic_cnt, direct_spatial_mv_pred_flag, cabac_init_idc, sp_for_switch_flag, slice_qs_delta, disable_deblocking_filter_idc, slice_alpha_c0_offset_div2, slice_beta_offset_div2, slice_group_change_cycle, depth_weighted_pred_flag, dmvp_flag, dps_id |
| pre_ref_lists_src | num_ref_idx_active_override_flag, num_ref_idx_l0_active_minus1, num_ref_idx_l1_active_minus1 and reference picture list modification syntax table |
| pre_pred_weight_table_src | pred_weight_table( ) syntax structure |
| pre_dec_ref_pic_marking_src | dec_ref_pic_marking( ) syntax structure |

- Table J-YY – Semantics of the values of pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src and pre_dec_ref_pic_marking_src

| Value of pre_slice_header_src, pre_ref_lists_src, pre_pred_weight_table_src or pre_dec_ref_pic_marking_src | Semantics |
| --- | --- |
| 0 | The respective syntax elements are not predicted but included in the slice header. |
| 1 | The values of the respective syntax elements are taken from the slice header of the first slice of the previous view component in decoding order having the same value of DepthFlag as the current slice, belonging to a dependent view of the current view, and residing in the same access unit. |
| 2 | The values of the respective syntax elements are taken from the first slice header of the first slice of the view component having the same view order index as the current slice and a different value of DepthFlag. |
| 3 | The values of the respective syntax elements are taken from the first slice header of the first slice of the view component in the same access unit having view order index equal to 0 and the same value of DepthFlag as the current slice. |

depth_weighted_pred_flag equal to 0 specifies that no depth-range-based weighted prediction is used for corresponding slice RBSP. depth_weighted_pred_flag equal to 1 specifies that depth-range-based weighted prediction is used for corresponding slice RBSP. When not present, depth_weighted_pred_flag is inferred to be equal to 0. When depth_weighted_pred_flag is equal to 1, the process of derivation of prediction weights specified in 8.3.4 applies.

dmvp_flag equal to 0 specifies that the inter and inter-view prediction process specified in I.8.3 is applied. dmvp_flag equal to 1 specifies that the inter, inter-view, and view synthesis prediction process specified in J.8.3 is applied.

dps_id specifies the depth parameter set in use. The value of dps_id shall be in the range of 0 to TBD, inclusive. When dps_id is equal to 0, depth parameters are set according to syntax elements in seq_parameter_set_3dvc_extension( ) of the active sequence parameter set. When present, the value of dps_id shall be the same in all slice headers within an access unit. [Ed. Note (CY): we may need to add a paragraph on generating the depth parameter set with id equal to 0 based on the SPS.]

nonlinear_depth_representation_enable_flag equal to 0 specifies that the current slice is represented in linear form. nonlinear_depth_representation_flag equal to 1 specifies that the current slice is represented in nonlinear form.

NOTE – The derivation of the variable NdrInverse[ i ], as specified below, depends on the value of nonlinear_depth_representation_enable_flag. When nonlinear_depth_representation_enable_flag in the current slice is equal to 0, NdrInverse[ i ] indicates a linear mapping and, in practice, the linear mapping indicates that input is transferred to the ouput without any change(e.g. scaling and offset).

When nonlinear_depth_representation_enable_flag in the current slice is equal to 1, NdrInverse[ i ], as specified below, is used to transform depth sample values from nonlinear representation to the linear representation. The shape of this transform is defined by means of line-segment-approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first (0, 0) and the last (255, 255) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (nonlinear_depth_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to 255, inclusive, with spacing depending on the value of nonlinear_depth_representation_num.

When nonlinear_depth_representation_enable_flag in the current slice is equal to 0, Variable NdrInverse[ i ] for i in the range of 0 to 255, inclusive, is specified as follows.

nonlinear_depth_representation_model[ 0 ] = 0
nonlinear_depth_representation_model[ nonlinear_depth_representation_num + 1 ] = 0
for( k=0; k<= nonlinear_depth_representation_num; ++k )
{
	pos1 = ( 255 * k ) / ( nonlinear_depth_representation_num + 1 )
	pos2 = ( 255 * ( k+1 ) ) / ( nonlinear_depth_representation_num + 1 ) )

	x1 = pos1
	y1 = pos1
	x2 = pos2
	y2 = pos2

	for ( x = Max( x1, 0 ); x <= Min( x2, 255 ); ++x )
		NdrInverse[ x ] = Clip3( 0, 255, Round( ( ( x - x1 ) * ( y2 - y1 ) ) ÷ ( x2 - x1 ) + y1 ) )
}

When nonlinear_depth_representation_enable_flag in the current slice is equal to 1, Variable NdrInverse[ i ] for i in the range of 0 to 255, inclusive, is specified as follows.

nonlinear_depth_representation_model[ 0 ] = 0
nonlinear_depth_representation_model[ nonlinear_depth_representation_num + 1 ] = 0
for( k=0; k<= nonlinear_depth_representation_num; ++k )
{
	pos1 = ( 255 * k ) / ( nonlinear_depth_representation_num + 1 )
	dev1 = nonlinear_depth_representation_model[ k ]
	pos2 = ( 255 * ( k+1 ) ) / ( nonlinear_depth_representation_num + 1 ) )
	dev2 = nonlinear_depth_representation_model[ k+1 ]

	x1 = pos1 - dev1
	y1 = pos1 + dev1
	x2 = pos2 - dev2
	y2 = pos2 + dev2

	for ( x = Max( x1, 0 ); x <= Min( x2, 255 ); ++x )
		NdrInverse[ x ] = Clip3( 0, 255, Round( ( ( x - x1 ) * ( y2 - y1 ) ) ÷ ( x2 - x1 ) + y1 ) )
}

In 7.4.4, Slice data semantics, add the following text at the end of the semantics of mb_skip_flag:

When mb_skip_flag is not present, it is inferred to be equal to 0.

In 7.4.4, Slice data semantics, add the following text

mb_skip_type_flag specifies the reference picture type if a macroblock is skipped. mb_skip_type_flag equal to 0 specifies that the current macroblock uses a reference picture as derived in subclause 8.4.1.1 or 8.4.1.2.2. mb_skip_type_flag equal to 1 specifies that the current macroblock uses a synthetic reference component as reference. When mb_skip_type_flag is not present, it is inferred to be equal to 0. When mb_skip_type_flag is present, the variable MbVSSkipFlag is set equal to mb_skip_type_flag.

mb_vsskip_flag equal to 1 specifies that for the current macroblock, when decoding a P slice, mb_type shall be inferred to be P_Skip using a synthetic reference component as a reference and the macroblock type is collectively referred to as P macroblock type, or for which, when decoding a B slice, mb_type shall be inferred to be B_Skip using a synthetic reference component as a reference and the macroblock type is collectively referred to as B macroblock type. mb_vsskip_flag equal to 0 specifies that the current macroblock is neither P_Skip using a synthetic reference component as a reference nor B_Skip using using a synthetic reference component as a reference. When mb_vsskip_flag is not present, it is inferred to be equal to 0. When mb_vsskip_flag is present, the variable MbVSSkipFlag is set equal to mb_vsskip_flag.

leftMbVSSkipped is derived to be 1 if the left macroblock adjacent to the current macroblock is available and the MbVSSkipFlag of the left macroblock is equal to 1, leftMbVSSkipped is derived to be 0 otherwise.

upMbVSSkipped is derived to be 1 if the upper macroblock adjacent to the current macroblock is available and the MbVSSkipFlag of the upper macrolbock is equal to 1, upMbVSSkipped is derived to be 0 otherwise.

mb_alc_skip_flag equal to 1 specifies that the adaptive luminance compensation is applied for the current macroblock. mb_alc_skip_flag equal to 0 specifies that the adaptive luminance compensation is not applied for the current macroblock. When not present, mb_alc_skip_flag is inferred to be equal to 0. When mb_alc_skip_flag is equal to 1, the current macroblock shall be coded as P_Skip.

In 7.4.5, Macroblock layer semantics, add the following text

mb_direct_type_flag specifies the reference picture type if a macroblock is coded in direct mode. mb_direct_type_flag equal to 0 specifies that the current macroblock uses a reference picture as derived in subclause 8.4.1.1 or 8.4.1.2.2. mb_direct_type_flag equal to 1 specifies that the current macroblock uses a synthetic reference component as reference. When mb_direct_type_flag is not present, it is inferred to be equal to 0.

IvmpEnabledFlag is set to 1 when all of the following conditions are satisfied:

–	inside_view_mvp_flag is equal to 1 (inside view motion prediction is enabled);

–	the current view component is a depth view component;

–	ViewCompOrder( 0, view_idx ) is smaller than ViewCompOrder( 1, view_idx );

–	avc_3d_extension_flag is equal to 1;

–	the current picture is a non-anchor picture and the slice type is not I or SI slice.

Otherwise, IvmpEnabledFlag is set to 0.

mb_ivmp_flag equal to 1 indicates that the mb_type and motion information of the current macroblock are not present and are derived from the one or four co-located macroblocks of the texture view component (depending on reduced_resolution_flag is 0 or 1, respectively) in the same view component pair. mb_ivmp_flag equal to 0 indicates that the mb_type and motion information are present. The motion information includes reference indices and motion vectors. When not present, this flag is inferred to be equal to 0.

mb_disp_flag equal to 1 specifies that the depth intra skip prediction specified in 8.7.1 is applied. mb_disp_flag equal to 0 specifies that the current macroblock is not intra skiped. When not present, this flag is inferred to be equal to 0.

mb_alc_flag equal to 1 specifies that the adaptive luminance compensation mode is in use for the current macroblock. mb_alc_flag equal to 0 specifies that the adaptive luminance compensation mode is not in use for the current macroblock. When mb_alc_flag is not present, it is inferred to be equal to 0. When mb_alc_flag is equal to 1, the current macroblock shall be coded as P_L0_16x16, P_L0_L0_16x8, P_L0_L0_8x16, P_8x8, or P_8x8ref0.

In Table D.1, add the following after “3d_reference_displays_info (payloadSize)”:

| else if( payloadType = = 54 ) |  |  |
| --- | --- | --- |
| constrained_depth_parameter_set_identifier( payloadSize ) |  |  |
| else if( payloadType = = 55 ) |  |  |
| depth_view_info ( payloadSize ) |  |  |

Annex J

Multiview and depth video coding

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies multiview and depth video coding, referred to as 3DVC.

  - Scope

Bitstreams and decoders conforming to the profile specified in this annex are completely specified in this annex with reference made to clauses Error: Reference source not found-Error: Reference source not found and Annexes Error: Reference source not found-I.

  - Normative references

The specifications in clause Error: Reference source not found apply.

  - Definitions

For the purpose of this annex, the following definitions apply in addition to the definitions in clause I.Error: Reference source not found. These definitions are either not present in clause I.Error: Reference source not found or replace definitions in clause I.Error: Reference source not found.

- MVC texture view component: A texture view component composed of coded slice NAL units of nal_unit_type not equal to 21.

- texture view component: A coded representation of the texture of a view in a single access unit. A texture view component may be a 3DVC texture view component or an MVC texture view component.

- view component pair: A texture view component and a depth view component of the same view within the same access unit.

- view synthesis: A process to generate a virtual picture using coded texture view and depth view from the same access unit. The process is used to generate a synthetic reference component.

- view synthesis prediction: A prediction derived from samples of synthetic reference components for decoding a view component.

- VSP reference index: An identifier of the output of the 3DVC decoding process for view synthesis reference component generation when the inputs are the texture and depth view components with view order index equal to ( vOIdx – 1 – i ), where vOIdx is the view order index of the present texture view component and i is equal to the VSP reference index.

- synthetic reference component: A picture synthesized from coded texture views and depth views. A view synthesis reference component contains samples that may be used for view synthesis prediction. A view synthesis reference components are reference pictures, and never being outputted.

- 3DVC texture view component: A texture view component composed of coded slice NAL units of nal_unit_type equal to 21.

  - Abbreviations

The specifications in clause Error: Reference source not found apply with the following additions:

VSP	View Synthesis Prediction

  - Conventions

The specifications in clause Error: Reference source not found apply.

  - Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specifications in clause Error: Reference source not found apply with substitution of 3DVC sequence parameter set for sequence parameter set.

  - Syntax and semantics

This clause specifies syntax and semantics for coded video sequences that conform to one or more of the profiles specified in this annex.

    - Method of specifying syntax in tabular form

The specifications in subclause I.Error: Reference source not found apply.

    - Specification of syntax functions, categories, and descriptors

The specifications in subclause I.Error: Reference source not found apply.

    - Syntax in tabular form

      - NAL unit syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - NAL unit header 3DVC extension syntax

| nal_unit_header_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| view_idx | All | u(8) |
| depth_flag | All | u(1) |
| non_idr_flag | All | u(1) |
| temporal_id | All | u(3) |
| anchor_pic_flag | All | u(1) |
| inter_view_flag | All | u(1) |
| } |  |  |

      - Raw byte sequence payloads and RBSP trailing bits syntax

        - Sequence parameter set RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Sequence parameter set data syntax

The syntax table is specified in subclause I.Error: Reference source not found.

            - Scaling list syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Sequence parameter set extension RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Subset sequence parameter set RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Sequence parameter set MVC extension syntax

The syntax table is specified in subclause I.7.3.2.1.4.

          - Sequence parameter set 3DVC extension syntax

The syntax table is specified in subclause I.7.3.2.1.5 is replaced by the following.

| seq_parameter_set_3dvc_extension( ) { | C | Descriptor |
| --- | --- | --- |
| for( i = 0; i <= num_level_values_signalled_minus1; i++ ) |  |  |
| for( j = 0; j <= num_applicable_ops_minus1[ i ]; j++ ) |  |  |
| applicable_op_num_depth_views_minus1[ i ][ j ] |  | ue(v) |
| depth_info_present_flag | 0 | u(1) |
| if( depth_info_present_flag ) { |  |  |
| 3dv_acquisition_idc | 0 | ue(v) |
| for( i = 0; i < num_views_minus1; i++ ) |  |  |
| view_id_3dv[ i ] | 0 | ue(v) |
| if( 3dv_acquisition_idc ) { |  |  |
| depth_ranges( 2, 0 ) |  |  |
| if( profile_idc = = 139 && num_views_minus1 ) |  |  |
| vsp_param( 2, 0 ) |  |  |
| } |  |  |
| if ( profile_idc = = 139 ) |  |  |
| depth_preceding_enable_flag | 0 | u(1) |
| if (depth_preceding_enable_flag) |  |  |
| for( i = 0; i<= num_views_minus1; i++ ) |  |  |
| depth_preceding_texture_flag[ i ] | 0 | u(1) |
| } |  |  |
| if( profile_idc = = 139 && depth_info_present_flag ) { |  |  |
| reduced_resolution_flag | 0 | u(1) |
| slice_header_prediction_flag | 0 | u(1) |
| inside_view_mvp_flag | 0 | u(1) |
| seq_view_synthesis_flag | 0 | u(1) |
| disp_flag | 0 | u(1) |
| psip_flag | 0 | u(1) |
| nonlinear_depth_representation_num | 0 | ue(v) |
| for( i = 1; i <= nonlinear_depth_representation_num; i++ ) |  |  |
| nonlinear_depth_representation_model[ i ] | 0 | ue(v) |
| } |  |  |
| if( profile_idc = = 139 && !depth_info_present_flag ) |  |  |
| alc_sps_enable_flag | 0 | u(1) |
| } |  |  |

        - Picture parameter set RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Supplemental enhancement information RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Supplemental enhancement information message syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Access unit delimiter RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - End of sequence RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - End of stream RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Filler data RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Slice layer without partitioning RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Slice data partition RBSP syntax

Slice data partition syntax is not present in coded video sequences conforming to one or more of the profiles specified in this annex.

        - RBSP slice trailing bits syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - RBSP trailing bits syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Prefix NAL unit RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Depth parameter set RBSP syntax

The syntax table is specified in subclause I.7.3.2.13 is replaced by the following.

| depth_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| depth_parameter_set_id | 11 | ue(v) |
| pred_direction | 11 | ue(v) |
| if( pred_direction = = 0 ¦ ¦ pred_direction = = 1 ) { |  |  |
| ref_dps_id0 | 11 | ue(v) |
| predWeight0 = 64 |  |  |
| } |  |  |
| if( pred_direction = = 0 ) { |  |  |
| ref_dps_id1 | 11 | ue(v) |
| pred_weight0 | 11 | u(6) |
| predWeight0 = pred_weight0 |  |  |
| } |  |  |
| num_views_minus1 | 11 | ue(v) |
| depth_ranges( pred_direction, depth_parameter_set_id ) |  |  |
| vsp_param_flag | 11 | u(1) |
| if( vsp_param_flag ) |  |  |
| vsp_param( pred_direction, depth_parameter_set_id ) |  |  |
| depth_param_additional_extension_flag | 11 | u(1) |
| if(depth_param_additional_extension_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| depth_param_additional_extension_data_flag | 11 | u(1) |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

          - Depth ranges syntax

| depth_ranges( predDirection, index ) { | C | Descriptor |
| --- | --- | --- |
| z_near_flag | 11 | u(1) |
| z_far_flag | 11 | u(1) |
| if( z_near_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 7, 0, ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |  |
| if( z_far_flag ) |  |  |
| 3dv_acquisition_element( 0, predDirection, 7, 0, ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |  |
| } |  |  |

          - 3DV acquisition element syntax

| 3dv_acquisition_element( deltaFlag, predDirection, precMode, expLen, OutSign, OutExp, OutMantissa, OutManLen ) { | C | Descriptor |
| --- | --- | --- |
| if( num_views_minus1 + 1 – deltaFlag > 1 ) |  |  |
| element_equal_flag | 11 | u(1) |
| if( element_equal_flag = = 0 ) |  |  |
| numValues = num_views_minus1 + 1 – deltaFlag |  |  |
| else |  |  |
| numValues = 1 |  |  |
| for( i = 0; i < numValues; i++ ) { |  |  |
| if( predDirection = = 2 && i = = 0 ) { |  |  |
| if( precMode = = 0 ) { |  |  |
| matissa_len_minus1 | 11 | u(5) |
| OutManLen[ index, i ] = manLen = mantissa_len_minus + 1 |  |  |
| } else |  |  |
| prec | 11 | u(5) |
| } |  |  |
| if( predDirection = = 2 ) { |  |  |
| sign0 | 11 | u(1) |
| OutSign[ index, i ] = sign0 |  |  |
| exponent0 | 11 | u(expLen) |
| OutExp[ index, i ] = exponent0 |  |  |
| if( precMode = = 1 ) { |  |  |
| if( exponent0 = = 0 ) |  |  |
| OutManLen[ index, i ] = manLen = Max( 0, prec – 30 ) |  |  |
| else |  |  |
| OutManLen[ index, i ] = manLen = Max( 0, exponent0 + prec – 31 ) |  |  |
| } |  |  |
| mantissa0 | 11 | u(manLen) |
| OutMantissa[ index, i ] = mantissa0 |  |  |
| } else { |  |  |
| skip_flag | 11 | u(1) |
| if( skip_flag = = 0 ) { |  |  |
| sign1 | 11 | u(1) |
| OutSign[ index, i ] = sign1 |  |  |
| exponent_skip_flag | 11 | u(1) |
| if( exponent_skip_flag = = 0 ) { |  |  |
| exponent1 | 11 | u(expLen) |
| OutExp[ index, i ] = exponent1 |  |  |
| } else |  |  |
| OutExp[ index, i ] = OutExp[ ref_dps_id0, i ] |  |  |
| mantissa_diff | 11 | se(v) |
| mantissaPred = (( OutMantissa[ ref_dps_id0, i ] * predWeight0 + OutMantissa[ ref_dps_id1, i ] * ( 64-predWeight0 ) + 32 ) >> 6 ) |  |  |
| OutMantissa[ index, i ] = mantissaPred + mantissa_diff |  |  |
| OutManLen[ index, i ] = OutManLen[ ref_dps_id0, i ] |  |  |
| } else { |  |  |
| OutSign[ index, i ] = OutSign[ ref_dps_id0, i ] |  |  |
| OutExp[ index, i ] = OutExp[ ref_dps_id0, i ] |  |  |
| OutMantissa[ index, i ] = OutMantissa[ ref_dps_id0, i ] |  |  |
| OutManLen[ index, i ] = OutManLen[ ref_dps_id0, i ] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( element_equal_flag = = 1 ) { |  |  |
| for( i = 1; i < num_views_minus1 + 1 – deltaFlag; i++ ) { |  |  |
| OutSign[ index, i ] = OutSign[ index, 0 ] |  |  |
| OutExp[ index, i ] = OutExp[ index, 0 ] |  |  |
| OutMantissa[ index, i ] = OutMantissa[ index, 0 ] |  |  |
| OutManLen[ index, i ] = OutManLen[ index, 0 ] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

          - View synthesis parameters syntax

| vsp_param( predDirection, index ) { | C | Descriptor |
| --- | --- | --- |
| for( i = 0; i <= num_views_minus1; i++ ) |  |  |
| for( j = 0; j < i; j++ ) { |  |  |
| disparity_diff_wji[ j ][ i ] | 0 | ue(v) |
| disparity_diff_oji[ j ][ i ] | 0 | ue(v) |
| disparity_diff_wij[ i ][ j ] | 0 | ue(v) |
| disparity_diff_oij[ i ][ j ] | 0 | ue(v) |
| } |  |  |
| } |  |  |

        - Slice layer extension RBSP syntax

The syntax table is specified in subclause I.Error: Reference source not found.

      - Slice header syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Reference picture list modification syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Reference picture list MVC modification syntax

The syntax table is specified in subclause I.7.3.3.1.1

          - Reference picture list 3DVC modification syntax

| ref_pic_list_3dvc_modification( ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type % 5 != 2 && slice_type % 5 != 4 ) { |  |  |
| ref_pic_list_modification_flag_l0 | 2 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |  |
| do { |  |  |
| modification_of_pic_nums_idc | 2 | ue(v) |
| if( modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| else if ( modification_of_pic_nums_idc = = 4 ¦ ¦ modification_of_pic_nums_idc = = 5 ) |  |  |
| abs_diff_view_idx_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 6 ) |  |  |
| vsp_ref_idx | 2 | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| if( slice_type % 5 = = 1 ) { |  |  |
| ref_pic_list_modification_flag_l1 | 2 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |  |
| do { |  |  |
| modification_of_pic_nums_idc | 2 | ue(v) |
| if( modification_of_pic_nums_idc = = 0 ¦ ¦ modification_of_pic_nums_idc = = 1 ) |  |  |
| abs_diff_pic_num_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 2 ) |  |  |
| long_term_pic_num | 2 | ue(v) |
| else if ( modification_of_pic_nums_idc = = 4 ¦ ¦ modification_of_pic_nums_idc = = 5 ) |  |  |
| abs_diff_view_idx_minus1 | 2 | ue(v) |
| else if( modification_of_pic_nums_idc = = 6 ) |  |  |
| vsp_ref_idx | 2 | ue(v) |
| } while( modification_of_pic_nums_idc != 3 ) |  |  |
| } |  |  |
| } |  |  |

        - Prediction weight table syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Decoded reference picture marking syntax

The syntax table is specified in subclause I.Error: Reference source not found.

      - Slice data syntax

The syntax table is specified in subclause I.Error: Reference source not found.

      - Macroblock layer syntax

The syntax table is specified in subclause I.Error: Reference source not found.

        - Macroblock prediction syntax

| mb_pred( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ){ |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] ) |  |  |
| rem_intra4x4_pred_mode[ luma4x4BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| if( psip_flag && Psip4x4Available && Intra4x4PredMode[luma4x4BlkIdx] = = 2 ){ |  |  |
| psip4x4_dir[ luma4x4BlkIdx ] | 2 | u(1) |
| for( idxRowCol=0; idxRowCol<4; idxRowCol++ ) |  |  |
| psip4x4_code_diff[ luma4x4BlkIdx ][ idxRowCol ] | 2 | se(v) ¦ ae(v) |
| [Ed. CAVLC syntax missing] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ){ |  |  |
| for( luma8x8BlkIdx=0; luma8x8BlkIdx<4; luma8x8BlkIdx++ ) { |  |  |
| prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra8x8_pred_mode_flag[ luma8x8BlkIdx ] ) |  |  |
| rem_intra8x8_pred_mode[ luma8x8BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| for( luma8x8BlkIdx =0; luma8x8BlkIdx <4; luma8x8BlkIdx ++ ) { |  |  |
| if( psip_flag && Psip8x8Available && Intra8x8PredMode[luma4x4BlkIdx] = = 2 ){ |  |  |
| psip8x8_dir[ luma8x8BlkIdx ] | 2 | u(1) |
| for( idxRowCol=0; idxRowCol<8; idxRowCol++ ) |  |  |
| psip8x8_code_diff[ luma8x8BlkIdx ][idxRowCol] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( psip_flag && Psip16x16Available && Intra16x16PredMode = = 2 ){ |  |  |
| psip16x16_dir | 2 | u(1) |
| for( idxRowCol=0; idxRowCol<16; idxRowCol++ ) |  |  |
| psip16x16_code_diff[ idxRowCol ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( ChromaArrayType = = 1 ¦ ¦ ChromaArrayType = = 2 ) |  |  |
| intra_chroma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } else if( MbPartPredMode( mb_type, 0 ) != Direct ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L1 && mb_alc_flag = = 0 ) |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode ( mb_type, mbPartIdx ) != Pred_L1 && VspRefL0Flag[ mbPartIdx ] ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 && VspRefL1Flag[ mbPartIdx ] ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |

        - Sub-macroblock prediction syntax

| sub_mb_pred( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| sub_mb_type[ mbPartIdx ] | 2 | ue(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && mb_type != P_8x8ref0 && sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 && mb_alc_flag = = 0 ) |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag != field_pic_flag ) && sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 ) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L1 && VspRefL0Flag[ mbPartIdx ] ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ subMbPartIdx ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 && SubMbPredMode( sub_mb_type[ mbPartIdx ] ) != Pred_L0 && VspRefL1Flag[ mbPartIdx ] ) |  |  |
| for( subMbPartIdx = 0; subMbPartIdx < NumSubMbPart( sub_mb_type[ mbPartIdx ] ); subMbPartIdx++) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ subMbPartIdx ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |

        - Residual data syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Residual luma syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Residual block CAVLC syntax

The syntax table is specified in subclause I.Error: Reference source not found.

          - Residual block CABAC syntax

The syntax table is specified in subclause I.Error: Reference source not found.

    - Semantics

Semantics associated with the syntax structures and syntax elements within these structures (in subclause J.7.3 and in subclause I.7.3 by reference in subclause J.7.3) are specified in this subclause and by reference to subclause I.7.4. When the semantics of a syntax element are specified using a table or a set of tables, any values that are not specified in the table(s) shall not be present in the bitstream unless otherwise specified in this Recommendation | International Standard.

      - NAL unit semantics

The semantics for the syntax elements in subclause 7.3.1 are specified in subclause I.7.3.1.

        - NAL unit header 3DVC extension semantics

- view_idx specifies the view oder index for the NAL unit.

- view_id is inferred to be equal to view_id[ view_idx ], where view_id[ ] is present in the active sequence parameter set.

- The variable VOIdx, representing the view order index of the view identified by view_id[ i ], is set equal to view_idx.

- depth_flag equal to 1 indicates that the current NAL unit belongs to a depth view component, depth_flag equal to 0 indicates that the current NAL unit belongs to a texture view component.

- non_idr_flag, temporal_id, anchor_pic_flag, and inter_view_flag have the same semantics as those syntax elements with the same names in Annex H.

        - Order of NAL units and association to coded pictures, access units, and video sequences

The specification of subclause I.7.4.1.2 applies.

          - Order of 3DVC sequence parameter set RBSPs and picture parameter set RBSPs and their activation

- The specification of subclause I.7.4.1.2.1 applies.

- In addition, the following applies for the activation of depth parameter set.

- A depth parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more texture view or depth view components of one or more coded pictures.

- Each depth parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one depth parameter set RBSP is considered as the active depth parameter set RBSP at any given moment during the operation of the decoding process, and when any particular depth parameter set RBSP becomes the active depth parameter set RBSP, the previously-active depth parameter set RBSP (if any) is deactivated.

- When a depth parameter set RBSP (with a particular value of depth_parameter_set_id) is not the active depth parameter set RBSP and it is referred to by a coded slice NAL unit (when dps_id is present in a slice header), it is activated. This depth parameter set RBSP is called the active depth parameter set RBSP until it is deactivated when another depth parameter set RBSP becomes the active depth parameter set RBSP. A depth parameter set RBSP, with that particular value of depth_parameter_set_id, shall be available to the decoding process prior to its activation. When a depth parameter set RBSP is activated, the same depth parameter set RBSP shall remain active for subsequent coded slice NAL units of the same access unit.

- If any depth parameter set RBSP is present that is never activated in the bitstream (i.e., it never becomes the active depth parameter set), its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise-conforming bitstream.

          - Order of access units and association to coded video sequences

The specification of subclause I.7.4.1.2.2 applies.

          - Order of NAL units and coded pictures and association to access units

The specification of subclause I.Error: Reference source not found applies with the following modifications.

The association of VCL NAL units to primary or redundant coded pictures is specified in subclause I.7.4.1.2.5.

          - Detection of the first VCL NAL unit of a primary coded picture

The specification of subclause I.7.4.1.2.4 applies.

          - Order of VCL NAL units and association to coded pictures

The specification of subclause I.7.4.1.2.5 applies with following modifications.

Each VCL NAL unit is part of a coded picture.

Let voIdx be the value of VOIdx of any particular VCL NAL unit. The order of the VCL NAL units within a coded picture is constrained as follows:

- For all VCL NAL units following this particular VCL NAL unit, the value of VOIdx shall be greater than or equal to voIdx.

- All VCL NAL units for a depth view component, if present, shall follow any VCL NAL unit of an MVC texture view component with a same value of VOIdx.

      - Raw byte sequence payloads and RBSP trailing bits semantics

        - Sequence parameter set RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Sequence parameter set data semantics

The semantics specified in subclause I.Error: Reference source not found apply.

            - Scaling list semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Sequence parameter set extension RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Subset sequence parameter set RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Sequence parameter set MVC extension semantics

The semantics specified in subclause I.7.4.2.1.4 apply with the substitution of texture view component or depth view component for view component

          - Sequence parameter set 3DVC extension semantics

The semantics specified in subclause I.7.4.2.1.4 apply with the following changes and additions.

depth_info_present_flag equal to 0 specifies that no depth view components are present in a coded video sequence in which this subset sequence parameter set RBSP is active. depth_info_present_flag equal to 1 specifies that depth view components may be present in a coded video sequence in which this subset sequence parameter set RBSP is active.

3dv_acquisition_idc equal to 0 indicates that no 3DV acquisition parameters (the closest and farthest depth values) are present in the sequence parameter set. 3dv_acquisition_idc equal to 1 indicates that 3DV acquisition parameters are present in the sequence parameter set and valid for the entire coded video sequence. 3dv_acquisition_idc equal to 2 indicates that 3DV acquisition parameters are present in the sequence parameter set and depth range parameter update NAL units may be present. 3dv_acquisition_idc values greater than 2 are reserved.

The function ViewIdTo3DVAcquisitionParamIndex( viewId ) is specified to return the value of i for which view_id_3dv[ i ] is equal to viewId in the active sequence parameter set.

depth_preceding_enable_flag equal to 1 indicates the syntax elements depth_preceding_texture_flag[ i ] are not present. depth_preceding_enable_flag equal to 0 indicates that the syntax elements depth_preceding_texture_flag[ i ] are present.

depth_preceding_texture_flag[ i ] specifies the decoding order of depth view components in relation to texture view components. depth_preceding_texture_flag[ i ] equal to 1 indicates that the depth view component of the view with view_idx equal to i precedes the texture view component of the same view in decoding order in each access unit that contains both the texture and depth view components. depth_preceding_texture_flag[ i ] equal to 0 indicates that the texture view component of the view with view_idx equal to i precedes the depth view component of the same view in decoding order in each access unit that contains both the texture and depth view components. When not present, depth_preceding_texture_flag[ i ] is inferred to be equal to 0.

The function ViewCompOrder( depthFlag, vOIdx ) is specified to return the following value:

vOIdx*2+ ( (depth_preceding_texture_flag[vOIdx] = = depthFlag) ? 0: 1).

reduced_resolution_flag equal to 1 specifies that the depth view components of a view component pair have a lower spatial resolution than the luma component of the texture view component of the same view component pair, and the width and height (as represented by pic_width_in_mbs_minus1+1 and pic_height_in_map_units_minus1+1 in the referred subset sequence parameter set) of the depth view components are both half of the width and height of all the texture view components. reduced_resolution_flag equal to 0 specifies that when both depth view components and texture view components are present, they have the same spatial resolution.

slice_header_prediction_flag equal to 0 indicates that slice header prediction from texture view component to depth view component or vice versa is disallowed. slice_header_prediction_flag equal to 1 indicates that the prediction is used.

inside_view_mvp_flag equal to 1 indicates inside view motion prediction is enabled for depth view components having view order index vOIdx when avc_3d_extension_flag is equal to 1 and ViewCompOrder( 0, vOIdx) is smaller than ViewCompOrder( 1, vOIdx). inside_view_mvp_flag equal to 0 inidicates that inside view motion prediction is disabled for all view components referring the current sequence parameter set.

seq_view_synthesis_flag equal to 1 indicates view synthesis prediction is enabled. seq_view_synthesis_flag equal to 0 indicates that view synthesis prediction is disbable for all view components referring the current sequence parameter set.

disp_flag equal to 1 indicates depth intra skip prediction is enabled for view components refereeing the current sequence parameter set. disp_flag equal to 0 indicates that depth intra skip prediction is disbable for all view components referring the current sequence parameter set. When not present, disp_flag is inferred to be equal to 0.

psip_flag equal to 1 indicates plane segmentation based intra prediction is enabled. psip_flag equal to 0 indicates that plane segmentation based intra prediction is disbable for all view components referring the current sequence parameter set. When not present, psip_flag is inferred to be equal to 0.

nonlinear_depth_representation_num + 1 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

nonlinear_depth_representation_model[ i ] specifies the piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

NOTE – When nonlinear_depth_representation_num is equal to 0, a depth sample represents a disparity normalized to the range of 0 to 255, inclusive, so that 0 corresponds to ZFar value and 255 corresponds to ZNear value. Depending on the value of nonlinear_depth_representation_num, a depth view component is composed of either depth samples that can be converted to disparity using a linear equation or nonlinearly transformed depth samples. If nonlinear_depth_representation_num is equal to 0, depth view component contains directly depth samples that are uniformly quantized in terms of disparity, i.e. depth samples that can be transformed to disparity using a linear equation. If nonlinear_depth_representation_num is greater than 0, depth view component contains nonlinearly transformed depth samples. The use of nonlinear depth representation is further controlled by the nonlinear_depth_representation_enable_flag syntax element in the slice header.

alc_sps_enable_flag equal to 0 specifies that mb_alc_skip_flag and mb_alc_flag are not present. alc_sps_enable_flag equal to 1 specifies that specifies that mb_alc_skip_flag and mb_alc_flag may be present.

        - Picture parameter set RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Supplemental enhancement information RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Supplemental enhancement information message semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Access unit delimiter RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - End of sequence RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - End of stream RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Filler data RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Slice layer without partitioning RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Slice data partition RBSP semantics

Slice data partition syntax is not present in bitstreams conforming to one or more of the profiles specified in Annex J.

        - RBSP slice trailing bits semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - RBSP trailing bits semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Prefix NAL unit RBSP semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Depth parameter set RBSP semantics

depth_parameter_set_id identifies the depth parameter set that is referred to in the slice header. The value of depth_parameter_set_id shall be in the range of 1 to TBD, inclusive.

pred_direction equal to 0 specifies that the 3DV acquisition variables (the closest and farthest depth value) for the base view may be predicted from the respective variables of two pictures. pred_direction equal to 1 specifies that the 3DV acquisition variables for the base view may be predicted from the respective variables of one picture. pred_direction equal to 2 specifies that the 3DV acquisition variables for the base view are not predicted. The value of pred_direction shall be in the range of 0 to 2, inclusive.

ref_dps_id0 specifies a first reference depth parameter set to be used in prediction of 3DV acquisition variables. The value of ref_dps_id0 shall be in the range of 0 to TBD, inclusive.

ref_dps_id1 specifies a second reference depth parameter set to be used in prediction of 3DV acquisition variables. The value of ref_dps_id1 shall be in the range of 0 to TBD, inclusive.

pred_weight0 specifies a weight associated with the a first reference depth parameter set derived from ref_dps_id0.

num_views_minus1 plus1 1 specifies the number of views for which depth parameters are specified in the included 3dv_acquisition_element ( ) structure.

vsp_param_flag equal to 1 indicates the presence of the vsp_param( ) syntax structure. vsp_param_flag equal to 0 indicates the absence of the vsp_param( ) syntax structure.

depth_param_additional_extension_flag equal to 0 indicates that no additional data follows within the depth range update RBSP prior to the RBSP trailing bits. The value of depth_param_additional_extension_flag shall be equal to 0. The value of 1 for depth_param_additional_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follows the value of 1 for depth_param_additional_extension_flag in a depth range update RBSP.

          - Depth ranges semantics

The contents of the syntax structure are controlled through input variables predDirection and index the semantics of which are as follows.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	index may be equal to the depth_parameter_set_id of the depth parameter set wherein the parameters are present.

z_near_flag equal to 0 specifies that the syntax elements specifying the closest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the closest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

          - 3DV acquisition element semantics

The syntax structure specifies the value of an element in 3DV acquisition information. The element may contain one or more loop entries i of the order specified by view_id_3dv syntax elements.

The contents of the syntax structure are controlled through input variables predDirection, precMode, and expLen the semantics of which are as follows.

–	deltaFlag equal to 0 specifies that the each loop entry corresponds to a value specific to the view indicated by i. deltaFlag equal to 1 specifies that each loop entry corresponds to a difference of values between the views indicated by i and i + 1.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	precMode equal to 0 specifies that the number of bits in the mantissa syntax element.

–	expLen specifies the number of bits in the exponent syntax element.

The syntax structure uses OutSign, OutExp, and OutMantissa variables for both input and output, where each variable is indexed by [ index, viewIdc ], index being an identifier (equal to either 0 when decoding depth ranges in sequence parameter set or depth_parameter_set_id value when decoding depth range parameter set) to a depth parameter set and viewIdc being a view indicator (in the order of views for 3DV acquisition parameters).

element_equal_flag equal to 0 specifies that the sign, exponent, and mantissa may not be identical to respective values for any two loop entries i and j. element_equal_flag equal to 1 specifies that the sign, exponent, and mantissa are identical to respective values for any two loop entries i and j.

mantissa_len_minus1 + 1 specifies the number of bits in the mantissa syntax element. The value of mantissa_len_minus1 shall be in the range of 0 to 31, inclusive.

prec specifies the exponent of the maximum allowable truncation error for the value represented by the sign, exponent, and mantissa given by 2−prec. The value of prec shall be in the range of 0 to 31, inclusive.

sign0 equal to 0 indicates that the sign of the value provided in the loop entry is positive. sign0 equal to 1 indicates that the sign is negative.

exponent0 specifies the exponent of the value provided by the loop entry. The value of exponent0 shall be in the range of 0 to 2expLen – 2, inclusive. The value 2expLen – 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 2expLen – 1 as indicating an unspecified value.

mantissa0 specifies the mantissa of the value provided by the loop entry.

skip_flag equal to 0 specifies that syntax elements sign1, exponent_skip_flag and mantissa_diff are present for the loop entry. skip_flag equal to 1 specifies that elements sign1, exponent_skip_flag and mantissa_diff are not present for the loop entry.

sign1 equal to 0 indicates that the sign of the value provided in the loop entry is positive. sign1 equal to 1 indicates that the sign is negative.

exponent1, if present, specifies the exponent of the value provided by the loop entry. The value of exponent1 shall be in the range of 0 to 2expLen – 2, inclusive. The value 2expLen – 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 2expLen – 1 as indicating an unspecified value.

mantissa_diff specifies the difference of the mantissa of the value provided by the loop entry relative to its prediction value.

          - View synthesis parameters semantics

The contents of the syntax structure are controlled through input variables predDirection and index the semantics of which are as follows.

–	predDirection equal to 2 specifies that the first loop entry of the element is not predicted and coded in the sign, exponent, and mantissa syntax elements. predDirection equal to 0 or 1 specifies that the first loop entry of the element is predicted and a difference relative to a prediction value is coded in the difference syntax element.

–	index identifies a depth parameter set.

disparity_diff_wji[ j ][ i ], disparity_diff_oji[ j ][ i ], disparity_diff_wij[ i ][ j ] and disparity_diff_oij[ i ][ j ] specify the variables DisparityScale and DisparityOffset as follows.

if( predDirection  = =  2 ) {
	DisparityScale[ index ][ j ][ i ] = disparity_diff_wji[ j ][ i ]
	DisparityOffset[ index ][ j ][ i ] = disparity_ diff_oji[ j ][ i ]
	DisparityScale[ index ][ i ][ j ] = disparity_diff_wij[ i ][ j ] – disparity_diff_wji[ j ][ i ]
	DisparityOffset[ index ][ i ][ j ] = disparity_diff_oij[ i ][ j ] – disparity_diff_oji[ j ][ i ]
} else {
	DisparityScale[ index ][ j ][ i ] = disparity_diff_wji[ j ][ i ] + ( DisparityScale[ ref_dps_id0 ][ j ][ i ] *
		predWeight0 + DisparityScale[ ref_dps_id1 ][ j ][ i ] * ( 64 – predWeight0 ) + 32 ) >> 6
	DisparityOffset[ index ][ j ][ i ] = disparity_diff_oji[ j ][ i ] + ( DisparityOffset[ ref_dps_id0 ][ j ][ i ] *
		predWeight0 + DisparityOffset[ ref_dps_id1][ j ][ i ] * ( 64 – predWeight0 ) + 32 ) >> 6
	DisparityScale[ index ][ i ][ j ] = disparity_diff_wij[ i ][ j ] + ( DisparityScale[ ref_dps_id0 ] i ][ j ] * 
		predWeight0 + DisparityScale[ ref_dps_id1 ][ i ][ j ] * ( 64 – predWeight0 ) + 32 ) >> 6
	DisparityOffset[ index ][ i ][ j ] = disparity_diff_oij[ i ][ j ] + ( DisparityOffset[ ref_dps_id0 ][ i ][ j ] * 
		predWeight0 + DisparityOffset[ ref_dps_id1 ][ i ][ j ] * ( 64 – predWeight0 ) + 32 ) >> 6
}

        - Slice layer extension RBSP semantics

The semantics specified in subclause I.7.4.2.14 apply.

      - Slice header semantics

The semantics specified in subclause H.Error: Reference source not found apply with the substitution of texture view component or depth view component for view component and with the following modifications.

When nal_unit_type is equal to 1, 5, 20 or 21 with DepthFlag equal to 0, all constraints specified in subclause H.Error: Reference source not found apply only to the texture view components with the same value of VOIdx. When nal_unit_type is equal to 21 and DepthFlag is equal to 1, all constraints specified in subclause H.Error: Reference source not found apply only to the depth view components with the same value of VOIdx.

The value of the following 3DVC sequence parameter set syntax elements shall be the same across all coded slice NAL units of nal_unit_type 1, 5, 20 and 21 with DepthFlag equal to 0 of an access unit: chroma_format_idc.

The value of the following slice header syntax elements shall be the same across all coded slice NAL units of nal_unit_type 1, 5, 20 and 21 with DepthFlag equal to 0 of an access unit: field_pic_flag and bottom_field_flag.

        - Reference picture list modification semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Reference picture list MVC modification semantics

The semantics specified in subclause I.7.4.3.1.1 apply.

          - Reference picture list 3DVC modification semantics

The semantics specified in subclause I.7.4.3.1.1 apply with the following additions.

The following additional table entry is inserted in Table H-1.

| modification_of_pic_nums_idc | Modification specified |
| --- | --- |
| 6 | vsp_ref_idx is present and corresponds to a VSP reference index |

modification_of_pic_nums_idc shall not be equal to 6, if seq_view_synthesis_flag is equal to 0.

        - Prediction weight table semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Decoded reference picture marking semantics

The semantics specified in subclause I.Error: Reference source not found apply to each view independently, with "sequence parameter set" being replaced by "3DVC sequence parameter set", with "primary coded picture" being replaced by "texture view component” for nal_unit_type 1, 5, and 20 as well as nal_unit_type 21 when DepthFlag is equal to 1, and by “depth view component” for nal_unit_type 21 when DepthFlag is equal to 0.

      - Slice data semantics

The semantics specified in subclause I.Error: Reference source not found apply.

      - Macroblock layer semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Macroblock prediction semantics

The semantics specified in subclause I.Error: Reference source not found apply with the following additions.

psip_4x4_dir specifies the pattern code direction for Intra_4x4_DC_PSIP mode luma block. psip_4x4_dir equal to 0 indicates that the PSIP pattern code is generated with row-by-row, and psip_4x4_dir equal to 1 indicates that the PSIP pattern code is generated with column-by-column.

psip_4x4_code_diff specifies the difference of pattern code between two adjacent rows or columns in the range of -4 to 3.

Psip4x4Available is set to 1 if eight neighbouring samples p[ x, y ] that are constructed luma samples prior to the deblocking filter process, with x = −1, y = 0..3 and with x = 0..3, y = −1 are marked as “available for Intra_4x4_prediction”.

psip_8x8_dir specifies the pattern code direction for Intra_8x8_DC_PSIP mode luma block. psip_8x8_dir equal to 0 indicates that the PSIP pattern code is generated with row-by-row, and psip_8x8_dir equal to 1 indicates that the PSIP pattern code is generated with column-by-column.

psip_8x8_code_diff specifies the difference of pattern code between two adjacent rows or columns in the range of -8 to 7.

Psip8x8Available is set to 1 if 16 neighbouring samples p[ x, y ] that are constructed luma samples prior to the deblocking filter process, with x = −1, y = 0..7 and with x = 0..7, y = −1 are marked as “available for Intra_8x8_prediction”.

psip_16x16_dir specifies the pattern code direction for Intra_16x16_DC_PSIP mode luma block. psip_16x16_dir equal to 0 indicates that the PSIP pattern code is generated with row-by-row, and psip_16x16_dir equal to 1 indicates that the PSIP pattern code is generated with column-by-column.

psip_16x16_code_diff specifies the difference of pattern code between two adjacent rows or columns in the range of -16 to 15.

Psip16x16Available is set to 1 if both the macroblocks with address mbAddrA and mbAddrB are all available.

        - Sub-macroblock prediction semantics

The semantics specified in subclause I.Error: Reference source not found apply.

        - Residual data semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Residual luma semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Residual block CAVLC semantics

The semantics specified in subclause I.Error: Reference source not found apply.

          - Residual block CABAC semantics

The semantics specified in subclause I.Error: Reference source not found apply.

  - 3DVC decoding process

This subclause specifies the decoding process for an access unit of a coded video sequence conforming to one or more of the profiles specified in Annex J. Specifically, this subclause specifies how the decoded picture with multiple texture view components and multiple depth view components is derived from syntax elements and global variables that are derived from NAL units in an access unit when the decoder is decoding the operation point identified by the target temporal level and the target output views.

The decoding process is specified such that all decoders shall produce numerically identical results for the target output views. Any decoding process that produces identical results for the target output views to the process described here conforms to the decoding process requirements of this Recommendation | International Standard.

Unless stated otherwise, the syntax elements and derived upper-case variables that are referred to by the decoding process specified in this subclause and all child processes invoked from the process specified in this subclause are the syntax elements and derived upper-case variables for the current access unit.

The target output views are either specified by external means not specified in this Specification, or, when not specified by external means, there shall be one target output view which is the base view. Let OutputVOIdxList be the list of VOIdx values, in increasing order of VOIdx, of all the target output views in one access unit. The list OutputVOIdxList shall not change within a coded video sequence.

For profiles specified in Annex J, a target output view includes both the texture view and the depth view, which have the same VOIdx.

All sub-bitstreams that can be derived using the sub-bitstream extraction process with pIdTarget equal to any value in the range of 0 to 63, inclusive, tIdTarget equal to any value in the range of 0 to 7, inclusive, viewIdTargetList consisting of any one or more viewIdTarget's identifying the views in the bitstream as inputs as specified in subclause J.8.5 shall result in a set of coded video sequences, with each coded video sequence conforming to one or more of the profiles specified in Annex Error: Reference source not found, Annex Error: Reference source not found, Annex I and Annex J.

Let vOIdxList be a list of integer values specifying the VOIdx values of the view components of the access unit. The variable VOIdxMax is set equal to the maximum value of the entries in the list vOIdxList, and the variable vOIdxMin is set to the minimum value of the entries in the list vOIdxList. VOIdxMax shall be the same for all access units within a coded video sequence. vOIdxMin shall be the same for all anchor access units within a coded video sequence. When the current access unit is an anchor access unit, the variable VOIdxMin is set to vOIdxMin.

The multiview video decoding process specified in this subclause is repeatedly invoked for each texture and depth view component with VOIdx from vOIdxMin to VOIdxMax, inclusive, which is present in the list vOIdxList, in increasing order of VOIdx and in decoding order of texture or depth view components as specified in subclause 7.4.1.2.5.

Outputs of the multiview video decoding process are decoded samples of the current primary coded picture including all decoded texture and depth view components.

For each texture view component and each depth view component, the specifications in clause J.Error: Reference source not found apply, with the decoding processes for reference picture lists construction and decoded reference picture marking being modified in subclauses J.8.1 and J.8.2, respectively. The 3DVC inter prediction, inter-view prediction, view synthesis prediction and inter prediction with adaptive luminance componesnation processes are specified in subclause J.8.3. The decoding process for depth range parameters is specified in subclause J.8.5. The decoding process for depth intra prediction is specified in subclause J.8.7. Additionally, the specification of bitstream subsets is specified in subclause J.8.4.

    - 3DVC decoding process for reference picture lists construction

The specifications of subclause I.8.1 apply with the following changes and additions:

–	Modification process for reference picture lists is specified in subclause 8.1.1 instead of subclause I.8.1.2.

–	Subclause 8.1.2 is invoked.

–	When DepthFlag is equal to 0, the variable VspRefExist is specified after applying subclause H.8.2 as follows.

–	If seq_view_synthesis_flag is equal to 0 (view synthesis prediction is disabled), VspRefExist is set to 0.

–	Otherwise, if the current slice is a P or SP slice and there exists at least one synthetic reference component in RefPicList0, or if  the current slice is a B slice and there exists at least one synthetic reference component in either RefPicList0 or RefPicList1, VspRefExist is set to 1;

–	Otherwise, VspRefExist is set to 0.

–	The variable VspRefL0Flag[ mbPartIdx ] is specified as follows.

–	If VspRefExist is equal to 1, ref_idx_l0[ mbPartIdx ] is present, and ref_idx_l0[ mbPartIdx ] indicates a synthetic reference component, the variable VspRefL0Flag[ mbPartIdx ] is set to 1.

–	Otherwise, the variable VspRefL0Flag[ mbPartIdx ] is set to 0.

–	The variable VspRefL1Flag[ mbPartIdx ] is specified as follows.

–	If VspRefExist is equal to 1, ref_idx_l1[ mbPartIdx ] is present, and ref_idx_l1[ mbPartIdx ] indicates a synthetic reference component, the variable VspRefL1Flag[ mbPartIdx ] is set to 1.

–	Otherwise, the variable VspRefL1Flag[ mbPartIdx ] is set to 0.

      - Modification process for reference picture lists

Input to this process is reference picture list RefPicList0 and, when decoding a B slice, also reference picture list RefPicList1.

Outputs of this process are a possibly modified reference picture list RefPicList0 and, when decoding a B slice, also a possibly modified reference picture list RefPicList1.

When ref_pic_list_modification_flag_l0 is equal to 1, the following ordered steps are specified:

- Let refIdxL0 be an index into the reference picture list RefPicList0. It is initially set equal to 0.

- The corresponding syntax elements modification_of_pic_nums_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies:

–	If modification_of_pic_nums_idc is equal to 0 or equal to 1, the process specified in subclause Error: Reference source not found is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise, if modification_of_pic_nums_idc is equal to 2, the process specified in subclause Error: Reference source not found is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise, if modification_of_pic_nums_idc is equal to 4 or equal to 5, the process specified in subclause 8.1.1.1 is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise, if modification_of_pic_nums_idc is equal to 6, the process specified in subclause 8.1.1.1 is invoked with RefPicList0 and refIdxL0 given as input, and the output is assigned to RefPicList0 and refIdxL0.

–	Otherwise (modification_of_pic_nums_idc is equal to 3), the modification process for reference picture list RefPicList0 is finished.

When ref_pic_list_modification_flag_l1 is equal to 1, the following ordered steps are specified:

- Let refIdxL1 be an index into the reference picture list RefPicList1. It is initially set equal to 0.

- The corresponding syntax elements modification_of_pic_nums_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies:

–	If modification_of_pic_nums_idc is equal to 0 or equal to 1, the process specified in subclause Error: Reference source not found is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise, if modification_of_pic_nums_idc is equal to 2, the process specified in subclause Error: Reference source not found is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise, if modification_of_pic_nums_idc is equal to 4 or equal to 5, the process specified in subclause 8.1.1.1 is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise, if modification_of_pic_nums_idc is equal to 6, the process specified in subclause 8.1.1.1 is invoked with RefPicList1 and refIdxL1 given as input, and the output is assigned to RefPicList1 and refIdxL1.

–	Otherwise (modification_of_pic_nums_idc is equal to 3), the modification process for reference picture list RefPicList1 is finished.

        - Modification process for reference picture lists for synthetic reference components

Inputs to this process are an index refIdxLX and a reference picture list RefPicListX (with X being 0 or 1).

Outputs of this process are an incremented index refIdxLX and a modified reference picture list RefPicListX.

The following procedure is conducted to place a synthetic reference component with VSP reference index equal to vsp_ref_idx into the index position refIdxLX, shift the position of any other remaining pictures to later in the list, and increment the value of refIdxLX:

for( cIdx = num_ref_idx_lX_active_minus1 + 1; cIdx > refIdxLX; cIdx− − )
	RefPicListX[ cIdx ] = RefPicListX[ cIdx − 1]
RefPicListX[ refIdxLX++ ] = synthetic reference component with VSP reference index equal to vsp_ref_idx

      - Initialisation of disparity vector derivation

A variable dVColDepthFlag is set to depth_preceding_texture_flag[viewIdx].

Disparity vector dvMB is set as follows: dvMB[ 0 ] = 0 and dvMB[ 1 ] = 0.

    - 3DVC decoded reference picture marking process

The specifications of subclause I.8.2 apply.

    - 3DVC inter prediction, inter-view prediction, view synthesis prediction and adaptive luminance compensation

[Ed. (CY): the changes in this subclause are considered as significant and related to various different modes. Current description is based on AVC specification with additions attached to different places of subclause 8.4. It might be better to write J.8.3 with all text in subclasue 8.4 explicitly described.]

Processes described in this subclause are invoked when decoding P and B macroblock types, and when nal_unit_type is equal to 21.

The variable log2Div is specified to be equal to BitDepthY + 6.

The function Disparity( depthSample, srcIndex, refIndex ) is specified to return disparityValue specified as follows.

disparityValue = ( NdrInverse[ depthSample ] * DisparityScale[ dps_id ][ srcIndex ][ refIndex ] + 
	( DisparityOffset[ dps_id ][ srcIndex ][ refIndex ] << BitDepthY ) + 
	( 1 << ( log2Div – 1 ) ) ) >> log2Div

When DepthFlag is equal to 0 and dmvp_flag is equal to 1, the variables DepthRefPicList0, DepthRefPicList1 for B slices, and DepthCurrPic are specified as follows. The variable DepthRefPicList0 is specified to consist of the depth view components of the view component pairs for which the texture view components are in RefPicList0 in the order that RefPicList0[ i ] and DepthRefPicList0[ i ] form a view component pair for any value of i = 0.. num_ref_idx_l0_active_minus1. The variable DepthRefPicList1 is specified for B slices to consist of the depth view components of the view component pairs for which the texture view components are in RefPicList1 in the order that RefPicList1[ i ] and DepthRefPicList1[ i ] form a view component pair for any value of i = 0.. num_ref_idx_l1_active_minus1. The variable DepthCurrPic is specified to be the upsampled decoded sample array of the depth view component of the view component pair for which the texture view component is the current texture view component.

The specifications in subclause 8.4 apply with the following changes.

–		When nal_unit_type is equal to 21, DepthFlag is equal to 0, dVColDepthFlag is equal to 0 and either dmvp_flag or seq_view_synthesis_flag is equal to 1, subclause 8.3.1.10 is invoked before decoding one macroblock.

–	If nal_unit_type is equal to 21, DepthFlag is equal to 0, dmvp_flag is equal to 1, mb_alc_flag is equal to 1 and mb_type is equal to P_8x8 or P_8x8ref0, subclause 8.3.7 is invoked instead of subclause 8.4 for inter prediction. [Ed.(MH): Is this if-condition in a correct place because it states “instead of subclause 8.4” but appears within a substitute of subclause 8.4? Subclause 8.3.7 seems to contain logic and invocations of other subclauses that are in the same level in this subclause – hence re-organizing subclause 8.3.7 should be considered.]

–	Otherwise, if nal_unit_type is equal to 21, DepthFlag is equal to 0, and dmvp_flag is equal to 1, the following applies:

–	If mb_alc_skip_flag is equal to 0 and mb_alc_flag is equal to 0, subclause 8.3.1 is invoked instead of subclause 8.4.1.

–	If mb_alc_skip_flag is equal to 1 or mb_alc_flag is equal to 1, subclause 8.3.5 is invoked instead of subclause 8.4.1 for the derivation of motion vector components and reference indices, and subclause 8.3.6 is invoked instead of subclause 8.4.3 for the derivation of prediction weights. [Ed.(MH): “subclause 8.3.6 is invoked instead of subclause 8.4.3 for the derivation of prediction weights” should be moved further down in the logic of subclause 8.4, that is between steps 3 and 4 in subclause 8.4.]

–	Otherwise, if nal_unit_type is equal to 21, DepthFlag is equal to 0, dmvp_flag is equal to 0, and mb_alc_skip_flag or mb_alc_flag is equal to 1, subclause 8.3.5 is invoked instead of subclause 8.4.1 for the derivation of motion vector components and reference indices, and subclause 8.3.6 is invoked instead of subclause 8.4.3 for the derivation of prediction weights. [Ed.(MH): “subclause 8.3.6 is invoked instead of subclause 8.4.3 for the derivation of prediction weights” should be moved further down in the logic of subclause 8.4, that is between steps 3 and 4 in subclause 8.4.]

–	When nal_unit_type is equal to 21, DepthFlag is equal to 1, and mb_ivmp_flag is equal to 1, subclause 8.3.3 is invoked instead of subclause 8.4.1.

–	The following additional step applied between steps 3 and 4 in subclause 8.4.

–	When (nal_unit_type is equal to 21 and depth_weighted_pred_flag is equal to 1 and (slice_type % 5) is equal to 0 or 3) or (depth_weighted_bipred_flag is equal to 1 and (slice_type % 5) is equal to 1), the derivation process for prediction weights in depth-range-based weighted prediction in subclause 8.3.4 is invoked.

–	The following sentence “The decoding process for Inter prediction samples as specified in subclause 8.4.2 is invoked.” in step 4 of subclause 8.4 is replaced by “The decoding process for Inter or view synthesis prediction samples as specified in subclause  is invoked.”

      - Derivation process for motion vector components and reference indices

[Ed. (MH): As there are quite many modifications to subcaluse 8.4.1 in this subclause, consider rewriting this subclause to be selfstanding, i.e. without references to 8.4.1.]

Inputs to this process are:

–	a macroblock partition mbPartIdx,

–	a sub-macroblock partition subMbPartIdx.

Outputs of this process are:

–	luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1,

–	a motion vector count variable subMvCnt.

The specifications of subclause 8.4.1 apply with the following changes.

–	The following paragraphs in subclause 8.4.1:

For the derivation of the variables mvL0 and mvL1 as well as refIdxL0 and refIdxL1, the following applies.

–	If mb_type is equal to P_Skip, the derivation process for luma motion vectors for skipped macroblocks in P and SP slices in subclause 8.4.1.1 is invoked with the output being the luma motion vectors mvL0 and reference indices refIdxL0, and predFlagL0 is set equal to 1. mvL1 and refIdxL1 are marked as not available and predFlagL1 is set equal to 0. The motion vector count variable subMvCnt is set equal to 1.

–	Otherwise, if mb_type is equal to B_Skip or B_Direct_16x16 or sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8, the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.4.1.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the motion vector count variable subMvCnt, and the prediction utilization flags predFlagL0 and predFlagL1.

are replaced by:

For the derivation of the variables mvL0 and mvL1 as well as refIdxL0 and refIdxL1, the following applies:

–	If mb_type is equal to P_Skip, the following applies:

–	If MbVSSkipFlag is equal to 0, the following applies:

–	If nal_unit_type is equal to 21 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the depth-based derivation process for luma motion vectors for skipped macroblock in P and SP slices in subclause 8.3.1.1 is invoked with the output being the luma motion vectors mvL0 and refernece indices refIdxL0, and predFlagL0 is set equal to 1.

–	Otherwise (nal_unit_type is not equal to 21 or DepthFlag is equal to 1 or dmvp_flag is equal to 0), the derivation process for luma motion vectors for skipped macroblock in P and SP slices in subclause 8.4.1.1 is invoked with the output being the luma motion vectors mvL0 and refernece indices refIdxL0, and predFlagL0 is set equal to 1.

–	Otherwise (MbVSSkipFlag is equal to 1), the derivation process for luma motion vectors for VSP skipped macroblock in P and SP slices in subclause 8.3.1.2 is invoked with the output being the luma motion vectors mvL0 and reference indices refIdxL0, and predFlagL0 is set equal to 1.

–	mvL1 and refIdxL1 are marked as not available and predFlagL1 is set equal to 0. The motion vector count variable subMvCnt is set equal to 1.

–	Otherwise, if mb_type is equal to B_Skip or B_Direct_16x16 or sub_mb_type[ mbPartIdx ] is equal to B_Direct_8x8, the following applies.

–	The variable vspFlag is specified as follows:

vspFlag = ! ( ( mb_type  = =  B_Skip  &&  MbVSSkipFlag  = =  0 )  | |  ( ( mb_type  = =  B_Direct_16x16  | |  sub_type[ mbPartIdx ]  = =  B_Direct_8x8 )  &&  !mb_direct_type_flag ) )

–	If vspFlag is equal to 0 and nal_unit_type is equal to 21 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the depth-based derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.3.1.3 is invoked with mbPartIdx and subMbPartIdx as the input and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the motion vector count variable subMvCnt, and the prediction utilization flags predFlagL0 and predFlagL1.

–	Otherwise, if vspFlag is equal to 0 either nal_unit_type is not equal to 21 or DepthFlag is equal to 1 or dmvp_flag is equal to 0, the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.4.1.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the motion vector count variable subMvCnt, and the prediction utilization flags predFlagL0 and predFlagL1.

–	Otherwise (vspFlag is equal to 1), the derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8 in B slices in subclause 8.3.1.6 is invoked.

–	The following paragraph in subclause 8.4.1:

4.	When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in subclause 8.4.1.3 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX. The luma motion vectors are derived by

mvLX[ 0 ] = mvpLX[ 0 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 0 ]	(8-172)

mvLX[ 1 ] = mvpLX[ 1 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 1 ]	(8-173)

is replaced by

4.	If VspRefLXFlag[ mbPartIdx ] is equal to 0, the following applies:

–	When predFlagLX is equal to 1 and DepthFlag is equal to 0 and dmvp_flag is equal to 1, the derivation process for luma motion vector prediction in subclause 8.3.1.7 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX.

–	When predFlagLX is equal to 1 and either DepthFlag is equal to 1 or dmvp_flag is equal to 0, the derivation process for luma motion vector prediction in subclause 8.4.1.3 is invoked with mbPartIdx subMbPartIdx, refIdxLX, and currSubMbType as the inputs and the output being mvpLX.

–	The luma motion vectors are derived by

mvLX[ 0 ] = mvpLX[ 0 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 0 ]	(J-XXX)

mvLX[ 1 ] = mvpLX[ 1 ] + mvd_lX[ mbPartIdx ][ subMbPartIdx ][ 1 ]	(J-XXX)

Otherwise (VspRefLXFlag[ mbPartIdx ] is equal to 1), the following applies:

–	The luma motion vectors are derived by

mvLX[ 0 ] = 0		(J-XXX)

mvLX[ 1 ] = 0		(J-XXX)

        - Depth-based derivation process for luma motion vectors for skipped macroblocks in P and SP slices

This process is invoked when mb_type is equal to P_Skip, nal_unit_type is equal to 21, DepthFlag is equal to 0, dmvp_flag is equal to 1 and MbVSSkipFlag is equal to 0.

Outputs of this process are:

–	the motion vector mvL0,

–	the reference index refIdxL0.

For the derivation of the motion vector mvL0 and refIdxL0 of a P_Skip macroblock type, the following ordered steps are specified:

- The process specified in subclause 8.3.1.4 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag equal to 0 as input and the output is assigned to the motion vector mvL0 and the reference index refIdxL0.

- When refIdxL0 is equal to -1, the following applies:

–	The reference index refIdxL0 is set to 0.

–	The derivation process for luma motion vector prediction in subclause 8.3.1.7 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, refIdxL0, and currSubMbType = “na” as the inputs and the output being mvL0.

        - Derivation process for luma motion vectors for VSP skipped macroblocks in P and SP slices

This process is invoked when mb_type is equal to P_Skip, nal_unit_type is equal to 21, DepthFlag is equal to 0, and MbVSSkipFlag is equal to 1.

Outputs of this process are the motion vector mvL0 and the reference index refIdxL0.

The reference index refIdxL0 for a VSP skipped macroblock is derived as the synthetic picture that appears first in RefPicList0.

The motion vector mvL0 is set equal to dvMB.

        - Derivation process for luma motion vectors for B_Skip, B_Direct_16x16, and B_Direct_8x8

Inputs to this process are current macroblock partition index mbPartIdx and subMbPartIdx.

Outputs of this process are the reference indices refIdxL0, refIdxL1, the motion vectors mvL0 and mvL1, the motion vector count variable subMvCnt, and the prediction list utilization flags, predFlagL0 and predFlagL1.

For the derivation of output, the following ordered steps are specified:

- Let the variable currSubMbType be set equal to sub_mb_type[ mbPartIdx ].

- The process specified in subclause 8.3.1.4 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType and listSuffixFlag set equal to 0 as input and the output is assigned to the motion vector mvL0 and the reference index refIdxL0.

- The process specified in subclause 8.3.1.4 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType and listSuffixFlag set equal to 1 as input and the output is assigned to the motion vector mvL1 and the reference index refIdxL1.

- When both reference indices refIdxL0 and refIdxL1 are equal to -1, the following applies:

–	The reference index refIdxL0 is set equal to 0.

–	The derivation process for luma motion vector prediction in subclause 8.3.1.7 is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, refIdxLX (with X being 0 or 1), and currSubMbType as the inputs and the output being mvLX.

        - Derivation process for the disparity vector and the inter-view reference

Inputs to this process are the listSuffixFlag and depth reference view component depthPic, the location of a top-left sample ( dbx1, dby1 ) of a partition if dVColDepthFlag is equal to 1.

Outputs of this process are a picture InterViewPic, an offset vector dv and a variable InterViewAvailable. [Ed. rename “dv” as a proper variable name should include camelCasing.] [Ed.(MH): As InterViewAvailable starts with an upper case letter, it can be used in other sub-clauses too. However, there are two other subclauses where a variable with the same name is specified. It may therefore be unclear which specification of InterViewAvailable is in effect.]

Set InterViewAvailable equal to 0.

The following applies to derive an inter-view reference picture or inter-view only reference picture InterViewPic:

for( cIdx = 0;cIdx<=num_ref_idx_l0_active_minus1  && !InterViewAvailable; cIdx ++)
	if ( view order index of RefPicList0[ cIdx ] is equal to 0) {
		InterViewPic = RefPicList0[ cIdx ]
		InterViewAvailable = 1
	}

When InterViewAvailable is equal to 1, the following steps apply in order.

–	The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 0 as input and with reference indices refIdxCandL0[ i ] and the motion vectors mvCandL0[ i ] as outputs with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

–	The process specified in subclause Error: Reference source not found is invoked with mbPartIdx set equal to 0, subMbPartIdx set equal to 0, currSubMbType set equal to "na", and listSuffixFlag set equal to 1 as input and with reference indices refIdxCandL1[ i ] and the motion vectors mvCandL1[ i ] as outputs with i equal to 0, 1, and 2 corresponding to neighbouring partition A, B, and C, respectively.

–	The variable dv is derived as specified by the following ordered steps:

–	Set DvAvailable[ i ] and mvCand[ i ] with i equal to 0, 1, and 2 corresponding to neighbouring partitions A, B, and C, respectively, as follows

for( i = 0; i < 3; i++ )
	if( view order index of RefPicList0[ refIdxCandL0[ i ] ] is equal to 0 ) {
		DvAvailable[ i ] = 1
		mvCand[ i ] = mvCandL0[ i ]
	} else if( view order index of RefPicList1[ refIdxCandL1[ i ] ] is equal to 0 ) {
		DvAvailable[ i ] = 1
		mvCand[ i ] = mvCandL1[ i ]
	} else
		DvAvailable[ i ] = 0

–	If DvAvailable[ 0 ] + DvAvailable[ 1 ] + DvAvailable[ 2 ] is equal to 1, the following applies:

–	dv[ 0 ] = mvCand[ i ][ 0 ]

–	dv[ 1 ] = mvCand[ i ][ 1 ]

–	Otherwise, the following steps apply in order:

–	If dVColDepthFlag is equal to 1, the variable maxDepth is specified as follows:

maxDepth = INT_MIN 
for( j = 0; j < partHeight; j += ( partHeight – 1 ) )
	for( i = 0; i < partWidth; i += ( partWidth – 1 ) )
		if( depthPic[ dbx1 + i, dby1 + j ] > maxDepth )
			maxDepth = depthPic[ dbx1 + i, dby1 + j ]

–	If dVColDepthFlag is equal to 1, the variable dispVector is specified as follows:

currIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the InterViewPic )

dispVector[ 0 ] = Disparity( maxDepth, currIndex, refIndex )
dispVector[ 1 ] = 0

–	For each value of i equal to 0, 1, and 2, when DvAvailable[ i ] is equal to 0, mvCand[ i ] is set to (dVColDepthFlag ? dispVector : dvMB ).

–	Each component of the variable dv is derived as follows:

dv[ 0 ] = Median( mvCand[ 0 ][ 0 ], mvCand[ 1 ][ 0 ], mvCand[ 2 ][ 0 ] )

dv[ 1 ] = Median( mvCand[ 0 ][ 1 ], mvCand[ 1 ][ 1 ], mvCand[ 2 ][ 1 ] )

        - Derivation process for the inter-view motion vector in inter-view reference

Inputs to this process are mbPartIdx, subMbPartIdx, and listSuffixFlag.

Outputs of this process are the motion vector mvCorrespond and the reference index refIdxCorrespond.

Inter-view reference picture InterViewPic and an offset vector dv are derived as specified by the following ordered steps:

–	The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

–	When dVColDepthFlag is equal to 1, the inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

–	When dVColDepthFlag is equal to 1, the inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

–	The process specified in subclause 8.3.1.4 is invoked withlistSuffixFlag, and when dVColDepthFlag is equal to 1 depthPic set to DepthCurrPic, dbx1 set to x1 + dx1 + dx2, dby1 set to y1 + dy1 + dy2, as inputs and an offset vector dv, and when dvColDepthFlag is equal to 1, an variable InterViewAvailable as outputs.

The refIdxCorrespond and mvCorrespond are set as follows.

–	If InterViewAvailable is equal to 0, refIdxCorrespond is set to -1, and mvCorrespond [ 0 ] and mvCorrespond [ 1 ] are both set to 0.

–	Otherwise, the following step applies in order.

–	The variable luma4x4BlkIdx is derived as  (4 * mbPartIdx + subMbPartIdx).

–	The inverse 4x4 luma block scanning process as specified in subclause Error: Reference source not found is invoked with luma4x4BlkIdx as the input and ( x, y ) as the output. In addition, ( xCorrespond, yCorrespond ) is set equal to ( x + ( dv[ 0 ] >> 4 ), y + ( dv [ 1 ] >> 4 ) ) and mbAddrCorrespond is set equal to ( ( CurrMbAddr / PicWidthInMbs ) + ( dv[1] >> 6 ) ) * PicWidthInMbs + ( CurrMbAddr % PicWidthInMbs ) + ( dv[0] >> 6 ).

–	Set mbTypeCorrespond to the syntax element mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic. When mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, subMbTypeCorrespond is set to be the syntax element sub_mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic.

–	Set mbPartIdxCorrespond to the macroblock partition index of the corresponding partition and subMbPartIdxCorrespond to the sub-macroblock partition index of the corresponding sub-macroblock partition. The derivation process for macroblock and sub-macroblock partition indices as specified in subclause Error: Reference source not found is invoked with the luma location equal to ( xCorrespond, yCorrespond ), the macroblock type equal to mbTypeCorrespond, and when mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, the list of sub-macroblock types subMbTypeCorrespond as the inputs and the outputs are the macroblock partition index mbPartIdxCorrespond and the sub-macroblock partition index subMbPartIdxCorrespond.

–	The motion vector mvCorrespond and the reference index refIdxCorrespond are derived as follows.

–	If the macroblock mbAddrCorrespond is coded in as Intra prediction mode, both components of mvCorrespond are set equal to 0 and refIdxCorrespond is set equal to –1.

–	Otherwise (the macroblock mbAddrCorrespond is not coded as Intra prediction mode), the prediction utilization flags predFlagLXCorrespond is set equal to PredFlagLX[ mbPartIdxCorrespond ], the prediction utilization flag of the macroblock partition mbAddrCorrespond\mbPartIdxCorrespond of the picture InterViewPic. In addition, the following applies.

–	When predFlagLXCorrespond is equal to 1, the mvCorrespond and the reference index refIdxCorrespond are set equal to MvLX[ mbPartIdxCorrespond ][ subMbPartIdxCorrespond ] and RefIdxLX[ mbPartIdxCorrespond ], respectively, which are the motion vector mvLX and the reference index refIdxLX that have been assigned to the (sub-)macroblock partition mbAddrCorrespond\mbPartIdxCorrespond\subMbPartIdxCorrespond inside the picture InterViewPic.

        - Derivation process for luma motion vectors for VSP skipped/direct macroblocks in B slices

Outputs of this process are the motion vector mvL0, mvL1 and the reference index refIdxL0, refIdxL1.

The reference index refIdxLX for a VSP skipped/direct macroblock is derived as the synthetic reference component that appears first in the reference picture list X, with X being replaced by 0 or 1. When there is no synthetic picture in the reference picture list X, refIdxLX is set equal to 0.

The motion vector mvLX is set equal to dvMBwith X being replaced by 0 or 1.

        - Derivation process for luma motion vector prediction

Inputs to this process are:

–	the macroblock partition index mbPartIdx,

–	the sub-macroblock partition index subMbPartIdx,

–	the reference index of the current partition refIdxLX (with X being 0 or 1),

–	the variable currSubMbType.

Output of this process is the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The specifications of subclause 8.4.1.3 apply with the following changes.

–	The following additional sentence is applied

If refIdxLX is not equal to refIdxLXN for any N = A, B, or C and X equal to 0 or 1, the following applies:

mbAddrN\mbPartIdxN\subMbPartIdxN is marked as not available
refIdxLXN = -1
mvLXN 0 ] = 0
mvLXN[ 1 ] = 0

after the following paragraph in subclause 8.4.1.3:

The derivation process for the neighbouring blocks for motion data in subclause Error: Reference source not found is invoked with mbPartIdx, subMbPartIdx, currSubMbType, and listSuffixFlag = X (with X being 0 or 1 for refIdxLX being refIdxL0 or refIdxL1, respectively) as the input and with mbAddrN\mbPartIdxN\subMbPartIdxN, reference indices refIdxLXN and the motion vectors mvLXN with N being replaced by A, B, or C as the output.

–	The following additional sentence is applied

–	Otherwise, if refIdxLX is a reference index to an inter-view reference component or an inter-view only reference component, the depth-based derivation process for median luma motion vector prediction in subclause 8.3.1.8 is invoked with mbAddrN\mbPartIdxN\subMbPartIdxN, mvLXN, refIdxLXN with N being replaced by A, B, or C, and refIdxLX as the inputs and the output is assigned to the motion vector predictor mvpLX.

–	Otherwise, if refIdxLX is a reference index to a reference picture which is not an inter-view reference component or an inter-view only reference component, the depth-based derivation process for median luma temporal motion vector prediction in subclause 8.3.1.9 is invoked with mbAddrN\mbPartIdxN\subMbPartIdxN, mvLXN, refIdxLXN with N being replaced by A, B, or C, and refIdxLX as the inputs and the output is assigned to the motion vector predictor mvpLX.

after the following paragraph in subclause 8.4.1.3

–	Otherwise, if MbPartWidth( mb_type ) is equal to 8, MbPartHeight( mb_type ) is equal to 16, mbPartIdx is equal  mvpLX = mvLXCto 1, and refIdxLXC is equal to refIdxLX, the motion vector predictor mvpLX is derived by:

mvpLX = mvLXC

        - Depth-based derivation process for median luma motion vector prediction

[Ed. Consider renumbering J.8.3.1.8 as a subclause of J.8.3.1.7]

Inputs to this process are:

–	the neighbouring partitions mbAddrN\mbPartIdxN\subMbPartIdxN (with N being replaced by A, B, or C),

–	the motion vectors mvLXN (with N being replaced by A, B, or C) of the neighbouring partitions,

–	the reference indices refIdxLXN (with N being replaced by A, B, or C) of the neighbouring partitions,,

–	the reference index refIdxLX of the current partition.

Output of this process is the motion vector prediction mvpLX.

When either partition mbAddrN\mbPartIdxN\subMbPartIdxN is not available or refIdxLXN is not equal to refIdxLX, mvLXN is derived as specified by the following:

If dVColDepthFlag is equal to 1, the following steps apply in order:

1.	The inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

2.	The inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

3.	The inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

4.	The modification process of inter-view motion vector in median luma motion vector prediction as specified in subclause 8.3.1.8.1 is invoked with depthPic being equal to DepthRefPicList0[ refIdxL0 ], dbx1 being equal to x1 + dx1 + dx2, dby1 being equal to y1 + dy1 + dy2, and mv being equal to mvL0 as inputs and the output is assigned to the motion vector mvLXN.

Otherwise, mvLXN is set equal to (dvMB[ 0 ], 0).

Each component of the motion vector prediction mvpLX is given by the median of the corresponding vector components of the motion vector mvLXA, mvLXB, and mvLXC:

mvpLX[ 0 ] = Median( mvLXA[ 0 ], mvLXB[ 0 ], mvLXC[ 0 ] )                 (J8-XX)

mvpLX[ 1 ] = Median( mvLXA[ 1 ], mvLXB[ 1 ], mvLXC[ 1 ] )                 (J8-XX)

          - Modification process for inter view motion vector in median luma motion vector prediction

Inputs to this process are

–	depth reference view component depthPic,

–	the location of a top-left sample ( dbx1, dby1 ) of a partition,

–	a motion vector mv,

Outputs of this process are:

–	the motion vector mv.

Let refViewId be the view_id value of depthPic.

The following ordered steps apply:

- The variable maxDepth is specified as follows:

maxDepth = INT_MIN
for( j = 0; j < partHeight; j += ( partHeight – 1 ) )
	for( i = 0; i < partWidth; i += ( partWidth – 1 ) )		(J-8-XX)
		if( depthPic[ dbx1 + i, dby1 + j ] > maxDepth ) 
			maxDepth = depthPic[ dbx1 + i, dby1 + j ]

- The variable mv is specified as follows:

srcIndex = ViewIdTo3DVAcquisitionParamIndex( view_id )
refIndex = ViewIdTo3DVAcquisitionParamIndex( refViewId )

mv[ 0 ] = Disparity( maxDepth, srcIndex, refIndex) 
mv[ 1 ] = 0

        - Depth-based derivation process for median luma temporal motion vector prediction

[Ed. Consider renumbering J.8.3.1.9 as a subclause of J.8.3.1.7]

Inputs to this process are:

–	the neighbouring partitions mbAddrN\mbPartIdxN\subMbPartIdxN (with N being replaced by A, B, or C),

–	the motion vectors mvLXN (with N being replaced by A, B, or C) of the neighbouring partitions,

–	the reference indices refIdxLXN (with N being replaced by A, B, or C) of the neighbouring partitions,

–	the reference index refIdxLX of the current partition.

Output of this process is the motion vector prediction mvpLX.

When either partition mbAddrN\mbPartIdxN\subMbPartIdxN is not available or refIdxLXN is not equal to refIdxLX, mvLXN is derived as specified by the following ordered steps:

1.	When dVColDepthFlag is equal to 1, the inverse macroblock scanning process as specified in subclause 6.4.1is invoked with CurrMbAddr as the input and the output is assigned to ( x1, y1 ).

2.	When dVColDepthFlag is equal to 1, the inverse macroblock partition scanning process specified in subclause 6.4.2.1 is invoked with mbPartIdx as the input and the output assigned to ( dx1, dy1 ).

3.	When dVColDepthFlag is equal to 1, the inverse sub-macroblock partition scanning process specified in subclause 6.4.2.2 is invoked with mbPartIdx and subMbPartIdx as the input and the output assigned to ( dx2, dy2 ).

4.	When dVColDepthFlag is equal to 1, the process specified in subclause 8.3.1.9.1 is invoked with depthPic set to DepthCurrPic, dbx1 set to x1 + dx1 + dx2, dby1 set to y1 + dy1 + dy2 and listSuffixFlag as input and InterViewPic, an offset vector dv and an variable InterViewAvailable as outputs.

5.		When dVColDepthFlag is equal to0, dv[ 0 ] is set equal to dvMB[ 0 ], dv[ 1 ] is set equal to 0 and the variable InterViewAvailable equal to IntViewAvaMB.

6.	The refIdxCorrespond and mvCorrespond are set as follows.

–	If InterViewAvailable is equal to 0, refIdxCorrespond is set to -1, and mvCorrespond[ 0 ] and mvCorrespond[ 1 ] are both set to 0.

–	Otherwise, the following steps apply in order.

–	The variable luma4x4BlkIdx is set equal to ( 4 * mbPartIdx + subMbPartIdx ).

–	The inverse 4x4 luma block scanning process as specified in subclause Error: Reference source not found is invoked with luma4x4BlkIdx as the input and ( x, y ) as the output. In addition, ( xCorrespond, yCorrespond ) is set equal to ( x + ( dv[ 0 ] >> 4 ), y + ( dv[ 1 ] >> 4 ) ) and mbAddrCorrespond is set equal to ( ( CurrMbAddr / PicWidthInMbs ) + ( dv[ 1 ] >> 6 ) ) * PicWidthInMbs + ( CurrMbAddr % PicWidthInMbs ) + ( dv[ 0 ] >> 6 ).

–	Set mbTypeCorrespond to the syntax element mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic. When mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, subMbTypeCorrespond is set to be the syntax element sub_mb_type of the macroblock with address mbAddrCorrespond inside the picture InterViewPic.

–	Set mbPartIdxCorrespond to the macroblock partition index of the corresponding partition and subMbPartIdxCorrespond to the sub-macroblock partition index of the corresponding sub-macroblock partition. The derivation process for macroblock and sub-macroblock partition indices as specified in subclause Error: Reference source not found is invoked with the luma location equal to ( xCorrespond, yCorrespond ), the macroblock type equal to mbTypeCorrespond, and when mbTypeCorrespond is equal to P_8x8, P_8x8ref0, or B_8x8, the list of sub-macroblock types subMbTypeCorrespond as the inputs and the outputs are the macroblock partition index mbPartIdxCorrespond and the sub-macroblock partition index subMbPartIdxCorrespond.

–	The motion vector mvCorrespond and the reference index refIdxCorrespond are derived as follows.

–	If the macroblock mbAddrCorrespond is coded in as Intra prediction mode, both components of mvCorrespond are set equal to 0 and refIdxCorrespond is set equal to –1.

–	Otherwise (the macroblock mbAddrCorrespond is not coded as Intra prediction mode), the prediction utilization flags predFlagLXCorrespond is set equal to PredFlagLX[ mbPartIdxCorrespond ], the prediction utilization flag of the macroblock partition mbAddrCorrespond\mbPartIdxCorrespond of the picture InterViewPic. In addition, the following applies.

–	When predFlagLXCorrespond is equal to 1, the mvCorrespond and the reference index refIdxCorrespond are set equal to MvLX[ mbPartIdxCorrespond ][ subMbPartIdxCorrespond ] and RefIdxLX[ mbPartIdxCorrespond ], respectively, which are the motion vector mvLX and the reference index refIdxLX that have been assigned to the (sub-)macroblock partition mbAddrCorrespond\mbPartIdxCorrespond\subMbPartIdxCorrespond inside the picture InterViewPic.

6.	The motion vectors mvLXN is derived as follows.

–	If refIdxCorrespond is equal to refIdxLX, the following applies:

mvLXN[ 0 ] = mvCorrespond[ 0 ]
mvLXN[ 1 ] = mvCorrespond[ 1 ]

–	Otherwise, the following applies:

mvLXN[ 0 ] = 0
mvLXN[ 1 ] = 0

7.	The following applies for the derivation of mvpLX[ 0 ] and mvpLX[ 1 ]:

mvpLX[  ] = Median( mvLXA[ 0 ], mvLXB[ 0 ], mvLXC[ 0 ] )                 (J-8-XX)
mvpLX 1 ] = Median( mvLXA[ 1 ], mvLXB[ 1 ], mvLXC[ 1 ] )                 (J-8-XX)

          - Derivation process for the disparity vector and the inter-view reference

Inputs to this process are depth reference view component depthPic, the location of a top-left sample ( dbx1, dby1 ) of a partition and the listSuffixFlag.

Outputs of this process are a picture InterViewPic, an offset vector dv and a variable InterViewAvailable

The variable InterViewAvailable is set equal to 0.

The following applies to derive an inter-view reference picture or inter-view only reference picture, InterViewPic, with X set to 1 when listFuffixFlag is 1 or 0 otherwise:

for( cIdx = 0;cIdx<num_ref_idx_l0_active_minus1 + 1 && !InterViewAvailable; cIdx ++)
	if ( view order index of RefPicList0[ cIdx ] is equal to 0) {
		InterViewPic = RefPicList0[ cIdx ]
		InterViewAvailable = 1
	}

When InterViewAvailable is equal to 1, the following steps apply in order.

- The variable maxDepth is specified as follows:

maxDepth = INT_MIN
for( j = 0; j < partHeight; j+=(partHeight-1) )
	for( i = 0; i < partWidth; i+=(partWidth-1))
		if( depthPic[ dbx1 + i, dby1 + j ] > maxDepth ) maxDepth = depthPic[ dbx1 + i, dby1 + j ]

- The variable dv is specified as follows:

currIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the InterViewPic )

dv[ 0 ] = Disparity(maxDepth, currIndex, refIndex )
dv[ 1 ] = 0

        - Macroblock-level neighbouring block based disparity vector derivation process

Input to this process is a macroblock currMB.

Let the variable availableDvFlag equal to 0, ( xP, yP ) be equal to the output of the subclause 6.4.2.1 (the location of upper-left luma sample for currMB partition 0).

- The variable dvMBCur is derived as specified by the following ordered steps:

- For X from 0 to 1, the following ordered steps apply:

      - When availableDvFlag is equal to 0 and RefPicListY[ 0 ] is available (with Y equal to 1-X) and , for each ( xN, yN ) being ( xP + 16,  yP + 16 ), ( xP + 8,  yP + 8 ), and ( xP ,  yP + 12 ) in RefPicListY[ 0 ], the following applies:

        - Set RefPicListCol0 is the reference picture list 0 of RefPicListY[ 0 ].

        - The reference index and the motion vector corresponding to RefPicListCol0 of the block covering ( xN, yN ) is set to MvL0N and RefIdxL0N respectively.

        - When the view order index of RefPicListCol0[ RefIdxL0N ] is unequal to view_idx or RefPicListCol0[ RefIdxL0N ] is a view synthetic reference component, and MvL0N[ 0 ] is unequal to 0, dvMBCur is set to MvL0N and availableDvFlag is set to 1.

- For each N being A, B, C and D, as specified in subclause Error: Reference source not found with the refIdxL0N and mvL0N being the reference index and motion vector corrponding to reference picture list 0, the following applies:

      - When availableDvFlag is equal to 0, the following applies

        - RefPicList0[ refIdxL0N ] is available and its the view order index is unequal to the view_idx or RefPicList0[ refIdxL0N ] is the view synthetic reference component, and MvL0N[ 0 ] is unequal to 0, dvMBCur is set to mvL0N and availableDvFlag is set to 1.

- When availableDvFlag is equal to 0, dvMBCur is set to dvMB.

- dvMBCur is further refined by the following steps if seq_view_synthesis_flag is equal to 1:

  - Let refDepthPic being the decoded sample array of the depth view component of the view component pair for which the texture view component is the one of base view, PicWidthInSamplesL and PicHeightInSamplesL be the picture width and height for the luma component of refDepthPic.

  - The variables currIndex and refIndex are derived by:

currIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of refDepthPic)

  - The variables dX, dY, and maxDepth are derived by:

maxDepth = INT_MIN
for( j = 0; j < 16; j+=15 )
	for( i = 0; i < 16; i+=15)
	{

dX = Clip3(0, PicWidthInSamplesL, (xP+i)+ (dvMBCur[ 0 ] >> 2)) >> reduced_resolution_flag)

dY = Clip3(0, PicHeightInSamplesL, (yP+j)+( dvMBCur[ 1 ] >> 2)) >> reduced_resolution_flag)
		if( refDepthPic[ dX, dY ] > maxDepth ) 
			maxDepth = refDepthPic[ dX, dY ]

}

  - dvMBCur[ 0 ] is set to Disparity( maxDepth, currIndex, refIndex ).

- dvMB is set to dvMBCur.

      - Decoding process for Inter or view synthesis prediction samples

Inputs to this process are:

–	a macroblock partition mbPartIdx,

–	a sub-macroblock partition subMbPartIdx,

–	variables specifying partition width and height for luma and chroma (if available), partWidth, partHeight, partWidthC (if available) and partHeightC (if available),

–	luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0 chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1,

–	variables for weighted prediction logWDC, w0C, w1C, o0C, o1C with C being replaced by L and, when ChromaArrayType is not equal to 0, Cb and Cr.

Outputs of this process are the prediction samples predPart, which are a (partWidth)x(partHeight) array predPartL of prediction luma samples, and when ChromaArrayType is not equal to 0 two (partWidthC)x(partHeightC) arrays predPartCb, predPartCr of prediction chroma samples, one for each of the chroma components Cb and Cr.

Let predPartL0L and predPartL1L be (partWidth)x(partHeight) arrays of predicted luma sample values and when ChromaArrayType is not equal to 0 predPartL0Cb, predPartL1Cb, predPartL0Cr, and predPartL1Cr be (partWidthC)x(partHeightC) arrays of predicted chroma sample values.

For LX being replaced by either L0 or L1 in the variables predFlagLX, RefPicListX, refIdxLX, refPicLX, predPartLX, the following is specified.

When predFlagLX is equal to 1, the following applies:

–	If refIdxLX indicates a synthetic reference component, the array predPartLXL and when ChromaArrayType is not equal to 0 the arrays predPartLXCb and predPartLXCr are derived by invoking the process specified in subclause 8.3.2.1 with the current partition specified by mbPartIdx\subMbPartIdx, the motion vectors mvLX, mvCLX (if available), the reference arrays with refPicLXL, refPicLXCb (if available), and refPicLXCr (if available), and currDepthPic being the decoded sample array of the depth view component of the view component pair for which the texture view component is the current texture view component, when dVColDepthFlag is equal to 1 or when decoded sample array of the depth view component of the view component pair for which the texture view component is the texture view component of the reference view given as input.

–	Otherwise, the following applies:

–	The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and when ChromaArrayType is not equal to 0 two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in subclause 8.4.2.1 with refIdxLX and RefPicListX given as input.

–	The array predPartLXL and when ChromaArrayType is not equal to 0 the arrays predPartLXCb and predPartLXCr are derived by invoking the process specified in subclause 8.4.2.2 with the current partition specified by mbPartIdx\subMbPartIdx, the motion vectors mvLX, mvCLX (if available), and the reference arrays with refPicLXL, refPicLXCb (if available), and refPicLXCr (if available) given as input.

For C being replaced by L, Cb (if available), or Cr (if available), the array predPartC of the prediction samples of component C is derived by invoking the process specified in subclause 8.4.2.3 with the current partition specified by mbPartIdx and subMbPartIdx, the prediction utilization flags predFlagL0 and predFlagL1, the arrays predPartL0C and predPartL1C, and the variables for weighted prediction logWDC, w0C, w1C, o0C, o1C given as input.

        - Decoding process for view synthesis prediction samples

Inputs of this process are:

–	the current partition of the current texture view component given by its partition index mbPartIdx and its sub-macroblock partition index subMbPartIdx

–	the width and height partWidth, partHeight of this partition in luma-sample units,

–	a luma motion vector mvLX given in quarter-luma-sample units,

–	when ChromaArrayType is not equal to 0, a chroma motion vector mvCLX with a precision of one-(4*SubWidthC)-th chroma-sample units horizontally and one-(4*SubHeightC)-th chroma-sample units vertically,

–	the selected reference picture sample arrays refPicLXL, and when ChromaArrayType is not equal to 0, refPicLXCb, and refPicLXCr, and

–	a decoded depth view component depthPic.

Outputs of this process are:

–	a (partWidth)x(partHeight) array predPartLXL of prediction luma sample values,

–	when ChromaArrayType is not equal to 0, two (partWidthC)x(partHeightC) arrays predPartLXCb, and predPartLXCr of prediction chroma sample values.

Let ( xAL, yAL ) be the location given in full-sample units of the upper-left luma sample of the current partition given by mbPartIdx\subMbPartIdx relative to the upper-left luma sample location of the given two-dimensional array of luma samples.

Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in quarter-sample units. These variables are used only inside this subclause for specifying general fractional-sample locations inside the reference sample arrays refPicLXL, refPicLXCb (if available), and refPicLXCr (if available), PicWidthInSamplesL and PicHeightInSamplesL be the picture width and height for the luma component of depthPic.

The variables currIndex and refIndex are derived by:

currIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view )
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of refPicLXL )

For each luma sample location (0 <= xL < partWidth, 0 <= yL < partHeight) inside the prediction luma sample array predPartLXL, the corresponding prediction luma sample value predPartLXL[ xL, yL ] is derived as specified by the following ordered steps:

- The variables dX, dY, and maxDepth are derived by: [Ed. (MH): Fix style/indents in the following.]

vspBlockSize = 4 >> reduced_resolution_flag
dX = xAL >> reduced_resolution_flag
dY = yAL >> reduced_resolution_flag
shiftX = ( (xL >> 2) << 2 ) >> reduced_resolution_flag
shiftY = ( (yL >> 2) << 2 ) >> reduced_resolution_flag
maxDepth = INT_MIN
for( j = 0; j < vspBlockSize; j+=( vspBlockSize – 1 ) )
	for( i = 0; i < vspBlockSize; i+=( vspBlockSize – 1 ) )
	{
		if (!dVColDepthFlag) {

dXC = Clip3(0, PicWidthInSamplesL, (xAL+i+((xL/vspBlockSize) * vspBlockSize)+ (mvL0[0] >> 2)) >> reduced_resolution_flag) 
			dYC = Clip3(0, PicHeightInSamplesL, (yAL+j+((yL/vspBlockSize) * vspBlockSize)+(mvL0[1] >> 2)) >> reduced_resolution_flag)
		}
		else {
			dXC = dX + shiftX + i
			dYC = dY + shiftY + j
		}

		if( depthPic[dXC, dYC] > maxDepth ) 
			maxDepth = depthPic[ dXC, dYC]
		}

- The variable disparityValue is derived by:

disparityValue = Disparity( maxDepth, currIndex, refIndex )

If dVColDepthFlag is equal to 0, the motion vector of the 8x8 block covering the position ( xAL+((xL>>3)<<3), yAL+((yL>>3)<<3) ) in reference picture list 0 is set equal to (disparityValue, 0).

- Variables xIntL, yIntL, xFracL, and yFracL are derived by:

xIntL = xAL + xL + ( disparityValue >> 2 )
yIntL = yAL + yL

xFracL = disparityValue & 3
yFracL = 0

- The prediction luma sample value predPartLXL[ xL, yL ] is derived by invoking the process specified in subclause 8.4.2.2.1 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLXL given as input.

When ChromaArrayType is not equal to 0, the following applies.

Let ( xAC, yAC ) be the location given in full-sample units of the upper-left chroma sample of the current partition given by mbPartIdx\subMbPartIdx relative to the upper-left chroma sample location of the given two-dimensional array of chroma samples.

Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in one-(4*SubWidthC)-th chroma-sample units horizontally and one-(4*SubHeightC)-th chroma-sample units vertically. These variables are used only inside this subclause for specifying general fractional-sample locations inside the reference sample arrays refPicLXCb, and refPicLXCr.

For each chroma sample location (0 <= xC < partWidthC, 0 <= yC < partHeightC) inside the prediction chroma sample arrays predPartLXCb and predPartLXCr, the corresponding prediction chroma sample values predPartLXCb[ xC, yC ] and predPartLXCr[ xC, yC ] are derived as specified by the following ordered steps:

- The variables vspBlockSize, dX, dY, shiftX, shiftY, and maxDepth are derived by: [Ed. (MH): Fix style/indents in the following.]

dX = xAL >> reduced_resolution_flag
dY = yAL >> reduced_resolution_flag
if( ChromaArrayType  = =  1  | |  ChromaArrayType  = =  2 ) {
	vspBlockSize = ( 4 >> 1) >> reduced_resolution_flag
	shiftX = ( ( xC >> 1 ) << 1 ) >> reduced_resolution_flag
	shiftY = ( ( yC >> 2 ) << 2 ) >> reduced_resolution_flag
} else {
	vspBlockSize = 4 >> reduced_resolution_flag
	shiftX = ( ( xC >> 2 ) << 2 ) >> reduced_resolution_flag
	shiftY = ( ( yC >> 2 ) << 2 ) >> reduced_resolution_flag
}
maxDepth = INT_MIN
for( j = 0; j < vspBlockSize; j += ( vspBlockSize -1) )
	for( i = 0; i < vspBlockSize; i += ( vspBlockSize -1 ) )
	{
		if(!dVColDepthFlag) {
			dXC = Clip3(0, PicWidthInSamplesC, (xAC+i+((xC/vspBlockSize)*vspBlockSize)+ (mvL0[0] >> 3)) >> reduced_resolution_flag) 
			dYC = Clip3(0, PicHeightInSamplesC, (yAC+j+((yC/vspBlockSize)*vspBlockSize)+(mvL0[1] >> 3)) >> reduced_resolution_flag)
		}else{
			dXC = dX + shiftX + i
			dYC = dY + shiftY + j
		}
		if( depthPic[dXC, dXY] > maxDepth ) 
			maxDepth = depthPic[dXC, dYC]

}

- The variable disparityValue is derived by:

disparityValue = Disparity( maxDepth, currIndex, refIndex )

- The variable yFracC is derived by:

yFracC = 0

- Depending on ChromaArrayType, the variables xIntC, yIntC, and xFracC are derived as follows:

–	If ChromaArrayType is equal to 1,

xIntC = ( xAL / SubWidthC ) + ( disparityValue >> 3 ) + xC
yIntC = ( yAL / SubHeightC ) + yC
xFracC = ( disparityValue & 7 )

–	Otherwise, if ChromaArrayType is equal to 2,

xIntC = ( xAL / SubWidthC ) + ( disparityValue >> 3 ) + xC
yIntC = ( yAL / SubHeightC ) + yC
xFracC = ( disparityValue & 7 )

–	Otherwise (ChromaArrayType is equal to 3),

xIntC = xAL + ( disparityValue >> 2 ) + xC
yIntC = yAL + yC
xFracC = ( disparityValue & 3 )

- Depending on ChromaArrayType, the following applies:

–	If ChromaArrayType is not equal to 3, the following applies:

–	The prediction sample value predPartLXCb[ xC, yC ] is derived by invoking the process specified in subclause 8.4.2.2.2 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCb given as input.

–	The prediction sample value predPartLXCr[ xC, yC ] is derived by invoking the process specified in subclause 8.4.2.2.2 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCr given as input.

–	Otherwise (ChromaArrayType is equal to 3), the following applies:

–	The prediction sample value predPartLXCb[ xC, yC ] is derived by invoking the process specified in subclause 8.4.2.2.1 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCb given as input.

–	The prediction sample value predPartLXCr[ xC, yC ] is derived by invoking the process specified in subclause 8.4.2.2.1 with ( xIntC, yIntC ), ( xFracC, yFracC ) and refPicLXCr given as input.

      - Derivation process for inside view motion prediction

Inputs to this process are:

–	macroblock types, luma motion vectors and reference indices of one/four co-located macroblocks in the texture view component of the current macroblock with address CurrMbAddr in the depth view component.

Outputs of this process are variables for the current macroblock in the depth view component:

–	macroblock type mb_type and sub-macroblock type sub_mb_type[ mbPartIdx ] for each sub-macroblock (with mbPartIdx being the macroblock partition index for the corresponding sub-macroblock)

–	motion vectors, reference indices refIdxLX (X being 0 and 1) and prediction modes

The co-located macroblocks are identified by 8.3.3.1.

When reduced_resolution_flag is equal to 0, the macroblock type, reference indices, prediction modes and luma motion vectors are derived by 8.3.3.3; when reduced_resolution_flag is equal to 1, the macroblock type, reference indices, prediction modes and luma motion vectors are derived by 8.3.3.4.

        - Deriviation process for inside view co-located macroblocks

Input to this process is the address of one macroblock in the depth view component.

Output to this process is the address of one/four co-located macroblocks of the texture view component (depending on reduced_resolution_flag is 0/1) in the same access unit.

When reduced_resolution_flag is equal to 0, the address of the co-located macroblock in the texture view componenent is set to CurrMbAddr.

When reduced_resolution_flag is equal to 1, the following applies.

–	For a macoblock with address CurrMbAddr in the depth view component, the addresses ColMbAddr[ mbPartIdx ] (mbPartIdx  = 0..3) of its four co-located macroblocks in the texture view component are defined as follows:

mb_x = ( CurrMbAddr % PicWidthInMbs ) << 1

mb_y = ( CurrMbAddr / PicWidthInMbs ) <<1		(J-8-XX)

ColMbAddr[ mbPartIdx ] = ( mb_y + mbPartIdx / 2 ) * ( PicWidthInMbs << 1 ) + ( mb_x + mbPartIdx % 2 )

        - Motion vectors scaling process

Input to this process is a motion vector.

Output to this process is the modified motion vector after scaling.

If reduced_resolution_flag is equal to 1, this motion vector scaling process is invoked.

Denote a motion vector by MV[ 2 ] with MV[ 0 ] and MV[ 1 ] represent the horizontal value and the vertical value, respectively. The scaling process is specified by the following formulas:

MV[ 0 ] = (MV[ 0 ] + 1) >> 1;

MV[ 1 ] = (MV[ 1 ] + 1) >> 1.		(J-8-XX)

        - Derivation process for motion information from one inside view co-located macroblock

This process is invoked when reduced_resolution_flag is equal to 0.

The reference indices, luma motion vectors of the co-located macroblock denoted by ColMb are assigned to the current macroblock.

Denote the reference indices of each reference picture list X of one 8x8 partition mbPartIdx by refIdxLXCol[ mbPartIdx ] (X being 0..1).

–	If macroblock type of ColMb specifies an intra coded type or view synthesis prediction type [Ed. (MH): it suggested use variables in this condition rather than plain English], the following applies:

–	Motion vectors [Ed. (MH): which motion vectors?] are set equal to zero.

–	refIdxL0[ mbPartIdx ] is set equal to 0 and refIdxL1[ mbPartIdx ] is set equal to -1.

–	PredMode[ mbPartIdx ] is set equal to Pred_L0.

–	The following applies:

–	If the current slice is a B slice, mb_type[ mbPartIdx ] is set equal to B_L0_8x8. [Ed. (MH): phrasing “is inferred to be equal to” is probably more appropriate to assign a value to a syntax element. Such a change should be done throughout the text.]

–	Otherwise, mb_type[ mbPartIdx ] is set equal to P_L0_8x8.

–	Otherwise, if both of the following conditions are false, macroblock type and sub-macroblock types of ColMb are inherited by current macroblock.

–	Macroblock type of ColMb is B_8x8 and at least one of the sub-macroblock types is B_Direct_8x8,

–	Macroblock type of ColMb is P_Skip, B_skip or B_Direct_16x16.

–	Otherwise (at least one of the above conditions is true)

–	If the current slice is a P slice, mb_type is set to P_8x8 and sub_mb_type[ mbPartIdx ] is set to P_L0_8x8

–	Otherwise (the current slice is a B slice), mb_type is set to B_8x8, and

–	If both refIdxLXCol[ mbPartIdx ] (X being 0..1) are not equal to -1, sub_mb_type[ mbPartIdx ] is set to B_Bi_8x8;

–	Otherwise, sub_mb_type[ mbPartIdx ] is set to B_LX_8x8, when refIdxLXCol[ mbPartIdx ] is not equal to -1.

        - Derivation process for motion information from inside view co-located macroblocks

This process is invoked when reduced_resolution_flag is equal to 1.

mb_type is set to P_8x8 if the current slice is a P slice or B_8x8 if the current slice is a B slice.

Denote the macroblock with address ColMbAddr[ mbPartIdx ] by ColMb and its macroblock type by mbtypeCol.

–	If mbtypeCol is equal to P_Skip, P_L0_16x16, B_LX_16x16, B_Bi_16x16 or B_Skip or B_Direct_16x16 for B slices (X being 0 or 1), the following ordered steps are specified:

- Reference indices are inherited by the 8x8 partition mbPartIdx. Luma motion vectors of ColMb are inherited by the 8x8 partition mbPartIdx and scaled by the subclause 8.3.3.2.

- SubPredMode[ mbPartIdx ] is derived as follows:

–	If both of list 0 and list 1 reference indices of ColMb are not equal to -1, BiPred is assigned to SubPredMode[ mbPartIdx ].

–	Otherwise (list X reference index of ColMb is not equal to -1), Pred_LX is assigned to SubPredMode[ mbPartIdx ].

- sub_mb_type[ mbPartIdx ] is derived as follows:

–	If the prediction mode is Pred_LX, sub_mb_type[mbPartIdx] is set to P_L0_8x8 for P slices and B_LX_8x8 for B slices, respectively.

–	Otherwise, sub_mb_type[ mbPartIdx ] is set to B_Bi_8x8 for B slices.

–	Otherwise, if mbTypeCol is equal to P_8x8, P_8x8ref0, B_8x8 for B slices or P_L0_L0_MxN, P_L0_L0_MxN, B_X0_X1_MxN for B slices (with X0, X1 being replaced by L0, L1, or Bi, and MxN being replaced by 16x8 or 8x16), let refIdxLXCol[ i ] (i being 0..3) denote the reference index of four 8x8 partitions of ColMb. The index i of the top-left and bottom-right 8x8 partition is 0 and 3, respectively, as depicted in Figure 6-11. The following ordered steps are specified:

- Reference indices, luma motion vectors of the 8x8 partition with index mbPartIdx are derived as follows:

–	If both of refIdxLXCol[ 0 ] and refIdxLXCol[ 3 ] are -1, refIdxLX[ mbPartIdx ] is set to -1.

–	Otherwise, if refIdxLXCol[ 0 ] is equal to refIdxLXCol[ 3 ], refIdxLXCol[ 0 ] is assigned to refIdxLX[mbPartIdx]. The sub-macroblock partition size for 8x8 partition mbPartIdx is set to 8x4, 4x8 or 4x8 correspoding to the partition size of ColMb equal to 16x8, 8x16 or 8x8, respectively. The luma motion vectors of the two 8x8 partitions are inherited by the two 8x4 or 4x8 partitions of the 8x8 partiotion mbPartIdx and scaled by the subclause 8.3.3.2.

–	Otherwise, if refIdxLXCol[ i ] (i being 0 or 3) is unequal to -1 and refIdxLXCol[ j ] (j being 3 or 0) is equal to -1, refIdxLXCol[ i ] is assigned to refIdxLX[mbPartIdx] and the luma motion vectors of 8x8 partition with index i in ColMb are inherited by the 8x8 partiotion mbPartIdx and scaled by the subclause 8.3.3.2.

–	Otherwise, refIdxLXCol[ i ] is assigned to refIdxLX[ mbPartIdx ] with i equal to mbPartIdx and the luma motion vectors of the 8x8 partition with index i in ColMb are inherited by the 8x8 partition mbPartIdx and scaled by the subclause 8.3.3.2.

- Sub-macroblock partition size MxN is derived as follows:

–	If the sub-macroblock partition size is set to 8x4 or 4x8 in the above step, MxN is set to 8x4 or 4x8.

–	Otherwise, MxN is set to 8x8.

- SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ] of the 8x8 partition mbPartIdx are derived as follows:

–	If current slice is a P slice, Pred_L0 and P_L0_MxN are assigned to SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ], respectively.

–	Otherwise, if refIdxLX[ mbPartIdx ] (X being 0 or 1) is equal to -1 and refIdxLY[ mbPartIdx ] (Y being 1 or 0) is unequal to -1, Pred_LY and B_LY_MxN are assigned to SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ], respectively.

–	Otherwise, BiPred and B_Bi_MxN are assigned to SubPredMode[ mbPartIdx ] and sub_mb_type[ mbPartIdx ], respectively.

–	Otherwise, if mbTypeCol specifies an intra coded type or view synthesis prediction type [Ed. (MH): it suggested use variables in this condition rather than plain English], the following applies:

–	Motion vectors [Ed. (MH): which motion vectors?] are set equal to zero.

–	refIdxL0[ mbPartIdx ] is set equal to 0 and refIdxL1[ mbPartIdx ] is set equal to -1.

–	SubPredMode[ mbPartIdx ] is set equal to Pred_L0.

–	The following applies:

–	If the current slice is a B slice, sub_mb_type[ mbPartIdx ] is set equal to B_L0_8x8.

–	Otherwise, sub_mb_type[ mbPartIdx ] is set equal to P_L0_8x8.

–	[Ed. (MH): This chain of “If; Otherwise, if” does not follow the conventions, as there is no otherwise-statement without a condition.]

      - Derivation of prediction weights in depth-range-based weighted prediction

For (nal_unit_type is equal to 21 and depth_weighted_pred_flag is equal to 1 and (slice_type % 5) is equal to 0 or 3) or (depth_weighted_bipred_flag is equal to 1 and (slice_type % 5) is equal to 1), the derivation process for prediction weights in depth-range-based weighted prediction described in this subclause is invoked.

Inputs to this process are

– the reference indices refIdxL0 and refIdxL1,

– the prediction utilization flags predFlagL0 and predFlagL1.

Outputs of this process are variables for weighted prediction logWDC, w0C, o0C, w1C, o1C, where C being replaced by L (luma).

When predFlagL0 is equal to 1, the following is applied.

–	The derivation process for a single prediction weight in depth-range-based weighted prediction specified in subclause J.8.3.4.1 is invoked with zN1 equal to ZNear[ dps_id1, index ], zF1 equal to ZFar[ dps_id1, index ], zN2 equal to ZNear[ dps_id2, refIndex ], zF2 equal to ZFar[ dps_id2, refIndex ], and X equal to 0.

When predFlagL1 is equal to 1 and depth_weighted_bipred_flag is equal to 1, the following is applied.

–	The derivation process for a single prediction weight in depth-range-based weighted prediction specified in sub-clause J.8.3.4.1 is invoked with zN1 equal to ZNear[ dps_id1, index], zF1 equal to ZFar[ dps_id1, index  ], zN2 equal to ZNear[ dps_id2, refIndex ], zF2 equal to ZFar[ dps_id2, refIndex ], and X equal to 1.

The variables index, refIndex, dps_id1 and dps_id2 are derived as following:

index = ViewIdTo3DVAcquisitionParamIndex( view_id of the current view );
refIndex = ViewIdTo3DVAcquisitionParamIndex( view_id of the view of the reference picture);
dps_id1 is set equal to the dps_id of the current picture;

dps_id2 is set equal to the dps_id of the reference picture;

        - Derivation of weight and offset parameteres

–	The variable wXC is calculated through the following steps.

- scale_w is set to 8;

- Calculate variable W_factora as follows:

temp1 = ZF2 - ZN2; 
temp2 = ZF2;
x = ( temp1 + ( temp2 >> 1 ) ) / temp2;
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;
W_factora = (x<<scale_w);
W_factora += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2;

- Calculate variable W_factorb as follows:

temp1 = ZF1;
temp2 = ZF1 – ZN1;
x = ( temp1 + ( temp2 >> 1 ) ) / temp2;
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;
W_factorb = (x<<scale_w);
W_factorb += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2;

- Calculate variable W_factorc as follows:

temp1 = ZN1;
temp2 = ZN2;
x = ( temp1 + ( temp2 >> 1 ) ) / temp2;
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;
W_factorc = (x<<scale_w);
W_factorc += ( ((temp1 - x * temp2) << scale_w) + sign * (temp2 >> 1) ) / temp2;

- Calculate variable wX:

wX= (W_factora*W_factorb*W_factorc + (1<<(scale_w*3- logWD-1)) ) >> (scale_w*3- logWD);
wX=Clip(-127,128,wX);

–	The variable oX is calculated through the following derivation process.

- scale_o is set to 8

- Calculate variable O_factora as follows:

O_factora=( (ZN1<<(scale_o))+( ZF2>>1) ) / ZF2;

- Calculate variable O_factorb:

temp1 = ZF1- ZF2;
temp2 = ZF1- ZN1;
sign= (temp1 < 0) ? -1 : 1;
x = ( temp1 + sign * ( temp2 >> 1 ) ) / temp2;
sign = ( (temp1 - x * temp2) < 0 ) ? -1 : 1;
O_factorb = (x<<scale_o);
O_factorb += ( ((temp1 - x * temp2) << scale_o) + sign * (temp2 >> 1) ) / temp2;

- Calculate variable o0:

oX=( O_factora*O_factorb + (1<<(scale_o*2-8-1)) ) >> (scale_o*2-8);
oX =Clip(-127,128, oX);

      - Derivation process for motion vector components and reference indices for inter prediction for adaptive luminance compensation

Inputs to this process are:

–	a macroblock partition mbPartIdx,

–	a sub-macroblock partition subMbPartIdx.

Outputs of this process are:

–	luma motion vectors mvL0 and mvL1 and when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1,

–	a motion vector count variable subMvCnt.

The motion vector count variable subMvCnt is set equal to 1.

Set InterViewAvailable equal to 0.

The reference index refIdxL0 for a skipped macroblock is derived as:

for( cIdx = 0;cIdx<=num_ref_idx_l0_active_minus1 && !InterViewAvailable; cIdx ++)
	if (RefPicList0[ cIdx ] and the current view component have different values of view oder index) {
		refIdxL0 = cIdx
		InterViewAvailable = 1
	}

If dmvp_flag is equal to 1 and mb_alc_skip_flag is equal to 1, subclause J.8.3.1.7 is invoked and mvL0 is set equal to mvpL0, the output of subclause J.8.3.1.7.

Otherwise, if dmvp_flag is equal to 1 and mb_alc_ flag is equal to 1, subclause J.8.3.1 is invoked for derivation of mvL0.

Otherwise, subclause 8.4.1 is invoked and mvL0 is set equal to mvpL0, the output of subclause 8.4.1.

      - Derivation process for prediction weights in adaptive luminance compensation

Inputs to this process are:

–	reference index refIdxL0

–	the luma sample array of the selected reference picture refPicL0L.

–	the current partition given by its partition index mbPartIdx and its sub-macroblock partition index subMbPartIdx

–	Luma4x4BlkIdx

–	the width and height partWidth, partHeight of this partition in luma-sample units

–	a luma motion vector mvL0 given in quarter-luma-sample units

–	array cSL containing already constructed luma samples prior to deblocking filter process.

Outputs of this process are:

–	variables for weighted prediction of the current partition logWDC, W0C, W1C,O0C, O1C, with C being replaced by L and, when ChromaArrayType is not equal to 0, Cb and Cr. [Ed. Some subscripts are incorrectly used or missing in this subclause and its siblings. For example W0C should be W0C.]

The variables W1C , O1C are derived as follows for C is equal to L, Cb or Cr:

W1C = 0

O1C = 0

The variables W0C , O0C are derived as follows for C if equal to Cb or Cr:

logWDC = 15

W0C = 1

O0C=0.

When C is equal to L for luma samples, subclauses J.8.3.6.1 through J.8.3.6.5 are invoked sequentially to derive LogWDL, W0C, and O0C.

        - Defining of coordinates and sizes of a luma block to be predicted

Let ( xM, yM ) be equal to the output of the subclause 6.4.1 (the location of upper-left luma sample for the current macroblock with address mbAddr relative to the upper-left sample of the picture).

Let ( xP, yP ) be equal to the output of the subclause 6.4.2.1 (the location of upper-left luma sample for the macroblock partition mbPartIdx).

Let ( xB, yB ) be equal to the output of the subclause 6.4.2.2 (the location of upper-left luma sample for the 4x4 luma block defined by Luma4x4BlkIdx that can be 0...15) relative to the top-left sample of the sub-macroblock.

The vairables xT, yT, xBlockWidth, yBlockHeight are set as follows:

–	xT is set equal to xM + xP;

–	yT is set equal to yM +yP;

–	xBlockWidth is set equal to MbPartWidth( mb_type );

–	yBlockHeight is set equal to MbPartHeight( mb_type );

–	When the mb_type is equal to P_8x8 or P_8x8ref0 and sub_my_type is not P_L0_8x8, the following applies

–	xT += xB;

–	yT += yB;

–	xBlockWidth and yBlockHeight are both set equal to 4.

If one or more of the following conditions are true, W0C is set to 1 and logWDC is set to 15.

–	xT is smaller than (xL +1) for xL = 3;

–	yT is smaller than (yL + 1) for yL = 3;

–	(mvL0[ 0 ] +((xT – xL – 1)<<2)) is smaller than 0, for xL = 3;

–	(mvL0[ 1 ] +((yT – yL – 1)<<2)) is smaller than 0 for yL = 3;

–	(mvL0[ 0 ] +((xT + xBlockWidth)<<2)) is larger or equal to (PicWidthInSamplesL<<2);

–	(mvL0[ 1 ] +((yT + yBlockHeight)<<2)) is larger or equal to (PicHeightInSamplesL<<2).

Otherwise LRef, URef, LRec, URec sample values are derived as it is specified in J.8.3.6.2 and J.8.3.6.3 followed by calculation of variables NeighborRefSum, NeighborSum and W0L, O0L specified in the subclause J.8.3.6.4 and J.8.3.6.5 correspondently.

        - Deriving of left and up reference samples of the current block

LRec and URec blocks belong to an (PicWidthInSamplesL)x(PicHeightInSamplesL) array cSL containing constructed luma samples prior to the deblocking filter process.

Each luma sample LRec[ xL, yL ]  (0<=xL<=3, 0<=yL< yBlockHeight) is defined as follows:

LRec[ xL, yL ] = cSL[ xT – 1 – xL, yT + yL ]

Each luma sample URec[ xL, yL ]  (0<=xL< xBlockWidth, 0<=yL<=3) is defined as follows:

URec[ xL, yL ] = cSL[ xT + xL, yT – 1 – yL]

        - Deriving of left and up reference samples of the reference block

For each luma sample location (xL, yL) such as: 0<=xL<=3 and 0<=yL< yBlockHeight inside LRef block, sample value LRef [xL, yL] is derived by the following ordered steps:

1.	The variables xIntL, yIntL, xFracL, and yFracL are derived by:

xIntL = xT + (mvL0[ 0 ] >> 2) - xL-1	(J-8-XX)

yIntL = yT + (mvL0[ 1 ] >> 2) + yL	(J-8-XX)

xFracL= mvL0[ 0 ] & 3	(J-8-XX)

yFracL= mvL0[ 1 ] & 3	(J-8-XX)

2.	LRef[ xL, yL ] sample is derived as an output of the process specified in the 8.4.2.2.1 sub-clause with ( xIntL, yIntL), ( xFracL, yFracL ) and refPicL0L given as input.

For each luma sample location ( xL, yL ) such as: 0<=xL< xBlockWidth, 0<=yL<=3 inside URef block sample value URef[ xL, yL ] is derived by the following ordered steps:

- The variables xIntL, yIntL, xFracL, and yFracL are derived by:

xIntL = xT + (mvL0[ 0 ] >> 2) + xL	(J-8-XX)

yIntL = yT + (mvL0[ 1 ] >> 2) - yL-1	(J-8-XX)

xFracL = mvL0[ 0 ] & 3	(J-8-XX)

yFracL= mvL0[ 1 ] & 3	(J-8-XX)

- URef[ xL, yL ] sample is derived as an output of the process specified in the 8.4.2.2.1 sub-clause with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicL0L given as input.

        - Deriviation of NeighborRefSum and NeighborSum

Both NeighborRefSum and NeighborSum are set to 1 and further calculated as follows:

for( i=0; i<4; i++ )
	for( j=0; j< yBlockHeight;j++ )
		if ( Abs( LRec[ i, j ] - LRef[ i,j ] ) < 31 ) {

NeighborRefSum = NeighborRefSum + LRef[ i, j ]

NeighborSum = NeighborSum + LRec[ i, j ]

}

for( i=0; i< xBlockWidth; i++ )
	for( j=0; j<4; j++ )
		if ( Abs( URec[ i, j ] - URef[ i ,j ] ) < 31 ) {

NeighborRefSum = NeighborRefSum + URef[ i, j ]

NeighborSum = NeighborSum + URec[ i, j ]

}

        - Deriviation of prediction weights

LogWDL is set to 15. O0L is set to 0 and W0L is derived as follows:

If (NeighborSum>>4) is equal to (NeighborRefSum >>4), W0L is set to 1.

Otherwise, W0L is equal to ((1<< LogWDL) * NeighborSum + (NeighborRefSum>>1)) / NeighborRefSum.

      - Inter prediction process with adaptive luminance compensation for 8x8 MB partitions

This subclause is invoked for a macroblock in a texture view component only when mb_alc_flag is equal to 1, and mb_type is equal to P_8x8 or P_8x8ref0.

The Inter prediction process for a macroblock consists of the following steps in order.

- Subclause J.8.3.5 is invoked to derive motion vector components, reference indices and prediction weights as follows.

- The variable MvCnt is incremented by subMvCnt.

- For each macroblock partition mbPartIdx from 0 to 3, inclusive, if NumSubMbPart( sub_mb_type[ mbPartIdx ] ) is not equal to 1, subMbType[ mbPartIdx ] is set to sub_mb_type[ mbPartIdx ] and sub_mb_type[ mbPartIdx ] is set to P_L0_4x4.

- The decoding process for Inter prediction samples for each macroblock partition mbPartIdx and a sub-macroblock partition subMbPartIdx consists of the following in ordered.

–	Invoke the subclause J.8.3.6 to derive the prediction weights in adaptive luminance compensation.

–	Invoke subclause 8.4.2 to derive the inter prediction samples.

–	Invoke subclause 8.5 to derive the transform coefficients and construct the picture prior to deblocking

filter process.

- For each macroblock partition mbPartIdx from 0 to 3, inclusive, if NumSubMbPart( sub_mb_type[ mbPartIdx ] ) is not equal to 1, sub_mb_type[ mbPartIdx ] is set to subMbType[ mbPartIdx ].

    - Specification of bitstream subsets

The specifications of subclause I.8.5 apply.

    - Decoding process for depth range parameters

This process is invoked for decoding of the depth_ranges( ) syntax structure and, if present, the vsp_param( ) syntax structure in sequence parameter set 3DVC extension and for decoding of depth parameter set RBSP.

Inputs of this process are the variables obtained from parsing the active depth range parameter set RBSP:

- the variables relative to the closest depth: ZNearSign[dps_id, i ], ZNearExp[dps_id, i ], ZNearMantissa[dps_id, i ], ZNearManLen[dps_id, i ];

- the variables relative to the farthest depth: ZFarSign[dps_id, i ], ZFarExp[dps_id, i ], ZNearMantissa[dps_id, i ], ZNearManLen[dps_id, i ]

where i is the index to the order of views for 3DV acquisition parameters.

Outputs of this process are

–	the closest depth values ZNear[ dps_id, i ] and the respective variables respective variables ZNearSign[ dps_id, i ], ZNearExponent[ dps_id, i ], ZNearMantissa[ dps_id, i ], ZNearManLen[ dps_id, i ];

–	the farthest depth values ZFar[ dps_id, i ] and the respective variables respective variables ZFarSign[ dps_id, i ], ZFarExponent[ dps_id, i ], ZFarMantissa[ dps_id, i ], ZFarManLen[ dps_id, i ].

Some of the views for which the 3DV acquisition information is specified may not be present in the coded video sequence.

The output variables x in Table I-X are derived as follows from the respective variables f, s, e, n, and v indicated in Table I-X.

–	If f is equal to 0 in depth range update RBSP, f in the active sequence parameter set 3DVC extension shall be equal to 1 and x[ dps_id, i ] = x[ SPS, i ], s[ dps_id, i ] = s[ 0, i ], e[ dps_id, i ] = e[ 0, i ], and n[ dps_id, i ] = n[ 0, i ] for all values of i and for “SPS” pointing to the active sequence parameter set. [Ed. Note (CY): if we have created a 0 depth parameter set, I believe this paragraph is not needed. In other words, shall we use this paragraph to create the depth parameter set with id equal to 0?]

–	Else if f is equal to 1, the variable x computed as follows for [ dps_id, i ] where i is index to the order of views for 3DV acquisition parameters:

–	If 0 < e < 127, x = (−1)s * 2e−31 * (1 + n ÷ 2v).

–	Otherwise (e is equal to 0), x = (−1)s * 2−(30+v) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989, Binary floating-point arithmetic for microprocessor systems.

- Table I-X – Association between depth parameter variables and syntax elements.

| x | f | s | E | n | v |
| --- | --- | --- | --- | --- | --- |
| ZNear | z_near_flag | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | z_far_flag | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |

    - (void)

(void) [Ed. (MH): This clause is to be removed completely, but kept for now as an empty subclause in order to keep the cross-references correct.]

    - Depth intra prediction process

This subclause is invoked for a coded slice with DepthFlag equal to 1, when disp_flag is equal to 1 or psip_flag is equal to 1.

Subclause 8.3 is applied with the following changes.

–	If mb_disp_flag is equal to 1, subclause 8.7.1 is invoked

–	Modification process for intra 4x4 precition is specified in subclause 8.7.1 instead of subclause 8.3.1.

–	Modification process for intra 8x8 precition is specified in subclause 8.7.1 instead of subclause 8.3.2.

–	Modification process for intra 16x16 precition is specified in subclause 8.7.14 instead of subclause 8.3.3.

      - Depth intra skip prediction process

Input to this process is a (PicWidthInSamplesL)x(PicHeightInSamplesL) array cSL containing constructed luma samples prior to the deblocking filter process of neighbouring macroblocks

Outputs of this process are Intra prediction luma samples for the current macroblock predL[ x, y ].

The 32 neighbouring samples p[ x, y ] that are constructed luma samples prior to the deblocking filter process, with x = −1, y = 0..15 and with x = 0..15, y = −1, are derived as specified by the following ordered steps:

- The derivation process for neighbouring locations in subclause Error: Reference source not found is invoked for luma locations with ( x, y ) assigned to ( xN, yN ) as input and mbAddrN and ( xW, yW ) as output.

- Each sample p[ x, y ] with x = −1, y = 0..15 and with x = 0..15, y = −1 is derived as follows.

–	If mbAddrA or mbAddrB are not available, the value of the sample p[ x, y ] is 128:

–	Otherwise, the value of the sample p[ x, y ] is derived as specified by the following ordered steps:

- The location of the upper-left luma sample of the macroblock mbAddrN is derived by invoking the inverse macroblock scanning process in subclause Error: Reference source not found with mbAddrN as the input and the output is assigned to ( xM, yM ).

- Depending on the variable MbaffFrameFlag and the macroblock mbAddrN, the sample value p[ x, y ] is derived as follows.

–	If MbaffFrameFlag is equal to 1 and the macroblock mbAddrN is a field macroblock,

p[ x, y ] = cSL[ xM + xW, yM + 2 * yW ]	(8-1)

–	Otherwise (MbaffFrameFlag is equal to 0 or the macroblock mbAddrN is a frame macroblock),

p[ x, y ] = cSL[ xM + xW, yM + yW ]	(8-2)

The depth intra skip prediction estimates the Intra 16x16PredMode from the upper-left neighboring luma samples.

Set left_line[k], up_line[k], left_i16mode, up_i16mode, left_change, and up_change as follows:

–	left_line[k] is equal to p[-1,y], with y=0…15, and k equal to y.

–	up_line[k] is equal to p[x,-1], with x=0…15, and k equal to x.

–	left_i16mode is equal to Intra 16x16PredMode of the left macroblock if the left macroblock is coded as I16x16 mode or DISP_mode. Otherwise, it is equal to 4.

–	up_i16mode is equal to Intra 16x16PredMode of the upper macroblock if the upper macroblock is coded as I16x16 mode or DISP_mode. Otherwise, it is equal to 4.

–	left_change is set to 0, for each k from 0 to 15 inclusive, if left_line[k] is not equal to left_line[k+1], left_change is increased by 1.

–	up_change is set to 0, for each k from 0 to 15, inclusive, if up_line[k] is not equal to up_line[k+1], up_change is increased by 1.

The Intra 16x16PredMode is derived as follows:

–	If mbAddrA and mbAddrB are available, the following applies.

–	If left_change is equal to 0 and up_change is equal to 0, the following applies.

–	If up_i16mode is equal to 0 and left_i16mode is equal to 0, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if up_i16mode is equal to 1 and left_i16modeis  equal to 1, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, if left_line[0] is less than up_line[0], the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if up_line[0] is less than left_line[0], the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, the variable Intra 16x16PredMode is set equal to 2. [Ed. (MH): this line should probably be indented one step to the right.]

–	Otherwise, if left_change is equal to 0 and up_change is larger than 0, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if left_change is larger than 0 and up_change is equal to 0, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, the following applies

–	If up_i16mode is equal to 0 and left_i16mode is equal to 0, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if up_i16mode is equal to 1 and left_i16mode is equal to 1, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, if up_i16mode is equal to 0 and left_i16mode is not equal to 1, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if left_i16mode is equal to 1 and up_i16mode is not equal to 0, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, if left_change is less than up_change, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if up_change is less than left_change, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, if abs(left_line[0] – left_line[15]) is less than abs(up_line[0] – up_line[15]), the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if abs(up_line[0] –up_line[15]) is less than abs(left_line[0] –left_line[15]), the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, the variable Intra 16x16PredMode is set equal to 2.

–	Otherwise, if mbAddrB is availabe, the variable Intra 16x16PredMode is set equal to 0.

–	Otherwise, if mbAddrA is availabe, the variable Intra 16x16PredMode is set equal to 1.

–	Otherwise, the variable Intra 16x16PredMode is set equal to 2.

Let predL[ x, y ] with x, y = 0..15 denote the prediction samples for the 16x16 luma block samples. Depending on Intra16x16PredMode, one of the Intra_16x16 prediction modes specified in subclauses Error: Reference source not found to Error: Reference source not found is invoked.

When mb_disp_flag is equal to 1, CodedBlockPatternLuma and all values of LumaLevel, ChromaDCLevel, ChromaACLevel are set equal to 0 for the current macroblock.

Subclause 8.7.2.1 is applied with the following changes. [Ed. Consider the best location of this section that modifies 8.7.2.1.]

      - Intra_4x4 prediction process for luma samples

The specification of subclause 8.3.1 apply

        - Derivation process for Intra4x4PredMode

The specification of subclause 8.3.1 apply with the following changes:

–	The Table 8-2 in subclause 8.3.1 is replaced by

- Table ‎8-1 – Specification of Intra4x4PredMode[ luma4x4BlkIdx ] and associated names

| Intra4x4PredMode[ luma4x4BlkIdx ] | Name of Intra4x4PredMode[ luma4x4BlkIdx ] |
| --- | --- |
| 0 | Intra_4x4_Vertical (prediction mode) |
| 1 | Intra_4x4_Horizontal (prediction mode) |
| 2 | Intra_4x4_DC_PSIP (prediction mode) |
| 3 | Intra_4x4_Diagonal_Down_Left (prediction mode) |
| 4 | Intra_4x4_Diagonal_Down_Right (prediction mode) |
| 5 | Intra_4x4_Vertical_Right (prediction mode) |
| 6 | Intra_4x4_Horizontal_Down (prediction mode) |
| 7 | Intra_4x4_Vertical_Left (prediction mode) |
| 8 | Intra_4x4_Horizontal_Up (prediction mode) |

–	The following term ‘Intra_4x4_DC’ is replaced by ‘Intra_4x4_DC_PSIP’.

The specification of subclause 8.3.1.1 is applied with the following changes.

–	The following paragraphs in 4th procedure of derivation of Intra4x4PredMode[luma4x4BliIdx] in subclause 8.3.1.1:

–	predIntra4x4PredMode = Min( intraMxMPredModeA, intraMxMPredModeB )

is replaced by

if(psip_flag)

predIntra4x4PredMode = Intra_4x4_DC_PSIP_MODE;

else

predIntra4x4PredMode = Min( intraMxMPredModeA, intraMxMPredModeB )

        - Derivation process for RepVal4x4 and PredictionMap4x4

Processes described in this subclause are invoked when Intra4x4PredMode is 2, Psip4x4Available is equal to 1, and nal_unit_type is equal to 21.

Inputs to this process are

–	the index of a 4x4 luma block luma4x4BlkIdx,

–	an (PicWidthInSamplesL)x(PicHeightInSamplesL) array cSL containing constructed luma samples prior to the deblocking filter process of neighbouring macroblocks.

Output of this process are the variable RepVal4x4[luma4x4BlkIdx][i], with i=0,1, and PredictionMap4x4[luma4x4BlkIdx][x,y], with x, y=0..3.

For the derivation of the RepVal4x4[luma4x4BlkIdx], the following ordered steps are specified:

- Let p0[x] = p[x,-1], with x=0..3, and p0[y+4] = p[-1,y], with y=0..3.

- Let Th = ( Min(p0[0], .. , p0[7] ) + Max(p0[0], .., p0[7]) ) >> 1.

- Set sum1 = sum2 = n1 = n2 = 0.

- For each n from 0 through 7 inclusive, sum1 = sum1 + p0[n] and n1 = n1 + 1, if p0[n] <= Th.

- For each n from 0 through 7 inclusive, sum2 = sum2 + p0[n] and n2 = n2 + 1, if p0[n] > Th.

- Set avg1 = sum1 ÷ n1, avg2 = sum2 ÷ n2.

- Set RepVal4x4[luma4x4BlkIdx][0] = avg1, RepVal4x4[luma4x4BlkIdx][1] = avg2.

- Reset Th = ( avg1 + avg2 ) >> 1.

For the derivation of the PredictionMap4x4[luma4x4BlkIdx], the following ordered steps are specified.

–	If psip4x4_dir[luma4x4BlkIdx] is equal to 0,

- If p[x,−1] <= Th, set PredictionMap4x4[luma4x4BlkIdx][x,-1] = 0, with x=0..3. Otherwise, set PredictionMap4x4[luma4x4BlkIdx][x,-1] = 1.

- Convert PredictionMap4x4[luma4x4BlkIdx][x,-1], with x=0…3. to PredictionCode[-1] by Table 8-3.

- For each n from 0 through 3 inclusive, PredictionCode[n] = (PredictionCode[n-1] + psip4x4_code_diff[luma4x4BlkIdx][n]+8)%8

- For each n from 0 through 3 inclusive, convert PredictionCode[n] to PredictionMap4x4[luma4x4BlkIdx][x,n] with x=0..3 by Table 8-3.

–	If psip4x4_dir [luma4x4BlkIdx] is equal to 1,

- If p[-1,y] <= Th, set PredictionMap4x4[luma4x4BlkIdx][-1,y] = 0, with y=0..3. Otherwise, set PredictionMap4x4[luma4x4BlkIdx][-1,y] = 1.

- Convert PredictionMap4x4[luma4x4BlkIdx][-1,y], with y=0…3. to PredictionCode[-1] by Table 8-3.

- For each n from 0 to 3 inclusive, PredictionCode[n] = ( PredictionCode[n-1] + psip4x4_code_diff[luma4x4BlkIdx][n]+8) % 8

- For each n from 0 to 3 inclusive, , convert PredictionCode[n] to PredictionMap4x4[luma4x4BlkIdx][n,y] with y=0..3 by Table 8-3.

- Table ‎8-3 – Specification of PredictionCode and PredictionMap

| PredictionCode | PredictionMap4x4[luma4x4BlkIdx][x,A] with x=0…3 PredictionMap4x4[luma4x4BlkIdx][B,y] with y=0…3 |
| --- | --- |
| 0 | 0 0 0 0 |
| 1 | 0 0 0 1 |
| 2 | 0 0 1 1 |
| 3 | 0 1 1 1 |
| 4 | 1 1 1 1 |
| 5 | 1 1 1 0 |
| 6 | 1 1 0 0 |
| 7 | 1 0 0 0 |

        - Intra_4x4 sample prediction

The specification of subclause 8.3.1.2 apply.

          - Intra_4x4 sample prediction

The specification of subclause 8.3.1.2.1 apply.

          - Specification of Intra_4x4_Horizontal prediction mode

The specification of subclause 8.3.1.2.2 apply.

          - Specification of Intra_4x4_DC_PSIP prediction mode

The specifications of subclause 8.3.1.2.3 apply with the following changes.

–	The following paragraphs in subclause 8.3.1.2.3:

–	If all samples p[ x, −1 ], with x = 0..3, and p[ −1, y ], with y = 0..3, are marked as "available for Intra_4x4 prediction", the values of the prediction samples pred4x4L[ x, y ], with x, y = 0..3, are derived by

pred4x4L[ x, y ] = ( p[ 0, −1 ] + p[ 1, −1 ] + p[ 2, −1 ] + p[ 3, −1 ] +

p[ −1, 0 ] + p[ −1, 1 ] + p[ −1, 2 ] + p[ −1, 3 ] + 4 ) >> 3  					(8-48)

is replaced by

–	If all samples p[ x, −1 ], with x = 0..3, and p[ −1, y ], with y = 0..3, are marked as "available for Intra_4x4 prediction", inputs of this process is RepVal[luma4x4BlkIdx] [n], with n=0,1, and PredictionMap4x4[luma4x4BlkIdx][x,y] with x, y=0..3, and the values of the prediction samples pred4x4L[ x, y ], with x, y = 0..3, are derived by

pred4x4L[ x, y ] = RepVal4x4[luma4x4BlkIdx][ PredictionMap4x4[luma4x4BlkIdx][ x, y] ] 		(8-48)

          - Specification of Intra_4x4_Diagonal_Down_Left prediction mode

The specification of subclause 8.3.1.2.4 apply.

          - Specification of Intra_4x4_Diagonal_Down_Right prediction mode

The specification of subclause 8.3.1.2.5 apply.

          - Specification of Intra_4x4_Vertical_Right prediction mode

The specification of subclause 8.3.1.2.6 apply.

          - Specification of Intra_4x4_Horizontal_Down prediction mode

The specification of subclause 8.3.1.2.7 apply.

          - Specification of Intra_4x4_Vertical_Left prediction mode

The specification of subclause 8.3.1.2.8 apply.

          - Specification of Intra_4x4_Horizontal_Up prediction mode

The specification of subclause 8.3.1.2.9 apply.

      - Intra_8x8 prediction process for luma samples

The specification of subclause 8.3.2 apply

        - Derivation process for Intra8x8PredMode

The specification of subclause 8.3.1 apply with the following changes :

–	The Table 8-3 in subclause 8.3.1 is replaced by

- Table ‎8-4 – Specification of Intra8x8PredMode[ luma8x8BlkIdx ] and associated names

| Intra8x8PredMode[ luma8x8BlkIdx ] | Name of Intra8x8PredMode[ luma8x8BlkIdx ] |
| --- | --- |
| 0 | Intra_8x8_Vertical (prediction mode) |
| 1 | Intra_8x8_Horizontal (prediction mode) |
| 2 | Intra_8x8_DC_PSIP (prediction mode) |
| 3 | Intra_8x8_Diagonal_Down_Left (prediction mode) |
| 4 | Intra_8x8_Diagonal_Down_Right (prediction mode) |
| 5 | Intra_8x8_Vertical_Right (prediction mode) |
| 6 | Intra_8x8_Horizontal_Down (prediction mode) |
| 7 | Intra_8x8_Vertical_Left (prediction mode) |
| 8 | Intra_8x8_Horizontal_Up (prediction mode) |

–	The following term ‘Intra_8x8_DC’ is replaced by ‘Intra_8x8_DC_PSIP’.

The specification of subclause 8.3.2.1 is applied with the following changes.

–	The following paragraphs in 4th procedure of derivation of Intra8x8PredMode[luma8x8BliIdx] in subclause 8.3.2.1:

–	predIntra8x8PredMode = Min( intraMxMPredModeA, intraMxMPredModeB )

is replaced by

if(psip_flag)

predIntra8x8PredMode = Intra_8x8_DC_PSIP_MODE;

else

predIntra8x8PredMode = Min( intraMxMPredModeA, intraMxMPredModeB )

        - Derivation process for RepVal8x8 and PredictionMap8x8

Processes described in this subclause are invoked when Intra8x8PredMode is 2, Psip8x8Available is equal to 1, and nal_unit_type is equal to 21.

Inputs to this process are

–	the index of a 8x8 luma block luma8x8BlkIdx,

–	an (PicWidthInSamplesL)x(PicHeightInSamplesL) array cSL containing constructed luma samples prior to the deblocking filter process of neighbouring macroblocks.

Output of this process are the variable RepVal8x8[luma8x8BlkIdx][i], with i=0,1, and PredictionMap8x8[luma8x8BlkIdx][x,y], with x, y=0..7.

For the derivation of the RepVal8x8[luma8x8BlkIdx], the following ordered steps are specified:

- Let p0[x] = p[x,-1], with x=0..7, and p0[y+8] = p[-1,y], with y=0..7.

- Let Th = ( Min(p0[0], .. , p0[15] ) + Max(p0[0], .., p0[15]) ) >> 1.

- Set sum1 = sum2 = n1 = n2 = 0.

- For each n from 0 through 15 inclusive, sum1 = sum1 + p0[n] and n1 = n1 + 1, if p0[n] <= Th.

- For each n from 0 through 15 inclusive, sum2 = sum2 + p0[n] and n2 = n2 + 1, if p0[n] > Th.

- Set avg1 = sum1 ÷ n1, avg2 = sum2 ÷ n2.

- Set RepVal8x8 [luma8x8BlkIdx][0] = avg1, RepVal8x8[luma8x8BlkIdx][1] = avg2.

- Reset Th = ( avg1 + avg2 ) >> 1.

For the derivation of the PredictionMap8x8[luma8x8BlkIdx], the following ordered steps are specified.

–	If psip8x8_dir[luma8x8BlkIdx] is equal to 0,

- If p[x,−1] <= Th, set PredictionMap8x8[luma8x8BlkIdx][x,-1] = 0, with x=0..7. Otherwise, set PredictionMap8x8[luma8x8BlkIdx][x,-1] = 1.

- Convert PredictionMap8x8[luma8x8BlkIdx][x,-1], with x=0..7. to PredictionCode[-1] by Table 8-5.

- For each n from 0 through 7 inclusive, PredictionCode[n] = ( PredictionCode[n-1] + psip8x8_code_diff[luma8x8BlkIdx][n]+16)%16

- For each n from 0 through 7 inclusive, convert PredictionCode[n] to PredictionMap8x8[luma8x8BlkIdx][x,n] with x=0..7 by Table 8-5.

–	If psip8x8_dir [luma8x8BlkIdx] is equal to 1,

- If p[-1,y] <= Th, set PredictionMap8x8[luma8x8BlkIdx][-1,y] = 0, with y=0..7. Otherwise, set PredictionMap8x8[luma8x8BlkIdx][-1,y] = 1.

- Convert PredictionMap8x8[luma8x8BlkIdx][-1,y], with y=0…7. to PredictionCode[-1] by Table 8-5.

- For each n from 0 through 7 inclusive , PredictionCode[n] = ( PredictionCode[n-1] + psip8x8_code_diff[luma8x8BlkIdx][n]+16)%16

- For each n from 0 through 7 inclusive, convert PredictionCode[n] to PredictionMap8x8[luma8x8BlkIdx][n,y] with y=0..7 by Table 8-5.

- Table ‎8-5 – Specification of PredictionCode and PredictionMap

| PredictionCode | PredictionMap8x8[luma8x8BlkIdx][x,A] with x=0..7 PredictionMap8x8[luma8x8BlkIdx][B,y] with y=0..7 |
| --- | --- |
| 0 | 0 0 0 0 0 0 0 0 |
| 1 | 0 0 0 0 0 0 0 1 |
| 2 | 0 0 0 0 0 0 1 1 |
| 3 | 0 0 0 0 0 1 1 1 |
| 4 | 0 0 0 0 1 1 1 1 |
| 5 | 0 0 0 1 1 1 1 1 |
| 6 | 0 0 1 1 1 1 1 1 |
| 7 | 0 1 1 1 1 1 1 1 |
| 8 | 1 1 1 1 1 1 1 1 |
| 9 | 1 1 1 1 1 1 1 0 |
| 10 | 1 1 1 1 1 1 0 0 |
| 11 | 1 1 1 1 1 0 0 0 |
| 12 | 1 1 1 1 0 0 0 0 |
| 13 | 1 1 1 0 0 0 0 0 |
| 14 | 1 1 0 0 0 0 0 0 |
| 15 | 1 0 0 0 0 0 0 0 |

        - Intra_8x8 sample prediction

The specification of subclause 8.3.2.2 apply.

          - Reference sample filtering process for Intra_8x8 sample prediction

The specification of subclause 8.3.2.2.1 apply.

          - Specification of Intra_8x8_Vertical prediction mode

The specification of subclause 8.3.2.2.2 apply.

          - Specification of Intra_8x8_ Horizontal prediction mode

The specification of subclause 8.3.2.2.3 apply.

          - Specification of Intra_8x8_ DC_PSIP prediction mode

The specifications of subclause 8.3.2.2.3 apply with the following changes.

–	The following paragraphs in subclause 8.3.2.2.3:

–	If all samples p[ x, −1 ], with x = 0..7, and p[ −1, y ], with y = 0..7, are marked as "available for Intra_8x8

prediction", the values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7, are derived by

pred8x8L[ x, y ]=(x'=07p'[x',−1]+y'=07p'[−1,y']+8)>>4	  					(‎8-91)

is replaced by

–	If all samples p[ x, −1 ], with x = 0..7, and p[ −1, y ], with y = 0..7, are marked as "available for Intra_8x8 prediction", inputs of this process is RepVal8x8[luma8x8BlkIdx] [n], with n=0,1, and predictionMap8x8[luma8x8BlkIdx][x,y] with x, y=0..7, and the values of the prediction samples pred8x8L[ x, y ], with x, y = 0..7, are derived by

pred8x8L[ x, y ] = RepVal8x8[ luma8x8BlkIdx ][ PredictionMap8x8[luma8x8BlkIdx][ x, y] ] 		(8-91)

          - Specification of Intra_8x8_ Diagonal_Down_Left prediction mode

The specification of subclause 8.3.2.2.5 apply.

          - Specification of Intra_8x8_ Diagonal_Down_Right prediction mode

The specification of subclause 8.3.2.2.6 apply.

          - Specification of Intra_8x8_ Vertical_Right prediction mode

The specification of subclause 8.3.2.2.7 apply.

          - Specification of Intra_8x8_ Horizontal_Down prediction mode

The specification of subclause 8.3.2.2.8 apply.

          - Specification of Intra_8x8_ Vertical_Left prediction mode

The specification of subclause 8.3.2.2.9 apply.

          - Specification of Intra_8x8_ Horizontal_Up prediction mode

The specification of subclause 8.3.2.2.10 apply.

      - Intra_16x16 prediction process for luma samples

The specification of subclause 8.3.3 apply with the following changes:

–	The Table 8-4 in subclause 8.3.3 is replaced by

- Table ‎8-6 – Specification of Intra16x16PredMode and associated names

| Intra16x16PredMode | Name of Intra16x16PredMode |
| --- | --- |
| 0 | Intra_16x16_Vertical (prediction mode) |
| 1 | Intra_16x16_Horizontal (prediction mode) |
| 2 | Intra_16x16_DC_PSIP (prediction mode) |
| 3 | Intra_16x16_Plane (prediction mode) |

        - Derivation process for RepVal16x16 and PredictionMap16x16

Processes described in this subclause are invoked when Intra16x16PredMode is 2, Psip16x16Available is equal to 1, and nal_unit_type is equal to 21.

Input to this process is a (PicWidthInSamplesL)x(PicHeightInSamplesL) array cSL containing constructed luma samples prior to the deblocking filter process of neighbouring macroblocks.

Output of this process are the variable RepVal16x16[i], with i=0,1, and PredictionMap16x16[x,y], with x, y=0..15.

For the derivation of the RepVal16x16, the following ordered steps are specified:

- Let p0[x] = p[x,-1], with x=0..15, and p0[y+16] = p[-1,y], with y=0..15.

- Let Th = ( Min(p0[0], .. , p0[31] ) + Max(p0[0], .., p0[31]) ) >> 1.

- Set sum1 = sum2 = n1 = n2 = 0.

- For each n from 0 through 31 inclusive, sum1 = sum1 + p0[n] and n1 = n1 + 1, if p0[n] <= Th.

- For each n from 0 through 31 inclusive, sum2 = sum2 + p0[n] and n2 = n2 + 1, if p0[n] > Th.

- Set avg1 = sum1 ÷ n1, avg2 = sum2 ÷ n2.

- Set RepVal16x16 [0] = avg1, RepVal16x16 [1] = avg2.

- Reset Th = ( avg1 + avg2 ) >> 1.

For the derivation of the PredictionMap16x16, the following ordered steps are specified.

–	If psip16x16_dir is equal to 0,

- If p[x,−1] <= Th, set PredictionMap16x16[x,-1] = 0, with x=0..15. Otherwise, set PredictionMap16x16[x,-1] = 1.

- Convert PredictionMap16x16[x,-1], with x=0…15, to PredictionCode[-1] by Table 8-7.

- For each n from 0 to 15 inclusive, PredictionCode[n] = ( PredictionCode[n-1] + psip16x16_code_diff[n]+32)%32

- For each n from 0 to 15 inclusive, convert PredictionCode[n] to PredictionMap16x16 [x,n] with x=0..15 by Table 8-7.

–	If psip16x16_dir is equal to 1,

- If p[-1,y] <= Th, set PredictionMap16x16[-1,y] = 0, with y=0..15. Otherwise, set PredictionMap16x16[-1,y] = 1.

- Convert PredictionMap16x16[-1,y], with y=0..15. to PredictionCode[-1] by Table 8-7.

- For each n from 0 to 15 inclusive, PredictionCode[n] = ( PredictionCode[n-1] + psip16x16_code_diff[n]+32) %32

- For each n from 0 to 15 inclusive, convert PredictionCode[n] to PredictionMap16x16n,y] with y=0..15 by Table 8-7.

- Table ‎8-7 – Specification of PredictionCode and PredictionMap

| PredictionCode | PredictionMap16x16[x,A] with x=0..15 PredictionMap16x16[B,y] with y=0..15 | PredictionCode | PredictionMap16x16[x,A] with x=0..15 PredictionMap16x16[B,y] with y=0..15 |
| --- | --- | --- | --- |
| 0 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 | 16 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 |
| 1 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 | 17 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 |
| 2 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 | 18 | 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 0 |
| 3 | 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 1 | 19 | 1 1 1 1 1 1 1 1 1 1 1 1 1 0 0 0 |
| 4 | 0 0 0 0 0 0 0 0 0 0 0 0 1 1 1 1 | 20 | 1 1 1 1 1 1 1 1 1 1 1 1 0 0 0 0 |
| 5 | 0 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 | 21 | 1 1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 |
| 6 | 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 | 22 | 1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 |
| 7 | 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 | 23 | 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 |
| 8 | 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 | 24 | 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 |
| 9 | 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 | 25 | 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 |
| 10 | 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1 | 26 | 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 |
| 11 | 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1 1 | 27 | 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 0 |
| 12 | 0 0 0 0 1 1 1 1 1 1 1 1 1 1 1 1 | 28 | 1 1 1 1 0 0 0 0 0 0 0 0 0 0 0 0 |
| 13 | 0 0 0 1 1 1 1 1 1 1 1 1 1 1 1 1 | 29 | 1 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 |
| 14 | 0 0 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 30 | 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 |
| 15 | 0 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 | 31 | 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 |

        - Specification of Intra_16x16_Vertical prediction mode

The specification of subclause 8.3.3.1 apply.

        - Specification of Intra_16x16_Horizontal prediction mode

The specification of subclause 8.3.3.2 apply.

        - Specification of Intra_16x16_DC_PSIP prediction mode

The specifications of subclause 8.3.3.3 apply with the following changes.

–	The following paragraphs in subclause 8.3.3.3:

–	If all samples p[ x, −1 ], with x = 0..15, and p[ −1, y ], with y = 0..15, are marked as "available for Intra_16x16

prediction", the prediction for all luma samples in the macroblock is given by

predL[ x, y ] = (x'=015px',−1+y'=015p−1,y'+16)>>5, with x, y = 0..15	(‎8-118)

is replaced by

–	If all samples p[ x, −1 ], with x = 0..15, and p[ −1, y ], with y = 0..15, are marked as "available for Intra_16x16

prediction",, inputs of this process is RepVal16x16 [n], with n=0,1, and PredictionMap16x16[x,y] with x, y=0..15, and the prediction for all luma samples in the macroblock is given by

predL[ x, y ] = RepVal16x16[ PredictionMap16x16[ x, y] ] 								(8-118)

        - Specification of Intra_16x16_Plane prediction mode

The specification of subclause 8.3.3.4 apply.

      - Intra prediction process for chroma samples

The specification of subclause 8.3.4 apply.

      - Sample construction process for I_PCM macroblocks

The specification of subclause 8.3.5 apply.

    - Deblocking filter process

The specifications in 8.7 apply.

      - Filtering process for block edges

The specifications in 8.7.1 apply.

      - Filtering process for a set of samples across a horizontal or vertical block edge

The specifications in 8.7.2 apply.

        - Derivation process for the luma content dependent boundary filtering strength

The specifications in 8.7.2.1 apply with the following changes:

–	The following paragraph is added after the paragraph starting with “Output of this process is the variable bS”:

The variable bS is derived as follows if psip_flag is equal to 1.

–	If psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded using a I16x16 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_DC_PSIP_MODE, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded as I8x8 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_8x8_DC_PSIP_MODE, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, both of the samples p0 and q0 is in a macroblock coded as I4x4 Mode and both of the samples p0 and q0 is in a macroblock coded using an Intra_4x4_DC_PSIP_MODE, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I16x16 Mode with Intra_16x16_DC_PSIP_MODE, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I8x8 Mode with Intra_8x8_DC_PSIP_MODE, intra prediction mode, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if psip_flag is equal to 1, either the samples p0 or q0 is in a macroblock coded as I4x4 Mode with Intra_4x4_DC_PSIP_MODE intra prediction mode, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 1 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

The variable bS is derived as follows if disp_flag is equal to 1. mb_type is DISPMODE if mb_disp_flag is equal to 1.

–	If verticalEdgeFlag is euqal to 1, both of the samples p0 and q0 is in a macroblock coded using a DISPMODE and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_Horizontal intra prediction mode, a value of bS equal to 0 is the output:

–	Otherwise, if verticalEdgeFlag is euqal to 0, both of the samples p0 and q0 is in a macroblock coded using a DISPMODE and both of the samples p0 and q0 is in a macroblock coded using an Intra_16x16_Vertical intra prediction mode, a value of bS equal to 0 is the output:

–	Otherwise, if both of the samples p0 and q0 is in a macroblock coded using a DISPMODE, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 2 is the output:

–	Otherwise, a value of bS equal to 0 is the output:

–	Otherwise, if either the samples p0 or q0 is in a macroblock coded using a DISPMODE, the following applies

–	If the block edge is also a macroblock edge, a value of bS equal to 4 is the output:

–	Otherwise, a value of bS equal to 3 is the output:

        - Derivation process for the thresholds for each block edges

The specifications in 8.7.2.2 apply.

        - Filtering process for edges with bS less than 4

The specifications in 8.7.2.3 apply.

        - Filtering process for edges for bS equal to 4

The specifications in 8.7.2.4 apply.

  - Parsing process

The specifications in clause Error: Reference source not found apply. Additionally, the following modifications related to the syntax element mb_ivmp_flag are specified and added.

    - Alternative CABAC parsing process for slice data and macroblock layer in depth extension

Subclause 9.1.1specifies the initialisation process for the alternative CABAC parsing process for slice data and macroblock layer in depth extension.

Subclause  specifies the binarization process for the alternative CABAC parsing process for slice data and macroblock layer in depth extension.

Subclause 9.1.3 specifies the decoding process flow for the alternative CABAC parsing process for slice data and macroblock layer in depth extension.

      - Initialisation process

Outputs of this process are the initialised CABAC context variables indexed by ctxIdx.

Tables I-3 and I-4 contain the values of the variables n and m used in the initialisation of context variables that are assigned to syntax elements mb_ivmp_flag, mb_vsskip_flag, and mb_direct_type_flag. Table I-5 contains the values of the variables n and m used in the initialisation of context variables that are assigned to syntax element mb_disp_flag, psip4x4_dir, psip4x4_code_diff, psip8x8_dir, psip8x8_code_diff, psip16x16_dir, and psip16x16_code_diff. Table I-6 contains the values of the variables n and m used in the initialisation of context variables that are assigned to syntax element mb_alc_skip_flag and mb_alc_flag. The initialization proces for two variables pStateIdx and valMPS is the same as other syntax elements, as defined in Equation 9-3. For all other syntax elements in subclause 7.3.5 the initialisation process of context variables as specified in subclause Error: Reference source not found applies.

In Table I-2, the ctxIdx for which initialisation is needed for I, P and B slices are listed. Also listed is the table number that includes the values of m and n needed for the initialisation.

- Table I-2 – Association of ctxIdx and syntax elements in the initialisation process

| Syntax element |  | Table |  | Slice type |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | I |  | P |  | B |
| mb_ivmp_flag | Error: Reference source not found3 |  |  |  | 1031..1033 |  | 1031..1033 |  |
| mb_vsskip_flag | Table I-4 |  |  |  | 1034..1036 |  | 1037..1039 |  |
| mb_direct_type_flag | Table I-4 |  |  |  |  |  | 1040..1042 |  |
| mb_disp_flag | Table I-5 |  | 1043..1045 |  | 1043..1045 |  | 1043..1045 |  |
| psip4x4_dir | Table I-5 |  | 1046 |  | 1046 |  | 1046 |  |
| psip4x4_code_diff | Table I-5 |  | 1047..1048 |  | 1047..1048 |  | 1047..1048 |  |
| psip8x8_dir | Table I-5 |  | 1046 |  | 1046 |  | 1046 |  |
| psip8x8_code_diff | Table I-5 |  | 1047..1048 |  | 1047..1048 |  | 1047..1048 |  |
| psip16x16_dir | Table I-5 |  | 1046 |  | 1046 |  | 1046 |  |
| psip16x16_code_diff | Table I-5 |  | 1047..1048 |  | 1047..1048 |  | 1047..1048 |  |
| mb_alc_skip_flag | Table I-6 |  |  |  | 1049 .. 1051 |  |  |  |
| mb_alc_flag | Table I-6 |  |  |  | 1052 .. 1054 |  |  |  |

- Table I-3 – Values of variables m and n for ctxIdx from 1031 to 1033

| Initialisation variables | ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 1031 | 1032 | 1033 |
| m | 0 | 0 | 0 |
| n | 64 | 64 | 64 |

- Table I-4 – Values of variables m and n for ctxIdx from 1034 to 1042

| Value of cabac_init_idc | Initialisation variables | ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1034 | 1035 | 1036 | 1037 | 1038 | 1039 | 1040 | 1041 | 1042 |
| 0 | m | 23 | 23 | 21 | 18 | 9 | 29 | -46 | -20 | 1 |
|  | n | 33 | 2 | 0 | 64 | 43 | 0 | 127 | 104 | 67 |
| 1 | m | 22 | 34 | 16 | 26 | 19 | 40 | -45 | -15 | -4 |
|  | n | 25 | 0 | 0 | 34 | 22 | 0 | 127 | 101 | 76 |
| 2 | m | 29 | 25 | 14 | 20 | 20 | 29 | -32 | -22 | -2 |
|  | n | 16 | 0 | 0 | 40 | 10 | 0 | 127 | -117 | 74 |

- Table I-5 – Values of variables m and n for ctxIdx from 1043 to 1048

| Initialisation variables | ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1043 | 1044 | 1045 | 1046 | 1047 | 1048 |
| m | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 64 | 64 | 64 | 64 | 64 |

- Table I-6 – Values of variables m and n for ctxIdx from 1049 to 1054

| Initialisation variables | ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1049 | 1050 | 1051 | 1052 | 1053 | 1054 |
| m | 14 | 14 | 10 | 14 | 14 | 10 |
| n | 4 | 27 | 52 | 4 | 27 | 52 |

      - Binarization process

Input to this process is a request for a syntax element.

Output of this process is the binarization of the syntax element, maxBinIdxCtx, ctxIdxOffset, and bypassFlag.

Associated with each binarization or binarization part of a syntax element is a specific value of the context index offset (ctxIdxOffset) variable and a specific value of the maxBinIdxCtx variable as given in Table I-5.

The variable bypassFlag is set equal to 0.

The possible values of the context index ctxIdx are in the range 1031 to 1054, inclusive. The value assigned to ctxIdxOffset specifies the lower value of the range of ctxIdx assigned to the corresponding binarization or binarization part of a syntax element.

- Table I-5 – Syntax elements and associated types of binarization, maxBinIdxCtx, and ctxIdxOffset

| Syntax element | Type of binarization | maxBinIdxCtx | ctxIdxOffset |
| --- | --- | --- | --- |
| mb_ivmp_flag (P, B slices) | FL, cMax=1 | 0 | 1031 |
| mb_vsskip_flag (P slices only) | FL, cMax=1 | 0 | 1034 |
| mb_vsskip_flag (B slices only) | FL, cMax=1 | 0 | 1037 |
| mb_direct_type_flag (B slices only) | FL, cMax=1 | 0 | 1040 |
| mb_disp_flag (I, P, B slices) | FL, cMax=1 | 0 | 1043 |
| psip4x4_dir psip8x8_dir psip16x16_dir | FL, cMax=1 | 0 | 1046 |
| psip4x4_code_diff psip8x8_code_diff psip16x16_code_diff | as specified in subclause 9.1.2.1 | 0 | 1047 |
| mb_alc_skip_flag | FL, cMax=1 | 0 | 1049 |
| mb_alc_flag | FL, cMax=1 | 0 | 1052 |

        - Binarization process for psip4x4_code_diff, psip8x8_code_diff, and psip16x16_code_diff

Input to this process is a request for a binarization for the syntax element psip4x4_code_diff, psip8x8_cod_diff, and psip16x16_code_diff.

Output of this process is the binarization of the syntax element.

The bin string of psip4x4_code_diff, psip8x8_cod_diff, and psip16x16_code_diff is derived by the U binarization of the mapped value of the syntax element, where the assignment rule between the signed value of psip4x4_code_diff, psip8x8_cod_diff, psip16x16_code_diff and its mapped value is given as specified in Table 9-3.

      - Decoding process flow

Input to this process is a binarization of the requested syntax element, maxBinIdxCtx, bypassFlag and ctxIdxOffset as specified in subclause Error: Reference source not found.

Output of this process is the value of the syntax element.

This process specifies how each bit of a bit string is parsed for each syntax element.

After parsing each bit, the resulting bit string is compared to all bin strings of the binarization of the syntax element and the following applies.

–	If the bit string is equal to one of the bin strings, the corresponding value of the syntax element is the output.

–	Otherwise (the bit string is not equal to one of the bin strings), the next bit is parsed.

While parsing each bin, the variable binIdx is incremented by 1 starting with binIdx being set equal to 0 for the first bin.

The parsing of each bin is specified by the following two ordered steps:

1.	Given binIdx, maxBinIdxCtx and ctxIdxOffset, ctxIdx is derived as specified in subclause  9.1.3.1.

2.	Given ctxIdx, the value of the bin from the bitstream as specified in subclause Error: Reference source not found is decoded.

        - Derivation process for ctxIdx

Inputs to this process are binIdx, maxBinIdxCtx and ctxIdxOffset.

Output of this process is ctxIdx.

Error: Reference source not found6 shows the assignment of ctxIdx increments (ctxIdxInc) to binIdx for all ctxIdxOffset values for the syntax mb_ivmp_flag, mb_vsskip_flag, mb_direct_type_flag, mb_disp_flag, psip4x4_dir, psip4x4_code_diff, psip8x8_dir, psip8x8_code_diff, psip16x16_dir, and psip16x16_code_diff, mb_alc_skip_flag and mb_alc_flag.

The ctxIdx to be used with a specific binIdx is the sum of ctxIdxOffset and ctxIdxInc, which is found in Error: Reference source not found6. When more than one value is listed in Table I-6 or Error: Reference source not found for a binIdx, the assignment process for ctxIdxInc for that binIdx is further specified in the subclauses given in parenthesis of the corresponding table entry.

All entries in Table I-6 labelled with "na" correspond to values of binIdx that do not occur for the corresponding ctxIdxOffset.

- Table I-6 – Assignment of ctxIdxInc to binIdx for the ctxIdxOffset values related to the syntax elements mb_ivmp_flag, mb_vsskip_flag, mb_direct_type_flag, mb_disp_flag, psip4x4_dir, psip4x4_code_diff, psip8x8_dir, psip8x8_code_diff, psip16x16_dir, psip16x16_code_diff, mb_alc_skip_flag and mb_alc_flag

| ctxIdxOffset | binIdx |  |
| --- | --- | --- |
|  | 0 | >= 1 |
| 1031 | 0,1,2 (subclause 9.1.3.2 ) | na |
| 1034 | 0,1,2 (subclause 9.1.3.3) | na |
| 1037 | 0,1,2 (subclause 9.1.3.3) | na |
| 1040 | 0,1,2 (subclause 9.1.3.4) | na |
| 1043 | 0,1,2 (subclause 9.1.3.5) | na |
| 1046 | 0 | na |
| 1047 | 0 | 1 |
| 1049 | 0,1,2 (subclause 9.1.3.6) | na |
| 1052 | 0,1,2 (subclause 9.1.3.7) | na |

        - Derivation process of ctxIdxInc for the syntax element mb_ivmp_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is not available or mb_ivmp_flag for the macroblock mbAddrN is equal to 0, condTermFlagN is set equal to 1.

–	Otherwise (mbAddrN is available and mb_ivmp_flag for the macroblock mbAddrN is equal to 1), condTermFlagN is set equal to 0.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(I-9-XX)

        - Derivation process of ctxIdxInc for the syntax element mb_vsskip_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is not available or mb_vsskip_flag for the macroblock mbAddrN is equal to 1, condTermFlagN is set equal to 0.

–	Otherwise (mbAddrN is available and mb_vsskip_flag for the macroblock mbAddrN is equal to 0), condTermFlagN is set equal to 1.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(I-9-XX)

        - Derivation process of ctxIdxInc for the syntax element mb_direct_type_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If any of the following conditions is true, condTermFlagN is set to 0:

–	If mbAddrN is not available,

–	Both mb_vsskip_flag and mb_direct_type_flag for the macroblock mbAddrN is equal to 0

–	Otherwise, condTermFlagN is set equal to 1.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(I-9-XX)

        - Derivation process of ctxIdxInc for the syntax element mb_disp_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause Error: Reference source not found is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is not available or mb_disp_flag for the macroblock mbAddrN is equal to 1, condTermFlagN is set equal to 0.

–	Otherwise (mbAddrN is available and mb_disp_flag for the macroblock mbAddrN is equal to 0), condTermFlagN is set equal to 1.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		(I-9-XX)

        - Derivation process of ctxIdxInc for the syntax element mb_alc_skip_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause 6.4.11.1 is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is available and mb_alc_skip_flag or mb_alc_flag for the macroblock mbAddrN is equal to 1, condTermFlagN is set equal to 1.

–	Otherwise (mbAddrN is not available or mb_alc_skip_flag and mb_alc_flag for the macroblock mbAddrN is equal to 0), condTermFlagN is set equal to 0.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB		 (I-9-XX)

        - Derivation process of ctxIdxInc for the syntax element mb_alc_flag

Output of this process is ctxIdxInc.

The derivation process for neighbouring macroblocks specified in subclause 6.4.10.1 is invoked and the output is assigned to mbAddrA and mbAddrB.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

–	If mbAddrN is available and mb_alc_skip_flag or mb_alc_flag for the macroblock mbAddrN is equal to 1, condTermFlagN is set equal to 1.

–	Otherwise (mbAddrN is not available or mb_alc_skip_flag and mb_alc_flag for the macroblock mbAddrN is equal to 0), condTermFlagN is set equal to 0.

The variable ctxIdxInc is derived by:

ctxIdxInc = condTermFlagA + condTermFlagB	(I-9-XX)

  - Profiles and levels

The specifications in Annex I apply. Additional profiles and specific values of profile_idc are specified in the following.

The profiles that are specified in subclause J.10.1 are also referred to as the profiles specified in Annex J.

    - Profiles

All constraints for picture parameter sets that are specified in the following are constraints for picture parameter sets that become the active picture parameter set or an active view picture parameter set inside the bitstream. All constraints for 3DVC sequence parameter sets that are specified in the following are constraints for 3DVC sequence parameter sets that become the active 3DVC sequence parameter set or an active view 3DVC sequence parameter set inside the bitstream.

      - 3D Enhanced High profile

Bitstreams conforming to the 3D Enhanced High profile shall obey the following constraints:

–	The base view bitstream as specified in subclause I.8.5.3 shall obey all constraints of the High profile specified in subclause Error: Reference source not found and all active sequence parameter sets shall fulfill one of the following conditions:

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1,

–	profile_idc is equal to 100.

–	Only I, P, and B slice types may be present.

–	NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

–	Arbitrary slice order is not allowed.

–	Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

–	Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

–	When the profile_idc is equal to 139 in a sequence parameter set, the frame_mbs_only_flag in the sequence parameter set shall be equal to 1.

–	3DVC sequence parameter sets shall have chroma_format_idc equal to 0 only.

–	3DVC sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

–	3DVC sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

–	3DVC sequence parameter sets shall have qpprime_y_zero_transform_bypass_flag equal to 0 only.

–	For each access unit, the value of level_idc for all active view 3DVC sequence parameter set RBSPs shall be the same as the value of level_idc for the active 3DVC sequence parameter set RBSP.

–	Reference picture list 0 for any coded slice shall contain at most one synthetic reference component. Reference picture list 1 for any coded slice shall contain at most one synthetic reference component that shall be the same as the synthetic reference picture included in reference picture list 0 for the same coded slice, if any. The synthetic reference component in reference picture list 0 or 1 of a coded slice of a view component shall be the same as the synthetic reference component in reference picture list 0 or 1, respectively, of any other coded slice of the same view component.

–	The level constraints specified for the 3D Enhanced High profile in subclause I.10.2 shall be fulfilled.

Conformance of a bitstream to the 3D Enhanced High profile is indicated by profile_idc being equal to 139.

Decoders conforming to the 3D Enhanced High profile at a specific level shall be capable of decoding all bitstreams in which both of the following conditions are true:

- All active 3DVC sequence parameter sets have one or more of the following conditions fulfilled:

–	profile_idc is equal to 139,

–	profile_idc is equal to 138,

–	profile_idc is equal to 128,

–	profile_idc is equal to 118 and constraint_set5_flag is equal to 1,

–	profile_idc is equal to 100,

–	profile_idc is equal to 77 or constraint_set1_flag is equal to 1.

- All active 3DVC sequence parameter sets have one or more of the following conditions fulfilled:

–	level_idc or (level_idc and constraint_set3_flag) represent a level less than or equal to the specific level,

–	level_idc[ i ] or (level_idc[ i ] and constraint_set3_flag) represent a level less than or equal to the specific level.

    - Levels

The specification of subclause I.10.2 apply.

      - Level limits common to 3D High profiles

The specification of subclause I.10.2.1 apply.

      - Profile specific level limits

- In bitstreams conforming to the 3D Enhanced High profile, 3DVC sequence parameter sets shall have frame_mbs_only_flag equal to 1 for the levels specified in Error: Reference source not found.

  - Byte stream format

The specifications in Annex Error: Reference source not found apply.

  - 3DVC hypothetical reference decoder

The specifications in Annex Error: Reference source not found apply with substituting 3DVC sequence parameter set for MVC sequence parameter set.

  - 3D-AVC SEI messages

The specifications in Annex D and clause I.13 together with the extensions and modifications specified in this subclause apply.

    - SEI message syntax

      - Constrained depth parameter set identifier SEI message syntax

| constrained_depth_parameter_set_identifier( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| max_dps_id | 5 | ue(v) |
| max_dps_id_diff | 5 | ue(v) |
| } |  |  |

      - Depth view information SEI message syntax

| depth_view_info ( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| depth_type | 5 | ue(v) |
| if( depth_type = = 0 ) { |  |  |
| num_residual_texture_views_minus1 | 5 | ue(v) |
| num_residual_depth_views | 5 | ue(v) |
| } |  |  |
| } |  |  |

    - SEI message semantics

      - Constrained depth parameter set identifier SEI message semantics

When present, this message shall be associated with an IDR access unit. The semantics of the message are valid for the current coded video sequence. A constrained depth parameter set identifier SEI message indicates that depth_parameter_set_id and dps_id values present in the coded video sequence are constrained as specified below.

NOTE 1: When a constrained depth parameter set identifier SEI message is present, decoders are able to conclude losses of depth parameter set NAL units.

max_dps_id plus 1 specifies the maximum allowed depth_range_parameter_set_id value.

max_dps_id_diff specifies the value range of depth_range_parameter_set_id values marked as “used”. max_dps_id_diff * 2 shall be less than max_dps_id.

For each coded slice, the following applies.

–	For the first coded slice of an IDR access unit, MaxUsedDpsId is set equal to “no value”, UsedDpsIdSet is an empty set of values, and all depth range parameter set RBSPs included in the bitstream or made available to the decoding process through external means prior to the access unit containing the IDR picture are marked unavailable.

–	When MaxUsedDpsId is not equal to “no value”, the value of dps_id of the slice header is constrained and the variable updateMaxUsedDpsIdFlag is set as follows.

–	If dps_id is equal to 0, the variable updateMaxUsedDpsIdFlag is set equal to 0.

–	Otherwise, the variable zeroBasedDpsId is equal to dps_id – 1 and dps_id is constrained so that zeroBasedDpsId fulfills the following:

–	If MaxUsedDpsId >= max_dps_id_diff and MaxUsedDpsId <= max_dps_id – max_dps_id_diff, zeroBasedDpsId is in the range of MaxUsedDpsId – max_dps_id_diff to MaxUsedDpsId + max_dps_id_diff, inclusive. The variable updateMaxUsedDpsIdFlag is set equal to ( zeroBasedDpsId > MaxUsedDpsId ).

–	Otherwise, if MaxUsedDpsId < max_dps_id_diff, zeroBasedDpsId is either in the range of 0 to MaxUsedDpsId + max_dps_id_diff, inclusive, or in the range of max_dps_id – ( max_dps_id_diff – MaxUsedDpsId – 1 ) to max_dps_id, inclusive. The updateMaxUsedDpsIdFlag is set equal to ( zeroBasedDpsId > MaxUsedDpsId  &&  zeroBasedDpsId < max_dps_id – ( max_dps_id_diff – MaxUsedDpsId – 1 ) ).

–	Otherwise (MaxUsedDpsId > max_dps_id – max_dps_id_diff), zeroBasedDpsId is either in the range of 0 to max_dps_id_diff – ( max_dps_id – MaxUsedDpsId ) – 1, inclusive, or in the range of MaxUsedDpsId – max_dps_id_diff to max_dps_id, inclusive. The updateMaxUsedDpsIdFlag is set equal to ( zeroBasedDpsId > MaxUsedDpsId  ||  zeroBasedDpsId < max_dps_id_diff – ( max_dps_id – MaxUsedDpsId ) ).

–	When updateMaxUsedDpsIdFlag is equal to 1, depth range parameter set RBSPs are marked as unavailable as follows.

prevMinUsedDpsId = MaxUsedDpsId – max_dps_id_diff
if( prevMinUsedDpsId < 0 )
	prevMinUsedDpsId += max_dps_id
minUsedDpsId = dps_id – 1 – max_dps_id_diff
if( minUsedDpsId < 0 )
	minUsedDpsId += max_dps_id
i = prevMinUsedDpsId
do {
	Mark depth range parameter set RBSP with depth_range_parameter_set_id equal to i + 1, 
	if present, as unavailable.
	i = ( i + 1 ) % ( max_dps_id + 1 )
} while( i != minUsedDpsId )

–	When updateMaxUsedDpsIdFlag is equal to 1 or MaxUsedDpsId is equal to “no value”, the following applies.

–	MaxUsedDpsId is set equal to dps_id – 1.

–	If MaxUsedDpsId is greater than or equal to max_dps_id_diff, UsedDpsIdSet is set to the values in the range of MaxUsedDpsId – max_dps_id_diff to MaxUsedDpsId, inclusive.

–	Otherwise (MaxUsedDpsId is smaller than max_dps_id_diff), UsedDpsIdSet is set to the values in the range of 0 to MaxUsedDpsId, inclusive, and in the range of max_dps_id – ( max_dps_id_diff – MaxUsedDpsId ) to max_dps_id, inclusive.

Any depth parameter set RBSP included in the bitstream or made available to the decoding process through external means and having depth_parameter_set_id equal to any value included in UsedDpsIdSet + 1 has the same content as the previous depth parameter set RBSP included in the bitstream or made available to the decoding process through external means having the same depth_range_parameter_id value.

NOTE 2: If a slice header includes a dps_id value marked as unavailable, a decoder should infer an unintentional loss of a depth parameter set with depth_parameter_set_id value equal to the dps_id value of the slice header.

      - Depth view information SEI message semantics

The depth view information SEI message specifies a different 3D format. The depth view information SEI message when present shall be associated with an IDR access unit. The information signaled in the SEI message applies to all the access units in the coded video sequence.

The 3D format data contains two texture views and one or two depth views. The coded video sequence contains information derived from up to five original texture views and five original depth views which are coded into two texture views and up to two depth views.

When this SEI message is present, inter-view prediction for either texture or depth views is disabled.

depth_type specifies the type of the depth view information. When depth_type is equal to 0, the different 3D format contains coded data corrponding to texture information from up to five views and depth information from up to five depth views. The non-base texture view composites texture information from four views, each of which has a quarter resolution of a texture view component of base view. The non-base depth view composites depth information from four views, each of which has a quarter resolution of a depth view component of base view. When depth_type is equal to 1, to be done. The values of depth_type larger than 2 are reserved.

num_residual_texture_views_minus1 plus1 specifies the number of texture views packed into a texture view component of the non-base view. num_residual_texture_views_minus1 is in the range of 0 to 3, inclusive. Each view that is packed into a texture view component of the non-base view has a quarter resolution of that of the textre view component of the base view. The available quarter-resolution views are present in a texture view component of the non-base view in the raster scan order. [Ed.(CY): specifying the dummy region is unnecessary for the decoder.]

num_residual_depth_views specifies the number of depth views packed into a depth view component of the non-base view. The value of num_residual_depth_views is in the range of 0 to 4, inclusive. When num_residual_depth_views is equal to 0, there is only one depth view present in the coded video sequence. Each view that is packed into a depth view component of the non-base view has a quarter resolution of that of the depth view component of the base view. The available quarter-resolution views are present in a depth view component of a non-base view in the raster scan order. [Ed.(CY): specifying the dummy region is unnecessary for the decoder.]

[Ed. (CY): more informative text that may be present e.g., in a note needs to be provided later.]

NOTE: each quarter-resolution textre or depth view that is packed into a texture or depth view component of the non-base view may be the residual signal between the current view and the base view and has been further downsampled.

  - Video usability information

The specifications in Annex I.14 apply with substituting 3DVC sequence parameter set for MVC sequence parameter set. The mvc_vui_parameters_extension() contained in 3DVC sequence parameter set apply for the 3DV operation points.

--End