| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0185 |
| --- | --- |

| Title: | CE2.a related: MB-level NBDV for 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Jewon Kang Li Zhang Xin Zhao Ye-Kui Wang Rajan Joshi Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8457 jewonk@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com xinzhao@qti.qualcomm.com yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The current 3D-AVC codec supports flexible decoding order for depth and texture. However, there is a big performance gap between the following 2 configurations:

- Configuration 1 (aka depth-first coding): texture followed by depth for the base view, depth followed by texture for other views.

- Configuration 2 (aka texture-first coding): texture followed by depth for all views.

Presently, configuration 1 outperforms configuration 2 by about 19%. In fact, configuration 2 provides only negligible coding gain (about 1%) for texture views compared to MVC+D. Since the decoding order in configuration 2 is needed for stereo/multiview compatibility, it is important that the coding of configuration 2 be improved to achieve significantly higher coding efficiency for texture views as compared to MVC+D.

In this document a technical solution is proposed to improve the performance of the texture-first coding mode with comparable (actually slightly better) performance as the depth-first coding mode (the current best performing mode of 3D-AVC). The technical solution consists of introduction of the so-called Neighboring Block based Disparity Vector (NBDV), which is included in 3D-HEVC, with simplifications, into 3D-AVC, to be applied in addition to all existing coding tools that bring coding gains for the depth-first coding mode but not the texture-first coding mode.

The proposed method provides 20% BD-rate saving for the texture only coding compared to MVC+D, as evaluated by the bitrates of the texture views and the average PSNR values of the texture views and about 25% BD-rate saving for the overall, as evaluated by the total bitrate and the average PSNR of the synthesized views. When enabling BVSP, 6% additional BD-rate saving for the overall can be achieved to reach the best coding performance. Even in this case, there is no need to decode each depth view component for a texture view component in the same non-base view.

# Introduction

This section introduces the flexible coding order which enables the two configurations. The coding performance under these two configurations is also studied.

## Flexible coding order in 3D-AVC

3D-AVC supports so-called “flexible coding order” in terms of the decoding order of the texture and depth view components within each access unit.

### Depth-first coding

In the current common test conditions (CTC), the decoding order is called “depth-first coding”, wherein for each non-base view, the depth view component is followed by the texture view component, although, in the base view, texture view component is followed by the depth view component.

The depth-first coding is illustrated in Figure 1. View components of one access unit are shown in this figure with the numbers being the view order index values and T/D being the texture/depth view. In Figure 1, the dependency within one view is illustrated with arrows in different colors while the inter-view dependency is not shown in this figure for simplicity. As shown in Figure 1, the base view is still AVC (i.e., high profile) compatible, and the depth view component can be coded pending on the texture view component of the base view. However, for each non-base view, the texture view component MUST depend on the associated depth view component for decoding. In addition, there are view dependency among the texture views and view dependency among the depth views.

Figure 1: Depth-first coding.

In such a configuration, to decode the texture view component of each non-base view, its associated depth view component (of the same view) needs also to be decoded.

The main motivation of depth-first coding is to facilitate the disparity vector generation for each non-base view such that inter-view prediction coding tools can be made more efficient. However it is questionable such a configuration is really needed if there is a method of deriving disparity vectors when just accessing the texture views.

### Texture-first coding

Texture-first coding is enabled in the current 3D-AVC codec to meet an MPEG requirement on stereo/multiview compatibility.

In the requirement document of the 3DV CfP (w12035) [1], it is stated that “The compressed data format shall include a mode that enables the simple extraction of bitstreams for stereo and mono output, and support high-fidelity reconstruction of samples from the left and right views of the stereo video.”

Also, it is desirable to have to have the multi-view compatibility. Therefore, the current 3D-AVC enables the texture-first coding to support the stereo/multiview compatibility.

The texture-first coding is illustrated in Figure 2, wherein there is within a view, a texture view component doesn’t depend on a depth view component.

Figure 2: Texture-first coding.

In this configuration, it is possible that only texture views need to be decoded when decoding a texture-only operation point. Therefore, bandwidth required for transmitting such a texture only operation point may be significantly reduced, and the complexity of decoding depth view components could be avoided.

## Coding performance for the two 3D-AVC configurations

We provide comparison between texture-first coding 3D-AVC, depth-first coding 3D-AVC, and MVC+D both under the ATM 7.0 software.

### 3D-AVC depth-first coding vs MVC+D

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB |
| S01 | 41.74 | -1.12 | -7.08 | 0.38 | 46.13 | -1.23 | 43.67 | -1.20 |
| S02 | 13.10 | -0.36 | -19.68 | 0.92 | 16.39 | -0.45 | 14.78 | -0.42 |
| S03 | 22.77 | -0.71 | -25.91 | 2.39 | 20.94 | -0.67 | 23.87 | -0.69 |
| S04 | 30.08 | -0.96 | 4.28 | -0.23 | 30.39 | -0.98 | 29.92 | -0.91 |
| S05 | 30.47 | -1.28 | -13.56 | 0.73 | 42.84 | -1.66 | 39.45 | -1.42 |
| S06 | 37.77 | -1.63 | -14.95 | 0.73 | 44.39 | -1.82 | 38.38 | -1.49 |
| S08 | 17.33 | -0.69 | -24.86 | 1.21 | 30.80 | -1.14 | 22.00 | -0.72 |
| Average | 27.61 | -0.96 | -14.54 | 0.87 | 33.13 | -1.14 | 30.29 | -0.98 |

The depth-first coding mode of 3D-AVC significantly outperforms MVC+D, as indicated by the synthesized results, by about 30%.

The texture coding results, however, are problematic since the bitrates used in the current template (provided by CTC) only considers the bitrates of the texture views, while a slimmest operation point providing the texture views requires all the depth views and texture views. Therefore the bitrates of the depth views need to be considered.

### 3D-AVC texture-first coding vs MVC+D

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB |
| S01 | 0.54 | -0.02 | -4.05 | 0.21 | 6.60 | -0.24 | 9.12 | -0.33 |
| S02 | 0.34 | -0.01 | -15.08 | 0.67 | 4.85 | -0.15 | 5.20 | -0.17 |
| S03 | 0.40 | -0.01 | -20.24 | 1.83 | 0.51 | -0.02 | 7.17 | -0.23 |
| S04 | 0.11 | 0.00 | 19.75 | -1.05 | 2.76 | -0.11 | 6.29 | -0.23 |
| S05 | 2.16 | -0.12 | -11.85 | 0.63 | 16.59 | -0.79 | 15.43 | -0.68 |
| S06 | 3.14 | -0.17 | -9.21 | 0.44 | 12.05 | -0.60 | 10.44 | -0.48 |
| S08 | 1.93 | -0.09 | -22.30 | 1.07 | 15.81 | -0.65 | 9.60 | -0.35 |
| Average | 1.23 | -0.06 | -9.00 | 0.54 | 8.45 | -0.37 | 9.03 | -0.35 |

The texture-first coding mode of 3D-AVC provides almost the same (1% better) coding performance as MVC+D considering the texture only operation point (evaluated by the PSNR of texture views and the bitrates of the texture views), and it outperforms MVC+D, by only about 9% when evaluated by the synthesized results and overall bitrates.

### 3D-AVC texture-first coding vs 3D-AVC depth-first coding

It is noticed that texture-first coding has a significant loss, by 19%, compared to depth-first coding, as configured in the common test condition (CTC) of JCT-3V.

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB |
| S01 | 40.72 | -1.09 | -2.87 | 0.19 | 36.69 | -1.01 | 31.27 | -0.89 |
| S02 | 12.64 | -0.35 | -6.49 | 0.30 | 10.94 | -0.31 | 9.08 | -0.26 |
| S03 | 22.24 | -0.70 | -6.58 | 0.56 | 20.33 | -0.66 | 15.45 | -0.45 |
| S04 | 29.93 | -0.96 | -12.22 | 0.80 | 26.94 | -0.88 | 22.00 | -0.69 |
| S05 | 27.57 | -1.18 | -2.88 | 0.14 | 22.35 | -0.93 | 20.78 | -0.79 |
| S06 | 33.30 | -1.47 | -7.22 | 0.34 | 28.58 | -1.25 | 25.26 | -1.03 |
| S08 | 14.97 | -0.60 | -3.88 | 0.19 | 12.80 | -0.51 | 11.43 | -0.39 |
| Average | 25.91 | -0.91 | -6.02 | 0.36 | 22.66 | -0.79 | 19.33 | -0.64 |

# Problems

The shortcomings of the current 3D-AVC design are summarized as follows:

- The best performing configuration (depth-first coding) always requires decoding of the depth view component of a non-base view to decode the texture view component of the same view. In other words, it is impossible to have a texture-only operation point. In applications where a texture only operation point is desired, the bandwidth and the decoding complexity are still high, as transmission and decoding of the depth view components are always needed. For example, when the bitrates of the depth views are taken into account, the depth-first coding configuration outperforms MVC+D for texture coding by around only 12%. Furthermore, it requires decoding of six view components for the 3-view case, instead of three view components for MVC+D.

- When configured in a way that texture view component can be decoded without accessing its depth view component, i.e. texture-first decoding, majority of the coding tools cannot work properly due to the missing disparity vector used for these coding tools. In such a configuration, 19% coding loss compared to the common test condition (CTC) with depth-first coding is observed. The performance if evaluated by texture views, is about 1% better compared to MVC+D, or MVC, as shown in section 1.2.2.

# Proposal

It is proposed that for texture-first coding in 3D-AVC, all the existing coding tools are enabled. An additional tool called Neighboring Block based Disparity Vector (NBDV) is proposed. This is a simplified version of a similar tool, included in 3D-HEVC. By utilizing the disparity vector derived from the NBDV, there is no need to access the depth view component of the same view for decoding any texture view component to get disparity vectors converted from depth values.

The proposed method enables all the existing tools in the texture-first mode.

## Proposal description

We propose to introduce MB-level NBDV into 3D-AVC. Similar to 3D-HEVC, neighboring blocks are checked in order, and, once a neighboring block contains a disparity motion vector (thus it is inter-view predicted), the motion vector is identified to be a disparity vector, and the whole process terminates. The identified disparity vector is the result of the NBDV process.

The result of the NBDV process is used to replace the disparity vector which is currently calculated based on the depth view component of the same view under the current 3D-AVC depth-first configuration.

Detailed spec. text changes are included, and the NBDV process described below is mainly included in subclause J.8.3.1.10.

### Spatial neighboring blocks

The spatial neighboring blocks that will be checked in AVC motion prediction process are checked in the order of A (left), B (above), C (above-right), and D (above-left) in the proposed NBDV process. The spatial neighboring blocks are shown as in Figure 3.

Figure 3: Spatial neighboring blocks for NBDV.

### Temporal neighboring blocks

Blocks from up to two reference pictures in the same view of the current picture are checked in order: RefPicList1[ 0 ] and RefPicList0[ 0 ] for B slices and RefPicList0[ 0 ] for P slices.

Currently three temporal blocks are checked picture by picture and for each picture, the co-located blocks relative to the co-located MB are checked as indicated below in order of BR (bottom-right), CT3 (center 3) and CO2 (corner 2). The temporal neighboring blocks are shown as in Figure 4.

Figure 4: Temporal neighboring blocks for NBDV.

### Termination of the process

The above mentioned neighboring blocks are checked in order. Similar to 3D-HEVC, temporal neighboring blocks are checked first and the spatial neighboring blocks are checked afterwards. Once a block contains an available disparity motion vector, the derivation process terminates.

## Highlights of the other aspects of the proposed method

- When accessing base view depth is enabled (e.g., in BVSP mode), disparity vector is further refined (namely NBDV-R) by the base view depth similar to 3D-HEVC. Based on the NBDV-R result, BVSP can be enabled similar to that in 3D-HEVC.

- A implicit disparity vector (IDV) may be stored in 3D-HEVC to help generate a disparity vector of a future block. This is not used in the currently proposed method in 3D-AVC. A simplified method of IDV, but without additional storage, is proposed in a different proposal JCT3V-D0186.

- The temporal neighboring blocks are checked before spatial blocks in the current implementation. Besides, if the order is changed (i.e., checking spatial neighboring blocks first), the coding performance of the spatial neighboring block first becomes slightly worse than the temporal neighboring block first.

- Only the motion vectors of neighboring blocks corresponding to RefPicList0 are checked during the NBDV process and motion vectors corresponding to RefPicList1 are not checked.

# Simulations results

In this section, simulation results are provided with different configurations and compared with MVC+D and 3D-AVC CTC respectively Error: Reference source not found. The software implementation as provided in this document is based on ATM 7.0.

“BVSP off” indicates that no texture view is coded based on any depth view.

“BVSP on” indicates that no texture view is coded based on any depth view of a non-base view, but non-base texture views are coded with access to the depth view components of the base view.

Note that unlike ATM 6.0, the IVMP has a bug when enabling texture-first coding, due to JCT3V-C0136 [2]. A simple bug fix is provided to avoid crash and/or mismatch. More detailed results on IVMP in a different proposal JCT3V-D0189.

## Proposal (BVSP off) vs MVC+D

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB |
| S01 | 27.48 | -0.82 | -3.04 | 0.16 | 33.03 | -0.98 | 32.73 | -0.99 |
| S02 | 7.88 | -0.23 | -14.61 | 0.65 | 12.19 | -0.35 | 11.62 | -0.34 |
| S03 | 16.88 | -0.55 | -18.14 | 1.60 | 16.10 | -0.54 | 20.52 | -0.61 |
| S04 | 19.71 | -0.66 | 18.74 | -1.00 | 21.60 | -0.73 | 23.06 | -0.73 |
| S05 | 22.45 | -1.02 | -11.46 | 0.60 | 36.70 | -1.50 | 33.14 | -1.26 |
| S06 | 29.75 | -1.35 | -8.46 | 0.40 | 38.25 | -1.63 | 32.56 | -1.32 |
| S08 | 13.08 | -0.54 | -22.11 | 1.06 | 27.38 | -1.04 | 19.05 | -0.64 |
| Average | 19.60 | -0.74 | -8.44 | 0.49 | 26.46 | -0.97 | 24.67 | -0.84 |

As shown in the above table, the proposed method when BVSP is turned off, outperforms MVC+D for 20% for texture only coding and 25% for the overall (evaluated by the synthesized views).

## Proposal (BVSP on) vs MVC+D

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB |
| S01 | 40.58 | -1.09 | -3.05 | 0.16 | 45.46 | -1.22 | 43.88 | -1.21 |
| S02 | 12.85 | -0.36 | -15.04 | 0.67 | 16.92 | -0.46 | 15.41 | -0.44 |
| S03 | 22.15 | -0.69 | -18.27 | 1.60 | 20.89 | -0.67 | 25.17 | -0.72 |
| S04 | 29.96 | -0.96 | 20.20 | -1.06 | 31.50 | -1.01 | 31.88 | -0.96 |
| S05 | 28.22 | -1.21 | -11.46 | 0.61 | 41.52 | -1.63 | 38.71 | -1.40 |
| S06 | 35.56 | -1.56 | -8.67 | 0.41 | 43.57 | -1.80 | 37.51 | -1.47 |
| S08 | 16.33 | -0.65 | -21.90 | 1.04 | 30.56 | -1.13 | 21.96 | -0.72 |
| Average | 26.52 | -0.93 | -8.31 | 0.49 | 32.92 | -1.13 | 30.65 | -0.99 |

As shown in the above table, after enabling BVSP, the proposed method outperforms MVC+D for 27% for texture only coding and 31% for the overall (evaluated by the synthesized views).

## Proposal vs 3D-AVC (depth-first)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB |
| S01 | 0.72 | -0.02 | -4.15 | 0.26 | 0.36 | -0.01 | -0.26 | 0.01 |
| S02 | 0.14 | 0.00 | -6.52 | 0.31 | -0.54 | 0.02 | -0.62 | 0.02 |
| S03 | 0.07 | 0.00 | -9.31 | 0.79 | -0.36 | 0.01 | -1.45 | 0.04 |
| S04 | 0.23 | -0.01 | -12.99 | 0.84 | -0.72 | 0.03 | -1.35 | 0.05 |
| S05 | 1.65 | -0.08 | -3.36 | 0.16 | 0.86 | -0.04 | 0.34 | -0.01 |
| S06 | 1.38 | -0.07 | -7.93 | 0.38 | 0.36 | -0.02 | 0.33 | -0.01 |
| S08 | 0.77 | -0.03 | -4.11 | 0.20 | 0.12 | 0.00 | -0.08 | 0.00 |
| Average | 0.71 | -0.03 | -6.91 | 0.42 | 0.01 | 0.00 | -0.44 | 0.01 |

As shown in the above table, after enabling BVSP, the proposed provides the same performance as 3D-AVC best performing configuration, when evaluated by the overall bitrates and the synthesized views.

## Additional results

The method as proposed in this proposal is technical the same as that discussed in USNB in early March 2013. The proposed NBDV can be further improved, as shown in JCT3V-D0186, especially when BVSP off, which is about 2.2% for texture and 1.8% for synthesized views. The summarized results, when BVSP is turned on, as compared with 3D-AVC, are shown in the table below.

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB |
| Average | 0.58 | -0.03 | -6.98 | 0.43 | -0.10 | 0.00 | -0.55 | 0.02 |

# Conclusion

To avoid relying on depth view component of the same view when decoding each texture view component of a non-base view, NBDV as in 3D-HEVC is used in 3D-AVC with simplifications to derive the disparity vector required for coding tools.

The proposed method enables all the existing coding tools thus even provides slight gain compared with the best performing 3D-AVC configuration.

The proposed method provides 20% gain for the texture only coding compared to MVC+D, as evaluated by the bitrates of the texture views and the PSNR values of the texture views and 25% gain for the overall, as evaluated by the total bitrate and the PSNR of the synthesized views. This method can be further improved with small additional changes as shown in JCT3V-D0186.

When enabling BVSP, 4~6% additional gain can be achieved to reach the best coding performance. Even in this case, there is no need to decode each depth view component for a texture view component in the same non-base view.

With the proposed method, depth-first coding becomes unnecessary.

# References

- Video and Requirements Group, " Applications and Requirements on 3D Video Coding," ISO/IEC JTC1/SC29/WG11, w12035, Geneva, Switzerland, March 2011.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1100, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

- J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek), " 3D-CE3.a related: Unconstrained inside-view motion prediction in 3D video coding," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C0136, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).