| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0186 |
| --- | --- |

| Title: | CE2.a related: Derived disparity vector in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Ying Chen Li Zhang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-5776-0696 xinzhao@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal improves the disparity vector derivation method (MB-level NBDV) as proposed in JCT3V-D0185. For a whole slice, one dynamic derived disparity vector (DDV) is maintained. When the MB-level NBDV fails to return an available disparity vector, the disparity vector is set to the DDV. The DDV is updated to be the same as the disparity vector of a current MB once it is coded. Therefore, unlike the similar method in 3D-HEVC, the proposed method has the advantage of reducing the potential storage of the disparity vectors for each block.

The proposed method provides 2.2% additional gain for the texture only coding and 1.8% gain if evaluated by synthesized views compared to JCT3V-D0185 when BVSP is turned off. Compared to MVC+D, the results become 22.4% for texture coding and 27.0% if evaluated by the synthesized views. It also slightly improves the performance when BVSP is turned on, such that the coding gain becomes 0.6% bitrate saving compared with CTC.

# Introduction

In JCT3V-D0185, NBDV has been proposed for multiview compatible 3D-AVC and can be used for BVSP when accessing the depth of the base view is allowed.

It is possible that the derived disparity vector is not accurate for certain MBs, when the NBDV process doesn’t reach a neighboring block containing a disparity motion vector, it may simply return an unavailable disparity vector and it is further set to 0.

# Proposal

It is proposed that a single derived disparity vector, namely Dynamic derived Disparity Vector (DDV) is maintained for the whole slice. After each current MB is decoded, the DDV is updated to be the derived disparity vector of the current MB.

In the NBDV process, as shown in Figure 1, if the neighboring blocks (temporal and spatial neighboring blocks) do not provide an available disparity motion vector that can be converted to disparity vector, the proposed DDV (highlighted in yellow) is set to be the disparity vector of the current MB.

Figure 1: Checking order of neighboring blocks and proposed DDV in NBDV process.

The working draft text (JCT3V-D0186_WD) of this proposal on top of JCT3V-D0185 is attached with this document. The JCT3V-D0186_WD is based on the text changes for JCT3V-D0185, with the newly added changes in this proposal highlighted in green. As can be seen from the provided working draft text, the changes for this proposal is very small.

# Simulations results

Four sets of results are provided:

- Proposed method compared with JCT3V-D0185 when BVSP is turned off.

- Proposed method compared with MVC+D when BVSP is turned off.

- Proposed method compared with CTC when BVSP is turned on.

- Proposed method compared with JCT3V-D0185 when BVSP is turned on.

For the test conditions in the above four tests, “BVSP off” indicates that no texture view is coded based on any depth view, and “BVSP on” indicates that no texture view is coded based on any depth view of a non-base view, but non-base texture views are coded with access to the depth view components of the base view.

## Proposal vs JCT3V-D0185 (BVSP off)

In this test, the anchor is the MB-level NBDV for 3D-AVC (JCT3V-D0185) based on ATM v7.0, and the tested method is the MB-level NBDV for 3D-AVC with proposed DDV based on ATM v7.0. For both anchor and tested method, texture first coding is applied and BVSP is turned off in this test.

Table 1: Proposal vs JCT3V-D0185 (BVSP off)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB |
| S01 | -5.7 | 0.2 | -0.4 | 0.0 | -5.2 | 0.2 | -4.7 | 0.2 |
| S02 | -0.5 | 0.0 | 0.2 | 0.0 | -0.4 | 0.0 | -0.3 | 0.0 |
| S03 | -1.0 | 0.0 | -0.3 | 0.0 | -0.9 | 0.0 | -0.8 | 0.0 |
| S04 | -0.6 | 0.0 | -0.1 | 0.0 | -0.6 | 0.0 | -0.4 | 0.0 |
| S05 | -2.4 | 0.1 | -0.2 | 0.0 | -2.1 | 0.1 | -1.9 | 0.1 |
| S06 | -2.8 | 0.1 | 0.1 | 0.0 | -2.5 | 0.1 | -2.1 | 0.1 |
| S08 | -2.5 | 0.1 | -0.1 | 0.0 | -2.2 | 0.1 | -2.1 | 0.1 |
| Average | -2.2 | 0.1 | -0.1 | 0.0 | -2.0 | 0.1 | -1.8 | 0.1 |

From the Table 1, it is reported that, compared to the MB-level NBDV for 3D-AVC as proposed in JCT3V-D0185, the proposed method further achieves -2.2% coding gain for texture coding, where the bitrates and PSNR values of the texture views are used for evaluation, and -1.8% coding gain for Total (synthesized PSNR), where the total bitrate and the PSNR of the synthesized views are used for evaluation.

## Proposal vs MVC+D (BVSP off)

In this test, the anchor is the MB-level NBDV for 3D-AVC (JCT3V-D0185) using proposed DDV, and the tested method is MVC+D based on ATM v7.0. For both anchor and tested method, texture first coding is applied and BVSP is turned off in this test.

Table 2: Proposal vs MVC+D (BVSP off)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB |
| S01 | 35.1 | -1.0 | -2.8 | 0.1 | 40.3 | -1.1 | 39.4 | -1.1 |
| S02 | 8.4 | -0.2 | -14.8 | 0.7 | 12.7 | -0.4 | 12.0 | -0.4 |
| S03 | 18.0 | -0.6 | -17.9 | 1.6 | 17.2 | -0.6 | 21.6 | -0.6 |
| S04 | 20.4 | -0.7 | 18.8 | -1.0 | 22.3 | -0.7 | 23.6 | -0.7 |
| S05 | 25.5 | -1.1 | -11.3 | 0.6 | 39.6 | -1.6 | 35.7 | -1.3 |
| S06 | 33.5 | -1.5 | -8.5 | 0.4 | 41.8 | -1.8 | 35.4 | -1.4 |
| S08 | 16.0 | -0.6 | -22.1 | 1.1 | 30.3 | -1.1 | 21.6 | -0.7 |
| Average | 22.4 | -0.8 | -8.4 | 0.5 | 29.2 | -1.0 | 27.0 | -0.9 |

From the Table 2, it is reported that, compared to MVC+D, the MB-level NBDV for 3D-AVC (JCT3V-D0185) using proposed method now achieves 22.4% coding gain for texture coding, where the bitrates and PSNR values of the texture views are used for evaluation, and 27% coding gain for Total (synthesized PSNR), where the total bitrate and the PSNR of the synthesized views are used for evaluation.

## Proposal vs CTC (BVSP on)

In this test, the anchor is the ATM v7.0 with depth first coding, and the tested method is the MB-level NBDV for 3D-AVC with proposed DDV based on ATM v7.0 with texture first coding. For both anchor and tested method, the BVSP is enabled with 8x8 size in this test.

Table 3: Proposal vs CTC (BVSP on)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB |
| S01 | 0.3 | 0.0 | -4.4 | 0.3 | 0.0 | 0.0 | -0.5 | 0.0 |
| S02 | 0.1 | 0.0 | -6.4 | 0.3 | -0.6 | 0.0 | -0.6 | 0.0 |
| S03 | 0.1 | 0.0 | -9.4 | 0.8 | -0.3 | 0.0 | -1.4 | 0.0 |
| S04 | 0.2 | 0.0 | -13.1 | 0.9 | -0.7 | 0.0 | -1.4 | 0.0 |
| S05 | 1.6 | -0.1 | -3.4 | 0.2 | 0.8 | 0.0 | 0.3 | 0.0 |
| S06 | 1.1 | -0.1 | -7.9 | 0.4 | 0.2 | 0.0 | 0.1 | 0.0 |
| S08 | 0.6 | 0.0 | -4.4 | 0.2 | -0.1 | 0.0 | -0.3 | 0.0 |
| Average | 0.6 | 0.0 | -7.0 | 0.4 | -0.1 | 0.0 | -0.6 | 0.0 |

From the above Table 3, compared to CTC, it is reported that the MB-level NBDV for 3D-AVC (JCT3V-D0185) has been improved to provided 0.6% coding gain for Total (synthesized PSNR), where the total bitrate and the PSNR of the synthesized views are used for evaluation.

## Proposal vs JCT3V-D0185 (BVSP on)

In this test, the anchor is the MB-level NBDV for 3D-AVC (JCT3V-D0185) based on ATM v7.0, and the tested method is the MB-level NBDV for 3D-AVC with proposed DDV based on ATM v7.0. For both anchor and tested method, texture first coding is applied and BVSP is turned on.

Table 4: DDV vs JCT3V-D0185 (BVSP on)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB | dBR,% | dPSNR,dB |
| S01 | -0.4 | 0.0 | -0.2 | 0.0 | -0.4 | 0.0 | -0.3 | 0.0 |
| S02 | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| S03 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| S04 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| S05 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| S06 | -0.2 | 0.0 | 0.0 | 0.0 | -0.2 | 0.0 | -0.2 | 0.0 |
| S08 | -0.2 | 0.0 | -0.3 | 0.0 | -0.2 | 0.0 | -0.2 | 0.0 |
| Average | -0.1 | 0.0 | -0.1 | 0.0 | -0.1 | 0.0 | -0.1 | 0.0 |

As shown in the above table, after enabling BVSP, compared to the MB-level NBDV for 3D-AVC (JCT3V-D0185), the proposed DDV further achieves -0.1% coding gain for texture coding, and -0.1% coding gain for Total (synthesized PSNR), where the total bitrate and the PSNR of the synthesized views are used for evaluation.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).