| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0188 |
| --- | --- |

| Title: | Simplified adaptive luminance compensation in 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jewon Kang Ying Chen Li Zhang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8457 jewonk@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal presents simplification of deriving a prediction weight of adaptive luminance compensation (ALC) in 3D/AVC. In the current ATM design, the above and the left regions of the current block are matched in the reference picture using a motion vector, and the prediction weight derivation process is performed to predict the luminance discrepancy of the sample pixels. The derivation process is performed once in a macroblock (MB) partition level if it does not contain any sub-marcoblock partition (sub-MB). However, in the other hand, if the MB partition contains more than one sub-MB partition, the current design still requires 4x4 block processing for all the blocks in the sub-MB. In this contribution, it is proposed to derive the prediction weights just once for a MB partition regardless of the sub-MB partition. Therefore, the 4x4 block processing is removed, and, thus the worst case number of prediction weight derivation in ALC is reduced with a minor change in coding efficiency.

# Introduction

In the current 3D-AVC [1], adaptive luminance compensation (ALC) is employed to compensate the luminance discrepancy in inter-view pictures, and achieves a significant coding gain [2]. Each luma sample in a macroblock (MB) partition is predicted with a set of prediction weights derived using above and left regions of the coded block. The above and left regions for the coded block, and the reference block are matched using motion compensation with interpolation. The estimated prediction weights are applied to a block in P_L0_16x16, P_L0_L0_16x8, P_L0_L0_8x16, P_8x8, and P_8x8ref0 modes, which is performed in a similar manner to the H.264/AVC weighted prediction

The above and the left regions, respectively, have the Mx4 and 4xN block shapes, wherein M and N are decided by the MB partition. The prediction weight derivation process is performed one time per MB partition if the MB partition does not contain more than one sub-MB partition [3]. For example, as shown in Figure 1, for the 16x8 MB partition, the left and the above region are decided with 4x8 and 16x4, respectively, in the neighbor. However, if an 8x8 MB partition has more than one sub-MB partition, the situation changes. All the 4x4 blocks in the sub-MB are coded with the ALC in series as shown in Figure 2, and this 4x4 block processing is the same regardless of sub-MB partitions. Hence, we propose a simplification of the prediction weight derivation in the ALC so as to reduce the complexity while changing a minimal coding gain in this contribution.

Figure 1: Example for the left and the above block regions used in 16x8 MB partition.

Figure 2: Example for the prediction weight derivation in series when an 8x8 MB contains more than one sub-MB partitions.

# Proposal

We modify the current ALC design in this proposal. The contributions of this proposal are summarized into two points:

- The prediction weight is derived one time for the MB partition. Previously, if an MB partition is 8x8 with multiple sub-MB partitions, the prediction weight derivation was performed for every 4x4 blocks. In this aspect, the worst number of prediction weight derivation is reduced.

- The prediction weight derivation is performed in the first sub-MB partition that has the left block and the above block located outside the coded MB-partition. This aspect provides a feature that can facilitate a parallel processing.

Figure 3 presents an example of the proposed prediction weight derivation when the MB partition has two 4x8 sub-MB partition. As can be seen, the prediction weight (w0) is derived using the first sub-MB partition, and applied to the second sub-MB partition. Furthermore, the first sub-MB partition does not need to be reconstructed ahead the second sub-MB partition for the decoding.

It is also noted that, in the proposed simplification, the size of the left and the above block regions are decided based upon the size of the sub-MB partition as the width of the sub-MB partition by 4 and 4 by the height of the sub-MB partition. For example, the blocks (4x8 and 4x4) are shown in Figure 3.

Figure 3: Proposed prediction weight derivation in sub-MB partition

# Experimental results

Simulation results of the proposal are shown in Table1. The implementation was based on ATM7.0, and simulations are done under common test conditions [4].

As shown in Table 1, the proposed algorithm affects only minor change (i.e. 0.01% BD-rate increase) in the coding efficiency, and can reduce the complexity in decoding throughput and in implementation. The rendering time is not reported because it is not relevant.

Table 1 – Proposed method VS ATM7.0

# References

- M. M. Hannuksela, Y. Chen, T. Suzuki, J.-R. Ohm, G. J. Sullivan, “3D-AVC draft text 5”, JCT3V-C1002, Jan. 2012.

- M. Mishurovskiy, A. Fartukov, I. Kovliga, J. Lee, “3D-CE2.a: A results on inter-view coding with adaptive luminance compensation,” JCT3V-B0031, Oct. 2012.

- J. Kang, Y. Chen, L. Zhang, and M. Karczewicz, “Simplifications for adaptive luminance compensation in 3D-AVC,” JCT3V-C0054, Jan. 2012.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-B1100, Oct. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).