| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0189 |
| --- | --- |

| Title: | Bug fix and simplifications for ATM IVMP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +86-10-5776-0696 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ATM v7.0, when texture-first (TF) coding order is applied, encoding failure occurs when a potential undefined encoding process is activated. Basically, the problem is caused by JCT3V-C0136 which enables Inside View Motion Prediction (IVMP) when the mb_type of co-located texture macroblock is P8x8. This proposal first provides a bug fix for this problem. Furthermore, when a simplification is proposed, it is reported that there is average 0.00% BD rate difference on synthesized views compared to ATM with proposed bug fix under common test condition.

# Introduction

In the last meeting, an unconstrained IVMP process proposed in JCT3V-C0136 is adopted in 3D-AVC. With the unconstrained IVMP process, IVMP flag can be enabled when partial partition of co-located texture macroblock (MB) is coded by inter_8x8, view synthesis prediction, or intra prediction. More specifically, when reduced depth resolution is utilized and the mb_type of co-located texture MB equals P8x8, the followings are applied to derive reference indices and motion vectors in IVMP, as specified in JCT3V-C1002 [1],

–	If both of refIdxLXCol[ 0 ] and refIdxLXCol[ 3 ] are -1, refIdxLX[ mbPartIdx ] is set to -1.

–	Otherwise, if refIdxLXCol[ 0 ] is equal to refIdxLXCol[ 3 ], refIdxLXCol[ 0 ] is assigned to refIdxLX[mbPartIdx]. The sub-macroblock partition size for 8x8 partition mbPartIdx is set to 8x4, 4x8 or 4x8 correspoding to the partition size of ColMb equal to 16x8, 8x16 or 8x8, respectively. The luma motion vectors of the two 8x8 partitions are inherited by the two 8x4 or 4x8 partitions of the 8x8 partiotion mbPartIdx and scaled by the subclause Error: Reference source not found.

–	Otherwise, if refIdxLXCol[ i ] (i being 0 or 3) is unequal to -1 and refIdxLXCol[ j ] (j being 3 or 0) is equal to -1, refIdxLXCol[ i ] is assigned to refIdxLX[mbPartIdx] and the luma motion vectors of 8x8 partition with index i in ColMb are inherited by the 8x8 partiotion mbPartIdx and scaled by the subclause Error: Reference source not found.

–	Otherwise, refIdxLXCol[ i ] is assigned to refIdxLX[ mbPartIdx ] with i equal to mbPartIdx and the luma motion vectors of the 8x8 partition with index i in ColMb are inherited by the 8x8 partition mbPartIdx and scaled by the subclause Error: Reference source not found.

The highlighted (in yellow) part of above process indicates a case when the left-top and right-bottom 8x8 blocks in co-located texture MB have non-negative but different reference index values. In this case, the MB partition of the co-located texture MB with the same partition index as that of the 8x8 block in the current depth MB is chosen to predict the motion information (i.e., reference index and motion vector) of the current 8x8 depth MB partition.

Figure 1: Examples of depth block partitioning

To describe the problem of the above highlighted part in JCT3V-C1002, as illustrate in Figure 1, for a depth MB including four 8x8 blocks (e.g., d0,…,d3), the co-located texture region includes four MBs (B0, B1, B2 and B3). Assume mb_type of B1 is P8x8 and both b0 and b3 are uni-directionally predicted from reference list X with different reference indexes, the motion information of b1 is inherited for d1 according to the above highlighted texts. However, when b1 is uni-directionally predicted from reference list 1-X, the motion information of b1 cannot be inherited and encoding failure occurs.

It is also noticed that although it sounds the handling of 8x8 MB partition is designed to reuse the existing logic for 16x8 and 8x16 MB partitions, they are totally different problems and quite some unclear places in the current working draft text are identified due to the inclusion of the case for 8x8 MB partitions.

Note that in addition to the above technique brick, JCT3V-C0136 also proposes to set reference index and motion vectors to zero values when the motion info. is not available. This part is useful.

# Proposal

## Bug fix for ATM

To resolve the encoding failure caused by an undefined motion inheritance process, it is proposed to modify the derivation of reference indices and motion vectors in IVMP as below, newly added texts are highlighted in green and modification to the original texts are in red.

–	If both of refIdxLXCol[ 0 ] and refIdxLXCol[ 3 ] are -1, refIdxLX[ mbPartIdx ] is set to -1.

–	Otherwise, if refIdxLXCol[ 0 ] is equal to refIdxLXCol[ 3 ], refIdxLXCol[ 0 ] is assigned to refIdxLX[mbPartIdx] and the sub-macroblock partition size for 8x8 partition mbPartIdx is set to 8x4, 4x8 or 4x8 correspoding to the partition size of ColMb equal to 16x8, 8x16 or 8x8, respectively. The luma motion vectors of the two 8x8 partitions are inherited by the two 8x4 or 4x8 partitions of the 8x8 partiotion mbPartIdx and scaled by the subclause Error: Reference source not found.

–	Otherwise, if refIdxLXCol[ i ] (i being 0 or 3) is unequal to -1 and refIdxLXCol[ j ] (j being 3 or 0) is equal to -1, refIdxLXCol[ i ] is assigned to refIdxLX[mbPartIdx], and the luma motion vectors of 8x8 partition with index i in ColMb are inherited by the 8x8 partiotion mbPartIdx and scaled by the subclause Error: Reference source not found.

–	Otherwise, the following applies:

- If refIdxLXCol[ mbPartIdx ] is not -1, i is set equal to mbPartIdx ;

- Otherwise, i = (refIdxLXCol[ 0 ]< refIdxLXCol[ 3 ] ? 0: 3);

- refIdxLXCol[ i ] is assigned to refIdxLX[ mbPartIdx ] with i equal to mbPartIdx and the luma motion vectors of the 8x8 partition with index i in ColMb are inherited by the 8x8 partition mbPartIdx and scaled by the subclause Error: Reference source not found.

With above proposed changes, the motion information is always available to a depth MB 8x8 partition when the co-located texture MB is coded by inter_8x8, and encoding/decoding failure does not occur.

In addition, the current software doesn’t handle the case when BVSP is turned on for texture-first coding, which is enabled in JCT3V-D0185. The bug is fixed by enabling switching the reference index to that of the inter-view reference in such a case.

## Proposed simplification to IVMP

In current 3D-AVC, specific rule (as specified in section 2.1) and different conditions need to be checked and followed to construct the motion information by accessing multiple MB partitions and sub-blocks when the co-located texture MB is coded with four 8x8 MB partitions.

This can be simplified by just accessing one 8x8 MB partition and reusing it for the current 8x8 depth MB partition, when the co-located texture MB is coded with four 8x8 MB partitions. The index to the 8x8 MB partition in the texture MB is the same as that of the current depth MB partition in the depth MB.

Figure 2: Co-located four texture MBs (left) of current depth MB (right)

As illustrated in Figure 2, for an 8x8 MB partition in the current depth MB, the 8x8 partition of one texture MB with the same color is directly used for motion parameter inheritance.

# Experimental results

The proposed bug fix and simplification are implemented both on 3D-ATM v7.0 and the JCT3V-D0185. Both the common test conditions [2] and texture first test condition are used in simulations. For TF test condition, the coding order is configured to be “T0D0T1D1T2D2”.

## Coding gain of IVMP with bug fix under texture-first coding

Two tests are performed in this subsection, as described below:

Test 1.1:

- Anchor: 3D-ATM v7.0 with IVMP disabled

- Tested: 3D-ATM v7.0 with proposed bug fix

- Test condition: TF test condition with BVSP off

Test 1.2:

- Anchor: JCT3V-D0185 with IVMP disabled

- Tested: JCT3V-D0185

- Test condition: TF test condition with BVSP on

The results for Test 1.1 are tabulated in Table 1. As it is reported in Table 1, based on 3D-ATM v7.0, IVMP achieves -0.98% gain for the synthesized views.

Table 1: Coding gain of IVMP on 3D-ATM v7.0

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Syn. PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB |
| S01 | 0.00 | 0.00 | -4.15 | 0.27 | -0.18 | 0.01 | -0.57 | 0.02 |
| S02 | 0.00 | 0.00 | -7.82 | 0.37 | -0.52 | 0.02 | -0.71 | 0.02 |
| S03 | 0.00 | 0.00 | -12.92 | 1.14 | -0.66 | 0.03 | -1.24 | 0.04 |
| S04 | 0.00 | 0.00 | -16.64 | 1.11 | -0.96 | 0.04 | -1.65 | 0.06 |
| S05 | 0.00 | 0.00 | -3.22 | 0.16 | -0.50 | 0.03 | -0.92 | 0.04 |
| S06 | 0.00 | 0.00 | -8.40 | 0.40 | -0.63 | 0.03 | -0.95 | 0.05 |
| S08 | 0.00 | 0.00 | -5.34 | 0.26 | -0.48 | 0.02 | -0.83 | 0.03 |
| Aver | 0.00 | 0.00 | -8.36 | 0.53 | -0.56 | 0.02 | -0.98 | 0.04 |

The results for Test 1.2 are tabulated in Table 2. As it is reported in Table 2, based on JCT3V-D0185, IVMP achieves 1.26% gain for the synthesized views.

Table 2: Coding gain of IVMP on JCT3V-D0185

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Syn. PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB |
| S01 | 0.10 | 0.00 | -5.28 | 0.33 | -0.16 | 0.01 | -0.70 | 0.02 |
| S02 | -0.09 | 0.00 | -7.93 | 0.38 | -0.66 | 0.02 | -0.87 | 0.03 |
| S03 | -0.08 | 0.00 | -15.48 | 1.37 | -0.89 | 0.03 | -1.71 | 0.05 |
| S04 | -0.08 | 0.00 | -17.44 | 1.16 | -1.33 | 0.05 | -2.11 | 0.07 |
| S05 | -0.05 | 0.00 | -3.71 | 0.18 | -0.69 | 0.03 | -1.28 | 0.05 |
| S06 | -0.09 | 0.00 | -9.10 | 0.43 | -0.86 | 0.04 | -1.22 | 0.05 |
| S08 | 0.02 | 0.00 | -5.59 | 0.27 | -0.52 | 0.02 | -0.94 | 0.03 |
| Aver | -0.04 | 0.00 | -9.22 | 0.59 | -0.73 | 0.03 | -1.26 | 0.05 |

## Coding performance for proposed simplification

Two tests are performed in this subsection, as described below, both with BVSP enabled:

Test 2.1:

- Anchor: 3D-ATM v7.0 with the proposed bug fix

- Tested: 3D-ATM v7.0 with the proposed bug fix and simplification method

- Test condition: Common test condition

Test 2.2:

- Anchor: JCT3V-D0185

- Tested: JCT3V-D0185 with proposed simplification method

- Test condition: TF test condition

The results for Test 2.1 are tabulated in Table 3. As it is reported in Table 3, based on 3D-ATM v7.0 with proposed bug fix, the proposed simplification method shows average 0.01% and 0.00% BD-rate for total coded views and synthesized views, respectively.

Table 3: Coding performance under common test conditions

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Syn. PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB |
| S01 | 0.04 | 0.00 | -0.03 | 0.00 | 0.04 | 0.00 | 0.01 | 0.00 |
| S02 | 0.04 | 0.00 | 0.03 | 0.00 | 0.04 | 0.00 | 0.04 | 0.00 |
| S03 | -0.02 | 0.00 | -0.32 | 0.03 | -0.03 | 0.00 | -0.09 | 0.00 |
| S04 | 0.03 | 0.00 | -0.16 | 0.01 | 0.01 | 0.00 | 0.03 | 0.00 |
| S05 | 0.02 | 0.00 | 0.03 | 0.00 | 0.01 | 0.00 | 0.04 | 0.00 |
| S06 | -0.01 | 0.00 | -0.04 | 0.00 | 0.01 | 0.00 | -0.02 | 0.00 |
| S08 | 0.01 | 0.00 | 0.03 | 0.00 | 0.01 | 0.00 | 0.00 | 0.00 |
| Aver | 0.02 | 0.00 | -0.07 | 0.01 | 0.01 | 0.00 | 0.00 | 0.00 |

The results for Test 2.2 are tabulated in Table 4. As it is reported in Table 4, based on JCT3V-D0185 with proposed bug fix, the proposed simplification method shows average -0.01% and -0.02% BD-rate for total coded views and synthesized views, respectively.

Table 4: Coding performance under texture first test condition

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Syn. PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB | dBR % | dPSNR dB |
| S01 | -0.05 | 0.00 | -0.19 | 0.02 | -0.06 | 0.00 | 0.02 | 0.00 |
| S02 | 0.01 | 0.00 | 0.09 | 0.00 | 0.01 | 0.00 | 0.00 | 0.00 |
| S03 | -0.01 | 0.00 | -0.16 | 0.01 | -0.01 | 0.00 | -0.08 | 0.00 |
| S04 | 0.01 | 0.00 | 0.17 | -0.01 | 0.01 | 0.00 | 0.02 | 0.00 |
| S05 | 0.01 | 0.00 | 0.07 | 0.00 | 0.00 | 0.00 | -0.04 | 0.00 |
| S06 | -0.02 | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 | -0.04 | 0.00 |
| S08 | 0.00 | 0.00 | -0.11 | 0.00 | -0.01 | 0.00 | -0.03 | 0.00 |
| Aver | -0.01 | 0.00 | -0.02 | 0.00 | -0.01 | 0.00 | -0.02 | 0.00 |

# References

- M. M. Hannuksela, Y. Chen, T. Suzuki, J.-R. Ohm, G. J. Sullivan, " 3D-AVC draft text 4," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1002, Geneva, CH, 17–23 Jan. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1100, Geneva, CH, 17–23 Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).