| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0190 |
| --- | --- |

| Title: | 3D-CE6-related: SDC without per-pixel prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document by the JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Wenhao Zhang1, Lidong Xu1, Yu Han1, Xiaoxia Cai1, Yi-Jen Chiu2, and Hong Jiang2 18F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 22200 Mission College Blvd. Santa Clara CA USA 95054 | Email: | {zhipin.deng, wenhao.zhang, lidong.xu, xiaoxia.cai, yu.han, yi-jen.chiu, hong.h.jiang } @intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

In this contribution, a simplification of calculating the constant prediction values of SDC prediction is proposed to reduce the computational complexity and enhance the hardware implementation for SDC. By this method, the constant prediction values of Intra_DC, Intra_Planar, and Intra_DMM1 can be directly computed by the above row neighboring pixels and left column neighboring pixels instead of per-pixel prediction and averaging processes. It reports an average of 3%/10% decrease of decoding time without any bitrate change, for CTC/All-Intra configuration.

# Introduction

- Simplified Depth Coding (SDC) approach is an alternative intra coding mode for depth map coding [1]. Whether SDC is used is signaled by an SDC flag at CU level. The SDC prediction mode (sdc_pred_mode) shall be one of the values of Intra_DC, Intra_DMM, and Intra_Planar. If sdc_pred_mode is Intra_DC or Intra_DMM, the number of segments is one. Otherwise (sdc_pred_mode is Intra_DMM1), the number of segment is two. For an SDC-coded CU, the residual is not coded as quantized transform coefficients but one or two constant residual values are signaled.

- In HTM6.1, the reconstruction process of an SDC-coded CU is as follows.

- Generating the prediction value of every sample one by one using the traditional Intra_DC/ Intra_Planar/Intra_DMM1 prediction.

- Computing a constant prediction value (PredDC[uiSeg]) for each segment by averaging all the prediction samples in each segment.

- Samples in each segment are reconstructed by adding a transmitted constant residual value (ResidualDC[uiSeg]) to the computed constant prediction value (PredDC[uiSeg]).

In this contribution, a simplification of calculating the constant prediction values of SDC without per-pixel prediction is proposed to reduce the computational complexity and enhance the hardware implementation for SDC. By this method, only the above row neighboring pixels and left column neighboring pixels need to be saved for SDC prediction. Figure 1 illustrates the main idea of the proposed method.

Figure 1. Comparison of SDC prediction in HTM6.1 and the proposed method, wherein (a) SDC prediction in HTM6.1, and (b) the proposed SDC prediction

# Algorithm description

## Proposed calculation of the constant prediction value for Intra_DC block

Suppose the CU size of current SDC-coded CU is NxN, the constant prediction value,PredDC(Intra_DC), for Intra_DC prediction is directly computed by PredDC(Intra_DC)=x=0N−1p(x,−1)+y=0N−1p(−1,y)+N2N, wherein p(x,−1) stands for the values of above row neighboring pixels, p(−1,y)stands for the values of left column neighboring pixels.

Compared to the PredDC(Intra_DC) calculation in HTM6.1, this contribution skips the boundary smoothing step and removes the exhaustive computations for averaging all prediction samples of the current CU.

## Proposed calculation of the constant prediction value for Intra_Planar block

Suppose the CU size of current SDC-coded CU is NxN, the constant prediction value, PredDC(Intra_Planar), for Intra_Planar prediction is directly computed by

PredDC(Intra_planar)=x=0N−1p(x,−1)+y=0N−1p(−1,y)+N∗p(N,−1)+p(−1,N)+2N4N,

whereinp(x,−1)stands for the values of above row neighboring pixels, p(−1,y) stands for the values of left column neighboring pixels, p(N,−1) stands for the value of the above-right neighboring pixel on the above row outside the current CU, p(−1,N) stands for the value of the left-bottom neighboring pixel on the left column outside the current CU.

Compared to the PredDC(Intra_Planar) calculation in HTM6.1, the proposed method skips the exhaustive calculations of prediction values for each pixel one by one and removes the computations for averaging the prediction samples of the current CU. Instead, the proposed method directly generates a PredDC(Intra_Planar) for all the pixels in the whole CU. The derivation process of PredDC(Intra_Planar) is attached as appendix I at the end of this contribution.

## Proposed calculation of the constant prediction value for Intra_DMM1 block

To simplify the calculations of constant prediction values for Intra_DMM1, the proposed method directly uses the two DC values obtained by the HTM6.1 function getWedgePredDCs( ) as the constant prediction values of the two segments for the Intra_DMM1 prediction. In getWedgePredDCs( ), the DC value of each segment is generated by the wedge Pattern based calculation from the available above row neighboring pixels and the available left column neighboring pixels. If there is no neighboring pixel existing for the current segment, then the DC value for the current segment is set to 128.

Compared to the PredDC(Intra_DMM1) calculation in HTM6.1, the proposed method removes the exhaustive per-pixel assignments of DC values to every prediction sample, and removes the computations for averaging the prediction samples of each segment. Moreover, it removes the per-pixel check for zero-segments. Instead, the proposed method directly generates PredDC(Intra_DMM1) for all the pixels in each segment.

# Simulation results

The simulations were performed according the common test conditions [3]. For the All-Intra coding scenario, configuration files from Core Experiment 6 on Depth Map Intra Coding Tools were used. Test results are summarized in Tables 1 and Table 2. It reports an average of 3% /10% decrease of decoding time without any bitrate change, for CTC/All-Intra configuration.

Table 1.	Simulation results under the Random Access configuration (CTC)

Table 2.	Simulation results under the All Intra configuration (All-Intra)

# Conclusion

In this contribution, a simplification of calculating the constant prediction values of SDC prediction is proposed to reduce the computational complexity and enhance the hardware implementation for SDC. By this method, the constant prediction values of Intra_DC, Intra_Planar, and Intra_DMM1 can be directly computed by the above row neighboring pixels and left column neighboring pixels instead of per-pixel prediction and averaging processes. It reports an average of 3%/10% decrease of decoding time without any bitrate change, for RA/AI configuration.

# References

- "3D-HEVC Test Model 3," JCT3V-C1005, Jan. 2013.

- 3DV-HTM version 6.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.1/

- "Common Test Conditions of 3DV Core Experiments," JCT3V-C1100, Jan. 2013.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix I

Figure 2   Illustration of Intra_Planar prediction in HTM6.1

The derivation process of the proposed method is as follows.

As illustrated in Figure 2, the planar prediction value of each pixel p(x, y) in the current CU can be interpreted as an average of a 1D horizontal linear prediction and a 1D vertical linear prediction,

p(x,y)=((N−x−1)∗p(−1,y)+(x+1)∗p(N,−1)+(N−y−1)∗p(x,−1)+(y+1)∗p(−1,N)+N)2N

Therefore, the constant prediction value, PredDC(Intra_Planar), for the whole CU is computed by,

PredDC(Intra_Planar)=x=0N−1y=0N−1p(x, y)N2=H+V+N22N2

wherein H stands for the sum of prediction values of 1D horizontal linear predictions, V stands for the sum of prediction values of 1D vertical linear predictions.

- Firstly, calculating the sum of prediction values of 1D horizontal linear prediction:

H=x=0N−1y=0N−1((N−x−1)∗p(−1,y)+(x+1)∗p(N,−1)N)N−12y=0N−1p(−1,y)+N+12∗N∗p(N,−1)

- Similarly, calculating the sum of prediction values of 1D vertical linear prediction:

V=x=0N−1y=0N−1((N−y−1)∗p(x,−1)+(y+1)∗p(−1,N)N)N−12x=0N−1p(x,−1)+N+12∗N∗p(−1,N)

- Finally:

PredDC(Intra_planar)=H+V+N22N2(N−1)∗x=0N−1p(−1,y)+y=0N−1p(x,−1)+(N+1)∗N∗(p(N−1)+p(−1,N))+2N24N2x=0N−1p(x,−1)+y=0N−1p(−1,y)+N∗p(N,−1)+p(−1,N)+2N4N