| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0191_proposed_text |
| --- | --- |

| Title: | Proposed text for JCT3V-D0191 based on 3D-HEVC Test Model 3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Liu He Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document provides the draft text changes of contribution JCT3V-D0191. The deleted part is highlighted in red strikethrough, the newly added text is in green.

- H.8.5.2.1	Derivation process for motion vector components and reference indices

Input to this process are

- ...

Outputs of this process are

- …

the following applies.

- If PredMode[ xC ][ yC ] is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause  is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPbW, nPbH and the partition index partIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, the disparity vectors ivpMvDispL0 and ivpMvDispL1, the flag vspModeFlag, the variable refViewIdx and the flag availableFlagIvMC.  .

- …

- When vspModeFlag is equal to 1, refIdxL0 is set to 0 when RefPicListX[ 0 ] is a long-term reference picture or AltRefIdxLX when RefPicListX[ 0 ] is a short-term reference picture.

- When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause 8.5.2.1.8 is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

- H.8.5.4	Derivation process for a disparity vector

Inputs to this process are:

- …

- Outputs of this process are:

…

- 9.	For each X from 0 to 1, the following applies:

    - When availableDV is equal to 0, availableN is equal to 1 and PredFlagLX[ xN ][ yN ] is equal to 1, the following applies:

      - If RefIdxLX[ xN ][ yN ] is greater than or equal to 0, the following applies:

        - If RefPicListX[ RefIdxLX[ xN ][ yN ] ] is an inter-view reference picture, the following applies:

            - refViewIdx = ViewIdx( RefPicListX[ RefIdxLX[ xN ][ yN ] ] )	(Error: Reference source not found-1)
mvDisp = MvLXN[ xN ][ yN ]		(Error: Reference source not found-2)
availableDV = 1		(Error: Reference source not found-3)

        - Otherwise (RefPicListX[ RefIdxLX[ xN ][ yN ] ] is not an inter-view reference picture), the following applies:

          - When availableIvpMvSearchFlagN is equal to 1 and PredMode[ xN ][ yN ] is equal to MODE_SKIP and IvpMvFlagLX[ xN ][ yN ] is equal to 1, the following applies:

              - ivpMvDispLXN = IvpMvDispLX[ xN ][ yN ]	(Error: Reference source not found-4)
availableFlagIvpMvLXN = 1		(Error: Reference source not found-5)

      - Otherwise ( RefIdxLX[ xN ][ yN ] is less than 0 ), the following applies:

        - When VspModeFlag[ xN ][ yN ] is equal to 1, the following applies:

            - refViewIdx = RefViewIdx[ xN ][ yN ]	(Error: Reference source not found-6)
mvDisp = MvLXN[ xN ][ yN ]		(Error: Reference source not found-7)
mvDisp[ 1 ] = 0		(Error: Reference source not found-8)
availableDV = 1		(Error: Reference source not found-9)

            - [Ed. (GT): In software refViewIdx is set equal to 0. (#19)]

- H.8.5.2.1.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode[ xC ][ yC ] is equal to MODE_SKIP or PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- …

Outputs of this process are

- …

The variables singleMCLFlag is derived as follows.

  - If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

  - Otherwise, singleMCLFlag is set to 0.

- …

- The merge candidate list, mergeCandList, is constructed as specified by the following ordered steps.

  - The variable numMergeCand is set equal to 0.

  - The variable vspCandInListFlag is set equal to 0.

  - When availableFlagT is equal to 1, the insertion process for a candidate from a potentially view synthesis predicted neighbour as specified in subclause Error: Reference source not found is invoked with the list mergeCandList, the variable numMergeCand, the variable vspCandInListFlag, the candidate position indicator N being equal to T, the luma location ( xP, yP ), and the variables nPbW and nPbH as the inputs and the outputs are the list mergeCandList, the variable numMergeCand and the flag vspCandInListFlag.

      - …

- The variable vspModeFlag is set equal to ( mergeCandList[ merge_idx[ xP][ yP ] ] = = VSP ).

[Ed. (GT): In software VspModeFlag[ x ][ y ] is used in several places (deblocking filter, AMVP) this is not yet reflected in the draft.]

- The disparity availability flag ivpMvFlagLX and the disparity vector ivpMvDispLX are derived as follows (with X being replace by 0 or 1).

    - If all of the following conditions are true, ivpMvFlagLX is set equal to 1 and ivpMvDispLX is set equal to mvDisp

      - merge_idx[ xP][ yP ] = = 0

      - predFlagLXIvMC = = 1

    - Otherwise, ivpMvFlagLX is set equal to 0 and both components of ivpMvDispLX are set equal to 0.