| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0192 |
| --- | --- |

| Title: | CE5.a related: Direct/skip mode simplification for 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vijayaraghavan Thirumalai Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1 858 845 5663 vthiruma@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current ATM software for the skip/direct MBs, when the inter-view motion vector is not available, the median value of motion vectors of the neighboring blocks is used. This proposal proposes as simplification in this scenario: to derive the MVP for the skip/direct MBs to be the motion vector of the first available spatial neighbour, including the reference index. Experimental results show that the proposed simplified MVP scheme gives an overall coding gain of 0.06% and synthesized coding gain of 0.08% when compared to the CTC results.

# Introduction

In the current ATM-7.0 software, inter-view motion prediction is performed for the coding of MBs in P-skip, B-skip, B-16x16 direct, and B-8x8 direct modes. In these modes, a disparity vector used to identify a reference block in the reference view picture; this is either derived from the neighboring blocks or from the derived disparity vector from the corresponding depth block associated with the current MB.

This derived disparity vector is used to get a reference block in a reference view picture. Within the reference block, the motion vector (namely inter-view motion vector) if available, is set equal to the motion vector of the current block and the reference index of the current block is derived accordingly.

If the inter-view motion vector is unavailable, i.e., the reference block in the base view pointed by the disparity vector is intra-coded,  conventional median-based motion vector prediction scheme is used [1], by setting the reference index to zero [2].

# Proposed Scheme

In this proposal, when the inter-view motion vector is unavailable for the skip/direct modes, the motion vector predictor is set to be the motion vector of the first available spatial neighbour, including the reference index. Note that this aspect is aligned with one previous version of the ATM software.

The procedure of predicting the motion vector is described as follows.

- Check in order (of the left, right and right-above blocks), if any of the spatial neighbours has the reference picture index greater than or equal 0.

- If true, the motion vector and the reference picture index of the current MB are set respectively to be equal to the motion vector and the reference index of the spatial neighboring block, and the checking process terminates.

- If false, the motion vector and the reference picture index of the current MB is set to zero.

# Experimental Results

The proposed simplified MVP scheme is integrated into ATM7.0 [2]. The simulations are carried out under CTC [3].

The coding performance of the proposed scheme is given in Table 1. As shown in Table 1, the overall average bitrate reduction is 0.08%, 0.06%, 0.08% for decoded texture views, coded and synthesized views, respectively.

The encoding, decoding and rendering time estimates may not be accurate due to parallel simulations in the cluster.

Table 1: Results of the proposed simplified MVP scheme (proposed scheme vs. CTC)

# Conclusions

In this proposal, a simplification to derive the MVP for the skip/direct MBs is proposed when the inter-view motion vector is unavailable. Experimental results show that the proposed simplified MVP scheme brings overall coding and synthesized gains of 0.06% and 0.08%, respectively.

# References

[1]J. Y. Lee, J. Lee and D.-S. Park, “3D-CE5.a related: results on median-based skip and direct motion vector prediction,” 2nd JCT3V Meeting, JCT3V-B0151, Shanghai, Oct. 2012.

[2]J .-L. Lin, Y. -W. Chen, Y. -W. Huang and S. Lei, “3D-CE5.a related: Direct MVP derivation with reduced complexity,” 3rd JCT3V Meeting, JCT3V-C1100, Geneva, Jan. 2013.

[3]ATM-7.0, http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/3DV-ATMv7.0/

[4]D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” 3rd JCT3V Meeting, JCT3V-C1100, Geneva, Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).