| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0193 |
| --- | --- |

| Title: | 3D-CE6-related: Cleanup for 64x64 SDC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document by the JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Wenhao Zhang1, Lidong Xu1, Yu Han1, Xiaoxia Cai1, Yi-Jen Chiu2 , and Hong Jiang2 18F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 22200 Mission College Blvd. Santa Clara CA USA 95054 | Email: | {zhipin.deng, wenhao.zhang, lidong.xu, xiaoxia.cai, yu.han, yi-jen.chiu, hong.h.jiang } @intel.com |
| Source: | Intel Corp. |  |  |

____________________________

# Abstract

This proposal reports a bug in current Simplified Depth Coding (SDC) of 3D-HEVC and presents methods to clean up it. Specifically, the bug happens when a 64x64 CU is SDC coded and its sdc_pred_mode is singled as DMM. In this case, the behaviour of decoder is not specified, since the maximum CU size of DMM is limited to be 32x32. Three solutions are proposed to fix this bug: Solution 1 proposes to remove SDC signalling from 64x64 CU. Solution 2 introduces a limitation to the semantic of sdc_pred_mode to avoid the ill-posed case. Solution 3 changes the signalling of sdc_pred_mode to a CU-size-dependent way. Based on the consideration of performance and complexity, it’s suggested to adopt Solution 3 in to HTM7 and 3D-HEVC Test Model 4.

# Introduction

In [1], the SDC approach is an alternative intra coding mode. Whether SDC is used is signaled by an SDC flag at CU level. The partition size of SDC-coded CU is always inferred as 2Nx2N. For an SDC-coded block, the prediction mode shall be one of the values of Intra_DC, Intra_DMM, and Intra_Planar. In HTM6.1, the binary codes for SDC prediction mode are 1, 01, and 00 for Intra_DC, Intra_DMM, and Intra_Planar, respectively, as shown in Table 1.

Table 1 sdc_pred_mode signalling in HTM6.1

| - sdc_pred_mode | - Binary Code | - associated Intra Prediction Mode |
| --- | --- | --- |
| - 0 | - 1 | - Intra_DC (1 segment) |
| - 1 | - 01 | - Intra_DMM Mode1 (2 segments) |
| - 2 | - 00 | - Intra_Planar (1 segment) |

# Proposed methods

## Solution 1- Removing SDC signalling from 64x64 CU

Modifications to Syntax and Semantics (highlight)

- H.7.3.9  Coding unit syntax

- H.7.3.9.1  General Coding unit syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode = = MODE_INTRA && DepthFlag && log2CbSize < 6) |  |
| sdc_flag[ x0 ][ y0 ] [Ed. (GT): In software there seems to be a dependency of position of sdc_flag and slice type.] | ae(v) |
| if( sdc_flag[ x0 ][ y0 ] ) { |  |
| sdc_pred_mode | ae(v) |
| if( sdc_pred_mode = = 1 ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( PredMode[ x0 ][ y0 ] ! = MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) && !predPartModeFlag) |  |
| part_mode | ae(v) |

## Solution 2- A restriction to the semantic of sdc_pred_mode

Modifications to Syntax and Semantics (highlight)

- H.7.4.9.1 General coding unit semantics

sdc_pred_mode shall be one of the values shown in . sdc_pred_mode specifies the intra prediction mode used for simplified depth coding. It is a requirement of bitstream conformance that the value of sdc_pred_mode shall be 0 or 2 when log2CbSize is equal to 6.

Table Error: Reference source not found-1 – Interpretation of sdc_pred_mode

| sdc_pred_mode | Associated Intra Prediction Mode |
| --- | --- |
| 0 | Intra_DC |
| 1 | Intra_DepthPartition( 35 ) |
| 2 | Intra_Planar |

## Solution 3- A CU-size-dependent sdc_pred_mode signalling

Main idea

It is proposed to use a 1 bit binary code to signal SDC prediction mode for a 64x64 SDC-coded CU, that is, 0 for Intra_DC and 1 for Intra_Planar. Thus, the possible types of segmentation/prediction of the SDC-coded 64x64 CU are shown in Table 2. And for other CU sizes such as 32x32, 16x16, and 8x8, the binary code allocations for SDC prediction mode are unchanged and can be found in Table 1.

Table 2   The proposed sdc_pred_mode signalling for 64x64 CU

| - sdc_pred_mode | - Binary Code | - associated Intra Prediction Mode |
| --- | --- | --- |
| - 0 | - 0 | - Intra_DC (1 segment) |
| 2 | 1 | Intra_Planar (1 segment) |

Modifications to Syntax and Semantics (highlight)

Table Error: Reference source not found-2 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| sdc_pred_mode | 0 | 9.3.2.1 U [Ed. (GT): in implementation 0 and 1 are switched. ] [Ed. (GT): Could be TU, cMax = 3] | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 3 |
|  | 2 |  | 0 | Error: Reference source not found | 6 |

#### H.9.3.2.1 Binarization process for sdc_pred_mode

Inputs to this process are a request for a binarization for the syntax element sdc_pred_mode, the current luma coding block size log2CbSize.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element sdc_pred_mode is specified in Table H-30.

Table H-30 – Binarization for sdc_pred_mode

| Value of sdc_pred_mode | Name of sdc_pred_mode | Bin string |  |
| --- | --- | --- | --- |
|  |  | log2CbSize != 6 | log2CbSize = = 6 |
| 0 | Intra_DC | 1 | 1 |
| 1 | Intra_DepthPartition( 35 ) | 01 | - |
| 2 | Intra_Planar | 00 | 0 |

# Simulation results

The simulations were performed according the common test conditions [3]. For the All-Intra coding scenario, configuration files from Core Experiment 6 on Depth Map Intra Coding Tools were used. Test results are summarized in Tables 3, wherein RA stands for the random access coding configuration, AI stands for the all intra coding configuration.

- Solution 1: totally removing SDC signalling from 64x64 CU

- Solution 2: a restriction to the semantic of sdc_pred_mode

- Solution 3: a CU-size-dependent sdc_pred_mode signalling

Specifically, the result of solution 1 reports an average bitrate saving of 0.1% for video PSNR vs. video bitrate and an average bitrate increasing of 0.1% for video PSNR vs. total bitrate, for CTC configuration. For All-Intra configuration, it reports an average bitrate increasing of 0.6% and 0.3%, for video PSNR vs. total bitrate and synth PSR vs. total bitrate, respectively. The results of solution 2 are the same with HTM6.1 anchor since no code is changed. The result of solution 3 reports an average bitrate saving of 0.1% for synthesis PSNR vs. total bitrate for CTC configuration. For All-Intra configuration, there is on average no bit rate change.

To sum up, it is suggested to adopt Solution 3 in to HTM7 and 3D-HEVC Test Model 4, since the performance and complexity of solution 3 is better than other solutions, as shown in Table 3.

Table 3.   Summary results of the proposed solution 1, solution 2, and solution 3

The detailed results of solution 1 are shown as Table 4 and Table 5. The detailed results of solution 3 are shown as Table 6 and Table 7.

Table 4.	Random Access Coding Configuration for solution 1 (CTC)

Table 5.	All-Intra Coding Configuration for solution 1 (All-Intra)

Table 6.	Random Access Coding Configuration for solution 3 (CTC)

Table 7.	All-Intra Coding Configuration for solution 3 (All-Intra)

# Conclusion

This proposal reports a bug in current Simplified Depth Coding (SDC) of 3D-HEVC and presents methods to clean up it. Specifically, the bug happens when a 64x64 CU is SDC coded and its sdc_pred_mode is singled as DMM. In this case, the behaviour of decoder is not specified, since the maximum CU size of DMM is limited to be 32x32. Three solutions are proposed to fix this bug: Solution 1 proposes to remove SDC signalling from 64x64 CU. Solution 2 introduces a limitation to the semantic of sdc_pred_mode to avoid the ill-posed case. Solution 3 changes the signalling of sdc_pred_mode to a CU-size-dependent way. Based on the consideration of performance and complexity, it’s suggested to adopt Solution 3 in to HTM7 and 3D-HEVC Test Model 4.

# References

- "3D-HEVC Test Model 3," JCT3V-C1005, Jan. 2013.

- 3DV-HTM version 6.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.1/

- "Common Test Conditions of 3DV Core Experiments," JCT3V-C1100, Jan. 2013.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).