| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0194 |
| --- | --- |

| Title: | CE2.h related: Derived disparity vector for 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In 3D-HEVC, the NBDV process checks the neighboring blocks for not only disparity motion vectors but also implicit disparity vectors (IDVs), generated and stored during the NBDV processes for the neighboring blocks. It is considered that the IDVs may slightly increase the storage. Therefore, in this proposal, IDV is simplified by using the so-called derived disparity vector (DDV). In this proposal, only one DDV is maintained for a slice and updated once a CU is coded. With the proposed method, there is no storage increase in the block-level. Small coding gain is also observed, i.e., 0.1% bitrate reduction for synthesized views.

# Introduction

In 3D-HEVC, the NBDV process checks neighboring blocks to derive a disparity vector. There might be cases when neighboring blocks do not contain any motion vector that is a disparity motion vector. To still return a disparity vector in the above cases, each block can contain not only motion vectors, but also implicit disparity vectors stored if the block utilizes a disparity vector for certain conditions. For simplicity, such a disparity vector is named implicit disparity vector in this document.

Each PU may contain implicit disparity vectors (IDVs) corresponding to different prediction direction (RefPicList0 or RefPicList1).

Although it has been optimized such that IDVs are never required to be stored into DPB, they need to be stored for the view component being processed. The storage used to store IDVs can be as large as that used for motion fields.

# Proposal

It is proposed that a single derived disparity vector, namely Derived Disparity Vector (DDV) is maintained for the whole slice. At the beginning of decoding one slice, DDV is initialized to be a zero motion vector. After each CU is decoded, the DDV is updated to be the derived disparity vector of that CU.

NBDV in this case is always enabled for each CU of a non-base view that uses inter-view prediction.

When decoding any PU of the current CU, if no available disparity motion vector is found from the neighboring blocks, DDV is returned as the result of NBDV.

Although DDV is used only to generate the result for NBDV, if DoNBDV is enabled, after a CU is coded, the DDV is set to the result of DoNBDV.

# Simulations results

Two sets of results are provided:

- Proposed method (with IDV is turned off) compared with 3D-HEVC CTC.

- Proposed method (with IDV is turned off) with BVSP off compared with 3D-HEVC CTC BVSP off.

For the second test set, “BVSP off” indicates that no texture view is coded based on any depth view, i.e., both BVSP and Do-NBDV are both disabled. The platform 3D-HTM 6.1 [1] is utilized and the proposed method is implemented on it. Simulation results of the above two cases are described in the following sub-sections.

## Proposal vs 3D-HTM(CTC)

In this test, the anchor is generated under CTC with 3D-HTM v6.1 wherein PU-level NBDV and IDV are both enabled, and the tested method is the CU-level NBDV with proposed DDV. As shown in Table 1, the overall average bitrate reduction is 0.02%, 0.02% and 0.12% for texture views, coded views, and synthesized views, respectively.

Table 1: Coding gain of DDV with respect to 3D-HTM anchor for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 101% | 108% | 102% |
| Kendo | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 104% | 101% | 97% |
| Newspapercc | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 104% | 103% | 100% |
| GhostTownFly | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100% | 97% | 99% |
| PoznanHall2 | -0.5% | -0.2% | -0.1% | -0.1% | -0.2% | 100% | 99% | 97% |
| PoznanStreet | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 101% | 106% | 96% |
| UndoDancer | 0.1% | -0.1% | 0.0% | 0.0% | -0.5% | 111% | 106% | 100% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 103% | 104% | 100% |
| 1920x1088 | -0.1% | 0.0% | 0.0% | 0.0% | -0.2% | 103% | 102% | 98% |
| average | -0.07% | -0.04% | -0.02% | -0.02% | -0.12% | 103% | 103% | 99% |

## Proposal vs 3D-HTM (BVSP off)

In this test, the anchor is generated with 3D-HTM v6.1 with BVSP disabled, PU-level NBDV and IDV enabled, and the tested method is the CU-level NBDV with proposed DDV and BVSP disabled. As shown in Table 2, the overall average bitrate reduction is 0.05%, 0.04% and -0.04% for texture views, coded views, and synthesized views, respectively.

Table 2: Coding gain of DDV with respect to 3D-HTM anchor for 3-view case

|  | Video 1 | Video 2 | Video PSNR / video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | -0.1% | -0.1% | 0.0% | 101% | 94% | 98% |
| Kendo | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 95% | 103% | 106% |
| Newspapercc | -0.4% | -0.5% | -0.2% | -0.1% | -0.1% | 100% | 105% | 104% |
| GhostTownFly | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 97% | 106% | 99% |
| PoznanHall2 | -0.4% | -0.4% | -0.1% | -0.1% | -0.2% | 108% | 108% | 105% |
| PoznanStreet | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 100% | 101% | 107% |
| UndoDancer | 0.1% | 0.1% | 0.0% | 0.0% | 0.6% | 96% | 101% | 103% |
| 1024x768 | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | 99% | 101% | 102% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 100% | 104% | 103% |
| average | -0.08% | -0.14% | -0.05% | -0.04% | 0.04% | 100% | 103% | 103% |

# References

- 3D-HTM version 6.1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-6.1/.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1100, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).