| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0195 |
| --- | --- |

| Title: | CE6.h related: Unification of new intra modes in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qin Yu1 Li Zhang2 Ying Chen2 Siwei Ma1 Marta Karczewicz2 1School of EECS, Peking University, Institute of Digital Media, Beijing, China 25775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-651-6660 lizhang@qti.qualcomm.com +1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Peking University, and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a unified intra mode singling design for all the new Intra modes added into 3D-HEVC, including Depth Modeling Mode (DMM) modes, the Chain Coding Mode (CCM) and Simplified Depth Coding (SDC) modes. The proposal significantly simplifies the syntax design of the current 3D-HEVC and enables to a large extent decoupling the parsing of HEVC syntax elements and the newly added syntax elements. Both test cases at common test condition (CTC) and all intra condition are verified. The proposed method achieves 0.14% BD-rate gains compared to 3DV-HTM6.0 under CTC. For all intra case, 0.1% bitrate reduction is observed for coded video PSNR using total bitrate while minor loss, i.e., 0.06% for synthsized views is reported.

# Introduction

In 3D-HEVC, three new sets of intra modes are introduced: depth coding mode (with mode 1, 2, 3 and 4), chain coding mode, and Simplified Depth Coding (SDC) modes applied on top of the other modes.

The current design has the following problems:

- The syntax design for the existing modes is managed not in a systematic manner: a lot of unnecessary flags are introduced to signal different modes and combinations.

- Too much interaction between the HEVC syntax elements and the 3D-HEVC syntax elements, therefore, unnecessary complication is introduced for 3D-HEVC parsers if built on top of an existing HEVC version 1 decoder implementation.

The syntax table of the current new Intra modes in 3D-HEVC is shown below, wherein syntax elements added on top of HEVC are highlighted in different colors, depending on which type of modes they belong to.

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode = = MODE_INTRA && DepthFlag ) |  |
| sdc_flag[ x0 ][ y0 ] [Ed. (GT): In software there seems to be a dependency of position of sdc_flag and slice type.] | ae(v) |
| if( sdc_flag[ x0 ][ y0 ] ) { |  |
| sdc_pred_mode | ae(v) |
| if( sdc_pred_mode = = 1 ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( PredMode[ x0 ][ y0 ] ! = MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) && !predPartModeFlag) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| if ( vps_dmm_flag[ nuh_layer_id ] && log2CbSize <= Log2MaxDmmCbSize ) { |  |
| log2PbSize = log2CbSize − ( PartMode = = PART_NxN ? 1 : 0 ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| dmm_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if (dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| dmm_mode[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULL ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULLDELTA ) ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| wedge_predtex_tab_idx[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| dmm_delta_end_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_abs_minus1[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| DmmDeltaFlag[ x0 + i ][ y0 + i ] = ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WFULLDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WPREDDIRDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WPREDTEXDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_CPREDTEXDELTA ) ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0 + i ] ) |  |
| dmm_dc_1_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_1_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_1_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] ) |  |
| dmm_dc_2_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_2_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_2_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| } else { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( !dmm_flag[ x0 + i ][ y0+ j ] ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else { |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| if( rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] = = 31 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) <= 5 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) >= 2 ) { |  |
| edge_intra_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if( edge_intra_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_start_left_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_start_position[ x0 + i ][ y0+ j ] | ae(v) |
| num_edge_codes_minus1[ x0 + i ][ y0+ j ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| edge_dc_flag[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_flag[ x0 + i ][ y0+ j ] ) { |  |
| edge_dc_1_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_1_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_1_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| edge_dc_2_abs[ x0 + i ][ y0+ j ] | ae(v) |
| if( edge_dc_2_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| edge_dc_2_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag && !sdc_flag) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| if( sdc_flag ) { |  |
| sdcNumSegments = ( sdc_pred_mode = = 1 ? 2 : 1 ) |  |
| for ( i = 0; i < sdcNumSegments ; i++) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i | ae(v) |
| if( sdc_residual_flag [ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Proposed method

The proposal is summarized as follows:

- All newly introduced modes, including DMM, SDC, and CCM are signalled by one syntax element: depth_intra_mode. Based on this value of this syntax element, other additional information is signalled.

- All depth intra prediction modes are signalled in a unique syntax table and interact with minimum interaction with the HEVC syntax elements.

The specification changes are provided in this proposal. Syntax and semantics are provided in this section, while the decoding process changes are provided in Section 5.

## Syntax

The deleted text is marked with red strikethrough and the newly added text is highlighted in green.

### Video parameter set extension 2 Syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| … |  |
| if ( DepthLayerFlag[ layerId ] ) { |  |
| vps_depth_modes_flagvps_dmm_flag[ layerId ] | u(1) |
| lim_qt_pred_flag[ layerId ] | u(1) |
| if ( vps_depth_modes_flag ) |  |
| dlt_flag[ layerId ] | u(1) |
| if( dlt_flag[ layerId ] ) { |  |
| num_depth_values_in_dlt[ layerId ] | ue(v) |
| for ( j = 0; j < num_depth_values_in_dlt[ layerId ] ; j++) { |  |
| dlt_depth_value[ layerId ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| iv_mv_scaling_flag | u(1) |
| } |  |

### General Coding Unit Syntax

| coding_unit( x0, y0, log2CbSize , ctDepth) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode = = MODE_INTRA && DepthFlag ) |  |
| sdc_flag[ x0 ][ y0 ] | ae(v) |
| if( sdc_flag[ x0 ][ y0 ] ) { |  |
| sdc_pred_mode | ae(v) |
| if( sdc_pred_mode = = 1 ) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( PredMode[ x0 ][ y0 ] ! = MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) && !predPartModeFlag) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 |  |
| if ( vps_dmm_flag[ nuh_layer_id ] && log2CbSize <= Log2MaxDmmCbSize ) { |  |
| log2PbSize = log2CbSize −( PartMode = = PART_NxN ? 1 : 0 ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| dmm_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if (dmm_flag[ x0 + i ][ y0+ j ] ) |  |
| dmm_mode[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) | for( j = 0; j <= pbOffset; j = j + pbOffset ) |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) | for( i = 0; i <= pbOffset; i = i + pbOffset ) |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULL ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WFULLDELTA ) ) |  |
| wedge_full_tab_idx[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| wedge_predtex_tab_idx[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if(dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIR ¦¦ dmm_mode[ x0 + i ][ y0+ j ] = = MODE_DMM_WPREDDIRDELTA ) ) |  |
| dmm_delta_end_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_abs_minus1[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( dmm_delta_end_flag[ x0 + i ][ y0 + i ] ) |  |
| dmm_delta_end_sign_flag[ x0 ][ y0 ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| DmmDeltaFlag[ x0 + i ][ y0 + i ] = ( dmm_flag[ x0 + i ][ y0+ j ] && ( dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WFULLDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WPREDDIRDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_WPREDTEXDELTA ¦¦ dmm_mode[ x0 + i ][ y0 + i ] = = MODE_DMM_CPREDTEXDELTA ) ) |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0 + i ] ) |  |
| dmm_dc_1_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_1_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_1_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] ) |  |
| dmm_dc_2_abs[ x0 + i ][ y0 + i ] | ae(v) |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) |  |
| if ( DmmDeltaFlag[ x0 + i ][ y0+ j ] && dmm_dc_2_abs[ x0 + i ][ y0 + i ] != 0 ) |  |
| dmm_dc_2_sign_flag[ x0 + i ][ y0 + i ] | ae(v) |
| } else { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( vps_depth_modes_flag[ nuh_layer_id ]!dmm_flag[ x0 + i ][ y0+ j ]) |  |
| depth_modeling_table( x0 + i , y0+ j ) |  |
| if(depth_intra_mode[ x0 + i ][ y0 + j ] = = HEVC_INTRA_PRED) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( depth_intra_mode[ x0 + i ][ y0 + j ] = = HEVC_INTRA_PRED !dmm_flag[ x0 + i ][ y0+ j ]) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else { |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| if( rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] = = 31 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) <= 5 && ( log2CbSize + ( PartMode = = PART_NxN ) ? − 1 : 0 ) >= 2 ) { |  |
| - edge_intra_flag[ x0 + i ][ y0 + j ] | ae(v) |
| - if( edge_intra_flag[ x0 + i ][ y0+ j ] ) { |  |
| - edge_start_left_flag[ x0 + i ][ y0+ j ] | ae(v) |
| - edge_start_position[ x0 + i ][ y0+ j ] | ae(v) |
| - num_edge_codes_minus1[ x0 + i ][ y0+ j ] | ae(v) |
| - for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| - edge_code[ k ] | ae(v) |
| - edge_dc_flag[ x0 + i ][ y0+ j ] | ae(v) |
| - if( edge_dc_flag[ x0 + i ][ y0+ j ] ) { |  |
| - edge_dc_1_abs[ x0 + i ][ y0+ j ] | ae(v) |
| - if( edge_dc_1_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| - edge_dc_1_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| - edge_dc_2_abs[ x0 + i ][ y0+ j ] | ae(v) |
| - if( edge_dc_2_abs[ x0 + i ][ y0+ j ] != 0 ) |  |
| - edge_dc_2_sign_flag[ x0 + i ][ y0+ j ] | ae(v) |
| - } |  |
| - } |  |
| - } |  |
| - } |  |
| - } |  |
| } |  |
| } |  |
| } |  |
| if ( ! sdcFlag[ x0 ][ y0 ] ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| … |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| if ( icEnableFlag ) |  |
| ic_flag | ae(v) |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag && sdc_flag) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| if( sdc_flag ) { |  |
| sdcNumSegments = ( sdc_pred_mode = = 1 ) ? 2 : 1 |  |
| for ( i = 0; i < sdcNumSegments ; i++) { |  |
| sdc_residual_flag[ x0 ][ y0 ][ i | ae(v) |
| if( sdc_residual_flag [ x0 ][ y0 ][ i ]) { |  |
| sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

### Depth modeling table syntax

| depth_modeling_table( x0 , y0 ) { | Descriptor |
| --- | --- |
| depth_intra_mode[ x0 ][ y0 ] | ae(v) |
| if (depth_intra_mode[ x0 ][ y0 ] = = MODE_DMM_WFULL ¦¦ depth_intra_mode[ x0 ][ y0 ] = = SDCM_DMM_1) |  |
| wedge_full_tab_idx[ x0 ][ y0 ] | ae(v) |
| else if(depth_intra_mode[ x0 ][ y0 ] = = MODE_DMM_WPREDTEX) |  |
| wedge_predtex_tab_idx[ x0 ][ y0 ] | ae(v) |
| else if (depth_intra_mode[ x0 ][ y0 ] = = MODE_DMM_WPREDDIR) { |  |
| dmm_delta_end_flag[ x0 ][ y0 ] | ae(v) |
| if ( dmm_delta_end_flag[ x0 ][ y0 ] ) { |  |
| dmm_delta_end_abs_minus1[ x0 ][ y0 ] | ae(v) |
| dmm_delta_end_sign_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if (depth_intra_mode[ x0 ][ y0 ] = = CCM ) { |  |
| edge_start_left_flag[ x0 ][ y0 ] | ae(v) |
| edge_start_position[ x0 ][ y0 ] | ae(v) |
| num_edge_codes_minus1[ x0 ][ y0 ] | ae(v) |
| for( k = 0; k <= num_edge_codes_minus1; k++ ) |  |
| edge_code[ k ] | ae(v) |
| } |  |
| if(dmmFlag[ x0 ][ y0 ]) { |  |
| dmm_delta_flag[ x0 ][ y0 ] | ae(v) |
| if ( dmm_delta_flag [ x0 ][ y0 ] ) |  |
| for( i = 0; i < 2; i ++ ) { |  |
| dmm_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if ( dmm_dc_abs[ x0 ][ y0 ][ i ]) |  |
| dmm_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| else if ( depth_intra_mode[ x0 ][ y0 ] = = CCM ) { |  |
| edge_dc_flag[ x0 ][ y0 ] | ae(v) |
| if( edge_dc_flag[ x0 ][ y0 ] ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| edge_dc_abs[ x0 ][ y0 ][ i ] | ae(v) |
| if( edge_dc_abs[ x0 ][ y0 ][ i ] != 0 ) |  |
| edge_dc_sign_flag[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| - else if( sdcFlag[ x0 ][ y0 ] ) { |  |
| - sdcNumSegments = depth_intra_mode[ x0 ][ y0 ] = = SDCM_DMM_1 ? 2 : 1 |  |
| - for ( i = 0; i < sdcNumSegments ; i++) { |  |
| - sdc_residual_flag[ x0 ][ y0 ][ i ] | ae(v) |
| - if( sdc_residual_flag [ x0 ][ y0 ][ i ]) { |  |
| - sdc_residual_sign_flag[ x0 ][ y0 ][ i ] | - ae(v) |
| - sdc_residual_abs_minus1[ x0 ][ y0 ][ i ] | - ae(v) |
| - } |  |
| - } |  |
| - } |  |
| } |  |

## Semantics

### Video parameter set extension 2 semantics

vps_depth_modes_flag[ layerId ] equal to 1 specifies that depth map modelling modes, the chain coding mode and the simplified depth coding modes may be used in the decoding process of the layer with layer_id equal to layerId. vps_dmm_flag[ layerId ] equal to 0 specifies that none of the depth map modelling modes, chain coding mode and the simplified depth coding modes, are not is used in the decoding process of the layer with layer_id equal to layerId. When not present vps_depth_modes_flag[ layerId ] shall be inferred to be equal to 0.

### General coding unit semantics

The specification in subclause F.7.4.9.1 apply with the following modifications and additions.

…

no_residual_syntax_flag (rqt_root_cbf in the lastest HEVC specification) equal to 1 specifies that transform_tree( ) syntax is present for the current coding unit. rqt_root_cbf equal to 0 specifies that transform_tree( ) syntax is not present for the current coding unit.

When rqt_root_cbf is not present and sdcFlag[ x0 ][ y0 ] equal to 0, its value is inferred to be equal to 1. When rqt_root_cbf is not present and sdcFlag[ x0 ][ y0 ] equal to 1, its value is inferred to be equal to 0.

dmm_flag[ x0 + i][ y0 + j ] equal to 0 specifies that depth map model modes are not used. dmm_flag[ x0 + i][ y0 + j ] equal to 1 specifies that depth map model modes are used that the dmm_mode syntax element is present. When the dmm_flag[ x0 + i][ y0 + j ] is not present, its value should be inferred to be equal to 0.

dmm_mode[ x0 + i][ y0 + j ] shall be one of the values shown in Table Error: Reference source not found-1. dmm_mode is used to derive the intra prediction mode when the dmm_flag[ x0 + i][ y0 + j ] is equal to 1.

Table Error: Reference source not found-1 – Interpretation of dmm_mode

| dmm_mode | Method identifier |
| --- | --- |
| 0 | MODE_DMM_WFULL |
| 1 | MODE_DMM_WFULLDELTA |
| 2 | MODE_DMM_WPREDTEX |
| 3 | MODE_DMM_WPREDTEXDELTA |
| 4 | MODE_DMM_CPREDTEX |
| 5 | MODE_DMM_CPREDTEXDELTA |
| 6 | MODE_DMM_WPREDDIR |
| 7 | MODE_DMM_WPREDDIRDELTA |

The following syntax elements are removed from the general coding unit syntax table to depth modelling table without semantics change: wedge_full_tab_idx, wedge_predtex_tab_idx, dmm_delta_end_flag, dmm_delta_end_abs_minus1, dmm_delta_end_sign_flag, dmm_dc_0_abs, dmm_dc_0_sign_flag, dmm_dc_1_abs, dmm_dc_1_sign_flag, edge_start_left_flag, edge_start_position, num_edge_codes_minus1, edge_code, edge_dc_flag, edge_dc_0_abs, edge_dc_0_sign_flag, edge_dc_1_abs, edge_dc_1_sign_flag, sdc_residual_flag, sdc_residual_abs_minus1, and sdc_residual_sign_flag.

edge_intra_flag[ x0 + i][ y0 + j ] equal to 0 specifies that region boundary chain coding is not used. edge_intra_flag[ x0 + i][ y0 + j ] equal to 1 specifies that region boundary chain coding is used.

sdc_flag equal to 0 specifies that simplified depth coding is not used. sdc_flag equal to 1 specifies that simplified depth coding is used. When sdc_flag is not present, it is inferred to be equal to 0.

sdc_pred_mode shall be one of the values shown in 2. sdc_pred_mode specifies the intra prediction mode used for simplified depth coding.

Table Error: Reference source not found-2 – Interpretation of sdc_pred_mode

| sdc_pred_mode | Associated Intra Prediction Mode |
| --- | --- |
| 0 | Intra_DC |
| 1 | Intra_DepthPartition( 35 ) |
| 2 | Intra_Planar |

### Depth modeling table semantics

depth_intra_mode[ x0 ][ y0 ] specifies the depth intra mode of the current prediction unit. The values of depth_intra_mode[ x0 ][ y0 ] are shown in Table xx, based on which variables including sdcFlag[ x0 ][ y0 ], dmmFlag[ x0 ][ y0 ] are derived. sdcFlag[x0][y0] is set to 1 if depth_intra_mode[ x0 ][ y0 ] is SDCM_Planar, SDCM_DMM_1 or SDCM_DC; otherwise, it is set to be 0. dmmFlag[ x0 ][ y0 ] is set to 1 if depth_intra_mode[ x0 ][ y0 ] is DMM_1, DMM_2, DMM_3, or DMM_4; otherwise, it is set to be 0.

Table xx: Interpretation of depth_intra_mode

| depth_intra_mode | DepthIntraMode |
| --- | --- |
| 0 | SDCM_Planar |
| 1 | HEVC intra (HEVC_INTRA_PRED) |
| 2 | SDCM_DMM_1 |
| 3 | DMM_1 (MODE_DMM_WFULL) |
| 4 | DMM_4 (MODE_DMM_CPREDTEX) |
| 5 | DMM_3 (MODE_DMM_WPREDTEX) |
| 6 | SDCM_DC |
| 7 | DMM_2 (MODE_DMM_WPREDDIR) |
| 8 | CCM |

dmm_delta_flag[ x0 ][ y0 ] equal to 1 specifies dmm_dc_abs[ x0 ][ y0 ][ i ] and  dmm_dc_sign_flag[ x0 ][ y0 ][ i ] are present, dmm_delta_flag[ x0 ][ y0 ] equal to 0 specifies that  dmm_dc_abs[ x0 ][ y0 ][ i ] and  dmm_dc_sign_flag[ x0 ][ y0 ][ i ] are not present.

# Experimental Results

The proposed method is implemented in the HTM6.0. And it was simulated under both common test condition configuration and all-intra configuration. Results are summarized as Table 2 and Table 3. An average of 0.1% BD-rate gain is achieved under CTC configurations [1] and minor loss is obtained under all intra configurations.

Table 1: Coding gain of proposed method compared to anchor (CTC)

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.11% | -0.11% | 104% | 108% | 103% |
| Kendo | 0.00% | -0.27% | -0.18% | 102% | 99% | 96% |
| Newspaper_CC | 0.01% | -0.05% | -0.07% | 104% | 102% | 101% |
| GT_Fly | 0.05% | 0.03% | 0.03% | 98% | 103% | 98% |
| Poznan_Hall2 | -0.11% | -0.08% | -0.18% | 98% | 95% | 92% |
| Poznan_Street | 0.01% | -0.02% | 0.06% | 102% | 106% | 100% |
| Undo_Dancer | 0.01% | 0.05% | -0.51% | 103% | 103% | 101% |
| 1024x768 | 0.01% | -0.14% | -0.12% | 103% | 103% | 100% |
| 1920x1088 | -0.01% | -0.01% | -0.15% | 100% | 102% | 98% |
| average | 0.00% | -0.06% | -0.14% | 102% | 102% | 99% |

Table 2: Coding gain of proposed method compared to anchor (All Intra)

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | Enc time | Dec time | Ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.16% | 0.03% | 102% | 98% | 94% |
| Kendo | 0.00% | -0.22% | 0.04% | 102% | 99% | 94% |
| Newspaper_CC | 0.00% | -0.12% | 0.08% | 96% | 95% | 88% |
| GT_Fly | 0.00% | -0.05% | 0.08% | 101% | 100% | 91% |
| Poznan_Hall2 | 0.00% | -0.14% | -0.04% | 101% | 93% | 89% |
| Poznan_Street | 0.00% | -0.04% | 0.10% | 101% | 95% | 90% |
| Undo_Dancer | 0.00% | 0.04% | 0.13% | 107% | 104% | 100% |
| 1024x768 | 0.00% | -0.16% | 0.05% | 100% | 97% | 92% |
| 1920x1088 | 0.00% | -0.05% | 0.07% | 103% | 98% | 93% |
| average | 0.00% | -0.10% | 0.06% | 101% | 98% | 92% |

# References

- Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-C1100, Geneva, CH, Jan. 2013.

# Decoding process

Several subclauses are modified as follows.

- H.8.4.2	Derivation process for luma intra prediction mode

…

IntraPredMode[ xB ][ yB ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1.

- If depth_intra_mode[ xB ][ yB ] is equal to CCM, IntraPredMode[ xB ][ yB ] is set equal to 43.

- Otherwise, ifIf sdc_flag sdcFlag[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is derived as follows.

  - If sdc_pred_mode[ xB ][ yB ] is equal to 0, IntraPredMode[ xB ][ yB ] is set equal to 1.

  - Otherwise, if sdc_pred_mode[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is set equal to 35.

  - Otherwise, if sdc_pred_mode [ xB ][ yB ] is equal to 2, IntraPredMode[ xB ][ yB ] is set equal to 0.

- Otherwise, if dmm_flagdmmFlag[ xB ][ yB ] is equal to 1, IntraPredMode[ xB ][ yB ] is derived as follows:

  - If depth_intra_mode[ xB ][ yB ] is equal to DMM_1, IntraPredMode[ xB ][ yB ] is set equal to 35.

  - Otherwise, if depth_intra_mode[ xB ][ yB ] is equal to DMM_2, IntraPredMode[ xB ][ yB ] is set equal to 41.

  - Otherwise, depth_intra_mode[ xB ][ yB ] is equal to DMM_3, IntraPredMode[ xB ][ yB ] is set equal to 37.

  - Otherwise (depth_intra_mode[ xB ][ yB ] is equal to DMM_4), IntraPredMode[ xB ][ yB ] is set equal to 39.

- IntraPredMode[ xB ][ yB ] = dmm_mode[ xB ][ yB ] + 35	(Error: Reference source not found-1)

- Otherwise (dmm_flag[ xB ][ yB ] is equal to 0 and sdc_flag[ xB ][ yB ] is equal to 0), IntraPredMode[ xB ][ yB ] is derived as the following ordered steps.

…

- H.8.4.4.1	General decoding process for intra blocks

…

- 3.	Depending on sdcFlag[ xB0 ][ yB0 ]sdc_flag, the following applies:

- If the sdcFlag[ xB0 ][ yB0 ]sdc_flag is equal to 0, following applies:

  - The scaling and transformation process as specified in subclause 8.6.2 is invoked with the location ( xB0, yB0 ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nT as the inputs and the output is a (nT)x(nT) array resSamples.

  - The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause 8.6.5 is invoked with the transform block location ( xB0, yB0 ), the transform block size nT, the variable cIdx, the (nT)x(nT) array predSamples, and the (nT)x(nT) array resSamples as the inputs.

- Otherwise (sdcFlag[ xB0 ][ yB0 ]sdc_flag is equal to 1) the following ordered steps apply:

  - The depth value reconstruction process as specified in subclause Error: Reference source not found is invoked with the location ( xB0, yB0 ), the transform size trafoSize set equal to nT, the (nT)x(nT) array predSamples, and the intra prediction mode intraPredMode, as the inputs and the output is a (nT)x(nT) array resSamples.

…

- H.9.3.1.1	Initialization process for context variables

…

Table Error: Reference source not found-3 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | dmm_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_mode | Error: Reference source not found | 0 | 1 | 2 |
|  | wedge_full_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | wedge_predtex_tab_idx | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_delta_end_flag dmm_delta_end_abs_minus1 | Error: Reference source not found | 0 | 1 | 2 |
|  | dmm_dc_1_abs dmm_dc_2_abs dmm_dc_abs | Error: Reference source not found | 0 | 1 | 2 |
|  | edge_code | Error: Reference source not found | 0 | 1 | 2 |
|  | ic_flag | Error: Reference source not found |  | 0 | 1 |
|  | mvp_l0_idx mpv_l1_idx | Error: Reference source not found |  | 0..1 | 2..3 |
|  | sdc_flag | … | 0..2 | 3..5 | 6..8 |
|  | sdc_residual_flag | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | sdc_residual_sign_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sdc_residual_abs_minus1 | Error: Reference source not found | 0..19 | 20..39 | 40..59 |
|  | sdc_pred_mode | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | depth_intra_mode | Table H-xx | 0...7 | 8..15 | 16..23 |
|  | dmm_delta_flag | Table H-xx | 0 | 1 | 2 |

…

Table Error: Reference source not found-4 – Values of variable initValue for dmm_flag ctxIdx

| Initialization variable | dmm_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table Error: Reference source not found-5 – Values of variable initValue for dmm_mode ctxIdx

| Initialization variable | dmm_mode |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

…

Table Error: Reference source not found-6 – Values of variable initValue for sdc_flag ctxIdx

| Initialization variable | sdc_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 0 | 0 | 64 | 0 | 0 | 0 | 64 | 0 | 0 |

…

Table Error: Reference source not found-7 – Values of variable initValue for sdc_pred_mode ctxIdx

| Initialization variable | sdc_pred_mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 9 | 255 | 70 | 0 | 9 | 255 | 70 | 0 | 9 |

Table Error: Reference source not found-8 – Values of variable initValue for sdc_flag ctxIdx

| Initialization variable | sdc_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 0 | 0 | 64 | 0 | 0 | 0 | 64 | 0 | 0 |

Table Error: Reference source not found-xx – Values of variable initValue for depth_intra_mode ctxIdx

| Initialization variable | sdc_residual_abs_minus1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| initValue | 0 | 0 | 64 | 0 | 154 | 0 | 154 | 0 |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 0 | 64 | 0 | 154 | 0 | 154 | 0 | 0 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| initValue | 64 | 0 | 154 | 0 | 154 | 0 | 0 | 0 |

Table Error: Reference source not found-xx – Values of variable initValue for dmm_delta_flag ctxIdx

| Initialization variable | dmm_dc_abs |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 0 | 0 | 64 |

- H.9.3.2	Binarization process

The specifications in subclause 9.3.2 apply with the following modifications.

–	The specifications in subclause 9.3.2 apply with the following modifications.  is appended to the end of Table 9-37.

–	A subclause is added after subclause 9.3.2.10.

- H.9.3.2.1	Binarization process for depth_intra_mode

Input to this process is a request for a binarization for the syntax element depth_intra_mode, a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, and a variable cLog2CbSize specifying the current luma coding block size.

Output of this process is the binarization of the syntax element.

The binarization for part_mode is given by xx depending on PartMode[ xC ][ yC ] and cLog2CbSize.

Table H-xx – Binarization for depth_intra_mode

| depth_intra_mode | DepthIntraMode | Bin String |  |  |
| --- | --- | --- | --- | --- |
|  |  | (cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N) ¦¦ (cLog2CbSize > 3 && cLog2CbSize < 6) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | cLog2CbSize = = 6 |
| 0 | SDCM_Planar | 0 0 | - | 0 |
| 1 | HEVC intra (HEVC_INTRA_PRED) | 0 10 | 0 | 1 0 |
| 2 | SDCM_DMM_1 | 0 1 1 | - | - |
| 3 | DMM_1 (MODE_DMM_WFULL) | 1 0 0 | 1 0 | - |
| 4 | DMM_4 (MODE_DMM_CPREDTEX) | 1 0 1 | - | - |
| 5 | DMM_3 (MODE_DMM_WPREDTEX) | 1 1 0 | 1 1 0 | - |
| 6 | SDCM_DC | 1 1 1 0 | - | 1 1 |
| 7 | DMM_2 (MODE_DMM_WPREDDIR) | 1 1 1 1 1 | - | - |

Table Error: Reference source not found-9 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| dmm_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| dmm_mode | 0 | FL, cMax = 7 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| … |  |  |  |  |  |
| dmm_dc_1_abs dmm_dc_2_abs dmm_dc_abs | 0 | UEG0 [Ed. (GT) To be specified] | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| dmm_dc_1_sign_flag dmm_dc_2_sign_flag | na | FL, cMax = 1 | na | na | na, (Bypass) |
| … |  |  |  |  |  |
| sdc_flag | 0 | FL, cMax = 1 | 0 | … | 0 |
|  | 1 |  | 0 | … | 3 |
|  | 2 |  | 0 | … | 6 |
| … |  |  |  |  |  |
| sdc_pred_mode | 0 | U [Ed. (GT): in implementation 0 and 1 are switched. ] [Ed. (GT): Could be TU, cMax = 3] | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 3 |
|  | 2 |  | 0 | Error: Reference source not found | 6 |
| depth_intra_mode | 0 | as specified in subclause H.9.3.2.1 | 0 | Error: Reference source not foundxx | 0 |
|  | 1 |  | 2 | Error: Reference source not foundxx | 8 |
|  | 2 |  | 2 | Error: Reference source not foundxx | 16 |
| dmm_dc_sign_flag | 0 | FL, cMax = 1 |  | Table H-xx | 0 |
|  | 1 |  |  | Table H-xx | 1 |
|  | 2 |  |  | Table H-xx | 2 |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).