| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0196 |
| --- | --- |

| Title: | AHG7: Comments on parameter sets for MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document includes the following two proposals on parameter sets for MV-HEVC: 1) clarification of ViewId; and 2) signaling of inter_view_mv_vert_constraint_flag.

# Proposal

## ViewId signaling in MV-HEVC

In the current MV-HEVC WD, the ViewId is signaled and used in a way that it has the same functionality as view order index in MVC, though it appears to be view ID. Therefore, it is proposed to change the name of “ViewId” to “ViewIdx” in the MV-HEVC specification.

### Editorial changes for view order index

The changes are described as follows.

F.3.9	view: a sequence of pictures with an identical value of ViewIdx.

F.7.4.3.1.1	Video parameter set extension semantics

Table F-1 –Mapping of ScalabiltyId to scalability dimensions

| scalability_mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | multiview | ViewIdx |
| 1-15 | Reserved |  |

The variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer and the variable ViewIdx[ layer_id_in_nuh[ i ] ] specifying the view order index of the i-th layer are derived as follows:

for (i = 0; i <= vps_max_layers_minus1; i++) {

for( smIdx= 0, j =0; smIdx< 16; smIdx ++ )

if( ( i  ! =  0 )  &&  scalability_mask[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

else

ScalabilityId[ i ][ smIdx ] = 0

ViewIdx[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ]

}

- F.8.1	General decoding process

The specifications in subclause 8.1 apply with following additions.

When the current picture has nuh_layer_id greater than 0, the following applies.

–	Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the following decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the following decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb, and SCr, respectively.

NOTE – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

–	The decoding process operates as follows for the current picture CurrPic.

–	For the decoding of the slice segment header of the first slice, in decoding order, of the current picture, the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	When ViewIdx[ nuh_layer_id ] is greater than 0, the decoding process for a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	After all slices of the current picture have been decoded, the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

- G.11.1.2	Stereo Main profile

Bitstreams conforming to the Stereo Main profile shall obey the following constraints:

- The sub-bitstream resulting from the sub-bitstream extraction process with any value of tIdTarget and a value of 0 in layerIdListTarget as inputs shall conform to the Main profile.

- The bitstream shall contain one layer with nuh_layer_id equal to i for which ViewIdx[ i ] is greater than 0.

- When ViewIdx[ i ] is greater than 0, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_extension( ) syntax structure of the active layer SPS of any coded pictures with nuh_layer_id equal to i.

### Singling of view id

In MVC and earlier versions the MV-HEVC WD, view_id is signaled in NAl unit header and the VPS extension, respectively.

The value of view_id usually corresponds to camera locations and may be used by systems specifications, e.g. as display hints, as well as by low-level coding tools, e.g. in motion vector scaling.

Therefore, we propose to add the signaling of view_id in the VPS extension, as shown below.

Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j <NumScalabilityTypes; j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| view_id_info_present_flag | u(1) |
| if( view_id_info_present_flag ) { |  |
| view_id_present_for_all_layers_flag | u(1) |
| for( i = 0; i < =vps_max_layers_minus1; i++ ) |  |
| if( view_id_present_for_all_layers_flag ¦ ¦ i % 2 = = 0 ) |  |
| view_id[ i ] | u(10) |
| } |  |
| for( lsIdx = 1; lsIdx <= vps_num_layer_sets_minus1; lsIdx ++ ) { |  |
| vps_profile_present_flag[ lsIdx ] | u(1) |
| if( !vps_profile_present_flag[ lsIdx ] ) |  |
| profile_layer_set_ref_minus1[ lsIdx ] | ue(v) |
| profile_tier_level( vps_profile_present_flag[ lsIdx ], vps_max_sub_layers_minus1) |  |
| } |  |
| num_output_layer_sets | ue(v) |
| for( i = 0; i < num_output_layer_sets; i++ ) { |  |
| output_layer_set_idx[ i ] | ue(v) |
| lsIdx = output_layer_set_idx[ i ] |  |
| for( j = 0 ; j <= vps_max_layer_id; j++) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ) |  |
| output_layer_flag[ lsIdx ][ j ] | u(1) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |

Video parameter set extension semantics

view_id_info_present_flag equal to 1 specifies that the view identifier are assigned for the layers specified by the VPS. view_id_info_present_flag equal to 0 specifies that the view identifiers are not assigned for the layers specified by the VPS.

view_id_present_for_all_layers_flag equal to 1 specified that the view identifier for each layer specified by the VPS is explicitly signalled. view_id_present_for_all_layers_flag equal to 0 specified that the view identifiers are explicitly signalled for some layers specified by the VPS and derived for other layers specified by the VPS.

view_id[ i ] specifies the view identifier of the i-th layer specified by the VPS. When view_id_present_for_all_layers_flag is equal to 0 and i % 2 is equal to 1, the value of view_id[ i ] is inferred to be equal to view_id[ i − 1 ].

NOTE – The same VPS syntax can be used by SHVC with the value of view_id_info_present_flag being required to be equal to 0. In a potential extension of MV-HEVC that includes depth, the texture and depth components of one view would be two neighbouring layers and have the same view_id. When texture and depth are always paired, setting view_id_info_present_flag equal to 1 and view_id_present_for_all_layers_flag equal to 0 would be desirable.

## Signaling of the vertical motion vector constraint in MV-HEVC

In MV-HEVC, a motion vector constraint flag is included in the SPS extension with the following semantics:

inter_view_mv_vert_constraint_flag equal to 1 specifies that vertical component of motion vectors used for inter-layer prediction are constrained in the CVS. When inter_view_mv_vert_constraint_flag is equal to 1, the vertical component of the motion vectors used for inter-layer prediction shall be equal to or less than 56 in units of luma samples. When inter_view_mv_vert_constraint_flag is equal to 0, no constraint for of the vertical component of the motion vectors used for inter-layer prediction is signalled by this flag. When not present, the inter_view_mv_vert_constraint_flag is inferred to be equal to 0.

This requires adding a syntax table for SPS extension and also unnecessary SPS instances and PPS instances even all the characteristics of the SPS of the enhancement view are equal to those in the SPS of the base view.

Therefore it is proposed to move this flag to VPS extension, as follows:

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| inter_view_mv_vert_constraint_flag | u(1) |
| } |  |

- G.7.3.2.1	Sequence parameter set extension syntax

The specifications in subclause F.7.3.2.1 apply.

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| } |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).