| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0199r1 |
| --- | --- |

| Title: | AHG7: Parallel decoding SEI message for MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen Adarsh K. Ramasubramonian Ye-Kui Wang Vadim Seregin Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-6589 cheny@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

A parallel decoding SEI message for MV-HEVC similar to the parallel decoding SEI message in MVC is proposed with three modifications: 1) the delay required for parallel decoding is signalled in unit of coding tree units (CTU); 2) the horizontal CTU delay is signalled together with CTU row delay to avoid large increased delay caused by the size of a CTU; 3) the delay is signalled for each reference view of a current view.

In the first revision of this document, it is proposed to use the collocated CTU in the reference view components to determine the unreferenced region, after discussion with experts working in this area.

# Introduction

Parallel decoding information SEI message is included in MVC, wherein the delays in the unit of MB rows are signalled and it applies only to the case when MBs are coded in a raster-scan order. In this document we propose to inherit the MVC parallel decoding information SEI message into MV-HEVC, with adaptation of the design taking into account the differences between MV-HEVC and MVC, including CTU-based coding and the introduction of SAO.

## Example of the LCU delay in horizontal and vertical directions

There is a delay of CTUs if it is desired to decode base view and enhancement views in parallel. In Fig.1, base and enhancement views are depicted. Assume that the current block in the enhancement view uses inter-view prediction with a disparity motion vector MV.

In the beginning, let’s assume that the in-loop filtering processes like deblocking filter and SAO are performed on CTU (also known as largest coding unit, LCU) basis.

To get the fully reconstructed pixels in the base view to be used for inter-view prediction, the pixels in the referenced LCU should be fully reconstructed, meaning in-loop filtered. To achieve this, the immediate neighboring LCUs have to be at least de-blocked, and the neighboring LCUs for those de-blocked LCUs have to be at least inverse transformed (e.g., after motion compensation). Therefore, for the case that decoder processes are performed on LCU basis there is a delay of three LCU rows in the vertical direction for the enhancement view decoding, unless the vertical component of MV is always 0. In other words, to start decoding the enhancement view at least three LCU rows of the base view should be available even if the MV has a very small magnitude. The LCU row containing the inter-view prediction has to have fully reconstructed data, the next LCU row has to have at least de-blocked pixels, and the third LCU row has to have at least the pixels after inverse transform.

Let’s simplify it to the case that three above mentioned LCU rows require full reconstruction. Assume the LCU address of the reference block is (LCUaddrX, LCUaddrY). In this case, the part of the base view picture located, starting from the LCUaddrY + 2, cannot be used for inter-view prediction.

Similarly, assuming that there is a horizontal restriction for the inter-view prediction, the part of the base view picture located after LCUaddrX + 2 cannot be referred for inter-view prediction.

If reference block is located at the LCU boundary in the base view, then the neighboring LCUs have to have fully reconstructed pixels that will be necessary for the interpolation.

Therefore, the delay can be increased by one in horizontal and vertical directions, i.e. LCUaddrY + 3 and LCUaddrX + 3.

Fig. 1. Illustration of the LCU rows delay.

## Discussions on impact on decoder implementations

In above section, there is an assumption that decoding processes are performed on LCU basis. If we assume that the decoding process can be done line by line, then the un-referenced base view picture regions can be expressed in the number of rows and columns of pixels. The part of the base view that should not be referenced starts from the point (BottomRightX + 8, BottomRightY + 8), in units of pixels, with the coordinates in horizontal and vertical directions, since 4 pixels are needed for interpolation, 3 pixels for deblocking, and 1 pixel for SAO. Here, BottomRight is the point at the bottom right corner of the referenced block in the base view, and BottomRightX and BottomRightY are its positions in the horizontal and vertical pixel coordinates, respectively. It is noticed that in this case, two LCU rows delay may be typically sufficient for parallel decoding if the magnitude of disparity motion vectors are relatively small.

# Proposal

It is proposed that the MVC parallel decoding SEI message is simplified and used for MV-HEVC, by allowing signaling of the horizontal delay. Similar to the parallel decoding SEI message in MVC, it is assumed that such an SEI message is present and applicable when raster scan decoding order applies.

## Parallel decoding information SEI message syntax

| parallel_decoding_info( payloadSize ) { | Descriptor |
| --- | --- |
| pdi_video_parameter_set_id | ue(v) |
| for( i = 1; i <= num_views_minus1; i++ ) |  |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) { |  |
| pdi_init_delay_ctu_vertical_minus2[ i ][ j ] | ue(v) |
| pdi_init_delay_ctu_horizontal_minus2[ i ][ j ] | ue(v) |
| } |  |
| } |  |

## Semantics

The parallel decoding information SEI message may be associated with any access unit. The information signalled in the parallel decoding information SEI message applies to all the access units from the access unit the SEI message is associated with to the next access unit, in decoding order, containing an SEI message of the same type, exclusively, or to the end of the CVS, whichever is earlier in decoding order.

Some view components of the access units for which the parallel decoding information is signalled in a parallel decoding information SEI message may not be present in the CVS.

pdi_video_parameter_set_id specifies the VPS that contains the inter-view dependency relationship information. The value of pdi_video_parameter_set_id shall be equal to the value of video_parameter_set_id of the active VPS for the coded pictures of the access unit containing the parallel decoding information SEI message.

The variable refLog2CtbSizeY, refPicWidthInCtbsY[ i ][ j ] and refPicHeightInCtbsY[ i][ j ] are set equal the Log2CtbSizeY, PicWidthInCtbsY and PicHeightInCtbsY, respectively, for the view component of the j-th reference view of the coded view component i.

pdi_init_delay_ctu_vertical_minus2[ i ][ j ] and pdi_init_delay_ctu_horizontal_minus2[ i ][ j ] specify the unavailable reference area in the j-th reference view component, which has a nuh_layer_id value to RefLayerId[ i ][ j ], of the i-th coded view component, which has a nuh_layer_id value equal to layer_id_in_nuh[ i ], that shall not be used for inter-view reference by the i-th coded view component. The range of pdi_unit_delay_ctu_vertical_minus2[ i ][ j ] shall be 0 to refPicHeightInCtbs - 3, inclusive. The range of pdi_unit_delay_ctu_horizontal_minus2[ i ][ j ] shall be 0 to refPicWidthInCtbs	 - 3, inclusive.

The variable horCtb[ i ][ j ] is derived as follows:

horCtb[ i ][ j ] =
 						pdi_init_delay_ctu_horizontal_minus2[ i ][ j ]

The variable verCtb[ i ][ j ] is derived as follows:

verCtb[ i ][ j ] = 
							pdi_init_delay_ctu_vertical_minus2[ i ][ j ]

The address of the collocated CTU in the reference view component is obtained by the following:

xAddrOfCtb = (ctbAddr % PicWidthInCtbs) << Log2CtbSizeY 
yAddrOfCtb = ( ctbAddr / PicWidthInCtbs ) << Log2CtbSizeY

xRefCtb = xAddrOfCtb >> refLog2CtbSizeY [ i ][ j ]
yRefCtb = yAddrOfCtb >> refLog2CtbSizeY[ i ][ j ]

-

The variable refCtbAddr[ i ][ j ] that denotes the address of refCtb[ i ][ j ] is derived as follows:

refCtbAddr[ i ][ j ] = Min( horCtb[ i ][ j ] + xRefCtb, RefPicWidthInCtbs - 1 ) + 														(verCtb[ i ][ j ] + yRefCtb)* refPicWidthInCtbsY[ i ][ j ]

The unavailable reference area in the j-th reference view of the coded view component i includes all the coding tree units with a raster-scan address greater than or equal to refCtbAddr[ i ][ j ].

It is a requirement of bitstream conformance that, when decoding the i-th coded view component, samples from the unavailable reference area from the view component of the j-th reference view of coded view component i shall not be referred to by the inter-view prediction process.

It is a requirement of bitstream conformance that when the parallel decoding information SEI message is present, the value of num_tile_columns_minus1 in the active picture parameters set shall be equal to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).