| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0202 |
| --- | --- |

| Title: | MV-HEVC/3D-HEVC HLS: on ViewId signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

The contribution attempts to provide a solution to two issues:

- In MV-HEVC, the ViewId value of the base view is constrained to be equal to 0, which reportedly causes a problem when using ViewId values for motion vector scaling in 3D-HEVC.

- When asymmetric quality is used between views, it is reported to be desirable to change the higher quality view between eyes periodically. However, it is asserted to be beneficial for rate-distortion performance that the base view has higher quality than the non-base view(s).

This contribution proposes:

- To derive ViewNum as a scalability dimension instead of ViewId on the basis of the VPS extension syntax. If a texture view and a depth view have the same ViewNum value, they represent the same view.

- The association of ViewNum with ViewId is proposed to be indicated in the picture parameter set (PPS). The ViewId values may indicate a relative view position in a 1D camera setup and can therefore be used in 3D-HEVC for motion vector scaling.

- The DPB is proposed to output pictures of the same output time in ascending order of ViewId.

# Introduction

The following is remarked in an editor’s note in MV-HEVC text JCT3V-C1004_d2:

ViewId[ 0 ], i.e. the ViewId value for the base view, is constrained to be equal to 0 according to the semantics above, whereas in the previous versions of MV-HEVC specification (e.g. JCT3V-B1004) it was possible to assign a non-zero ViewId value for the base view and  ViewId was defined in a way similar to MVC. Consequently, it was possible to assign a non-base view with a smaller ViewId value than the ViewId value of the base view, which is no longer possible in the present version of the annex. In the current 3D-HEVC working draft, the ViewId values are used for motion vector scaling and essentially indicate relative camera positions. The constraint that the ViewId value for the base view is equal to 0 has a consequence that it is essentially considered to be an outmost view in the camera setup. It is an open issue whether to define ViewId in the same way as in previous versions of MV-HEVC and similar to MVC.

It is noted that it is a common practice to use unequal quality between the views in stereoscopic video, which is utilized for example in the JCT-3V common test conditions (JCT3V-C1100). It is preferred to encode the base view with a higher quality, as it serves therefore as a good-quality inter-view reference picture.

Related to asymmetric quality between views, the following has been reported in [W.J. Tam, “Image and depth quality of asymmetrically coded stereoscopic video for 3D-TV”, JVT-W094]:

One issue with using asymmetrically coded stereoscopic images as a display format for 3D-TV is that for an unvarying presentation of reduced-quality images to one eye, depending on the individual, that eye might actually be his or her dominant eye.  In this case, the masking effect will not be optimal and might not even take place.  Another major concern is that the continuous presentation of degraded images to one eye might have an undesirable long-term effect on viewers.  One obvious solution is to present images with the reduced quality alternately to the left and the right eyes so that the image quality is distributed over both eyes in a balanced manner over time.

This contribution therefore proposes:

- To derive ViewNum as a scalability dimension instead of ViewId on the basis of the current VPS extension syntax. If a texture picture and a depth picture with the same PicOrderCntVal have the same ViewNum value, they represent the same view. However, ViewNum values do not necessarily indicate a physical camera/view order or a view decoding order.

- The association of ViewNum to ViewId is proposed to be indicated in the picture parameter set (PPS). The ViewId values may indicate a relative view position in a 1D camera setup and can therefore be used in 3D-HEVC for motion vector scaling. The ViewId value of the layer with nuh_layer_id equal to 0 is not constrained. As the association resides in the PPS, it is possible to switch the base view (with nuh_layer_id equal to 0) to represent a different viewpoint and hence balance the image quality over both eyes over time.

- The DPB is proposed to be modified to output pictures of the same output time in ascending order of ViewId. Consequently, the decoder outputs the pictures consistently left view first, followed by the right view (or vice versa) even if the view association was changed.

# Proposed Specification Text

The specification text changes relative to MV-HEVC (JCT3V-C1004_d3) are presented below in red font.

- F.7.4.3.1.1	Video parameter set extension semantics

…

scalability_mask[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table Error: Reference source not found-1 are present. scalability_mask[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present.

Table Error: Reference source not found-1 – Mapping of ScalabiltyId to scalability dimensions

| scalability_mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | multiview | ViewNumViewId |
| 1-15 | Reserved |  |

…

The variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer and the variable ViewNumViewId[ layer_id_in_nuh[ i ] ] specifying the view numberidentifier of the i-th layer and NumViews specifying the number of unique ViewNum values are derived as follows:

NumViews = 0
for (i = 0; i <= vps_max_layers_minus1; i++) {

for( smIdx= 0, j =0; smIdx< 16; smIdx ++ )

if( ( i  ! =  0 )  &&  scalability_mask[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

else

ScalabilityId[ i ][ smIdx ] = 0

ViewNumViewId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ]

newViewFlag = 1

for( k = i – 1; k >= 0; k-- )

if( ScalabilityId[ i ][ 0 ]  = =  ScalabilityId[ k ][ 0 ] )

newViewFlag = 0

NumViews += newViewFlag

}

- F.7.x.x.x	Picture parameter set extension

The picture parameter set extension is proposed to include the following syntax structure:

| view_num_to_view_id( ) { | Descriptor |
| --- | --- |
| num_views | ue(v) |
| for( i = 0; i < num_views; i++ ) { |  |
| view_num[ i ] | ue(v) |
| view_id[ i ] | ue(v) |
| } |  |
| } |  |

num_views specifies the number of unique ViewNum values. num_views shall be equal to NumViews.

view_num[ i ] specifies the i-th ViewNum value. view_num[ i ] shall not be equal to view_num[ j ] for any value of j not equal to i.

view_id[ i ] specifies the ViewId value for view_num[ i ].

nuhLayerId is set equal to a value for which ViewNum[ nuhLayerId ] is equal to view_num[ i ]. The variable ViewId[ nuhLayerId ] is set equal to view_id[ i ].

It is a requirement of bitstream conformance that num_views, view_num[ i ] and view_id[ i ] are identical in the active PPS and all active layer PPSs for pictures having the same PicOrderCntVal.

- F.8.1	General decoding process

…

–	The decoding process operates as follows for the current picture CurrPic.

–	For the decoding of the slice segment header of the first slice, in decoding order, of the current picture, the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	When ViewNumViewId[ nuh_layer_id ] is greater than 0, the decoding process for a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	After all slices of the current picture have been decoded, the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

- F.13.3	Operation of the decoded picture buffer (DPB)

The specifications in subclause C.3 apply separately for each set of decoded pictures with a particular value of nuh_layer_id.

PicOutputFlag for pictures that are not included in a target output layer is set equal to 0.

If NumViews is greater than 1, decoded pictures with the same DPB output time and with PicOutputFlag equal to 1 are output in ascending order of ViewId values of these decoded pictures. Otherwise, decoded pictures with the same DPB output time and with PicOutputFlag equal to 1 are output in ascending order of nuh_layer_id values of these decoded pictures. [Ed. (MH): if combined scalability is allowed, the output order of different scalability dimensions should be specified.]

- G.11.1.2	Stereo Main profile

Bitstreams conforming to the Stereo Main profile shall obey the following constraints:

- The sub-bitstream resulting from the sub-bitstream extraction process with any value of tIdTarget and a value of 0 in layerIdListTarget as inputs shall conform to the Main profile.

- The bitstream shall contain one layer with nuh_layer_id equal to i for which ViewNumViewId[ i ] is greater than 0.

- When ViewNumViewId[ i ] is greater than 0, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_extension( ) syntax structure of the active layer SPS of any coded pictures with nuh_layer_id equal to i.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).