| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0209 |
| --- | --- |

| Title: | Inter-view motion field mapping for MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Done Bugdayci, Kemal Ugur, Miska M. Hannuksela, Dmytro Rusanovskyy Visiokatu 3, Tampere, Finland | Email: | ext-done.bugdayci@nokia.com kemal.ugur@nokia.com miska.hannuksela@nokia.com dmytro.rusanovskyy@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Motion information of different views consisting the multiview video data are correlated. In order to utilize this redundancy, 3D-HEVC allows a disparity compensated motion vector of a base view to be utilized in motion vector prediction (MVP) of dependent view. MV-HEVC specification is restricted from such low-level operations with motion information, thus redundancy in motion information between views is not utilized. This contribution proposes to use temporal motion vector prediction of HEVC to enable the prediction of motion vectors from the inter-view reference picture. For this purpose, inter-view reference picture is marked as the collocated reference picture and motion field of such reference picture can be used in MVP of currently coded picture. However, the disparity present between two views would penalize such MVP, unless motion field of reference picture is modified to compensate for the disparity. This contribution presents a scheme that utilizes a global disparity compensation for motion field adjustment. Global disparity is estimated at the encoder side for each slice and signaled to the decoder side at the slice header. Simulations conducted in the MV-HEVC configuration of 3D-HTM v6.0 show that the proposed method brings 1.1% of dBR for view 1 and 0.8% for view 2 which results in 0.6% gain on average of all views.

# Introduction and Proposal

Scalable extension of HEVC includes a mechanism called motion field mapping, where base layer motion vectors are used as temporal motion vector predictor candidates to code enhancement layer. When base layer is upsampled, the motion data of the base layer is also mapped to the resolution of enhancement layer. If the enhancement layer picture utilizes temporal motion vector prediction from base layer picture, the corresponding motion vector predictor is originated from the mapped base layer motion field. This way the correlation between the motions of different layers is exploited to improve the coding efficiency of a scalable video coder.

Same principles can be applicable to MV-HEVC if operation motion field mapping is redefined as proposed. In our proposal, in order to be utilized for coding of current view, coordinates of motion field of inter-view reference picture are adjusted/shifted with global disparity value.

In our design, global disparity shift is estimated at the encoder side as average disparity value from depth map picture associated with currently coded video picture. However, alternative method of global disparity estimation, such disparity from motion vectors, can be directly applied here.

Global disparity shift offset_x estimated at the encoder side typically would have accuracy defined by the depth to disparity conversion process, e.g. would have 1/4 –pixel accuracy. To be applicable for motion field mapping, offset_x is quantized to achieve “block-level” accuracy. Namely, it is quantized by 16x16 block levelQuantized value of the horizontal offset_x is applied to the motion field adjustment and signaled in the slice header for the decoder side.

The concept of motion field adjustment (shifting) is visualized in Figure 1.

Figure 1-1 Horizontal component of the motion field for Kendo sequence is plotted to illustrate the effect of motion mapping

It should be noted, the current design specifies that selection of collocated picture is based on view id, assuming that view 0 is the independent (base) view and there is only one inter-view reference picture available. However, this design is motived by software issues inherited in 3D-HTM. In case of multiple inter-view reference pictures available, collocated picture selection may be based on a different algorithm considering the view distance and/or QP cascading parameters.

# Specification text

Two new syntax elements are introduced.

- sps_mfm_enable_flag: Flag indicating the use of motion field mapping for inter-layer reference pictures.

- avg_disp_col: A horizontal offset for the motion field mapping process.

Below are the possible specification text changes for the proposal. Changes are according to JCT3V- C1004 however the implementation differs from what is shown here.

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_mfm_enable_flag | u(1) |
| } |  |

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( slice_temporal_mvp_enabled_flag && sps_mfm_enabled_flag && nuh_layer_id > 0 ) |  |
| avg_disp_col | se(v) |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| … |  |

# Experimental Results

Proposed in this paper technique has been integrated to the HTM-6.0 software configured in the MV-HEVC. Simulation results have been conducted according to the JCT-3V Common Test conditions, adjusted for MV_HEVC configuration. The HTM6.0 in default configuration was utilized to produce the MV-HEVC anchor results.

In our simulation we have tested 2 schemes. In the first scheme (Experiment 1) inter-view reference picture is utilized as a collocated picture in MVP, and its motion field is utilized without adjustment. The second scheme (Experiment 2) utilizes inter-view reference picture as collocated and perform motion field mapping as specified in this contribution. Experiments have been conducted with compressed motion field of 16x16 block size.

Table 1 Compression efficiency of experiment 1 - MFM disabled

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | -2.0% | -1.1% | -0.9% |
| Kendo | 0.0% | -0.6% | 0.3% | -0.1% |
| Newspaper_CC | 0.0% | 0.7% | 2.2% | 0.7% |
| GT_Fly | 0.0% | 0.9% | 1.1% | -0.2% |
| Poznan_Hall2 | 0.0% | -2.1% | 0.7% | -0.1% |
| Poznan_Street | 0.0% | 0.7% | 1.3% | 0.3% |
| Undo_Dancer | 0.0% | -0.5% | 0.6% | 0.0% |
| 1024x768 | 0.0% | -0.6% | 0.4% | -0.1% |
| 1920x1088 | 0.0% | -0.3% | 0.9% | 0.0% |
| Average | 0.0% | -0.4% | 0.7% | -0.1% |

Table 2 Compression efficiency of experiment 2 - MFM enabled

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | -2.7% | -2.5% | -1.4% |
| Kendo | 0.0% | -1.3% | -1.3% | -0.7% |
| Newspaper_CC | 0.0% | -0.4% | -0.1% | -0.1% |
| GT_Fly | 0.0% | 0.5% | 0.5% | -0.4% |
| Poznan_Hall2 | 0.0% | -3.0% | -2.0% | -1.1% |
| Poznan_Street | 0.0% | 0.6% | 0.9% | 0.2% |
| Undo_Dancer | 0.0% | -1.1% | -1.2% | -0.5% |
| 1024x768 | 0.0% | -1.5% | -1.3% | -0.7% |
| 1920x1088 | 0.0% | -0.7% | -0.5% | -0.4% |
| average | 0.0% | -1.1% | -0.8% | -0.6% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).