| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0211_v1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Extending the supported number of layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Gerhard Tech Robert Skupin Thomas Schierl | Email: | karsten.suehring@hhi.fraunhofer.de gerhard.tech@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes an extension mechanism for layer identifiers to increase the number of supported layers in MV-HEVC and 3D-HEVC. The range of nuh_layer_id is extended by an additional syntax element within the NAL units. The concept of so-called layer clusters allows using the existing extraction processes to select groups of related layers. As second part of the proposal an extension of the layer dependency signalling applicable to layer id extension is proposed.

# Introduction

HEVC allows 64 values of nuh_layer_id to be used for scalable layers, views and depth. If no scalability is used, this implies a constraint of max. 64 views or 32 view plus depth combinations.

For the most common application this is totally sufficient. On the other hand we see huge camera arrays with more the 100 views showing up for future applications like light field imaging [4], [5] or holographic displays [6].

In this contribution we propose an extension mechanism for nuh_layer_id that does not hurt coding performance in the common cases (i.e. which does not change the NAL unit header) but allows increasing the number of views.

In the special applications, it will become more complicated to access the layer identifier. But a pre-selection can still be achieved using the traditional bitstream extraction process based on nuh_layer_id.

# Proposal

## Layer extension

A new variable LayerId is used to identify layers. If the layer extension is disabled, LayerId is equal to nuh_layer_id. If the extension is enabled, nuh_layer_id is used as MSB of LayerId, while a new syntax element (layer_id_ext) forms the LSB. The new syntax element is coded inside the NAL units (e.g. parameter set extensions, slice header extension).

When the layer extension is enabled nuh_layer_id can be used to pre-select a set of related layers (a "layer cluster"), which can e.g. exist of neighboring views or a texture and a depth layer related to the same position.

We believe that composing LayerId from these two syntax elements provides sufficient flexibility for special case applications that require a huge number of layers, without hurting the efficiency of the common case with less than 65 layers.

## Layer dependencies

As a second part of the contribution it is proposed to modify the signalling of layer dependencies, in case that the layer extension is used. In case of large number of layers it would be a common use case that the dependencies between layers within a cluster is the same for all clusters and that the dependencies between layers with same layer_id_ext value in different clusters is the same for all layer_id_ext values.

Therefore it is proposed to signal the dependencies within a cluster as an additional set of dependency flags and combine them with existing direct_dependency_flag values to determine the reference layers of a particular layer when the layer extension is used.

An example for the proposed layer dependency signalling is depicted in Figure 1.

Figure 1: Views of a 20x5 view camera array, a view cluster contains all views of cameras with same horizontal position

In this example the layer extension is used to cluster views of the same horizontal camera position and different vertical camera position. For coding views from such a setup it would be a common use case to utilize the same dependency structure (depicted by the solid arrows) between horizontal positions, independent from the vertical position and vice versa. For signalling the horizontal dependencies the existing direct_dependency_flag syntax elements are utilized. For signalling the vertical dependencies (depicted by the dotted arrows) an additional set of dependency flags is transmitted.

Figure 2: Views of a 64x1 view camera array with associated depth maps, a cluster contains the texture and the depth of a view.

A second example is depicted in Figure 2, here the layer extension is utilized to cluster a texture view and a depth view. For the signalling the view dependencies existing direct_dependency_flag syntax elements are utilized. For signalling the dependencies between texture and depth an additional set of dependency flags is transmitted.

# Syntax and semantics

## Layer extension

The following syntax element is added to the VPS extension:

| video_parameter_set_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| layer_id_ext_len | u(3) |
| … |  |
| } |  |

layer_id_ext_len indicates the number of bits used for extending the LayerId range.

For SPS and PPS, the extensions are used to signal the new identifier using ue(v) to avoid a parsing dependency to the VPS:

| sps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| layer_id_ext | ue(v) |
| … |  |
| } |  |

| pps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| layer_id_ext | ue(v) |
| … |  |
| } |  |

SEI messages can generically be extended by using the payload extension mechanism:

| generic_sei_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| layer_id_ext | ue(v) |
| … |  |
| } |  |

In slice segment headers the first reserved field is used for the additional syntax element. Alternatively the syntax element could be moved to the slice header extension.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| layer_id_ext | u(v) |
| for( i = 0; i < (num_extra_slice_header_bits- layer_id_ext_len); i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| … |  |
| } |  |

layer_id_ext is used in combination with nuh_layer_id to identify the layer. The syntax element layer_id_ext is coded using layer_id_ext_len bits.

The variable LayerId is determined as follows:

LayerId = (nuh_layer_id << layer_id_ext_len) + layer_id_ext

The following changes add the flexibility to map the additional layer identifier value range to different dimensions types (i.e. depth and texture view components) and to signal the dependencies between layers.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i < VpsMaxNumLayers; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nalu[ i ] | u(v) |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( lsIdx = 1; lsIdx <= vps_num_layer_sets_minus1; lsIdx ++ ) { |  |
| vps_profile_present_flag[ lsIdx ] | u(1) |
| if( !vps_profile_present_flag[ lsIdx ] ) |  |
| profile_layer_set_ref_minus1[ lsIdx ] | ue(v) |
| profile_tier_level( vps_profile_present_flag[ lsIdx ], vps_max_sub_layers_minus1) |  |
| } |  |
| num_output_layer_sets | ue(v) |
| for( i = 0; i < num_output_layer_sets; i++ ) { |  |
| output_layer_set_idx[ i ] | ue(v) |
| lsIdx = output_layer_set_idx[ i ] |  |
| for( j = 0 ; j <= vps_max_layer_id; j++) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ) |  |
| output_layer_flag[ lsIdx ][ j ] | u(1) |
| } |  |
| for( i = 1; i <= VpsMaxNumLayers – 1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |

The variable VpsMaxNumLayers specifying the maximum number of layer in the extended layer range is set equal to ( vps_max_layers_minus1 + 1 ) * 2  layer_id_ext_len.

layer_id_in_nalu[ i ] specifies the value of the LayerId value associated with VCL NAL units of the i-th layer. For i in a range from 0 to VpsMaxNumLayers – 1, inclusive, when not present, the value of layer_id_in_nalu[ i ] is inferred to be equal to i.

When i is greater than 0, layer_id_in_nalu[ i ] shall be greater than layer_id_in_nalu[ i – 1 ].

For i in a range from 0 to vps_max_layers_minus1, inclusive, the variable LayerIdInVps[ layer_id_in_nalu[ i ] ] is set equal to i.

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th layer. When not present, the value of dimension_id[ i ][ j ] is inferred to be equal to 0. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits. When splitting_flag is equal to 1, it is a requirement of bitstream conformance that dimension_id[ i ][ j ] shall be equal to ( ( layer_id_in_nalu[ i ] & ( (1 << dimBitOffset[ j + 1 ] ) − 1) ) >> dimBitOffset[ j ] ).

The variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer, the variable ViewId[ layer_id_in_nuh[ i ] ] specifying the view identifier of the i-th layer and DependencyId[ layer_id_in_nalu[ i ] ] specifying the spatial/SNR scalability identifier of the i-th layer are derived as follows:

for (i = 0; i < VpsMaxNumLayers; i++) {
	for( smIdx= 0, j =0; smIdx< 16; smIdx ++ ) 
		if( ( i  ! =  0 )  &&  scalability_mask[ smIdx ] ) 
			ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]
		else 
			ScalabilityId[ i ][ smIdx ] = 0 
	ViewId[ layer_id_in_nalu[ i ] ] = ScalabilityId[ i ][ 0 ]
	DependencyId [ layer_id_in_nalu[ i ] ] = ScalabilityId[ i ][ 1 ]
}

## Layer dependencies

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for( i = 1; i < ( 1 << layer_id_ext_len ) ; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_ext_dependency_flag[ i ][ j ] | u(1) |

The variables cId[ i ] and eId[ i ], specifying the cluster and the extension index of the layer with index i are derived as specified in the following:

for( i = 1; i < VpsMaxNumLayers; i++ ) {
	cId[ i ] = i >> layer_id_ext_len 
	eId[ i ] = i – ( cId[ i ] << layer_id_ext_len)
}

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer or cluster with index j is not a direct reference layer or reference cluster for the layer or cluster with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer or cluster with index j may be a direct reference layer or reference cluster for the layer or reference cluster with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

direct_ext_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with extension index j is not a direct reference layer for the layer with extension index i within the same layer cluster. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with extension index j may be a direct reference layer for the layer with extension index i within the same cluster. When direct_ext_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

The variables NumDirectRefLayers[ i ] and RefLayerId[ i ][ j ] are derived as follows:

for( i = 1; i < VpsMaxNumLayers; i++ ) {
	for( j = 0, NumDirectRefLayers[ i ] = 0; j < i; j++ ) { 
		if(	( direct_dependency_flag[ cId[ i ]][ cId[ j ]]  = =  1 && eId[ i ] = = eId[j] ) | |
			( direct_ext_dependency_flag[ eId[ i ]][ eId[ j ]]  = =  1  &&  cId[ i ] = = cId[ j ]  )
				RefLayerId[ i ][ NumDirectRefLayers[ i ]++ ] = layer_id_in_nalu[ j ]
	}
}

[Ed. Please note that the derivation above is equal to the current derivation when layer_id_ext_len is equal to 0.]

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand (Eds.), “High Efficiency Video Coding (HEVC) text specification draft 10“, JCTVC-L1003, Geneva, CH, Jan. 2013

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J.Boyce (Eds.), “MV-HEVC Draft Text 3 (ISO/IEC 23008-2 PDAM2)“, JCT3V-C1004, Geneva, CH, Jan. 2013

- G. Tech, K. Wegner, Y. Chen, S. Yea (Eds.), “3D-HEVC Test Model Description, draft specification“, JCT3V-C1005, Geneva, CH, Jan. 2013

- WILBURN, Bennett, et al. High performance imaging using large camera arrays. ACM Transactions on Graphics, 2005, 24. Jg., Nr. 3, S. 765-776.

- WILBURN, Bennett S., et al. Light field video camera. In: Electronic Imaging 2002. International Society for Optics and Photonics, 2001. S. 29-36.

- HORIMAI, Hideyoshi, et al. Full-color 3D display system with 360 degree horizontal viewing angle. In: Proc. Int. Symposium of 3D and Contents. 2010. S. 7-10.

- Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).