| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0213 |
| --- | --- |

| Title: | 3D-CE1.h: Adaptive block size on VSP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto, Hideaki Kimata (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

View synthesis prediction adopted in 3D-HTM is performed by two-step disparity compensated prediction (DCP). The first DCP is performed to generate virtual depth map at PU level, and prediction signals are generated by the second DCP which is operated for each 4x4 pixels. Since no 4x4 motion compensation is supported in the HEVC specification, this contribution proposes to use either 4x8 or 8x4 block size in VSP. The block size is adaptively decided by analyzing virtual depth map generated by the first DCP. Experiments show that the average bitrate increases are 0.1% and 0.0% for coded and synthesis views, respectively, while reducing decoding time and removing 4x4 DCP.

# Introduction

View synthesis prediction (VSP) was adopted into the test model of 3D-HEVC at Geneva meeting [1]. VSP is performed two-step disparity compensated prediction (DCP) where virtual depth map is first generated by DCP with DoNBDV (depth oriented neighbouring disparity vector) and prediction signals are generated by DCP with disparity derived from the virtual depth map.

The second DCP is performed for each 4x4 pixels. However, it might be better not to use 4x4 DCP since 4x4 motion compensation is excluded from the specification of HEVC. In this contribution, it is proposed to decide either 4x8 or 8x4 DCP using virtual depth map.

# Proposals

The proposed method is to decide block partitioning for 2nd DCP on VSP by using virtual depth map. In the proposed method, the block partition is decided for each 8x8 block in VSP predicted block. For each 8x8 block, either 4x8 or 8x4 partition is selected by comparing depth values at 4 corners.

The block partition is decided by the following rule.

if vdepthTL−vdepthTR+vdepthBL−vdepthBR>vdepthTL−vdepthBL+vdepthTR−vdepthBR use 8x4partitionelse use4x8partition

, where TL, TR, BL, and BR represent top-left, top-right, bottom-left, and bottom-right corner pixels, respectively.

In addition to the method how to decide block partition, we also propose to clip disparity vectors once per 2nd DCP block. It is noted that disparity vector is clipped at each pixel in VSP block. This means that the current implementation utilizes the pixel-based DCP in some blocks.

# Experiments

The proposed methods are built on the top of the reference software 3DV-HTM v6.0. And the experiments are conducted according to the common test condition [2].

TABLE 1 shows the simulation results by removing the pixel-based DCP in VSP. There are small coding losses for dependent views, but the overall loss is negligible.

TABLE 1: HTM v6.0 vs. Removal of pixel-based DCP in VSP

TABLE 2 shows the simulation results by the proposed adaptive block partitioning for the 2nd DCP in VSP. It can be seen that there are some coding losses, but the overall losses for synthesis views are still negligible. And the proposed method achieves some complexity reduction by removing 4x4 DCP completely.

TABLE 2: HTM v6.0 vs. Proposed adaptive partitioning for 2nd DCP in VSP

# Conclusion

In this document, VSP with 4x8 or 8x4 DCP is proposed. In the proposed method, block partition of 2nd DCP in VSP is decided by analyzing depth values at four corner samples of PU. The overall performance losses are negligible, but decoder complexity is slightly decreased. And the proposed method achieves better alignment with the HEVC design. We therefore recommend adopting the proposed method.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “3D-HEVC Test Model 3”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT2-C1005, March 2013, Geneva, Switzerland.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT2-C1100, January 2013, Geneva, Switzerland.