| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0222 |
| --- | --- |

| Title: | CE2.a: Crosscheck on Qualcomm proposal JCT3V-D0185 |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Dong Tian 201 Broadway, 8th floor Cambridge, MA 02139 | Tel: Email: | +1-617-621-7556 tian@merl.com |
| Source: | Mitsubishi Electric Research Labs (MERL) |  |  |

_____________________________

# Abstract

This document reports the outcome of cross-verification of JCT3V-D0185, "CE2.a related: MB-level NBDV for 3D-AVC", performed by Mitsubishi Electric Research Labs (MERL). The coding/synthesis results obtained for this cross-verification match those provided by the proponents. In addition, the decoding time observed by crosscheckers is about 4~5% increase compared to EHP anchor configuration. Crosscheckers confirmed that the software and the text provided by the proponent are consistent.

# Introduction

In contribution JCT3V-D0185 [1], a texture-first coding method is proposed, in which the coding tools for texture depending on depth is implemented by NBDV. Two testing were performed in the proposal. One is with the BVSP turned OFF; and the other is with BVSP turned ON. Upon request, MERL performed the crosschecking on the two experiments.

# Performance of proposed method

The cross checker received the software package, which is compiled as is. The tests for BVSP ON/OFF are switched with the run-time configurations.

The coding and rendering results are matched with the reported results by proponents. Three tables are generated from the experiments. All the coding and synthesis results are exactly matched, the cross checker provided the Excel files in HTM template, in addition to ATM template, as it is easier to report decoding time in HTM template. In this document, the results from HTM template are pasted for quick reference.

- Compare the proposal with BVSP off with ATM HP anchor

The proposal with BVSP off is set as benchmark in the Excel forms. A positive number for bitrate difference indicates coding penalty for ATM HP anchor relative to the proposal. Table 1 is copied from proponents' document. Table 2 is copied from crosscheckers' results.

- Compare the proposal with BVSP on with ATM HP anchor

The proposal with BVSP on is set as benchmark in the Excel forms. A positive number for bitrate difference indicates coding penalty for ATM HP anchor relative to the proposal. Table 3 is copied from proponents' document. Table 4 is copied from crosscheckers' results.

- Compare the proposal with BVSP on with ATM EHP anchor

The ATM EHP anchor is set as benchmark in the Excel forms. A positive number for bitrate difference indicates coding penalty for the proposal relative to ATM EHP anchor. Table 5 is copied from proponents' document. Table 6 is copied from crosscheckers' results.

Table 1. Proponent’s results – ATM HP anchor vs. Proposal with BVSP off

Table 2. Crosschecker’s results – ATM HP anchor vs. Proposal with BVSP off

Table 3. Proponent’s results – ATM HP anchor vs. Proposal with BVSP on

Table 4. Crosschecker’s results – ATM HP anchor vs. Proposal with BVSP ofn

Table 5. Proponent's results – Proposal with BVSP off vs. ATM EHP anchor

Table 6. Crosschecker's results – Proposal with BVSP off vs. ATM EHP anchor

# References

- Y. Chen, J. Kang, L. Zhang, X. Zhao, Y.-K. Wang, R. Joshi, M. Karczewicz, "CE2.a related: MB-level NBDV for 3D-AVC", JCT3V-D0185, Incheon, KR, April 2013

- D. Rusanovskyy, K. Mueller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-C1100, Geneva, CH, January 2013