| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0228 |
| --- | --- |

| Title: | 3D-CE1.h related: Cross check of Improved DV for BVSP in depth coding (JCT3V-D0137) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck on the improved disparity vector for BVSP in depth coding proposed in JCT3V-C0137. The verification task was done successfully. No mismatch is identified. The simulation result reportedly shows the average bitrate reduction of 0.03% and 0.05% with 1.8% decoding runtime decrease in terms of coded texture quality and synthesis quality over total bitrate, respectively. It is asserted that there is potential mismatch between the proposed method and the implementation.

# Introduction

The contribution JCT3V-C0137 proposed the improved disparity vector for BVSP in depth map coding. In this crosscheck report, the simulations are conducted with the same software used in [1].

# Crosscheck Results

The simlations are conducted on the common test condition [2]. The provided source code was compiled on 64-bit Windows 7 environment by using Microsoft Visual Studio 2010 Professional Edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7. Regarding the decoding time, we use the average decoding times of three decoding trials.

No mismatch is identified between encoder and decoder. The coding results matched with those provided by the proponents. The obtained results are reported in Error: Reference source not found. Detailed results are available in the attached EXCEL files.

TABLE 1: Results of crosscheck simulation

Source codes are reviewed. And it is found that, in the implementation, disparity vector derived by DoNBDV is over-written before constructing merge candidate list. Such disparity vector is originally utilized to derive VSP and inter-view motion, inter-view disparity candidates. Therefore, it could be happen that such candidates are changed in the implementation even though such changes are not proposed.

# Conclusions

The results obtained during this crosscheck match those provided by the proponents. Experimental result reportedly shows the average bitrate reduction of 0.03% and 0.05% with 1.8% decoding runtime decrease in terms of coded texture quality and synthesis quality over total bitrate, respectively. Source code was also reviewed, and it is asserted that there is potential mismatch between the proposed method and the implementation.

# References

- J. Heo, T. Kim, H. Liu, and S. Yea, “3D-CE1.h related: Improved DV for BVSP in depth coding,” JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/ WG 11 JCT3V-D0137, April 2013, Incheon, KR.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-C1100, January 2013, Geneva, CH.