| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0235 |
| --- | --- |

| Title: | Reference picture marking process for MV-HEVC and other HEVC extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the current MV-HEVC and SHVC specification, the reference picture marking process has several drawbacks:

- Redundant marking process: The inter-layer reference pictures are marked as “used for long-term reference” and “used for short-term reference” iteratively as many as the number of layers that have the dependency from the layer of the inter-layer reference picture.

- Confliction with multiple layer decoding: When multiple layers, which may be decoded in parallel, access the same inter-layer reference picture at the same time, the reference picture marking processes can be mixed up.

- No explicit marking for inter-layer reference pictures.

In order to simplify and improve the reference picture marking processing for inter-layer prediction, three alternative reference picture marking processes are proposed. The proposed solutions remove the redundant marking process while one of them provides an explicit distinction from other decoded pictures in a decoder picture buffer, which are not used for inter-layer prediction.

This contribution proposes the same topic and contents as JCTVC-M0161.

# Problem

In the reference picture marking process in the current MV-HEVC and SHVC specification text, short-term inter-layer reference pictures are temporarily marked as “used for long-term reference” for the decoding of the current picture and marked back as “used for short-term reference” after the decoding of the current picture, as shown in figure 1.

The benefit of the reference picture marking process in HEVC v1 is its simplicity. By marking initially the decoded picture as “used for short-term reference”, which is the most frequently used marking type, and updating the marking to “unused for reference” or “used for long-term reference” when the slice header of the following picture is decoded, the number of reference picture markings can be minimized.

Figure 1. The current inter-layer reference picture marking process in MV-HEVC and SHVC

However, in the current MV-HEVC and SHVC design, the reference picture marking process is not optimized yet. The shortcomings of the current design are as follows:

- Redundant marking process: The inter-layer reference picture is firstly marked as “used for short-term reference”. It is marked again as “used for long-term reference”, when it is included RefPicSetInterLayer[ i ] for inter-layer prediction. After a coded picture with nuh_layer_id greater than 0, which refers to the inter-layer reference picture, is decoded, the reference picture is marked back as "used for short-term reference". The above marking process is iterated as many as the number of layers that have the dependency from the layer of the inter-layer reference picture. Hence, the redundant marking processes are present.

- Confliction with multiple layer decoding: When multiple layers, which may be decoded in parallel, access the same inter-layer reference picture at the same time, the reference picture marking processes can be mixed up. Hence, while an inter-layer reference picture has been marked as “used for long-term reference” by a non-base layer for an inter-layer prediction, another layer can mark again the inter-layer reference picture as "used for short-term reference". It may confuse the inter-layer prediction.

- No explicit marking for inter-layer reference pictures: The inter-layer reference pictures are initially marked as “used for short-term reference”. Therefore, it is not straightforward to distinguish whether the pictures are used for inter-layer prediction.

# Proposed solutions

In order to simplify and improve the reference picture marking processing for inter-layer prediction, we propose three alternative reference picture marking processes for MV-HEVC and SHVC.

## Proposed solution 1

Figure 2. The proposed inter-layer reference picture marking process 1

To simplify the reference picture marking process for MV-HEVC and SHVC, a simple inter-view reference marking process is proposed. In figure 2, a decoded picture is firstly marked as “used for long-term reference” after the coded picture is decoded. Since it is marked as “used for long-term reference” by default, it is not needed to mark again the decoded picture for inter-layer prediction. After all coded pictures of the current access unit are decoded, the following pictures will be decoded. The decoded picture is marked as “unused for reference” or “used for short-term reference” when the slice header of the first slice segment of the following picture in the same layer is decoded.

In the proposed design, the decoded picture is not temporally marked for inter-layer prediction. Once the picture is marked as “used for long-term reference”, this mark is kept until the decoding of the next coded picture in the same layer starts. In the proposed design, there is no iterative marking process for each dependent layer. The firstly proposed solution is simplest.

## Proposed solution 2

Figure 3. The proposed inter-layer reference picture marking process 2

In the second proposed solution, a decoded picture is initially marked as “used for short-tem reference” after the coded picture is decoded, same as the current reference picture marking process in MV-HEVC and SHVC. The decoded picture is temporally marked as “used for long-term reference”, when it is included in RefPicSetInterLayer for inter-layer prediction. However, the inter-layer picture is not marked back as “used for short-tem reference” immediately after the coded picture, which refers to the inter-layer reference picture, is decoded. After all coded pictures of all layers in the current access unit are decoded, all inter-layer reference pictures temporally marked as “used for long-tem reference” are marked back as “used for short-tem reference” at the same time for the temporal prediction of the following pictures. Differently from the current reference picture marking process, the temporal marking for inter-layer prediction is done once. The marking process is not iterated for all layers with nuh_layer_id greater than 0 referring to the inter-layer reference picture. The second solution simplifies the current reference picture marking process without changing the first marking as “used for short-tem reference” and the last marking as “used for long-tem reference” or “unused for reference”.

## Proposed solution 3

Figure 4. The proposed inter-layer reference picture marking process 3

The third solution minimizes the reference picture marking process by utilizing the inter-layer dependency information. Assuming that the InterlayerPredictionFlag, which indicates whether the current layer is used as reference picture for inter-layer prediction of other layers, is signalled or can be inferred from other syntaxes, the reference picture marking process can be more optimized.

In the third solution, if InterlayerPredictionFlag is equal to 1, a decoded picture is firstly marked as “used for long-term reference” after the coded picture is decoded. Otherwise, it is initially marked as “used for short-term reference”. When an interlayer reference picture is included in RefPicSetInterLayer for inter-layer prediction, there is no need to mark it again as “used for long-term reference”, because all pictures to be used as inter-layer references are already marked as “used for long-term reference”. Finally, same as the current MV-HEVC and SHVC designs, the decoded picture is marked as “unused for reference” or “used for long-term reference” when the slice header of the first slice segment of the following picture in the same layer is decoded.

The interlayer reference pictures can be explicitly differentiated from other non-interlayer reference pictures in DPB. It is because the inter-layer pictures with the same POC are marked as “used for long-term reference”, while the pictures with the same POC, which are not referenced for inter-layer prediction, are marked as “used for short-term reference”.

# Text in specification

## Current text

- 8.1.1 Decoding process for a coded picture with nuh_layer_id equal to 0

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause 8.2.

- The processes in subclause 8.3 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause Error: Reference source not found. This needs to be invoked only for the first slice segment of a picture.

–	The decoding process for RPS in subclause 8.3.2 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice segment of a picture.

–	When the current picture is a BLA picture or is a CRA picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.3 is invoked, which needs to be invoked only for the first slice segment of a picture.

–	PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause 8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0) and, when decoding a B slice, reference picture list 1 (RefPicList1).

- The processes in subclauses 8.4, 8.5, 8.6, and 8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

- After all slices of the current picture have been decoded, the decoded picture is marked as "used for short-term reference".

- G.8.1.1 Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer pictures RefPicSetInterLayer.

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) {
	RefPicSetInterLayer[ i ]  = the picture with picture order count equal to PicOrderCnt and
		nuh_layer_id equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ][ i ] ]
	RefPicSetInterLayer[ i ] is marked as "used for long-term reference"

- G.8.1.2 Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies.
	
for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) 
	RefPicSetInterLayer[ i ] is marked as "used for short-term reference"

## Modified text for the proposed solution 1

- 8.1.1 Decoding process for a coded picture with nuh_layer_id equal to 0

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause 8.2.

- The processes in subclause 8.3 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause Error: Reference source not found. This needs to be invoked only for the first slice segment of a picture.

–	The decoding process for RPS in subclause 8.3.2 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference" "used for short-term reference". This needs to be invoked only for the first slice segment of a picture.

–	When the current picture is a BLA picture or is a CRA picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.3 is invoked, which needs to be invoked only for the first slice segment of a picture.

–	PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause 8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0) and, when decoding a B slice, reference picture list 1 (RefPicList1).

- The processes in subclauses 8.4, 8.5, 8.6, and 8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

- After all slices of the current picture have been decoded, the decoded picture is marked as "used for short-term reference" "used for long-term reference".

- G.8.1.1 Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer pictures RefPicSetInterLayer.

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) {
	RefPicSetInterLayer[ i ]  = the picture with picture order count equal to PicOrderCnt and
		nuh_layer_id equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ][ i ] ]
	RefPicSetInterLayer[ i ] is marked as "used for long-term reference"

- G.8.1.2 Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies.
	
for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) 
	RefPicSetInterLayer[ i ] is marked as "used for short-term reference"

## Modified text for the proposed solution 2

- 8.1.1 Decoding process for a coded picture with nuh_layer_id equal to 0

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause 8.2.

- The processes in subclause 8.3 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause Error: Reference source not found. This needs to be invoked only for the first slice segment of a picture.

–	The decoding process for RPS in subclause 8.3.2 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice segment of a picture.

–	When the current picture is a BLA picture or is a CRA picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.3 is invoked, which needs to be invoked only for the first slice segment of a picture.

–	PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause 8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0) and, when decoding a B slice, reference picture list 1 (RefPicList1).

- The processes in subclauses 8.4, 8.5, 8.6, and 8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

- After all slices of the current picture have been decoded, the decoded picture is marked as "used for short-term reference".

- G.8.1.1 Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer pictures RefPicSetInterLayer.

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) {
	RefPicSetInterLayer[ i ]  = the picture with picture order count equal to PicOrderCnt and
		nuh_layer_id equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ][ i ] ]
	RefPicSetInterLayer[ i ] is marked as "used for long-term reference"

- G.8.1.2 Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies.
	
for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) 
	RefPicSetInterLayer[ i ] is marked as "used for short-term reference"

- G.8.1.2 Marking process for ending the decoding of all coded pictures with nuh_layer_id greater than 0 in the current access unit

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies.

for( i = 0; i <= vps_max_layers_minus1; i++ ) 
	the picture, marked as "used for long-term reference", with picture order count equal to PicOrderCntVal and with nuh_layer_id equal to LayerIdInVps[ nuh_layer_id ] is marked as "used for short-term reference"

## Modified text for the proposed solution 3

- 8.1.1 Decoding process for a coded picture with nuh_layer_id equal to 0

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause 8.2.

- The processes in subclause 8.3 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause Error: Reference source not found. This needs to be invoked only for the first slice segment of a picture.

–	The decoding process for RPS in subclause 8.3.2 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice segment of a picture.

–	When the current picture is a BLA picture or is a CRA picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.3 is invoked, which needs to be invoked only for the first slice segment of a picture.

–	PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause 8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0) and, when decoding a B slice, reference picture list 1 (RefPicList1).

- The processes in subclauses 8.4, 8.5, 8.6, and 8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

- After all slices of the current picture have been decoded, the decoded picture is marked as "used for short-term reference".

- After all slices of the current picture have been decoded, if InterlayerPredictionFlag is equal to 1,  the decoded picture is marked as "used for long-term reference". If InterlayerPredictionFlag is equal to 0, the decoded picture is marked as "used for short-term reference".

- G.8.1.1 Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer pictures RefPicSetInterLayer.

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) {
	RefPicSetInterLayer[ i ]  = the picture with picture order count equal to PicOrderCnt and
		nuh_layer_id equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ][ i ] ]
	RefPicSetInterLayer[ i ] is marked as "used for long-term reference"

- G.8.1.2 Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies.
	
for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) 
	RefPicSetInterLayer[ i ] is marked as "used for short-term reference"

- G.8.1.2 Marking process for ending the decoding of all coded pictures with nuh_layer_id greater than 0 in the current access unit

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies.

for( i = 1; i <= vps_max_layers_minus1; i++ ) 
	the picture with picture order count equal to PicOrderCntVal and 
		with InterlayerPredictionFlag equal to1 is marked as "used for short-term reference"

# References

- J. Chen et. al., “SHVC Test Model 1 (SHM 1),” Document JCTVC-L1007, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

- G. Tech et. al., “MV-HEVC Draft Text 3,” Document JCT3V-C1004, the 3rd JCT3V meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).