| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0237 |
| --- | --- |

| Title: | On video parameter set extension for MV-HEVC and other HEVC extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In order to improve the syntaxes related to mapping nuh_layer_id with splitting_flag in video parameter set. Several changes are proposed:

- Signalling the maximum number of layers for each scalability dimension.

- Mandatory signalling layer_id_in_nuh[ i ], when splitting_flag is equal to 1.

- Replacing dimension_id[ i ][ j ] with split_dimension_id[ i ][ j ], which specifies the j-th value of the identifier of the i-th scalability dimension type, when splitting_flag is equal to 1. Or skipping the signal of dimension_id[ i ][ j ].

- No signalling of the last dimension_id_len_minus1[ i ], when splitting_flag is equal to 1.

The proposed changes are beneficial for managing multi-layered HEVC bitstreams in memory-limited systems, and efficient in terms of header compression.

This contribution proposes the same topic and contents as JCTVC-M0163.

# Introduction

JCTVC-L0200 proposed a flag, splitting_flag, in the VPS extension syntax to indicate a sparse allocation of nuh_layer_id values that allow identification of dedicated scalable identifiers within nuh_layer_id when the mapping approach for nuh_layer_id is used, and was agreed.

In this proposal, the improvement of video parameter set extension design is proposed, focusing on syntaxes and semantics related splitting_flag.

First, signalling max_layers_scalability_dimension_minus1 [ i ], which specifies the maximum number of layers for the scalability dimension which the i-th scalability mask index indicates, is proposed. Informing the maximum number of layers for each scalability dimension is useful for session negotiation and notification of bitstream formation. Moreover, when splitting_flag is equal to 1, dimension_id can be separately signalled for each scalability dimension type as many as the maximum number of layers of the scalability dimension. It may save the bits.

Second, when splitting_flag is equal to 1, layer_id_in_nuh[ i ] shall be signalled to enable a sparse allocation of nuh_layer_id values. If layer_id_in_nuh[ i ] is not signalled, the value of layer_id_in_nuh[ i ] is linearly mapped, not sparse.

Third, split_dimension_id[ i ][ j ] can be signalled instead of signalling dimension_id[ i ][ j ].  split_dimension_id[ i ][ j ] specifies the j-th value of the identifier of the i-th scalability dimension type, when split_dimension_present_flag is equal to 1. When split_dimension_present_flag is equal to 0, split_dimension_id[ i ][ j ] is equal to j. From this modification, the total number of bits for layer identifier mapping can be reduced, and nuh_layer_id can be directly mapped into dedicated scalability identifiers without lookup table matching.

Fourth, if splitting_flag is equal to 1, dimension_id_len_minus1[ NumScalabilityTypes - 1] can be inferred from dimBitOffset[ NumScalabilityTypes - 1 ] without signalling.

The proposed changes are beneficial for managing multi-layered HEVC bitstreams in memory-limited systems, and efficient in terms of syntax compression.

# Proposed specification text

The syntaxes are highlighed, with yellow for proposal 1, green for proposal 2, sky for proposal 3, and red for proposal 4.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( i = 0;i <NumScalabilityTypes; i++ ) { |  |
| if(i < NumScalabilityTypes-1 ¦¦ !splitting_flag) |  |
| dimension_id_len_minus1[ i ] | u(3) |
| max_layers_scalability_dimension_minus1[ i ] | u(v) |
| } |  |
| if(splitting_flag) { |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| layer_id_in_nuh[ i ] | u(6) |
| split_dimension_present_flag | u(1) |
| for( i = 0;i <NumScalabilityTypes; i++ ) |  |
| for( j = 1; j <= max _layers _scalability_dimension_minus1[ i ]; j++ ) |  |
| split_dimension_id[ i ][ j ] | u(v) |
| } else { |  |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| for( lsIdx = 1; lsIdx <= vps_num_layer_sets_minus1; lsIdx ++ ) { |  |
| vps_profile_present_flag[ lsIdx ] | u(1) |
| if( !vps_profile_present_flag[ lsIdx ] ) |  |
| profile_layer_set_ref_minus1[ lsIdx ] | ue(v) |
| profile_tier_level( vps_profile_present_flag[ lsIdx ], vps_max_sub_layers_minus1) |  |
| } |  |
| num_output_layer_sets | ue(v) |
| for( i = 0; i < num_output_layer_sets; i++ ) { |  |
| output_layer_set_idx[ i ] | ue(v) |
| lsIdx = output_layer_set_idx[ i ] |  |
| for( j = 0 ; j <= vps_max_layer_id; j++) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ) |  |
| output_layer_flag[ lsIdx ][ j ] | u(1) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |

max_layers_scalability_dimension_minus1 [ i ] specifies the maximum number of layers for the scalability dimension which the i-th scalability mask index indicates. The number of bits used for the representation of max_layers_scalability_dimension_minus1 [ i ] is dimension_id_len_minus1[ j ] + 1 bits.

split_dimension_present_flag equal to 1 specifies whether split_dimension_id[ i ][ j ] is present. When not present, the value of split_dimension_present_flag is inferred to be equal to 0.

split_dimension_id[ i ][ j ] specifies the j-th identifier of the i-th scalability dimension type, when splitting_flag is equal to 1. When not present, the value of split_dimension_id[ i ][ j ] is inferred to be equal to 0. The number of bits used for the representation of split_dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits.

The variables ScalabilityId[ layerIdInVps ][ scalabilityMaskIndex ] and ViewId[ layerIdInNuh ] are derived as follows:

if splitting_flag is equal to 1 and split_dimension_present_flag is equal to 1

for (i = 0; i <= vps_max_layers_minus1; i++) {

for( smIdx= 0, j =0; smIdx< 16; smIdx ++ )

if( ( i  ! =  0 )  &&  scalability_mask[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = split_dimension_id[ smIdx ][( layer_id_in_nuh[ i ] & ( (1 << ( dimension_id_len_minus1[ j ] + 1) − 1) >> dimBitOffset[ j ] ) ) ]

else

ScalabilityId[ i ][ smIdx ] = 0

ViewId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ]

}

if splitting_flag is equal to 1 and split_dimension_present_flag is equal to 0

for (i = 0; i <= vps_max_layers_minus1; i++) {

for( smIdx= 0, j =0; smIdx< 16; smIdx ++ )

if( ( i  ! =  0 )  &&  scalability_mask[ smIdx ] )

ScalabilityId[ i ][ smIdx ] 		= ( layer_id_in_nuh[ i ] & ( (1 << ( dimension_id_len_minus1[ j ] + 1) − 1) >> dimBitOffset[ j ] ) )

else

ScalabilityId[ i ][ smIdx ] = 0

ViewId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ]

}

else

for (i = 0; i <= vps_max_layers_minus1; i++) {

for( smIdx= 0, j =0; smIdx< 16; smIdx ++ )

if( ( i  ! =  0 )  &&  scalability_mask[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

else

ScalabilityId[ i ][ smIdx ] = 0

ViewId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ]

}

if splitting_flag is equal to 1, dimension_id_len_minus1[NumScalabilityTypes - 1] can be inferred from dimBitOffset[NumScalabilityTypes - 1 ] as follows:

dimension_id_len_minus1[NumScalabilityTypes - 1] = 6 - dimBitOffset[NumScalabilityTypes - 1 ]

# References

- J. Chen et. al., “SHVC Test Model 1 (SHM 1),” Document JCTVC-L1007, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

- G. Tech et. al., “MV-HEVC Draft Text 3,” Document JCT3V-C1004, the 3rd JCT3V meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).