| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0242 |
| --- | --- |

| Title: | Decoded picture buffer for MV-HEVC and other HEVC extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this proposal, to support both the synchronized management of decoded picture buffers (DPBs) in all layers and the independent management of each DPB to minimize the DPB size, two flags are signalled in VPS. A dbp_synchronize_all_layers_flag indicates whether the DPBs of all layers are synchronized with a base layer, while dbp_synchronize_flag indicates whether a DPB of a layer is synchronized with a base layer. When a DPB of a layer is synchronized, signaling the syntaxes related to RPS and sub-layer ordering in SPS or slice header can be skipped.

This contribution proposes the same topic and contents as JCTVC-M0169.

# Introduction

In HEVC multi-layered extension, MV-HEVC or SHVC, the decoded picture buffer (DPB) of each layer can be synchronized with the DPB of the base layer, or independently operated. In the proposed design, both are supported operationally.

The benefit of the DPB synchronization of all layers is easy manipulation of all DPBs. Moreover, the syntaxes related to the reference picture set (RPS) and sub-layer ordering cannot be signaled in sequence parameter set or slice header of enhancement layers.

The benefit of the independent operation of DPB for each layer is the minimization of DPB size. The pictures which are not used for inter- or inter-layer prediction can be immediately marked as “unused for reference”, and removed from the DPB. It enables to maintain the size of DPB minimally.

In this contribution, A flag which indicates whether the DPBs of all layers are synchronized with the base layer is proposed, named dbp_synchronize_all_layers_flag. A flag dbp_synchronize_flag which indicates whether a DPB of a layer is synchronized with the base layer (or the first directly dependent layer) is signalled, when the dbp_synchronize_all_layers_flag is equal to 0. Both flags are signalled in VPS or other parameter set.

When the DPB of a layer is synchronized with the base layer (or the first directly dependent layer), the syntaxes related to RPS and sub-layer ordering cannot be signaled in sequence parameter set or slice header.

# Specification text

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| dbp_synchronize_all_layers_flag | u(1) |
| if( !dbp_synchronize_all_layers_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| dbp_synchronize_flag[ i ] | u(1) |
| } |  |

dbp_synchronize_all_layers_flag equal to 1 specifies that the DPBs of all layers are operated same as the DPB of the base layer. dbp_synchronize_all_layers_flag equal to 0 specifies that the DPB of each layer can be independently operated by signalling the RPS syntax in the slice headers of each layer.

dbp_synchronize_flag[ i ] equal to 1 specifies that the DPB of the ith layer is operated same as the DPB of the base laye. dbp_synchronize_flag[ i ] equal to 0 specifies that the DPB of the ith layer is independently operated by the RPS syntaxes signalled in sequence parameter sets and slice headers.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| … |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| if( nuh_layer_id > 0) |  |
| sps_dpb_sychronized_flag | u(1) |
| if(!sps_dpb_sychronized_flag) { |  |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| … |  |
| if(!sps_dpb_sychronized_flag) { |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

sps_dpb_sychronized_flag specifies that the DPB of the layer associated with the current SPS is operated same as the DPB of the base layer without signalling the syntaxes related to RPS and sub-layer ordering. dbp_synchronize_flag[ i ] equal to 0 specifies that the DPB of the ith layer is independently operated by the RPS syntaxes signalled in sequence parameter set and slice header.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| … |  |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| if(!sps_dpb_sychronized_flag) { |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ i ] | u(v) |
| } else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| … |  |
| byte_alignment( ) |  |
| } |  |

# References

- J. Chen et. al., “SHVC Test Model 1 (SHM 1),” Document JCTVC-L1007, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

- G. Tech et. al., “MV-HEVC Draft Text 3,” Document JCT3V-C1004, the 3rd JCT3V meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).