| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0244 |
| --- | --- |

| Title: | Slice header design for MV-HEVC and other HEVC extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In terms of header compression, it is considerable to reduce the bits of slice headers, because the sum of bits of all slice headers in multi-layered bitstreams, occupy a sizable portion of the total bits, especially in a low bit rate. In this proposal, to enable the inter-layer prediction of slice header syntaxes, allowing a dependent slice segment at the first of slices in enhancement layers is proposed. If the first slice segment is a dependent slice segment in an enhancement layer, the value of each slice segment header syntax element that is not present is inferred to be equal to the value of the first slice segment header syntax element in the first slice header in a base layer in the same access unit.

This contribution proposes the same topic and contents as JCTVC-M0171.

# Introduction

In terms of header compression, it is considerable to reduce the bits of slice headers, because the sum of bits of all slice headers in multi-layered bitstreams, such as SHVC or MV-HEVC, occupy a sizable portion of the total bits, especially in a low bit rate. The best way to reduce the total bits of slice headers in multi-layered streams could be the syntax prediction of slice header across layers.

In this proposal, to enable the inter-layer prediction of slice header syntaxes, a simplest method is proposed. That is allowing a dependent slice segment at the first of slices in enhancement layers. In HEVC v1, the first slice segment in a slice shall be an independent slice segment. In SHVC or MV-HEVC, the first slice segment need not be an independent slice segment. If the first slice segment is a dependent slice segment in an enhancement layer, the value of each slice segment header syntax element that is not present is inferred to be equal to the value of the first slice segment header syntax element in the first slice header in a base layer in the same access unit. It means that all slice segments in enhancmenet layers can be dependent slice segments.

Since each layer may have a layer dependency, the slice syntax can be predicted from a directly referred layer. Then, an explicit signaling is needed to indiciate which layer is the reference for inter-layer slice prediction. In the proposed solution, a slice_header_pred_layer_id indicates a nuh_layer_id of a reference layer for inter-layer slice prediction.

# Specification text

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ¦¦ nuh_layer_id > 0) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| if( !first_slice_segment_in_pic_flag) |  |
| slice_segment_address | u(v) |
| if( first_slice_segment_in_pic_flag && dependent_slice_segment_flag ) |  |
| slice_header_pred_layer_id | u(6) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| } |  |
| } |  |

dependent_slice_segment_flag equal to 1 specifies that the value of each slice segment header syntax element that is not present is inferred to be equal to the value of the corresponding slice segment header syntax element in the slice header. When not present, the value of dependent_slice_segment_flag is inferred to be equal to 0. When nuh_layer_id is greater than 0 and first_slice_segment_in_pic_flag is equal to 1, dependent_slice_segment_flag equal to 1 specifies that the value of each slice segment header syntax element that is not present is inferred to be equal to the value of the first slice segment header syntax element in the first slice header in a reference layer indicated by slice_header_pred_layer_id.

slice_header_pred_layer_id indicates which layer is used for the slice header inter-layer prediction. When nuh_layer_id is greater than 0, first_slice_segment_in_pic_flag is equal to 1, and dependent_slice_segment_flag equal to 1, the value of each slice segment header syntax element that is not present is inferred to be equal to the value of the first slice segment header syntax element in the first slice header in a reference layer with nuh_layer_id equal to slice_header_pred_layer_id. When not present, slice_header_pred_layer_id is inferred to be equal to 0.

# References

- J. Chen et. al., “SHVC Test Model 1 (SHM 1),” Document JCTVC-L1007, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

- G. Tech et. al., “MV-HEVC Draft Text 3,” Document JCT3V-C1004, the 3rd JCT3V meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).