| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0245 |
| --- | --- |

| Title: | MV-HEVC: Crosscheck on Nokia proposal JCT3V-D0209 |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Dong Tian (MERL) 201 Broadway, 8th floor Cambridge, MA 02139 | Tel: Email: | +1-617-621-7556 tian@merl.com |
| Source: | Mitsubishi Electric Research Labs (MERL) |  |  |

_____________________________

# Abstract

This document reports the outcome of cross-verification of JCT3V-D0209, "Inter-view motion field mapping for MV-HEVC", performed by Mitsubishi Electric Research Labs (MERL). The coding results for texture video obtained for this cross-verification match those provided by the proponents, which shows 0.6% bitrate savings for video PSNR vs. total video bitrates. In addition, the decoding time (texture decoding only) is increased by 11%, which was not included in the proponents' report.

# Introduction

In contribution JCT3V-D0209 [1], it is proposed to adjust/shift the motion field of interview reference pictures with a global disparity value, which is estimated at encoder and signaled at slice level. The proposal reported 0.6% gain for video PSNR vs. video bitrate. Upon request, MERL performed the crosschecking.

# Performance of proposed method

The cross checker received the software package. Two macros, REF_IDX_MFM and MFM_COMPRESS_MOTION are used to configure the anchor coding and experimental coding.

For anchor coding, both above macros are set to 0; while for experimental coding, both macros are set to 1.

In proponent contribution, depth is not coded and thus no rendering results are reported. Coding time are not reported either.

The cross checker ran simulations for anchor coding and experimental coding. In addition to code the texture video as proponents' proposal, the cross checker also codes the depth using the same configuration as the anchor texture coding, i.e., the two macros are turned off for depth coding.

Table 1 is copied from the Excel file provided by the proponents. Table 2 is created from simulation conducted by the cross checker. The decoding time was measured on a standalone machine and should be reliable. The full data sets are in the accompanying Excel files.

From the tables, it is confirmed that the reported results from proponents are verified. Some additional statistic numbers are provided for information.

Table 1. Proponent’s results

Table 2. Crosschecker’s results

# References

- D. Bugdayci, K. Ugur, M. Hannuksela, D. Rusanovskyy, "Inter-view motion field mapping for MV-HEVC", JCT3V-D0209, Incheon, KR, April 2013

- D. Rusanovskyy, K. Mueller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-C1100, Geneva, CH, January 2013