| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0249 |
| --- | --- |

| Title: | 3D-CE5.h related: cross-check of AMVP candidate list construction (JCT3V-D0122) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of AMVP simplification schemes which reduces the number of candidates reduced to two, or the same number of HM version 1, from the current three.  Experiment results perfectly match with those provided by the proponent. The software are also studied and confirmed that it follows the description.

- It is reported that two candidates without inter-view candidates shows 0.05 and 0.04 % loss in total video and total synthesis while two candidates with inter-view candidates shows 0.01 % loss and 0.05 % gain in total video.

# Introduction

Summary of test schemes are shown in Table 1

Table 1. Test scheme summary

|  | Number of AMVP candidates | Usage of inter-view candidate | Note |
| --- | --- | --- | --- |
| Test 1 | 2 | No | Same as HM version1 |
| Test 2 | 2 | Yes |  |
| HTM60 | 3 | Yes |  |

# Experimental results

Table 2. Test 1 (Two AMVP candidates without inter-view candidate)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.1% | 0.00% | 0.00% | -0.04% | 98.9% | 98.5% | 98.8% |
| Kendo | 0.0% | 0.1% | 0.4% | 0.09% | 0.06% | 0.10% | 96.3% | 100.5% | 100.1% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | -0.01% | 0.02% | 0.02% | 95.6% | 101.1% | 100.3% |
| GT_Fly | 0.0% | 0.1% | 0.0% | 0.12% | 0.14% | 0.04% | 99.5% | 100.1% | 100.0% |
| Poznan_Hall2 | 0.0% | -0.2% | 0.3% | -0.01% | -0.04% | 0.06% | 98.9% | 98.6% | 99.7% |
| Poznan_Street | 0.0% | 0.2% | 0.0% | 0.05% | 0.04% | 0.03% | 97.7% | 98.4% | 100.2% |
| Undo_Dancer | 0.0% | -0.1% | 0.0% | 0.07% | 0.10% | 0.10% | 98.1% | 100.4% | 100.1% |
| 1024x768 | 0.0% | 0.0% | 0.2% | 0.03% | 0.03% | 0.03% | 96.9% | 100.0% | 99.7% |
| 1920x1088 | 0.0% | 0.0% | 0.1% | 0.06% | 0.06% | 0.06% | 98.5% | 99.4% | 100.0% |
| average | 0.0% | 0.0% | 0.1% | 0.04% | 0.05% | 0.04% | 97.8% | 99.7% | 99.9% |

Table 3. Test 2 (Two AMVP candidates with inter-view candidate)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | -0.03% | 0.00% | -0.08% | 100.6% | 98.8% | 99.2% |
| Kendo | 0.0% | 0.1% | 0.0% | 0.02% | 0.01% | 0.05% | 100.6% | 100.2% | 99.9% |
| Newspaper_CC | 0.0% | -0.1% | 0.1% | 0.01% | 0.02% | 0.01% | 99.1% | 100.6% | 100.9% |
| GT_Fly | 0.0% | 0.2% | 0.1% | 0.03% | 0.03% | 0.01% | 100.2% | 100.5% | 99.7% |
| Poznan_Hall2 | 0.0% | 0.0% | -0.1% | -0.01% | -0.02% | 0.00% | 99.7% | 98.7% | 99.4% |
| Poznan_Street | 0.0% | -0.1% | 0.1% | 0.00% | -0.01% | 0.00% | 99.3% | 97.8% | 99.9% |
| Undo_Dancer | 0.0% | 0.0% | 0.2% | 0.04% | 0.04% | -0.38% | 98.1% | 99.9% | 100.0% |
| 1024x768 | 0.0% | 0.0% | 0.1% | 0.00% | 0.01% | -0.01% | 100.1% | 99.8% | 100.0% |
| 1920x1088 | 0.0% | 0.0% | 0.1% | 0.01% | 0.01% | -0.09% | 99.3% | 99.2% | 99.8% |
| average | 0.0% | 0.0% | 0.1% | 0.01% | 0.01% | -0.06% | 99.7% | 99.5% | 99.9% |

# Conclusion

It is recommended to adopt the use of two candidates but retain inter-view candidate for further study.

# References

[1] J. Y. Lee, M. W. Park, C. Kim, “3D-CE5.h related: AMVP candidate list construction”, JCT3V-D0112, JCT3V 4th Meeting: Incheon, KR, 20–26 Apr. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).