| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0253 |
| --- | --- |

| Title: | 3D-CE4.h related: cross-check of Limited inter-view residual prediction by CBF checking (JCT3V-D0148) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of residual prediction simplification scheme.  Experiment results perfectly match with those provided by the proponent. The software are also studied and confirmed that it follows the description.

- It is reported that the scheme shows negligible bd-rate change (0.02 % gain in total video) and decrease decoding time by 7 %.

# Introduction

Summary of test schemes are shown in Table 1

Table 1. Test scheme summary

|  | Parsing | On / Off decision |
| --- | --- | --- |
| Test | No flag (Same as HTM60) | HTM60 plus cbfAllZero |
| HTM60 | No flag | availableFlagIvMC && PartMode[ xP ][ yP ] == PART_2Nx2N |

# Experimental results

Table 2. Residual prediction on/off control based on cbf

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.4% | -0.4% | -0.13% | -0.09% | -0.14% | 100.1% | 93.4% | 99.3% |
| Kendo | 0.0% | -0.2% | -0.1% | -0.05% | -0.02% | -0.03% | 99.7% | 94.2% | 101.4% |
| Newspaper_CC | 0.0% | -0.4% | -0.3% | -0.11% | -0.06% | -0.10% | 99.2% | 91.2% | 101.0% |
| GT_Fly | 0.0% | 0.1% | -0.1% | 0.01% | 0.03% | -0.01% | 99.9% | 94.5% | 99.8% |
| Poznan_Hall2 | 0.0% | -0.2% | -0.1% | -0.07% | -0.08% | -0.05% | 99.1% | 90.8% | 99.7% |
| Poznan_Street | 0.0% | 0.1% | 0.3% | 0.09% | 0.09% | 0.07% | 99.8% | 89.7% | 100.1% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 0.10% | 99.2% | 94.3% | 100.4% |
| 1024x768 | 0.0% | -0.3% | -0.2% | -0.09% | -0.06% | -0.09% | 99.7% | 92.9% | 100.6% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.01% | 0.01% | 0.03% | 99.5% | 92.3% | 100.0% |
| average | 0.0% | -0.1% | -0.1% | -0.03% | -0.02% | -0.02% | 99.6% | 92.6% | 100.2% |

# References

[1] J. An, K. Zhang, J.-L. Lin, S. Lei, “3D-CE4.h related: cross-check of Limited inter-view residual prediction by CBF checking”, JCT3V-D0148, JCT3V 4th Meeting: Incheon, KR, 20–26 Apr. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).