| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0255 |
| --- | --- |

| Title: | 3D-CE5.h related: cross-check of Removal of parsing dependency for illumination compensation (JCT3V-D0152) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of parsing dependency removal schemes.  One scheme is merge mode fix and another scheme is encoder only one. Experiment results perfectly match with those provided by the proponent. The software are also studied and confirmed that it follows the description.

- It is reported that the merge mode fix scheme shows negligible bd-rate change (0.03 % loss in total video) and encoder only scheme shows 0.15 % loss on average.

# Experimental results

Table 1. Test results of merge fix scheme (anchor: HTM6.0)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.1% | 0.09% | 0.07% | -0.01% | 97.5% | 99.0% | 99.6% |
| Kendo | 0.0% | 0.7% | 0.6% | 0.27% | 0.18% | 0.10% | 96.6% | 101.1% | 100.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.1% | 0.04% | 0.00% | -0.05% | 96.2% | 102.2% | 100.7% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 99.6% | 100.6% | 100.1% |
| Poznan_Hall2 | 0.0% | -0.3% | -0.2% | -0.08% | -0.09% | 0.05% | 96.5% | 98.9% | 99.9% |
| Poznan_Street | 0.0% | 0.4% | -0.3% | 0.02% | 0.02% | 0.02% | 98.0% | 98.6% | 99.9% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | 0.02% | 98.1% | 100.8% | 100.1% |
| 1024x768 | 0.0% | 0.3% | 0.3% | 0.13% | 0.08% | 0.01% | 96.8% | 100.8% | 100.2% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | -0.01% | -0.01% | 0.02% | 98.1% | 99.7% | 100.0% |
| average | 0.0% | 0.1% | 0.1% | 0.05% | 0.03% | 0.02% | 97.5% | 100.2% | 100.1% |

Table 2. Test results of encoder only scheme(anchor: HTM6.0)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.6% | 0.6% | 0.26% | 0.23% | 0.19% | 96.9% | 99.3% | 99.1% |
| Kendo | 0.0% | 1.6% | 2.1% | 0.76% | 0.64% | 0.60% | 96.2% | 99.8% | 99.8% |
| Newspaper_CC | 0.0% | 0.2% | 0.5% | 0.14% | 0.11% | 0.13% | 96.1% | 101.0% | 100.1% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 99.4% | 100.5% | 100.0% |
| Poznan_Hall2 | 0.0% | -0.2% | 0.5% | 0.05% | 0.05% | 0.07% | 96.7% | 98.6% | 100.2% |
| Poznan_Street | 0.0% | -0.1% | 0.0% | -0.01% | -0.01% | -0.02% | 97.5% | 98.5% | 99.9% |
| Undo_Dancer | 0.0% | 0.1% | 0.0% | 0.01% | 0.01% | 0.01% | 96.8% | 100.8% | 100.0% |
| 1024x768 | 0.0% | 0.8% | 1.0% | 0.38% | 0.33% | 0.30% | 96.4% | 100.0% | 99.6% |
| 1920x1088 | 0.0% | -0.1% | 0.2% | 0.01% | 0.01% | 0.02% | 97.6% | 99.6% | 100.0% |
| average | 0.0% | 0.3% | 0.5% | 0.17% | 0.15% | 0.14% | 97.1% | 99.8% | 99.9% |

# References

[1] K. Zhang, Y.-W. Chen, J. An, J.-L. Lin, S. Lei, “3D-CE5.h related: Removal of parsing dependency for illumination compensation”, JCT3V-D0152, JCT3V 4th Meeting: Incheon, KR, 20–26 Apr. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).