| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0256 |
| --- | --- |

| Title: | 3D-CE5.h related: cross-check of Additional inter-view merging candidate (JCT3V-D0109) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of additional merge candidate scheme.  Experiment results perfectly match with those provided by the proponent. The software are also studied and confirmed that it follows the description.

- It is reported that the scheme shows 0.1 % gain in total video.

# Experimental results

Table 2. Test results (anchor: HTM6.0)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | -0.3% | -0.14% | -0.13% | -0.17% | 100.3% | 98.9% | 99.1% |
| Kendo | 0.0% | -0.1% | -0.2% | -0.12% | -0.14% | -0.16% | 99.6% | 100.5% | 100.3% |
| Newspaper_CC | 0.0% | -0.2% | 0.0% | -0.04% | -0.04% | -0.04% | 99.0% | 101.4% | 100.3% |
| GT_Fly | 0.0% | 0.1% | 0.0% | -0.04% | -0.04% | -0.04% | 99.7% | 100.2% | 100.2% |
| Poznan_Hall2 | 0.0% | -0.6% | -0.1% | -0.17% | -0.17% | -0.14% | 99.2% | 100.4% | 99.8% |
| Poznan_Street | 0.0% | 0.0% | 0.1% | 0.00% | -0.01% | -0.01% | 99.9% | 99.3% | 99.7% |
| Undo_Dancer | 0.0% | -0.1% | -0.2% | -0.08% | -0.08% | -0.21% | 98.8% | 100.4% | 100.3% |
| 1024x768 | 0.0% | -0.2% | -0.2% | -0.10% | -0.10% | -0.12% | 99.6% | 100.2% | 99.9% |
| 1920x1088 | 0.0% | -0.1% | 0.0% | -0.07% | -0.07% | -0.10% | 99.4% | 100.1% | 100.0% |
| average | 0.0% | -0.2% | -0.1% | -0.08% | -0.09% | -0.11% | 99.5% | 100.2% | 100.0% |

# References

[1] J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei, “3D-CE5.h related: Additional inter-view merging candidate”, JCT3V-D0109, JCT3V 4th Meeting: Incheon, KR, 20–26 Apr. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).