| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0259 |
| --- | --- |

| Title: | 3D-AVC: Cross-check result on the package of NBDV proposed by Qualcomm (JCT3V-D0186) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Cross check |  |  |
| Author(s) or Contact(s): | Gun Bang, * Young Su Heo, * Kyung Yong Kim, *Gwang Hoon Park, Gwang Soon Lee, Namho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-6791 {gbang, gslee, namho} @etri.re.kr +82-31-201-3680 {* medist, * kimky, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This document presents the cross-check results of JCT3V-D0186 [1] which is improving the disparity vector derivation method on MB-level.

# Experimental Condition

ETRI and KHU received the package of crosscheck data to be presented by JCT3V-D0186 [1] from Qualcomm and the result cross-checked. Then we found no problem in compiling, building, encoding and decoding.

The test configurations are summarized in Table 1. Note that the results of the tests were simulated with Texture First Coding “T0D0T1D1T2D2”.

Table 1. Test configurations

| Test number | VSP at anchor picture | VSP at non-anchor picture | DDV |
| --- | --- | --- | --- |
| 1 | No | No | No |
| 2 | No | No | Yes |
| 3 | yes | yes | Yes |
| 4 | yes | yes | No |

# Results

R-D performance and execution time of each experimental result are summarized in Table 2 to .

Table 2 shows the results of Test 2 vs Test 1.

Table 3 shows the results of Test 2 vs MVC+D.

Table 4 shows the results of Test 3 vs MVC+D.

Table 5 shows the results of Test 4 vs Test 3.

It was confirmed that these results perfectly match with the one provided by the proponent.

Table 2. Result of Test 1 DDV OFF(BVSP OFF) vs Test 2 DDV ON(BVSP OFF)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -5.66 | 0.19 | -0.45 | 0.03 | -5.21 | 0.17 | -4.74 | 0.16 |
| S02 | -0.46 | 0.01 | 0.20 | -0.01 | -0.43 | 0.01 | -0.30 | 0.01 |
| S03 | -0.98 | 0.03 | -0.31 | 0.02 | -0.93 | 0.03 | -0.84 | 0.03 |
| S04 | -0.63 | 0.02 | -0.08 | 0.00 | -0.60 | 0.02 | -0.44 | 0.02 |
| S05 | -2.41 | 0.12 | -0.17 | 0.01 | -2.07 | 0.10 | -1.87 | 0.08 |
| S06 | -2.80 | 0.14 | 0.09 | 0.00 | -2.48 | 0.12 | -2.11 | 0.10 |
| S08 | -2.48 | 0.11 | -0.14 | 0.01 | -2.22 | 0.10 | -2.08 | 0.08 |
| Average | -2.20 | 0.09 | -0.12 | 0.01 | -1.99 | 0.08 | -1.77 | 0.07 |

Table 3. Result of Test 2DDV ON(BVSP OFF) vs MVC+D

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 35.09 | -1.00 | -2.76 | 0.14 | 40.32 | -1.14 | 39.36 | -1.14 |
| S02 | 8.38 | -0.24 | -14.81 | 0.66 | 12.68 | -0.36 | 11.95 | -0.35 |
| S03 | 18.04 | -0.58 | -17.93 | 1.57 | 17.18 | -0.57 | 21.56 | -0.64 |
| S04 | 20.44 | -0.68 | 18.79 | -1.00 | 22.31 | -0.75 | 23.61 | -0.74 |
| S05 | 25.48 | -1.13 | -11.31 | 0.60 | 39.59 | -1.59 | 35.66 | -1.33 |
| S06 | 33.49 | -1.49 | -8.49 | 0.40 | 41.77 | -1.75 | 35.42 | -1.41 |
| S08 | 15.97 | -0.65 | -22.09 | 1.05 | 30.29 | -1.13 | 21.57 | -0.71 |
| Average | 22.41 | -0.82 | -8.37 | 0.49 | 29.16 | -1.04 | 27.02 | -0.90 |

Table 4. Result of Test 3 DDV ON(BVSP ON) vs MVC+D

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 41.14 | -1.11 | -2.54 | 0.13 | 45.98 | -1.23 | 44.27 | -1.22 |
| S02 | 12.90 | -0.36 | -15.19 | 0.68 | 16.95 | -0.46 | 15.42 | -0.44 |
| S03 | 22.12 | -0.69 | -18.21 | 1.59 | 20.87 | -0.67 | 25.13 | -0.72 |
| S04 | 29.98 | -0.96 | 20.39 | -1.07 | 31.54 | -1.01 | 31.91 | -0.96 |
| S05 | 28.28 | -1.21 | -11.47 | 0.61 | 41.59 | -1.63 | 38.82 | -1.40 |
| S06 | 35.86 | -1.57 | -8.62 | 0.41 | 43.85 | -1.81 | 37.76 | -1.47 |
| S08 | 16.56 | -0.66 | -21.79 | 1.04 | 30.80 | -1.14 | 22.24 | -0.73 |
| Average | 26.69 | -0.93 | -8.20 | 0.48 | 33.08 | -1.14 | 30.79 | -0.99 |

Table 5. Result of Test 4 DDV OFF(BVSP ON) vs Test 3 DDV ON(BVSP ON)

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -0.41 | 0.01 | -0.22 | 0.02 | -0.36 | 0.01 | -0.28 | 0.01 |
| S02 | -0.05 | 0.00 | 0.18 | -0.01 | -0.03 | 0.00 | -0.02 | 0.00 |
| S03 | 0.02 | 0.00 | -0.07 | 0.01 | 0.02 | 0.00 | 0.04 | 0.00 |
| S04 | -0.02 | 0.00 | -0.12 | 0.01 | -0.02 | 0.00 | -0.03 | 0.00 |
| S05 | -0.05 | 0.00 | -0.04 | 0.00 | -0.05 | 0.00 | -0.07 | 0.00 |
| S06 | -0.22 | 0.01 | -0.01 | 0.00 | -0.20 | 0.01 | -0.19 | 0.01 |
| S08 | -0.20 | 0.01 | -0.26 | 0.01 | -0.18 | 0.01 | -0.23 | 0.01 |
| Average | -0.13 | 0.01 | -0.08 | 0.01 | -0.12 | 0.00 | -0.11 | 0.00 |

# Conclusion

The results of JCT3V-D0186 were confirmed. Experimental results perfectly match with the one provided by the proponent.

# References

- Xin Zhao, Ying Chen, Li Zhang, Marta Karczewicz, “CE2.a related: Derived disparity vector in 3D-AVC”, The 4th JCT3V meeting, JCT3V-D0186, April. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, The 3rd JCT3V meeting, JCT3V-C1100, Geneva, CH, Jan. 2013.