| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0269 |
| --- | --- |

| Title: | CE6.H: Cross check of efficient DLT coding in SPS (JCT3V-D0151) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Philipp Merkle | Email: | philipp.merkle@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Cross check results for JCT3V-D0151 on efficient DLT coding in SPS by MediaTek are reported. Performance evaluation results for random access and all-intra configuration reported by the proponent were verified.

# Introduction

In JCT3V- D0151 and according to the tool under test described in JCT3V-C1106, MediaTek proposes a range constrained bit map (RCBM) approach to code DLT. This cross-check corresponds to test 3 in section 3.3 of JCT3V-C1106, where the DLT is transmitted in the SPS. As cross-checker, we received their source code and simulation results for random access and all-intra configuration according to JCT3V-C1106. The source code was compiled and the corresponding simulations were conducted within our lab on a clustered system with Linux OS and GCC 4.5.5 compiler. Compilation required fixing some errors, such as shadow declarations and unused variables.

# Results

Encoding and decoding YUV files (md5sum) matched. More details of the cross check results are summarized in Tables 1 and 2.

Table 1. Result for 3-view scenario under CTC

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.02% | -0.02% | 100.4% | 96.4% | 100.0% |
| Kendo | 0.00% | -0.02% | -0.02% | 100.3% | 105.7% | 98.6% |
| Newspapercc | 0.00% | -0.02% | -0.02% | 100.3% | 100.1% | 99.5% |
| GhostTownFly | 0.00% | 0.00% | 0.00% | 100.5% | 109.2% | 100.4% |
| PoznanHall2 | 0.00% | -0.03% | -0.03% | 100.8% | 104.8% | 100.3% |
| PoznanStreet | 0.00% | 0.00% | 0.00% | 100.1% | 112.3% | 104.1% |
| UndoDancer | 0.00% | 0.00% | 0.00% | 100.2% | 100.6% | 102.0% |
| 1024x768 | 0.00% | -0.02% | -0.02% | 100.3% | 100.7% | 99.4% |
| 1920x1088 | 0.00% | -0.01% | -0.01% | 100.4% | 106.7% | 101.7% |
| average | 0.00% | -0.01% | -0.01% | 100.4% | 104.2% | 100.7% |

Table 2. Result for 3-view scenario under all-intra configuration

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 100.2% | 99.4% | 101.5% |
| Kendo | 0.00% | 0.00% | 0.00% | 99.9% | 100.4% | 100.4% |
| Newspapercc | 0.00% | 0.00% | 0.00% | 100.2% | 101.5% | 100.9% |
| GhostTownFly | 0.00% | 0.00% | 0.00% | 99.8% | 102.9% | 98.9% |
| PoznanHall2 | 0.00% | 0.00% | 0.00% | 99.9% | 101.2% | 101.9% |
| PoznanStreet | 0.00% | 0.00% | 0.00% | 100.0% | 101.0% | 101.1% |
| UndoDancer | 0.00% | 0.00% | 0.00% | 100.5% | 100.2% | 100.6% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 100.1% | 100.5% | 100.9% |
| 1920x1088 | 0.00% | 0.00% | 0.00% | 100.0% | 101.3% | 100.6% |
| average | 0.00% | 0.00% | 0.00% | 100.1% | 101.0% | 100.7% |

# Conclusion

MediaTek provided the source code of their proposal for HTM-6.0 together with a short explanation on how to enable and compile the proposed algorithm. All the modifications to the reference software were encapsulated in preprocessor statements for conditional compilation. Overall the implementation of the proposed algorithm is done in a clean and maintainable way and does not interfere with other coding tools, even though a minor cleanup is recommended.

The cross check results were matched to those provided by the proponent.