| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0272r3 M29173r3 |
| --- | --- |

| Title: | AHG8: Draft Text for Signaling Global View and Depth as an Alternative 3D Format in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Yasuyuki Ichihashi, Hisayuki Sasaki, and Kenji Yamamoto Nukui-Kitamachi 4-2-1, Koganei, Tokyo 184-8795, Japan Masayuki Tanimoto, Kazuyoshi Suzuki Yotsuya-dori 1-13, Chikusa-ku, Nagoya 464-0819, Japan | Tel: Email: | +81-42-327-7262 {senoh, y-ichihashi, sasaki, k.yamamoto}@nict.go.jp, +81-52-781-1883 tanimototentative3@yahoo.co.jp kazuyoshi-suzuki@cb3.so-net.ne.jp |
| Source: | National Institute of Information and Communications Technology(NICT), Nagoya Industry Science Research Institute(NISRI) |  |  |

_____________________________

# Abstract

In this document, syntax and semantics for an alternative depth format SEI message are proposed. In particular, syntax elements are presented which allow deriving Global View and Depth from decoded multi-view and depth samples. This proposal was provided in addition to the scope of mandate 4 of AhG8 to discuss and prepare draft text for a SEI message for 3D-HEVC which signals warping maps. It is proposed to adopt the SEI message presented in this document and JCT3V-D0103 in the current specification of 3D-HEVC Test Model 3.

# Introduction

Responding to the 3DV CFP (N12036, March 2011) on efficient coding methods based on AVC, MVC and HEVC, concepts of Global View and Depth (GVD) format and Warp format were proposed. Through several core experiments (JCT2-A0071/M26013) and subjective quality evaluations (JCT2-A0172/ M26320), they were recognized to be a simple, fast and subjectively comparable coding method and merged to the Alternative 3D Format. Additionally to the AhG8 mandate to discuss and prepare draft text for a SEI message for 3D-HEVC, which signals warping maps as an alternative 3D format, this contribution proposes an extension of the draft text for a SEI message for 3D-HEVC (JCT3V-D0103).

# Proposed syntax and semantics

An SEI message is proposed which signals that decoded view and depth samples have to be interpreted as an alternative 3D format. A syntax element of this SEI message called depth_type is used to discriminate different types of alternative 3D format. A depth type of 0 identifies warp maps as proposed in JCT3V-D0103. A depth type of 1 is extensionally proposed to identify Global View and Depth format. The extension of SEI message syntax and semantics are proposed below with respect to the current specification of 3D-HEVC Test Model 3:

H.14	3D-HEVC SEI messages

The specifications in Annex D together with the extensions and modifications specified in this subclause apply.

H.14.1	SEI message syntax

H.14.1.1	Alternative depth information SEI message syntax

| alternative_depth_info ( payloadSize ) { | Descriptor |
| --- | --- |
| depth_type | u(2) |
| if( depth_type == 0 ) { |  |
| min_x_offset_int | se(v) |
| min_x_offset_frac | u(8) |
| max_x_offset_int | se(v) |
| max_x_offset_frac | u(8) |
| y_offset_present_flag | u(1) |
| if (y_offset_present_flag){ |  |
| min_y_offset_int | se(v) |
| min_y_offset_frac | u(8) |
| max_y_offset_int | se(v) |
| max_y_offset_frac | u(8) |
| } |  |
| warp_map_size_present_flag | u(1) |
| if( warp_map_size_present_flag ) { |  |
| warp_map_width | ue(v) |
| warp_map_height | ue(v) |
| } |  |
| } |  |
| if( depth_type == 1 ) { |  |
| num_residual_texture_views_minus1 | ue(v) |
| residual_depth_flag | u (1) |
| } |  |
| } |  |

H.14.2	SEI message semantics

H.14.2.1	Alternative depth information SEI message semantics

The alternative depth information SEI message indicates that decoded depth samples have to be interpreted as an alternative depth format. To discriminate different alternative depth formats, a depth_type syntax element is used, where a depth_type of 0 indicates that decoded depth samples have to be used to derive a warp map and view synthesis has to be performed by image-domain warping. Additional syntax elements contained in this SEI message are used to derive corresponding warp maps from decoded depth samples of each view.

depth_type	identifies an alternative depth type according to the following table:

| depth_type | interpretation |
| --- | --- |
| 0 | Warp map |
| 1 | Global View and Depth |
| 2-3 | Reserved |

min_x_offset_int, min_x_offset_frac specify an integer and a fractional part of a min offset for the horizontal direction of a warp map. A min_x_offset value is derived as follows:

min_x_offset = min_x_offset_int + min_x_offset_frac / 256

max_x_offset_int, max_x_offset_frac specify an integer and a fractional part of a max offset for the horizontal direction of a warp map. A max_x_offset value is derived as follows:

max_x_offset = max_x_offset_int + max_x_offset_frac / 256

y_offset_present_flag equal to 1 specifies that min and max offset values for the vertical direction are present.

min_y_offset_int, min_y_offset_frac specify an integer and a fractional part of a min offset for the vertical direction of a warp map. A min_y_offset value is derived as follows:

if ( y_offset_present_flag )

min_y_offset = min_x_offset_int + min_x_offset_frac / 256

else

min_y_offset = 0

max_y_offset_int, max_y_offset_frac specify an integer and a fractional part of a max offset for the vertical direction of a warp map. A max_y_offset value is derived as follows:

if ( y_offset_present_flag )

max_y_offset = max_x_offset_int + max_x_offset_frac / 256

else

max_y_offset = 0

warp_map_size_present_flag equal to 1 specifies that a new warp map size is present, which is valid for the current and all following warp maps in decoding order until a new warp map size is again specified. warp_map_size_present_flag  equal to 0 specifies that the warp map size is not changed.

warp_map_width, warp_map_height specify width and height of a warp map, respectively. Valid values are constraint as follows:

0 <= warp_map_width <= pic_width_in_luma_samples

if ( ! y_offset_present_flag )

0 <= warp_map_height <= pic_height_in_luma_samples

else

0 <= warp_map_height <= pic_height_in_luma_samples / 2

View Synthesis by Image-domain Warping

Decoded depth samples of each view are used to derive a corresponding warp map for that view. A warp map is derived as follows from decoded depth samples d[x][y] and syntax elements provided by this SEI message:

deltax =  pic_width_in_luma_samples / (warp_map_width - 1)

deltay =  pic_height_in_luma_samples / (warp_map_height - 1)

for (x=0;x<warp_map_width;++x)

for (y=0;y<warp_map_height;++y){

w[x][y][0] = x * deltax + min_x_offset + (max_x_offset-min_x_offset) / 255 * d[x][y]

if (y_offset_present_flag) {

w[x][y][1] =  y * deltay + min_y_offset

+ (max_y_offset-min_y_offset) / 255 * d[x][y + pic_height_in_luma_samples/2]

} else

w[x][y][1] = y * deltay

}

}

Each warp map derived by this process is assigned to its corresponding input view and has the property to map the corresponding input view to a central camera position as illustrated in Figure 1. Note that only the warp map of the most left input view maps the view to a central camera position on the right, while all other warps maps map their corresponding input views to a central camera position on the left.

Figure 1 – Camera positions and associated warps.

To synthesize a view at an arbitrary position Q using an input view at position P (in Figure 1 position P is either A, B, C, or D), a new warp map wP,Q, which maps an input view at position P to a view at position Q, is derived from a warp map wP,M, which maps the same input view to an central camera position M according to  Figure 1. A warp map wP,Q is derived as follows:

deltax =  pic_width_in_luma_samples / (warp_map_width - 1)

deltay =  pic_height_in_luma_samples / (warp_map_height - 1)

for (x=0;x<warp_map_width;++x)

for (y=0;y<warp_map_height;++y){

w_pq [x][y][0] =  (M-P)/(Q-P) * (w_pm[x][y][0] - x*deltax) + x * deltax

w_pq [x][y][1] =  (M-P)/(Q-P) * (w_pm[x][y][1] - y*deltay) + y * deltay

}

}

The warp map w_pq, which is derived by this process, is used to synthesize the texture for a view at position Q from texture of a view at position P. Warp map samples w_pq[x][y][0] and w_pq[x][y][1] are interpreted as a 2-dimensional sub-pel position in the synthesized view, to which the color samples of the input view at the sub-pel position (x*deltax, y*deltax) are mapped. A warp map also represents a mapping of color samples of quads of the input view, which are defined by the four positions (x*deltax, y*deltay), ((x+1)*deltax, y*deltay), ((x+1)*deltax, (y+1)*deltay), and (x*deltax, (y+1)*deltay), to corresponding quads in the synthesized view, which are defined by the corresponding positions (w_pq[x][y][0], w_pq[x][y][1]), (w_pq[x+1][y][0], w_pq[x+1][y][1]), (w_pq[x+1][y+1][0], w_pq[x+1][y+1][1]), und (w_pq[x][y+1][0], w_pq[x][y+1][1]). This quad-based color mapping process defined here describes the synthesis of an arbitrary view at position Q from an input view at position P and a warp map w_pq, where the warp map w_pq is derived from a warp map w_pm that was derived from decoded depth samples and syntax elements provided by this SEI message.

Note that the position of a view to be synthesized can be located between input views or not. In case a view to be synthesized is located between input views, it is recommended to synthesize two different versions, one based on the closest input view on the left and one based on the closest input view on the right of the view to be synthesized. A final synthesized view can be computed by combining both results, e.g. by pixel-wise blending. If the view to be synthesized in not located between two input views, it is recommended to use the closest input view to synthesize the view.

When depth_type is equal to 1, this SEI message signals Global View and Depth (GVD) type of 3D format. In this case, the 3D format data contains two texture views and one or two depth views. The coded video sequence contains information derived from up to five original texture views and five original depth views which are coded into two texture views and up to two depth views.

Note: When depth_type is equal to 1, tools for inter-view prediction and depth dependent texture coding should be disabled.

The texture and depth view components of the non-base view contain up to four residual views, each represents a difference picture that is horizontally and vertically subsampled relative to the corresponding texture or depth base view components.

depth_type equal to 0 indicates that N = num_residual_texture_views_minus1 + 2 input texture views and associated depth views (N=2, 3, 4 and up to 5) are converted into Global texture view (one base texture view and one residual texture view) and Global depth view (one base depth view or one base depth view plus one residual depth view).

Base texture view (B) is the input from central camera (camera ID=3) as shown in Figure 2. In the case N=2, camera ID=2 and 3 are used. In the case N=3, camera ID=2, 3 and 4 are used. In the case N=4, camera ID=1, 2, 3 and 4 are used. In the case N=5, camera ID=1, 2, 3, 4 and 5 are used. If the number of input texture views is even, base texture view is the first right view from the center (V3). The other N-1 input texture views are converted to N-1 half-size sub-residual texture views (R-x) and packed in the residual texture view from top-left (R-2), to bottom-left (R-4), top-right (R-1) and bottom-right (R-5) order. Their top-left co-ordinates in the residual view of width=W and height=H is shown in Table 1. The residual texture view represents occluded area or out-of-frame area of the base texture view when it is projected to the N-1 input texture views. The number of sub-residual texture views is signalled by num_residual_texture_views_minus1.

num_residual_texture_views_minus1 plus1 specifies the number of sub-residual texture views packed in the residual texture view. num_residual_texture_views_minus1 is in the range of 0 to 3, inclusive.

1     2     3     4     5

R-1   R-2    B    R-4   R-5

camera ID

GVD view

Figure 2. Relation between camera ID and GVD texture views

Table 1. Top-left corner co-ordinates of GVD residual views packed in a Residual view of width=W and height=H

| R-1 | (W/2, 0) |
| --- | --- |
| R-2 | (0, 0) |
| R-4 | (0, H/2) |
| R-5 | (W/2, H/2) |

Base depth view (B) is a median of all input depth views and is associated with the base texture view (camera ID=3) as shown in Figure2. Residual depth view may exist or may not exist. residual_depth_flag =1signals the existence of residual depth view. The other N-1 input depth views are converted to N-1 half-size sub-residual depth views (R-x) and packed in the residual depth view in the same positions as the residual texture view shown in Table 1. Each sub-residual depth view associates with its sub-residual texture view.

When residual_depth_flag = 0 and the number of input views is even (2 or 4), base depth view (B) is the center of input views (camera ID=2.5).

Since the input depth views are originally sub-sample to the half-size of the input texture views, the sub-residual depth views are sub-sampled to the quarter-size of input texture views. The residual depth view represents occluded area or out-of-frame area of the base depth view when it is projected to the N-1 input depth views.

residual_depth_flag equal to 1 specifies that the number of packed depth views in a depth view component of non-base view is equal to the number of packed texture views in a texture view component. residual_depth_flag equal to 0 specifies there is only one depth view.

1     2     3     4     5

(R-1)  (R-2)  B   (R-4)  (R-5)

camera ID

GVD depth

1       2       3       4

2.5

B

camera ID

(even)

GVD depth

wo/res.

- General case      (b) case for even number of input views w/o residual depth

Figure 2. Relation between camera ID and GVD depth view

- Base depth view is generated by projecting N-input depth views to the base depth view (camera ID=3) and taking the median of them.

- Residual depth view is generated by encoding and decoding the base depth view and then projecting it to the other input depth views (camera ID=1,2, 4 and 5), and then extracting the un-covered area by the projected base depth view to sub-residual depth views and then packing them in a residual depth view.

- Residual texture view is generated by projecting the base texture view to the other input texture views (camera ID=1, 2, 4 and 5) by the decoded base depth view, and then extracting the un-covered area in the other input texture views (camera ID=1, 2, 4 and5) by the projected base texture view into sub-residual texture views and then packing them in a residual texture view in the case of residual_depth_flag = 0.

- In the case of residual_depth_flag = 1, residual texture view is generated by encoding and decoding the residual depth view, and then projecting the base texture view to the other input texture views (camera ID=1, 2, 4 and 5) by the decoded base depth view and the decoded residual depth views, and then extracting the un-covered area in the other input texture views (camera ID=1, 2, 4 and5) by the projected base texture view into sub-residual texture views and then packing them in a residual texture view.

# Conculsion

This document proposes an extension of SEI message for signaling warp maps and Global View and Depth as alternative depth formats. It is proposed to adopt this SEI message in the current specification of 3D-HEVC Test Model 3 (JCT3V-C1005).

# References

- JCT2-A0071/M26013, “3DV CE7 summary on Global Depth and View Prediction”, T. Senoh, JCT2, Jul 2012, Stockholm, SE

- JCT2-A0172/M26320, “3DV CE7h Subjective Test Results on Global Depth and View Prediction”, T. Senoh, M. Tanimoto, JCT2, Jul 2012, Stockholm, SE

- JCT3V- C0043/M27777, “AHG8: Report on Relation of GVD Format with Current 3D Video Standardization Tracks”, T. Senoh, M. Tanimoto, N. Stefanoski, JCT-3V, Jan 2013, Geneva, Switzerland

- JCT3V-D0070/M28873, “AHG8 Revision on Depth View Information SEI message in 3D-AVC DT5”, T. Senoh, M. Tanimoto, JCT-3V, April 2013, Incheon, Korea

- JCT3V-D0071/M28874, “AHG8 Description of Global View and Depth Format for Depth View SEI Message”, T. Senoh, M. Tanimoto, JCT-3V, April 2013, Incheon, Korea

- JCT3V-D0103, “AHG8 Draft Text for Signaling Warp Maps as an Alternative 3D Format in 3D-HEVC”, JCT-3V, April 2013, Incheon, Korea

# Patent rights declaration(s)

National Institute of Information and Communications Technology (NICT), and Nagoya Industry Science Research Institute(NISRI) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).